3GPP TSG-RAN WG2 Meeting #132												R2-2508219

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.2.2

Source: 	Sharp

Title:  	Discussion for TN/NTN integration

Document for: 	Discussion

Introduction

In RAN2#131bis meeting [1], RAN2 discussed the needs for maximizing commonality between TN and NTN in 6G radio, and has made the conclusion for TN/NTN integration as follows:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

This document provides a discussion on common and essential aspects for TN/NTN integration.

Discussion

First, essential functions for NTN will be examined. Subsequently, it will be considered whether each function should be harmonized with TN or addressed through NTN-specific solutions. The following functions are identified as key aspects to be studied for NTN in 6G:

- Long sync signal periodicity

- GNSS-resilient access

- Ephemeris-free access

- HARQ process scaling / feedback disable

- Timer offset handling

- TA maintenance & pre-compensation

- Mobility (support for RACH-less, CHO, (C-)LTM)

- TN–NTN mobility

- Multi-orbit & payload support

- Control signaling efficiency

Some of these functions were already studied in 5G and are generally regarded as essential for supporting NTN. In addition, functions not considered in 5G—such as GNSS-resilient access and ephemeris-free access—are considered important technologies, for example for enabling TN–NTN mobility.

Long sync signal periodicity

Short sync signal periodicities (typically 5–20 ms) are used to reduce access delay and increase synchronization reliability. However, in NTN, large propagation delays, large cell footprints, and power constraints of satellite and HAPS call for longer sync signal periodicity values. Allowing long sync signal periodicity in 6GR allows the system to reduce control overhead and conserve transmission power. GEO or MEO systems experience slower channel variations compared to LEO systems, meaning synchronization signals need not be transmitted frequently once the UE is aligned in time and frequency. In addition, longer periodicity reduces unnecessary beaconing in sparsely populated and intermittent coverage areas.

From spec viewpoint, support for long sync signal periodicity can be considered common between TN and NTN. But the use of long periodicity in practice can be limited to NTN, as long periodicities in TN may lead to excessive access delays and degraded synchronization performance.

GNSS-resilient access

Relying fully on GNSS can be risky as the availability and integrity of GNSS signals can be degraded or denied due to obstruction and environmental factors or even jamming and spoofing. For NTN, the network may need to support alternative synchronization and positioning methods that remain operational without continuous GNSS assistance. A GNSS-resilient access design enables the UE to perform initial access, timing acquisition, and mobility procedures when GNSS is unavailable or unreliable. Techniques include deriving timing from sync signals or system information, maintaining time/frequency alignment through predictive modeling of satellite motion, and leveraging inter-node synchronization or network-based positioning.

From the spec viewpoint, GNSS-resilient access can be generally considered common between TN and NTN. Resilience to GNSS outage can be critical if the service continuity and robust access is to be ensured, and it should be considered essential when full GNSS dependency is not acceptable. GNSS-resilient access is important and potentially essential for NTN robustness, but that may not necessarily mean a mandatory capability for all NTN implementations.

Ephemeris-free access

According to early NTN releases in 5G NR, the UE needs knowledge of the satellite’s orbital parameters (ephemeris) to perform timing, frequency, and Doppler pre-compensation for initial access, mobility, etc. Ephemeris-assisted procedures can improve access accuracy and reduce acquisition time. However, obtaining and maintaining up-to-date ephemeris information requires signaling overhead and additional coordination with external systems or network assistance, which may not always be feasible for all UE types or service scenarios. Ephemeris-free access refers to the ability of a UE to perform initial access and establish synchronization without prior knowledge of satellite position or trajectory. This can be achieved through extended search ranges for timing and frequency uncertainty, or through adaptive acquisition algorithms that rely solely on the received sync signals. It should be noted that while this approach increases access robustness and autonomy, it may result in longer access delays and higher UE processing complexity.

From the spec viewpoint, ephemeris-free access can be considered common between TN and NTN. The feature primarily impacts parameter ranges (search windows, frequency offsets, etc.) rather than the fundamental design of NR access mechanisms. However, although related to GNSS-resilient access, we may consider ephemeris-free access not essential, since most NTN deployments will benefit from ephemeris-assisted access.

HARQ process scaling / feedback disable

The number of parallel HARQ processes and the use of feedback control (ACK/NACK) in TN are designed to match typical propagation delays / RTTs. However, in NTN (LEO/MEO/GEO), standard HARQ configurations can become inefficient or infeasible. RTTs may exceed typical HARQ timings, which then results in idle resources or delayed retransmissions. To address this, scaling the number of HARQ processes and allowing feedback disablement can accommodate long RTTs while maintaining throughput efficiency and reliability. Consequently, the system can continue transmitting new data while waiting for ACK/NACK from prior transmissions. Similarly, feedback disablement allows forward-error correction to operate without waiting for explicit ACK/NACK, which is useful when feedback delay is large.

From the spec viewpoint, HARQ process scaling and feedback disablement should be a common 6GR feature applicable to both TN and NTN, as it is a mechanism to manage reliability versus latency tradeoffs. In practice, standard TN deployments may rarely need extreme scaling or feedback disablement, but the capability can still support long-delay terrestrial links (e.g., rural, high-latency backhaul) or future high-speed transport links. Nevertheless, the feature is considered essential for NTN.

Timer offset handling

Various timers are used to govern procedures such as random access, retransmissions, mobility, and inactivity. These timers are typically defined with the assumption of terrestrial propagation delays, i.e., in the order of µs up to a few ms. In NTN, propagation delays can be significantly longer, e.g., tens or hundreds of ms for LEO/MEO/GEO, which can cause the timers to expire prematurely or lead to unnecessary retransmissions, failed access attempts, or incorrect mobility decisions.

Timer offset handling introduces adjustments to the nominal timer values to account for large propagation delays. For example, random access response windows, HARQ timers, and RRC inactivity timers can be offset by the expected satellite link delay to avoid spurious timeouts. This adjustment is therefore specific to NTN. It is also essential for NTN, as otherwise protocols may not behave correctly/properly.

TA maintenance & pre compensation

Propagation delays in NTN (LEO/MEO/GEO) are much larger and can vary significantly depending on satellite orbit and movement as well as UE location. TA pre-compensation is applied at the UE based on predicted propagation delay and satellite trajectory, and TA maintenance allows ongoing adjustment as the satellite moves relative to the UE. From a spec viewpoint, TA maintenance and pre-compensation are NTN-specific. These mechanisms are essential for NTN.

Mobility (support for RACH-less, CHO, (C-)LTM)

Quick overview:

- RACH-less mobility allows UEs to complete handovers without performing a full random-access procedure, which can reduce access delay and signaling overhead.

- CHO allows the network to preconfigure handover conditions such that the UE can trigger a handover automatically once conditions are met.

- (C-)LTM allows mobility decisions to be initiated at the lower layer to reduce dependency on higher layer signaling and enable faster handover.

Such mobility enhancements are common to both TN and NTN. Although the underlying physical and propagation conditions differ between the two, the framework for RACH-less access, CHO, and (C-)LTM can be specified once and applied to both network types, possibly with different parameter values. These mechanisms can be considered at least as essential for NTN as they are for TN.

TN–NTN mobility

Seamless handover when a UE moves across TN and NTN coverage areas is considered essential for 6GR. TN-NTN mobility mechanisms should manage differences in propagation delay, coverage footprint, Doppler effects, and timing between the two network types, with the aim of maintaining continuous service and QoE regardless of the UE’s position relative to TN and NTN infrastructure.

From the spec viewpoint, TN-NTN mobility mechanisms can be integrated with the TN-only mechanisms to obtain a unified mobility framework to handle handovers, measurements, and state transitions across both network types. Core procedures such as measurement reporting, handover triggers, and conditional mobility mechanisms can be applied in a unified way while leveraging RRC enhancements to reduce signaling overhead/latency across TN and NTN. This approach will allow lean spec as well as interoperability and implementation efficiency.

Multi orbit & payload support

6GR should support NTN deployed in multiple satellite orbits: LEO, MEO, GEO, each with different coverage areas, propagation delays, and Doppler characteristics. Supporting multiple payload types (transparent, regenerative) may lead to additional complexity for managing user access, mobility, and resource allocation. Multi-orbit and payload support enables a single UE or network node to communicate seamlessly across different orbital layers and payload configurations, which enhances resilience and service continuity.

Multi-orbit and payload support is NTN-specific and essential, to some extent, as its importance depends on the future deployment scenarios as well as service requirements. For example, in a multi-orbit NTN constellation designed for global coverage, support for UE connectivity across orbits is crucial. However, in single-orbit or regionally focused NTN deployments, the feature may remain optional. Similarly, payload flexibility enhances operational efficiency but may not be equally critical across all service classes.

Control signaling efficiency

Efficiency of control-plane signaling is key, especially in scenarios of integrated TN-NTN where signal quality is unpredictable and access from each of the TN and NTN is intermittent. In such scenarios, conventional control mechanisms may fall short and impose an unnecessary signaling overhead for link management and recovery. It is possible to enhance the control-plane signaling with lean spec with the aim of reducing control overhead and improving responsiveness and power efficiency under varying link conditions. The enhancements are considered essential for seamless integration of TN and NTN.

Observation: Aspects to consider for NTN in 6GR are as follows:

| Aspect | Common with TN | NTN-specific | Essentiality |
| --- | --- | --- | --- |
| Long sync signal periodicity |  | – |  |
| GNSS-resilient access |  | – | – |
| Ephemeris-free access |  | – | – |
| HARQ process scaling / feedback disable |  | – |  |
| Timer offset handling | – |  |  |
| TA maintenance & pre-compensation | – |  |  |
| Mobility (support for RACH-less, CHO, (C-)LTM) |  | – |  |
| TN–NTN mobility |  | – |  |
| Multi-orbit & payload support | – |  |  |
| Control signaling efficiency |  | – |  |

Proposal 1: Consider the following aspects for NTN in 6GR: (1) long sync signal periodicity, (2) GNSS-resilient access, (3) ephemeris-free access, (4) HARQ process scaling / feedback disable, (5) timer offset handling, (6) TA maintenance and pre-compensation, (7) mobility (support for RACH-less, CHO, (C-)LTM), (8) TN-NTN mobility, (9) multi-orbit and payload support, and (10) control signalling efficiency.

Proposal 2: From spec viewpoint, consider aspects (1) long sync signal periodicity, (2) GNSS-resilient access, (3) ephemeris-free access, (4) HARQ process scaling / feedback disable, (7) mobility (support for RACH-less, CHO, (C-)LTM), (8) TN-NTN mobility, and (10) control signalling efficiency as common between TN and NTN, while aspects (5) timer offset handling, (6) TA maintenance and pre-compensation, and (9) multi-orbit and payload support can be considered NTN-specific.

Proposal 3: Consider aspects (1) long sync signal periodicity, (4) HARQ process scaling / feedback disable, (5) timer offset handling, (6) TA maintenance and pre-compensation, (7) mobility (support for RACH-less, CHO, (C-)LTM), (8) TN-NTN mobility, (9) multi-orbit and payload support, and (10) control signalling efficiency as essential, but the system should be able to operate without aspects (2) GNSS-resilient access and (3) ephemeris-free access.

Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observation and agree the following proposals:

Observation: Aspects to consider for NTN in 6GR are as follows:

| Aspect | Common with TN | NTN-specific | Essentiality |
| --- | --- | --- | --- |
| Long sync signal periodicity |  | – |  |
| GNSS-resilient access |  | – | – |
| Ephemeris-free access |  | – | – |
| HARQ process scaling / feedback disable |  | – |  |
| Timer offset handling | – |  |  |
| TA maintenance & pre-compensation | – |  |  |
| Mobility (support for RACH-less, CHO, (C-)LTM) |  | – |  |
| TN–NTN mobility |  | – |  |
| Multi-orbit & payload support | – |  |  |
| Control signaling efficiency |  | – |  |

Proposal 1: Consider the following aspects for NTN in 6GR: (1) long sync signal periodicity, (2) GNSS-resilient access, (3) ephemeris-free access, (4) HARQ process scaling / feedback disable, (5) timer offset handling, (6) TA maintenance and pre-compensation, (7) mobility (support for RACH-less, CHO, (C-)LTM), (8) TN-NTN mobility, (9) multi-orbit and payload support, and (10) control signalling efficiency.

Proposal 2: From spec viewpoint, consider aspects (1) long sync signal periodicity, (2) GNSS-resilient access, (3) ephemeris-free access, (4) HARQ process scaling / feedback disable, (7) mobility (support for RACH-less, CHO, (C-)LTM), (8) TN-NTN mobility, and (10) control signalling efficiency as common between TN and NTN, while aspects (5) timer offset handling, (6) TA maintenance and pre-compensation, and (9) multi-orbit and payload support can be considered NTN-specific.

Proposal 3: Consider aspects (1) long sync signal periodicity, (4) HARQ process scaling / feedback disable, (5) timer offset handling, (6) TA maintenance and pre-compensation, (7) mobility (support for RACH-less, CHO, (C-)LTM), (8) TN-NTN mobility, (9) multi-orbit and payload support, and (10) control signalling efficiency as essential, but the system should be able to operate without aspects (2) GNSS-resilient access and (3) ephemeris-free access.

References

[1] RAN2#131-bis Chair Notes, Prague, Czech Republic, October 2025.