3GPP TSG-RAN WG2#132	R2-2508265

Dallas, USA, Nov. 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	8.7.2

Title:	Discussions on XR remaining issues [RLC-N02], [RLC-H02] and rate query

Document for:	Discussion and decision

Introduction

During the RAN2#131bis meeting [1], there are several RLC open issues left for discussion in this meeting, namely [RLC-N02] and [RLC-H02]. The agreements on these issues are as follows:

Agreements for RLC - (RLC-N02) In NOTE in 5.3.3.3 we add “If stopReTxDiscardedSDU is configured” at the beginning of the sentence. - (RLC-N02) FFS whether any other changes are needed, e.g. as in R2-2507159 - (RLC-N02) Companies will bring contribution in next meeting to explain the use cases and the TP. - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

In this contribution, we would like to provide our view on the RLC open issues and provide our TPs to TS38.322 [2]. We also discuss a MAC remaining issue on bit rate query and provide a TP to TS38.321 [3].

Discussion

[RLC-N02]

In [4], the proponent or [RLC-N02] proposes the following changes on the NOTE in clause 5.3.3.3. The intention is that the transmitting side is waiting for ACK/NACK after retransmission or before discard, and hence, the transmitter should be able to keep polling.

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

During the RAN2 meeting, the first change on adding stopRetxDiscardedSDU configuration has been agreed. The issue that needs to be discussed is whether “or negatively” should be removed or not.

In TS38.322 v19.0.0, The behavior of receiving a STATUS PDU from a peer AM RLC entity is defined as follows:

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; and - if stopReTxDiscardedSDU is configured and no discard indication for the RLC SDU has been received from upper layer, or if stopReTxDiscardedSDU is not configured: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

From above specification, it is clear that a NACKed RLC SDU or the RLC SDU segment is pending for retransmission. In this case, there is no need to poll if all RLC SDUs with SNs up to and including POLL_SN are already ACKed or NACKed. One may argue that the retransmission may have already been performed, and no ACK/NACK has been received after that, then polling in this scenario may be helpful for fast retransmission. In this corner case, the more accurate change to address this issue is as follows:

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged and the retransmissions of the negatively acknowledged RLC SDUs have not been performed or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Based on above analysis, we have two options for this RLC open issue.

Proposal 1: [RLC-N02] RAN2 to choose from the following two options:

- Option 1: No need to remove "negatively" in the NOTE.

- Option 2: Add “and the retransmissions of the negatively acknowledged RLC SDUs have not been performed” after “negatively acknowledged” in the NOTE.

In our view, both options are feasible. However, changes in option 2 is more difficult to comprehend and may over-specify UE behaviors. For simplicity, option 1 is preferable.

Proposal 1bis: [RLC-N02] If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer, the transmitting side of AM RLC entity may stop and reset the running t-PollRetransmit, i.e., no need to remove "negatively" in the NOTE. (Option 1)

[RLC-H02]

In [5], it is proposed to confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status, and to specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate condition. During RAN2#131bis meeting, it has been confirmed that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. The solutions on this issue are left to discussion in this meeting.

In our view, the reason for this misalignment of RX_Next and RX_Highest_Status is that when t-RxDiscard expires, the RX_Highest_Status is not updated. To follow similar behavior when t-Reassembly expires, where the RX_Highest_Status is updated for generating the STATUS PDU, RX_Highest_Status shoud be updated when t-RxDiscard expires as well.

Observation: [RLC-H02] The inconsistency of RX_Next and RX_Highest_Status is due to the reason that RX_Highest_Status is not updated when t-RxDiscard expires.

To decide the upper bound of the SN (ACK_SN) for constructing a STATUS PDU, the state variable RX_Highest_Status needs to be updated to reflect the latest status, considering some PDU(s) may have been received after t-Reassembly expires. The principle of updating RX_Highest_Status is the same as that when t-Reassembly expires, but the difference is to set it to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger (instead of RX_Next_Status_Trigger) for which not all bytes have been received. With this update, the RX_Highest_Status will have the same value as RX_Next. The inconsistency issue between the two variables is therefore addressed, and the procedural text for constructing a STATUS PDU in 5.3.4 can keep unchanged.

Proposal 2: [RLC-H02] When t-RxDiscard expires, update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received (same as RX_Next).

Also similar to the expiry of t-Reassembly, the status report for the expiry of t-RxDiscard shall be triggered after RX_Highest_Status is updated. Current NOTE 3 can be changed adaptively to reflect this order of tiggering (please refer to Annex for the change).

Proposal 3: [RLC-H02] The expiry of t-RxDiscard triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

Proposal 4: [RLC-H02] Adopt the TPs in Annex 1 for P2 and P3.

Cancellation of bit rate query

In the current MAC specification, upon request by upper layers for a preferred UL bit rate for a QoS flow, the MAC entity shall trigger a bit rate query for the QoS flow, if no other bit rate query is already pending for the same QoS flow. A bit rate query remains pending after being triggered, until it is cancelled. When UL-SCH resources are available for a new transmission, a UL Rate Control MAC CE may be generated. For each QoS flow whose bit rate query is included in the UL Rate Control MAC CE, the bit rate query for the QoS flow is cancelled.

An issue to be discussed is whether the UE should cancel the pending bit rate query for a QoS flow if a UL Rate Control MAC CE including the bit rate for the same QoS flow is received from the gNB. In the following example in Figure 1, a bit rate query for a QoS flow (e.g., QoS flow i) is triggered and not cancelled, i.e., pending. At this time the gNB transmits a UL Rate Control MAC CE to the UE including the bit rate for QoS flow i. According to the current MAC specification, the UE may still transmit the UL Rate Control MAC CE including the bit rate query for QoS flow i to the gNB when UL-SCH for new transmission is available.

Figure 1 An example of Bit rate query

From our perspective, it is unnecessary for the UE to transmit the bit rate query for QoS flow i anymore, since the bit rate for QoS flow i has already been received. Therefore, we propose to cancel the pending bit rate query and start the bitRateQueryProhibitTimer for QoS flow i in this case.

Proposal 5: The bit rate query for a QoS flow is cancelled if the UE receives a UL Rate Control MAC CE including the bit rate for this QoS flow.

The text proposal can be found in Annex 2.

Proposal 6: Adopt the TP in Annex 2 for cancellation of a bit rate query for a QoS flow.

Conclusion

In this paper we discuss two RLC open issues and uplink rate query and have come up with the following proposals:

Proposal 1: [RLC-N02] RAN2 to choose from the following two options:

- Option 1: No need to remove "negatively" in the NOTE.

- Option 2: Add “and the retransmissions of the negatively acknowledged RLC SDUs have not been performed” after “negatively acknowledged” in the NOTE.

Proposal 1bis: [RLC-N02] If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer, the transmitting side of AM RLC entity may stop and reset the running t-PollRetransmit, i.e., no need to remove "negatively" in the NOTE. (Option 1)

Observation: [RLC-H02] The inconsistency of RX_Next and RX_Highest_Status is due to the reason that RX_Highest_Status is not updated when t-RxDiscard expires.

Proposal 2: [RLC-H02] When t-RxDiscard expires, update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received (same as RX_Next).

Proposal 3: [RLC-H02] The expiry of t-RxDiscard triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

Proposal 4: [RLC-H02] Adopt the TPs in Annex 1 for P2 and P3.

Proposal 5: The bit rate query for a QoS flow is cancelled if the UE receives a UL Rate Control MAC CE including the bit rate for this QoS flow.

Proposal 6: Adopt the TP in Annex 2 for cancellation of a bit rate query for a QoS flow.

References

- R2-2507705, Report from session on XR and LTE-based 5G Broadcast, 3GPP RAN2#131bis

- TS38.322, Radio Link Control (RLC) protocol specification, v19.0.0

- TS38.321 v19.0.0, NR; Medium Access Control (MAC) protocol specification (Release 19)

- R2-2507159, UP open issues, 3GPP RAN2#131bis

- R2-2507056, Discussion on remaining issues for RLC for R19 XR, 3GPP RAN2#131bis

Annex 1 (TP to TS38.322 v19.0.0)

Start of the Change

##### 5.2.3.2.5	Actions when t-RxDiscard expires

When t-RxDiscard expires, the receiving side of an AM RLC entity shall:

-	discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any;

-	update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received;

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received;

-	if RX_Next_Highest > RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-RxDiscard;

-	set RX_Next_Discard_Trigger to RX_Next_Highest.

Next Change

### 5.3.4	Status reporting

>>> unchanged part skipped>>>

NOTE 3:	The expiry of t-RxDiscard triggers both RX_Next and RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next and RX_Highest_Status are updated.

End of Change

Annex 2 (TP to TS38.321 v19.0.0)

Start of the Change

### 5.18.40	UL Rate Control

The UL Rate Control procedure provides information on UL physical-layer bit rate available to a QoS flow.

The MAC entity is configured by upper layers with a set of QoS flows for which UL bit rate indication and UL bit rate query are supported.

The gNB may transmit the UL Rate Control MAC CE (defined in clause 6.1.3.86) to the MAC entity to recommend UL bit rate(s) for one or multiple QoS flows. Upon reception of the UL Rate Control MAC CE, the MAC entity shall indicate the recommended bit rate(s) to upper layers.

The MAC entity may transmit the UL Rate Control MAC CE to the serving gNB to request preferred UL bit rate(s) for one or multiple QoS flows. Upon request by upper layers for a preferred UL bit rate for a QoS flow, the MAC entity shall trigger a bit rate query for the QoS flow, if no other bit rate query is already pending for the same QoS flow. A bit rate query remains pending after being triggered, until it is cancelled.

When UL-SCH resources are available for a new transmission, the MAC entity shall:

1>	for each QoS flow with a pending bit rate query:

2>	if bitRateQueryProhibitTimer for the QoS flow is configured but not running:

3>	include the QoS flow and its preferred bit rate in the MAC entity’s list of pending bit rate queries.

1>	if there is at least one entry in the MAC entity’s list of pending bit rate queries; and

1>	if the UL-SCH resources can accommodate the UL Rate Control MAC CE, including its subheader and preferred bit rate of at least one of the pending queries, as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the UL Rate Control MAC CE.

1>	for each QoS flow whose bit rate query is included in the UL Rate Control MAC CE:

2>	start its bitRateQueryProhibitTimer;

2>	cancel its bit rate query.

When a bit rate query for a QoS flow is pending, if the MAC entity receives the UL Rate Control MAC CE from the gNB and the UL Rate Control MAC CE includes UL bit rate for the QoS flow whose bit rate query is pending, the bit rate query for the QoS flow is cancelled and the bitRateQueryProhibitTimer for the QoS flow is started.

NOTE:	If the UL-SCH resources cannot accommodate all the pending bit rate queries, it is up to UE implementation to determine whether and which queries are included in the UL Rate Control MAC CE.

End of Change