3GPP TSG-RAN WG2 Meeting #132	R2-2508313

Dallas, USA, 17 – 21 November 2025

Agenda item:	9.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On support of voice over NB-IoT-NTN via GEO

WID/SID:	IoT_NTN_Ph4 - Release 20

Document for:	Discussion and Decision

# 1	Introduction

At RAN #109 the work item on NB-IoT NTN Phase 4 [1] was agreed with the following objective:

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

At RAN2-131bis meeting, RAN2 reached agreements as included in Annex, and concluded that majority of companies in RAN2 prefer the UP over CP solution. Furthermore, RAN2 agreed to further study and discuss how to handle the voice packets of different sizes. In this document, we elaborate more on scheduling of the variable packet size as well as CP/UP solution.

# 2	Discussion

## 2.1	CP vs. UP solution

As indicated in [1], at RAN2-131bis meeting, majority companies prefer the UP over CP solution, considering the higher specifications impact to support the CP than the UP solution. For example, the CP solution may need to introduce a new SRB for RLC UM and a new QoS framework, yet it offers no advantage over the UP solution in terms of overall packet overhead size, while also requiring coordination with other working groups (e.g., CT1) to further address NAS header optimization.

RAN2 Agreement: RAN2 also discussed the CP and UP solution for the support of IMS voice over NB-IoT NTN. Majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

Additionally, at SA2-171 meeting, SA2 also reached interim conclusions on KI#1 Support of IMS voice call over NB-IoT NTN via GEO [3] where the voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

To facilitate the final decision on down-selecting between the CP and UP solutions at the RAN Plenary meeting this December, we believe RAN2 could confirm the selection of the UP solution. However, the choice between IP and non-IP over UP could be left to SA2 for decision.

Proposal 1: From RAN2’s perspective, the UP solution should be selected to support voice over NB-IoT-NTN. The choice between UP IP and UP non-IP can be further down-selected by SA2.

## 2.2	Voice packets scheduling

At RAN2-131bis meeting, RAN2 agreed to further study and discuss the scheduling method to handle the voice packets of different sizes.

RAN2 agreement: Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

Due to limited PDCCH resource and large RTT in GEO NB-IoT NTN, SPS should be used in both UL and DL to support voice over NB-IoT-NTN, given the voice packets follow a fixed traffic pattern in both the packet size and the packet interval.  However, according to current specification, only UL SPS can be configured in NB-IoT, and the corresponding UL grants can be used only for BSR or SPS confirmation. Therefore, RAN2 should first confirm to support SPS for voice packet transmission in both UL and DL.

36.321 5.10.2: For NB-IoT UEs, a configured uplink grant shall be used only for BSR or SPS confirmation transmission, and skipUplinkTxSPS is implicitly configured.

Proposal 2: RAN2 to introduce UL and DL SPS used for voice packet transmission in NB-IoT-NTN.

To make the SPS works well, ideally, the transport block size (TBS) of each SPS occasion should be large enough to accommodate the voice payload in the UE’s buffer, and the SPS occasion periodicity should match the voice packet interval. This ensures that the generated voice packets can be delivered exactly as intended using the allocated SPS resources. Otherwise, the additional payload which cannot be transmitted via SPS TBS after each of the SPS transmission may need further scheduling enhancement to handle it.

Observation 1: To deliver voice packets using SPS occasions only, the SPS TBS should be sufficient to accommodate the voice packet payload, and the SPS periodicity should align with the voice packet interval.

However, the voice packet payload size and/or packet interval may be changed during the call. Taking the UL as an example, such variations can arise from transitions between talkspurt and silence period, changes in voice codec bitrate, adjustments in voice packet aggregation/bundling level as well as the change of ROHC header size (if IP-based solution is selected). Consequently, the network may need to dynamically adapt the corresponding SPS configurations, such as the SPS TBS and/or periodicity, to match the evolving voice packet pattern. To support this dynamic adapation, the legacy mechanism typically requires the UE to first send a BSR to eNB, indicating the updated voice payload size. Based on the BSR, the eNB can infer changes in voice packet size and/or the packet interval and then decide whether to reconfigure the SPS configurations accordingly.

Observation 2: SPS configuration adapation should be supported to align with the changes in voice payload size and/or packet interval.

For NB-IoT, the SPS configuration is provisioned by the eNB via RRC signalling, and activation or release of the configured SPS is controlled through DCI fromat N0. For TN, the mechansim works effectively because the RTT between UE and eNB is significantly shorter than the typical voice packet interval (e.g., 20ms), allowing timely SPS reconfiguration in response to traffic variations. However, for NTN over GEO, the RTT is more than 500 ms (max 541 ms propagation for transparent case and 271 ms for regenerative). From the UE’s perspective, this results in a delay of more than 500ms between the transmission of a BSR and the subsequent activation of a new UL SPS configuration.

Such a long latency severely undermines the effectiveness of SPS adaptation for real-time voice traffic. For example, when the UE transitions from silence period to talkspurt, it sends a BSR to indicate the arrival of a new voice packet. However, due to the long RTT in NB-IoT-NTN, the UE continues to generate and buffer voice packets—e.g., every 80 ms—while waiting for the network to activate a new UL SPS configuration via DCI. By the time the SPS is activated (i.e., when the activation DCI is received), the UE may have already accumulated multiple voice packets in its buffer. However, the activated SPS TBS is typically configured to carry a single voice packet, as this ensures efficient and consistent resource allocation across all subsequent SPS occasions. Consequently, the initial SPS grant may be insufficient to transmit the backlog of buffered voice packets, leading to potential packet loss, increased delay, or the need for fallback to dynamic scheduling—thereby negating the resource-saving benefits of SPS in NB-IoT-NTN.

In another example, when the UE transitions from a talkspurt to a silence period, it may generate fewer and smaller size voice packets compared to the talkspurt. Due to the delay in updating the SPS configuration, the network may continue to allocate periodic SPS resources sized for the higher talkspurt traffic. This mismatch can lead to inefficient resource usage, resulting in wasted system capacity and UE power consumption—either through the transmission of unnecessary padding bits within the SPS TBS or through entirely unused periodic SPS resources.

Observation 3: The legacy SPS configuration adaptation mechanism is ineffective in NB-IoT-NTN due to up to 541ms RTT delay between the UE and eNB, which prevents timely configuration update. As a result, the newly activated SPS grant may be insufficient to transmit the backlog of voice packets buffered during the adaptation latency, or, conversely, the allocated periodic SPS resource may be wasted when the traffic decreases.

In our understanding, the voice traffic pattern variation (e.g., voice payload size and/or packet interval) will anyway happen during the call. Therefore, RAN2 should study how to support effective SPS configuration adaptation and reconfiguration.

Proposal 3: RAN2 to study mechanism for effectively adapting and reconfiguring SPS configurations for NB-IoT-NTN in a timely manner to support changes in voice traffic patterns.

In last meeting, on top of SPS, dynamic scheduling was proposed as the other mechanism to handle the dynamic changes in voice traffic. Specifically, after each of the SPS transmission, if there is still residual payload remains that cannot fit within the SPS TBS, the network may issue a dynamic scheduling to transmit the excess data. This is quite important for bursty, non-periodic traffic variations. For example, if the ROHC header size changes due to occasional ROHC state transitions or a change of the ROHC packet format (e.g., from UO-0 to UO-1), the overall packet size may temporarily deviate from the SPS TBS. In such case, dynamic scheduling is more suitable than reconfiguring the entire SPS configurations. However, compared to SPS, dynamic scheduling not only costs more PDCCH resource but also suffers from additional scheduling delay caused by the long RTT in NB-IoT-NTN. It is currently unclear whether the additional scheduling delay will cause the voice packet discards since the allowable packet delay budget (PDB) for voice over GEO is yet to be specified by SA2. In this regard, it may be open whether SPS is needed to be activated in advance from transmission of the first voice packet or not. However, it is still worth to minimize the possiblity to use the dynamic scheduling for voice over NB-IoT NTN. Furthermore, as the PUSCH/PDSCH repetition will be used by both the dynamic scheduling and SPS transmissions, it may be hard for the network to always find suitable slots to schedule the dynamic scheduling in time without overlap with the granted SPS transmissions. Therefore, we think RAN2 should target to study the co-existence between dynamic scheduling and SPS while trying to minimize the occasions to use dynamic scheduling.

Proposal 4: RAN2 to study the co-existence of dynamic scheduling and SPS for voice over NB-IoT NTN, with the objective of minimizing the occasions requiring to use dynamic scheduling.

Another scenario that may trigger the network to issue dynamic scheduling is the presence of non-voice data traffic during an ongoing voice call. This is common in VoLTE deployment, where an LTE UE is typically configured with multiple EPS bearers. For example, QCI 1 for voice media, QCI 5 for SIP signalling, and a default QCI 9 for best-effort data traffic. When such background data needs to be transmitted concurrently with voice, dynamic scheduling becomes necessary since the SPS TBS is typically designed to accomodate voice traffic only. For voice over NB-IoT-NTN via GEO, RAN2 may need to confirm whether the best-effort data traffic can be transmitted concurrently when the UE has an ongoing voice call.

Proposal 5: RAN2 to confirm whether the best-effort data traffic can be transmitted concurrently when the UE has an ongoing voice call in NB-IoT-NTN via GEO.

To minimize the need for SPS configuration reconfiguration as well as dynamic scheduling, it is essential to constrain the degree of variation in voice packets, both in terms of packet size and packet interval. Among the factors discussed in this paper which may impact the voice packet variation, the transitions between talkspurt and silence period and changes in voice codec bitrate, are still under discussion in SA4 under the ULBC SI [3]. RAN2 can await further progress from SA4. Regarding the change of ROHC header size, it falls out of RAN2 scope on the ROHC implementation hence we may not have mechanism to control them. However, for the voice packet bundling (i.e., the number of voice frames aggregated into a single data packet for transmission), we think RAN2 can further discuss it.

In VoLTE, the voice traffic typically arrives at a rate of one frame per packet every 20ms. However, to improve resource efficiency and reduce UE power consumption while still meet the packet delay budget, two or more voice frames can be aggregated for one transmission. This bundling reduces protocol overhead (e.g., by up to 50%) by sharing headers across multiple frames. Such packet bundling/aggregation can be implemented either in the UE’s voice application layer or in the UE’s AS layer.

For voice over NB-IoT-NTN via GEO, SA4 is currently discussing the codec options and it seems they are also considering the voice bundling at the application layer. For example, as show in the table excerpt from TR26.940 [4],  when the codec frame size is 80ms, the voice bundling period can be 160ms or 320ms which means 2 or 4 voice packets are aggregated respectively.

TR26.940 v0.3.0: Table 5.1.2-2 UE delay estimation for ULBC

| Voice bundling period | Codec frame size | UE delay excluding solution specific delay (Note 1) |  |
| --- | --- | --- | --- |
|  |  | lower bound | upper bound |
| 80 | 20 | 268 | 355 |
|  | 40 |  | 395 |
|  | 80 |  | 475 |
| 160 | 20 | 428 | 515 |
|  | 40 |  | 555 |
|  | 80 |  | 635 |
|  | 160 |  | 795 |
| 320 | 20 | 748 | 835 |
|  | 40 |  | 875 |
|  | 80 |  | 955 |
|  | 160 |  | 1115 |
|  | 320 |  | 1435 |
| Note 1: UE_delay excluding the solution specific delay = 2x voice bundling period + 2x vendor specific encoder/decoder processing delay + vendor delay budget + JBM Note 2: Solution specific delay X = [TBD ms] |  |  |  |

Clearly, changing the voice bundling period directly impacts both the size of the resulting packet to be transmitted (due to aggregation of more frames) and the transmssion interval (since the packets are sent less frequently). This dual effect introduces significant variability in traffic patterns, which may trigger frequent SPS reconfiguration or even more dynamic scheduling – both of them are problematic in NB-IoT-NTN. Therefore, to enable  SPS operate effectivly and smoothly in NB-IoT-NTN system, the network should have control over whether and how the UE performs the voice packet bundling. For the packet bundling performed in the UE’s application layer, it is up to SA4 to decide the details. For the packet bundling performed in the UE’s AS layer, RAN2 can decide whether and how it should be performed.

Observation 4: For a UE configured with an SPS configuration, voice packet bundling/aggregation in the UE may trigger SPS reconfiguration or dynamic scheduling, as it changes both of the size of resulting packet to be transmitted and the transmission interval.

Proposal 6: To avoid traffic pattern variations in terms of transmitted packet size and packet interval, whether and how to perform the voice packet bundling/aggregation should be clearly defined and controlled by the network.

# 3	Conclusion

This document has made the following observations and proposals:

CP vs. UP solution

Proposal 1: From RAN2’s perspective, the UP solution should be selected to support voice over NB-IoT-NTN. The choice between UP IP and UP non-IP can be further down-selected by SA2.

Voice packets scheduling

Proposal 2: RAN2 to introduce UL and DL SPS used for voice packet transmission in NB-IoT-NTN.

Observation 1: To deliver voice packets using SPS occasions only, the SPS TBS should be sufficient to accommodate the voice packet payload, and the SPS periodicity should align with the voice packet interval.

Observation 2: SPS configuration adapation should be supported to align with the changes in voice payload size and/or packet interval.

Observation 3: The legacy SPS configuration adaptation mechanism is ineffective in NB-IoT-NTN due to up to 541ms RTT delay between the UE and eNB, which prevents timely configuration update. As a result, the newly activated SPS grant may be insufficient to transmit the backlog of voice packets buffered during the adaptation latency, or, conversely, the allocated periodic SPS resource may be wasted when the traffic decreases.

Proposal 3: RAN2 to study mechanism for effectively adapting and reconfiguring SPS configurations for NB-IoT-NTN in a timely manner to support changes in voice traffic patterns.

Proposal 4: RAN2 to study the co-existence of dynamic scheduling and SPS for voice over NB-IoT NTN, with the objective of minimizing the occasions requiring to use dynamic scheduling.

Proposal 5: RAN2 to confirm whether the best-effort data traffic can be transmitted concurrently when the UE has an ongoing voice call in NB-IoT-NTN via GEO.

Observation 4: For a UE configured with an SPS configuration, voice packet bundling/aggregation in the UE may trigger SPS reconfiguration or dynamic scheduling, as it changes both of the size of resulting packet to be transmitted and the transmission interval.

Proposal 6: To avoid traffic pattern variations in terms of transmitted packet size and packet interval, whether and how to perform the voice packet bundling/aggregation should be clearly defined and controlled by the network.

# Reference

[1] R2-2507784 Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

[2] S2-2509293 Interim conclusions on KI#1 Support of Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

[3] SP-250635 New SID on Ultra Low Bitrate Speech Codec

[4] https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_Audio/Inbox/Drafts/TR26.940-V0.3.0_rm_r1.docx

[5] R1-2508096 Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

[6] R2-2507790 Reply LS on the RAN simulation assumptions, bundling period and SPS for ULBC

# Annex

RAN2-131bis Agreements:

Agreements regarding the reply LS to SA2: 1. Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope 2. Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues. 3. Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20. 4. Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet 5. We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary. Agreements regarding the reply LS to SA4: 1. RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes. 2. For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size 3. For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size 4. RAN2 will provide further feedback on the expected average RoHC header size 5. We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues