3GPP TSG-RAN WG2 Meeting #132	                               																		  R2-2508317

Dallas, USA, Nov. 17 – 21, 2025

Source: 		 ZTE Corporation, Sanechips

Title:                 Consideration on QoS, QoE and Service-awareness in 6G

Agenda item:   10.3.1.2

Document for: Discussion and Decision

# Introduction

In this contribution we identify the essential issues related QoS, QoE and Service-awareness in 6G that should be studied and give our proposals.

# RAN level QoS for 6G

# Background – QoS handling in 5G

One of the main changes from 4G to 5G is the move away from the end-to-end bearer concept (in 4G) to a QoS-flow based modelling (in 5G). In case, of 5G, whilst the packets between CN and RAN are marked with the QoS flow identifier, on the RAN side, the bearer concept is still kept and the finest granularity of QoS at radio level is still the radio bearer (D/S)RB. Thus, in Rel-15, the 5G QoS model is characterised by the following principles:

- QoS flow is the finest granularity for QoS treatment in the system

- All packets within a QoS flow are treated equally

- At radio level the finest granularity for QoS handling is the RB (radio bearer) and the scheduling priorities for all packets within a given radio bearer are the same

This overall QoS model is depicted in Figure 1 below.

Figure 1: QoS model in 5G

However, with 5G-Advanced, for XR like traffic, it has been appreciated that some of the packets within the same QoS flow may have different characteristics with respect to inter packet dependencies and need different treatment especially with respect to handling of discard mechanism. In order to handle this the concept of PDU Set has been introduced in later releases of 5G advance, specifically for XR like traffic. With in the PDU set, if one packet is discarded, other packets belonging the same set may also become redundant and hence may be subject to discard. In order to handle this, new QoS related identifiers PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and/or PDU Set Integrated Handling Information (PSIHI) were introduced.

# New 6G use cases and requirements

6G use cases and service requirements have been studied in SA1. Based on the SA1 output TR [3], it may be challenging to meet these requirements based on the 5G QoS model and service awareness.

Use case on real time VR live service with deterministic user experience

In this use case, the immersive service (e.g. VR) is a real time service which includes audio, video and haptic related data. A typical transmission data rate for the immersive service is considerable, which makes the user experience can hardly be guaranteed at the condition when a user is at poor coverage or a bandwidth limited place. With the advent of advanced algorithms (AI or non-AI), it can help to recover the information of the receiving end even when the receiving end lost a part of information incurred by the poor communication link, which is called “Error tolerant communication” in this use case. By leveraging AI techniques, the error-tolerant information even with being discarded via transmission can still be recovered based on the traditional codec (FEC, Error Concealment) at the receiving end with a deterministic user experience.

[Potential New Requirement 9.11.6-2] The 6G system shall be able to transmit the error tolerant data traffic with adaptive QoS which is used for efficient delivery of the error tolerant data traffic, i.e. the reliability and latency can be adjusted automatically without influence from 3rd party application.

[Potential New Requirement 9.11.6-4] The 6G system shall provide a suitable means for communication with error tolerance to evaluate the high data transmission error rate and accept the data packet that fulfil the condition of high data transmission error rate (e.g. allow a percentage of error bits for a successfully delivered packet).

Use case on personalized interactive immersive guided tour

In this use case, localized (i.e. location-aware) mobile Mixed Reality (MR) services provide users with information and services that are of local relevance. This use case focuses on a personalized interactive immersive guided tour service of a city including outdoor/indoor configurations.

[Potential New Requirement 9.12.6-2]	The 6G system shall provide a means to synchronize media/flow and data for rendering in time and in XR space (e.g. coherent gestures, facial expressions the personalized avatars of the group of participants.

[Potential New Requirement 9.12.6-3] The 6G system shall support a mechanism, including enabling one or more authorized third party(ies) to adapt XR session QoS dynamically at different levels of granularity (e.g. per type of media stream, per data flow, per burst) based on the fluctuations of the 6G connection conditions and QoE policies.

[Potential New Requirement 9.12.6-4] The 6G system shall provide a means to synchronize heterogeneous data flows from/to a set of UEs (e.g. phone, glasses, tethered ring) associated with a single user and between UEs associated with multiple users in a multimodal communication (e.g. each user having glasses and/or phones).

[Potential New Requirement 9.12.6-5] Subject to operator policy and privacy considerations, the 6G system shall provide a means to keep heterogeneous media flows synchronized within an acceptable threshold whether rendered on a single or multiple UEs, for each user in a multimodal communication.

Use case on intelligent transmission service for user experience improvement

[Potential New Requirement 9.13.6-1] Subject to operator’s policy, the 6G network shall support QoS adjustment based on real time network status to ensure user experience.

Use case on Application Context Enhanced Communication Service

[Potential New Requirement 9.17.6-1] Subject to operator policy, 6G network shall provide means to receive information related to the application from an authorized UE or 3rd party application.

Proposal 1: In order to address the new requirements for 6G identified by SA1, RAN2 needs to study the following areas of enhancements:

- Traffic with fast and dynamic QoS requirements

- XR/AI traffic with multi-modality with synchronization requirements

- Error tolerant data traffic

- Ability to exchange application related information from authorized UE/3rd party applications

Based on the above, the overall QoS model for 6G is discussed in sections below.

# QoS framework for 6G

For 6G, the overall QoS architecture should be designed by SA2 and RAN2. Considering the current 5G QoS model, and the general direction of traffic getting even more diverse over the years (and not less), it is unlikely that the QoS model between CN and RAN will be any coarser than what is available in 5G. If anything, an even finer granularity (e.g. per packet handling) is more likely. However, the QoS-flow based model is flexible enough to be extended to such finer granularity at least on the CN interfaces. Furthermore, in 5G, with the flexibility to map different QoS flows to different DRBs, it is clear that RAN also can provide the same granularity of QoS handling as that is available in other parts of the system. In 5G, especially in UL, currently a given QoS flow can only be mapped to 1 DRB. This is under the assumption that within a given QoS flow same QoS requirements apply to all packets. The only exception to this (which was studied and specified for XR) is to allow selective discard of a sub set of packets within the QoS flow based on some additional information (e.g. PDU Set handling). For 6G, we need to first understand if such framework can be reused or if a new QoS framework will be defined. There is a very clear dependency here with the work done in SA2 and hence, it would be good to highlight this to SA2.

Observation 1: In 5G, all packets within a QoS flow are subject to the same QoS requirements. We need to understand if this general principle will also be extended to 6G (coordination with SA2 is needed)

Based on the above, if all packets within a QoS flow are subject to the same QoS requirements, then it is possible that the general RB based handling of 5G is taken as baseline for 6G and enhancements for this can be studied mainly focussing on what has been studied for XR (e.g. PDU set based handling for discard during congestion etc).

One problem with this model however is that some higher layer control information (e.g. TCP control packets) may be subject to head of line blocking and this will cause the overall link throughput to suffer. It should be discussed whether such packets need special treatment. If this is the case, there may be two ways to deal with this:

- Packets that need different treatment (e.g. TCP control information) are mapped to a different QoS flow (up to SA2 to decide this and is then transparent to RAN2)

- Packets that need different treatment may exist within the same QoS flow and it is up to RAN to differentiate these (this will have more impact to RAN2)

If RAN has to deal with packets that need different QoS treatment within the same QoS flow (e.g. TCP control packets), then we need a standardized mechanism to deal with these. One option could be to map these to a different DRB. But this needs further discussion whether such option is needed or not. Since this depends on the SA2 decisions on the QoS architecture, we need to first understand the overall QoS model first. So, we propose the following:

Proposal 1: 	Ask SA2 if all packets within a QoS flow will have the same QoS requirements in case of 6G.

- If yes, the radio bearer based QoS handling from 5G, where a given QoS flow is mapped to only one DRB and more than one QoS flows (with similar QoS requirements) can be mapped to the same DRB, can be taken as baseline for 6G

- If no (e.g. some packets like TCP feedback mapped to same QoS flow needs special treatment to avoid HOL blocking issues), RAN2 should study enhancements (e.g. mapping one QoS flow to different DRBs etc)

One critical issue to keep in mind here is that any standardized feature should work across the protocol layers both in UL and DL. Whilst the QoS marking for packets in DL is done by a network node within CN; in UL, this is done by the UE. The QoS flow identification is configured by NAS level signalling and is fully specified. Any further enhancements to this model, especially for UL (e.g. to identify packets that need different QoS treatment within the same QoS flow etc) should also be based on standardized configuration (e.g. enhanced NAS level TFT filter configuration at the UE). Unless this is the case, it is very difficult to ensure uniform implementations at the UE side and this creates problems for network to implement these features and guarantee QoS. One of the short-comings of the XR features developed in 5G-A is that the identification of various UL QoS parameters (such as priority/PDU set identification etc) are all left to UE implementation. This is not ideal and should be avoided for 6G.

Proposal 2: 	The per packet QoS parameters (such as QoS flow/PDU set identification etc) in DL should be done at the CN and indicated to RAN in the UP header (similar to GTP-U header) whilst this should be based on standardized mechanisms using NAS configuration at the UE in UL (SA2 and CT1 involvement is needed)

# Traffic/Service Awareness at RAN

For services such as XR/AI applications, traffic generated by a given application can have multiple modalities (e.g. Audio/Video/Text/Sensory information etc). The application may also quickly switch between these modalities. The generated traffic will also have dependencies between the traffic belonging to different modalities (e.g. synchronization requirements etc). In 5G XR, multi-modal application use case has been discussed, but the only specified enhancement was to provide MMSID (Multi-modal Service ID) to NG-RAN, as part of the QoS parameters of the QoS flow, for QoS flows to DRB mapping. In case of 6G, a generic framework to enable RAN awareness for multi-modal traffic and the QoS requirements for multi-modal traffic can be considered from day 1 in cooperation with SA WGs.

Proposal 3: 	6GR should support RAN awareness of multi-modal traffic and the associated QoS requirements for each modality.

In addition, SA1 has already identified a wide range of new 6G specific use cases, notably involving AI-driven applications. A clear observation from these studies is that the QoS requirements for future AI-enabled services are highly dynamic. For example, an AI agent service may support multiple interaction modalities (text, speech, video), and its instantaneous QoS requirements vary depending on the specific task being processed. Traditional QoS adaptation procedures, based on NAS/NG-level signaling, are not well-suited for these scenarios due to long reaction times and heavy signaling overhead. Such procedures were designed with relatively static or predictable service requirements in mind. In contrast, future AI-driven tasks often involve sudden, unpredictable, and rapidly changing QoS demands that should be fulfilled in real time to maintain service quality. To address these challenges, enhancements on QoS attributes and QoS framework/mechanism (e.g. service awareness and cross-layer coordinated QoS adaptation) should be studied in 6G. In 5G, any QoS renegotiation requires NAS signalling and this is not suitable for quick changes in QoS that are applicable to multi-modal applications such as AI based XR applications. In order for the RAN to be able to switch between different QoS profiles, RAN-level QoS reconfiguration should be considered as a baseline for such applications.

Observation 2: Traditional QoS adaptation procedures involving NAS/NG signalling are not well suited for AI/XR applications that involve rapid changes of QoS requirements.

Proposal 4: 	6GR shall support RAN based switching between different preconfigured QoS profiles without involving NAS/NG signalling to enable fast QoS adaptation at RAN level

Furthermore, some XR/AI applications are built in such a way that the upper layers are tolerant to errors at the lower layers. This may be due to FEC at upper layers (e.g. application level FEC in some XR like applications) or due to differential importance for the data exchanged in lower layers (e.g. in some AI/ML applications where some of the raw data is converted in to upper layer PDUs that are error tolerant). For such use cases, RAN2 should study if the UP should enable modes of operation where RAN is aware of the error tolerance in upper layers and can use this information, e.g. in discard mechanisms in L2. In case of XR, one mechanism that was studied was the content ratio based discard for a given PDU set (see Solution #3 in 3GPP TR 23.700-70). RAN2 can study such mechanisms if L2 is aware of the error tolerance reception for upper layers.

Proposal 5: 	RAN2 shall study mechanisms to enhance the L2 operation based on the knowledge of error tolerant reception at upper layers (e.g. solutions such as content ratio based PDU discard in PDCP layer).

# QoE awareness at RAN

QoE report mechanism was introduced to allow NW to know the user experience of a specific service type for a specific UE or UEs in certain area. Based on the QoE report, NW can know whether the QoS policies specified for a service can fulfill the goal to deliver a high-quality, productive experience for the end-user. In 6G QoE awareness for the network shall be enabled to fulfill the same requirement.

QoE framework first introduced in LTE allows collection of user experienced metrics (e.g. playout delay etc) for a specific service type for a specific UE or UEs in certain area. The collected QoE information then can be used by MCE in CN for analysis and KPIs calculation. In R17-R18, QoE report is introduced in NR with further enhancements. For example, the supported service types are extended to include VR, AR services. Subsequently, RAN visible QoE is introduced to allow RAN node to gain knowledge of QoE metrics to assist configuration optimization, e,g., resource management, scheduling adjustment. With the introduction of RAN visible QoE, there are two termination points for the QoE report, one is at RAN another is at MCE located in CN. It is understood that QoE still play an important role in 6G for service awareness and NW optimization, e.g., scheduling enhancements. Therefore, QoE report shall still be one of the use cases to be considered in data collection.

Most of requirement for 6G QoE report, e.g., data size, report frequency, QoS requirement, can be inherited from 5G. One thing that needs further discussion is the termination point. In 5G, because RAN visible QoE report is introduced at late stage, the RAN visible QoE can only be initiated when MCE has initiated a container QoE task towards UE, which limited the applicability of RAN visible QoE at RAN.

Observation 3: RAN visible QoE in 5G can only be initiated when CN (MCE) initiates a container based QoE task and this creates unnecessary dependencies/redundancies for this function.

In order to enable a unified, efficient data collection for different use cases, including QoE in 6G, the framework should enable RAN to also initiate the QoE task based on its own needs. The solution requires RAN to have the visibility of QoE report container from UE and the visibility of the QoE task requirement from CN. In order to simplify the overall design and avoid redundancy between CN and RAN level QoE framework, a unified framework should be considered for 6G where both RAN and CN can share a common mechanism for QoE reporting.

Proposal 6:  	6G shall support a unified data collection framework to support QoE reporting which can be initiated either by RAN or CN with full visibility of collected data both at RAN and CN

# Conclusion

Based on the above, the following observations/proposals is made:

Proposal 1: In order to address the new requirements for 6G identified by SA1, RAN2 needs to study the following areas of enhancements:

- Traffic with fast and dynamic QoS requirements

- XR/AI traffic with multi-modality with synchronization requirements

- Error tolerant data traffic

- Ability to exchange application related information from authorized UE/3rd party applications

Observation 1: In 5G, all packets within a QoS flow are subject to the same QoS requirements. We need to understand if this general principle will also be extended to 6G (coordination with SA2 is needed)

Proposal 2: 	Ask SA2 if all packets within a QoS flow will have the same QoS requirements in case of 6G.

- If yes, the radio bearer based QoS handling from 5G, where a given QoS flow is mapped to only one DRB and more than one QoS flow (with similar QoS requirements) can be mapped to the same DRB, can be taken as baseline for 6G

- If no (e.g. some packets like TCP feedback mapped to same QoS flow needs special treatment to avoid HOL blocking issues), RAN2 should study enhancements (e.g. mapping one QoS flow to different DRBs etc)

Proposal 3: 	The per packet QoS parameters (such as QoS flow/PDU set identification etc) in DL should be done at the CN and indicated to RAN in the UP header (similar to GTP-U header) whilst this should be based on standardized mechanisms using NAS configuration at the UE in UL (SA2 and CT1 involvement is needed)

Proposal 4: 	6GR should support RAN awareness of multi-modal traffic and the associated QoS requirements for each modality – inform SA2 about this requirement

Observation 2: Traditional QoS adaptation procedures involving NAS/NG signalling are not well suited for AI/XR applications that involve rapid changes of QoS requirements.

Proposal 5: 	6GR shall support RAN based switching between different preconfigured QoS profiles without involving NAS/NG signalling to enable fast QoS adaptation at RAN level – inform SA2 about this requirement

Proposal 6: 	RAN2 shall study mechanisms to enhance the L2 operation based on the knowledge of error tolerant reception at upper layers (e.g. solutions such as content ratio based PDU discard in PDCP layer).

Observation 3: RAN visible QoE in 5G can only be initiated when CN (MCE) initiates a container based QoE task and this creates unnecessary dependencies/redundancies for this function.

Proposal 7:  	6G shall support a unified data collection framework to support QoE reporting which can be initiated either by RAN or CN with full visibility of collected data both at RAN and CN

# References

- 3GPP TS 38.415 V19.0.0 (2025-09), NG-RAN; PDU Session User Plane Protocol(Release 19)

- 3GPP TR 26.927 V19.0.0 (2025-06), Study on Artificial Intelligence and Machine Learning in 5G media   services;

- 3GPP TR 22.870 V0.4.1 (2025-10), Study on 6G Use Cases and Service Requirements

- R2_131bis_ChairNotes_25-10-17_13-00