3GPP TSG-RAN WG2 #132	R2-2508336

Dallas, USA, 17th – 21st November, 2025

Agenda item:	   9.3.2

Source:		Lenovo

Title:                             RRM measurement prediction

Document for:		Discussion and Decision

# Introduction

As instructed in the WID, the AI/ML-assisted mobility WI will focus on RRM measurement prediction and measurement event prediction. In this paper, we discuss essential LCM functionality management aspects for RRM measurement prediction, considering UE capability, applicability reporting, and inference configurations for temporal and frequency domain predictions for UE-sided model.

# Discussion

For AI mobility use cases, initial agreements have been made on functionality management based on the progress of AI/ML PHY work item. Given that all UE-side AI/ML functionalities are up to UE capabilities and as agreed in AI/ML PHY, the supported functionalities can be indicated by the UE using the UE capability framework. Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. In response to which, UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. The granularity of UE capabilities for AI mobility use cases can be at the sub-use case level, such as temporal and frequency domain RRM measurement predictions. Additionally, RAN2 should discuss whether inter-frequency and intra-frequency predictions must be considered as individual UE capabilities.

Proposal 1: UE capability for RRM measurement and RRM event prediction are two separate capabilities; the granularity of UE capability can be at least at the sub-use case level (e.g., temporal and frequency domain measurement predictions).

To apply AI/ML functionalities for inference, the respective configuration and thus the functionalities must be applicable from both the network and UE perspectives. Therefore, determining applicability is a crucial step for the smooth functioning of all AI/ML features. Both Option A and Option B for inference configuration or set of inference parameters are considered. Upon receiving the set of parameters for inference or the full inference configuration from the network, the UE is expected to report the applicability of the configured functionalities. It has been agreed that UE reports the applicability via RRCReconfigurationComplete and/or UAI message.

Agreements

- Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case)

•	Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress)

•	Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

•	Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

•	Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

•	Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

•	No prohibit timer is introduced.

- Associated ID should be optionally configurable for training and  inference (i.e. it may not mandatorily required for all training/inference configurations).  FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

How UE can determine the applicability of an AI/ML functionality has been intensively discussed in RAN2, and it has been concluded that the applicability status depends on the configuration, network-side additional conditions, UE-side additional conditions, and the model availability at the UE. The network-side additional conditions and UE-side additional conditions are essential to maintain the consistency in training and inference of a functionality. The UE-side additional conditions can be internally known to the UE, which do not require specification impact. Whereas, network-side additional conditions, if available, must be considered necessary to decide the applicability status of the functionalities/configurations by the UE. We believe this criterion for applicability determination should be common to AI/ML framework and is not limited to certain use cases. It is practical considering different scenarios and conditions experienced by the network, which can also be extended beyond the serving cell. Following the progress of AI/ML PHY, additional conditions are represented by associated ID(s), and as agreed in AI Mobility study item, associated ID is optionally configurable for training and inference (it may not be mandatorily required for all training/inference). It is important to discuss the impact of associated ID(s) on the applicability of functionality when network-side additional conditions change during UE mobility, for example, depending on the properties of the DL Tx beams.

Proposal 2: RAN2 clarifies for which cases the associated ID can be absent, and whether/how consistency in training and inference can be maintained when the associated ID is absent.

Considering associated ID(s) is optional, the UE needs to determine applicability based on the respective network configuration, UE internal conditions, and model availability only. Since the UE internal conditions and model availability are up to UE implementation, the determination of applicability can up to UE implementation following the progress in AI PHY.

Proposal 3: If associated ID is absent, UE determines applicability based on UE implementation. RAN2 discusses whether UE should consider any additional criteria, e.g., model availability.

In particular, for mobility use cases, it is essential to maintain the AI/ML functionalities while the UE traverses from one cell to another. Limiting the associated ID to be cell-specific can restrict the performance of the AI-assisted mobility. Therefore, associated ID(s) beyond cell-specific ID(s) will be useful for an uninterrupted transition in inference during/after handovers, and for efficient LCM processes, such as monitoring model performance. Unlike AI PHY, for AI mobility, associated ID(s) spanning across certain areas can be beneficial. This can avoid frequent fallbacks to non-AIML functionality due to the lack of an associated ID spanning across the areas. The associated ID(s) should be defined per use case/sub-use case, as the network-side conditions considered at the time of training can naturally vary for different use cases. The validity of associated ID corresponds to whether the interpretation of the associated ID is consistent with one or more cells or over an area.

Proposal 4: Associated ID(s) can be single-cell or multi-cell. An additional indicator of the validity of associated ID can be useful to indicate the consistency in the associated ID scoping beyond an area or a cell.

The UE reports applicability status in RRCReconfigurationComplete and subsequent updates in UAI. Since the applicability reporting via UAI is crucial to be reported if any change in the applicability status occurs, in AI/ML PHY, RAN2 agreed that the applicability reporting via UAI must always be enabled, which will allow the UE to always report the updated status. Unlike other UAI reports, applicability reporting should not be considered as a UE preference.  Therefore, the previous agreement related to the flag to enable or disable applicability reporting via UAI should also be revisited for AI Mobility. It is logical to apply the same principle of always enabling an applicability report via UAI for AI Mobility as well.

Proposal 5: Applicability reporting via UAI is always enabled for options A and B, and it is considered configured when option A or option B is configured.   The flag for enabling or disabling the applicability reporting via UAI is not required.

Along with the inapplicability reporting, UE further includes a release configuration indication. It was agreed to replace the inapplicability cause with a release configuration flag to indicate the UE’s preference to release a configuration, for example, due to model inavailability in the local device. For example, if the UE does not have the possibility to load a trained model, it is reasonable that the respective configuration must be released by the network. The release configuration indication should be a simple single-bit flag with respect to each functionality that is inapplicable and does not require any further enhancements, such as the reason for release or a model inavailability indication.

Proposal 6: A single bit “Release configuration” flag will be used, and RAN2 will not consider any enhancements for cause value or reasons (e.g., model inavailability) for release configuration.

The high-level categorization of RRM measurement prediction for UE-side prediction is conducted in two ways: prediction in the temporal domain and frequency domain. At the same time, pure temporal prediction is studied as the baseline for measurement predictions. The AI/ML for mobility study considered both UE-sided and NW-sided models for RRM measurement prediction. Cell-level measurements are the focus for UE-sided models; they can be either generated or predicted based on the beam-level or cell-level inputs.

For intra-cell measurement predictions with different cases, the final output will be predicted cell-level measurements, while the inputs will be different for types of measurement predictions (temporal/frequency). RAN2 is suggested to discuss whether temporal domain and frequency domain RRM measurement prediction can be applied, to generate respective inferences at the same time. As the granularity of functionality can be on per sub-use case level, e.g., temporal measurement prediction and frequency domain measurement predictions, it should be possible to configure these measurement predictions simultaneously. The purpose of generating inference using frequency domain predictions is to predict measurements of other frequencies, unlike temporal predictions. Thus, outcomes from both types of predictions can be useful for the network. The UE can determine the applicability and report applicability status for each respective case of measurement prediction individually (e.g., temporal prediction, frequency prediction). This will be important to define the granularity of functionality.

Proposal 7: Temporal frequency and frequency domain predictions can be configured simultaneously, and UE can report their applicability/inapplicability individually in the applicability report.

For all cases of RRM measurement prediction, configuration from the network is always required, which is used for applicability determination and eventually in the process of measurement inference. However, the measurement configuration may be different for each case and depends on whether the cell-level measurements are predicted based on predicted beam-level results, cell-level results, or beam-level results of the same cell or a different cell. In some cases, it may be a configuration of a set of beams, as agreed in the R19 AI/ML PHY for BM. Based on the progress of LCM in AI/ML PHY and AI mobility study item, the legacy measurement configuration framework can be used as a baseline for inference configuration and reporting, which can be extended further, considering different cases of measurement prediction.

A general set of parameters that can be part of the inference configuration for RRM measurement prediction is the cell ID of the serving cell and neighbor cell, cell frequencies, set A and set B beam configurations, and the length of the prediction window. The above parameters can be individually categorized for the different sub-cases of measurement prediction.

UE can be configured with different periodic configurations for RRM measurement predictions, for example, a periodic temporal domain inference configuration and a periodic frequency domain inference configuration, or periodic inference configurations per frequency. Upon receiving inference configurations, UE will determine their applicability individually for each case, and thus, one or more periodic inference configurations can be applicable in parallel. For certain applicable configurations, UE can start inference immediately upon receiving the configuration and sending the applicability report; whereas, UE can be configured to trigger inference upon an event being fulfilled, e.g., based on radio conditions thresholds. It would be beneficial for RAN2 to discuss the UE behaviour in this case and activation for inference of such applicable inference configurations, and how multiple applicable configurations can be handled. Each set of inference configuration for the sub-cases of measurement prediction can be associated with an individual identifier (e.g., measId) such that independent measurement configurations for the respective cases/sub-cases can be activated and reported, if applicable.

Proposal 8: After receiving the full inference configuration for measurement prediction and reporting the applicable AI/ML functionalities, the inference/report can be triggered by UE immediately (e.g., for periodic configurations) or upon a certain event.

Proposal 9: RAN2 clarifies whether multiple periodic RRM measurement predictions per frequency can be activated at the same time.

Proposal 10: For RRM measurement prediction, one set of inference configuration can be identified with a measId.

# Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

Proposal 1: UE capability for RRM measurement and RRM event prediction are two separate capabilities; the granularity of UE capability can be at least at the sub-use case level (e.g., temporal and frequency domain measurement predictions).

Proposal 2: RAN2 clarifies for which cases the associated ID can be absent, and whether/how consistency in training and inference can be maintained when the associated ID is absent.

Proposal 3: If associated ID is absent, UE determines applicability based on UE implementation. RAN2 discusses whether UE should consider any additional criteria, e.g., model availability.

Proposal 4: Associated ID(s) can be single-cell or multi-cell. An additional indicator of the validity of associated ID can be useful to indicate the consistency in the associated ID scoping beyond an area or a cell.

Proposal 5: Applicability reporting via UAI is always enabled for options A and B, and it is considered configured when option A or option B is configured.   The flag for enabling or disabling the applicability reporting via UAI is not required.

Proposal 6: A single bit “Release configuration” flag will be used, and RAN2 will not consider any enhancements for cause value or reasons (e.g., model inavailability) for release configuration.

Proposal 7: Temporal frequency and frequency domain predictions can be configured simultaneously, and UE can report their applicability/inapplicability individually in the applicability report.

Proposal 8: After receiving the full inference configuration for measurement prediction and reporting the applicable AI/ML functionalities, the inference/report can be triggered by UE immediately (e.g., for periodic configurations) or upon a certain event.

Proposal 9: RAN2 clarifies whether multiple periodic RRM measurement predictions per frequency can be activated at the same time.

Proposal 10: For RRM measurement prediction, one set of inference configuration can be identified with a measId.

# References

[1] RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[2] TR 38.744 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

[3] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

[4] TS 38.300 running CR for AI for air interface