3GPP TSG RAN WG2 Meeting #132                                                          R2-2508364

# Dallas, USA, Nov. 17th-21st, Oct. 13th-17th, 2025

Agenda item:	8.2.2

Source:	Ofinno

Title:	Views on remaining issue 3-7

Document for:	Discussion and decision

# Introduction

The following RAN2#131bis agreements were taken for the cases that A-IoT NAS does not respond:

Agreements

1.	RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

2.	For cases other than integrity failure, AS will indicate no NAS response expected to reader.   FFS how (e.g. using 0 SDU & MDI, or new indication).

3.	For integrity failure, for now RAN2 assumes that there is no AS response to the reader.  Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

Related open issues were also discussed in the email discussion [POST131bis][017][AIoT] [1]:

- Issue 3-7: Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field

# Discussion

CT1 TS 24.369 capture two kind of scenarios in which a NAS command request is considered as “not successful” by A-IoT NAS device point of view (as shown in Annex for reference):

- Scenarios (A) A-IoT NAS does not respond. The scenarios (A) include when A-IoT fails the integrity protection check, or other error scenarios (e.g., message being too short,  unknown/unforeseen IE in the non-imperative message part, or non-imperative message part errors). These scenarios (A) are highlighted in pink in the references of TS 24.369 copied in Annex, and for all of them, the UE ignores the message/IE.

For two cases of this scenario (A), when there is a failure of the integrity protection check and when the message is too short, NAS also provides an indication to the lower layers. However, this is not done for all the cases in which NAS does not respond.

- Scenarios (B) A-IoT NAS responds with a NAS message (reject or status kind of message). The scenarios (B) include unknown/unforeseen message type, non-semantical mandatory information element errors, not supporting a command message (to write or read),  a command type specific parameter being invalid, not being able to execute successfully a command (write or read), application data not being found to be read in the requested place of the user memory, or not being able to implement a write command due to running out of energy. These scenarios (B) are highlighted in blue in the references of TS 24.369 copied in Annex, and for all of them, a NAS message is sent by the A-IoT NAS device to the AIOTF including a corresponding NAS cause value.

CT1 TS 24.369 defines a group of scenarios, described in this document as scenarios (A), in which the A-IoT NAS does not respond (i.e., ignores). Scenarios (A) includes a failure of the integrity protection check, other error scenarios (e.g., message being too short, unknown/unforeseen IE in the non-imperative message part, or non-imperative message part errors).

For a failure of the integrity protection check or detecting a NAS message being too short (two out of the 4 cases of the scenario (A)), CT1 TS 24.369 captures that the AIoT device (NAS) provides an indication to the lower layers that the message was ignored at the NAS layer.

CT1 TS 24.369 defines a group of cases, described in this document as scenarios (B), in which the A-IoT NAS responds including a cause value information in a NAS response message (as a reject command for write or for read, or as a status). Scenarios (B) includes not supporting a command, not being able to successfully execute a command, or not being able to implement a command, and other errors (e.g., unknown/unforeseen message type, non-semantical mandatory information element errors).

For A-IoT, a reader pre-allocates the D2R scheduling information when providing the R2D message including inventory or a command to be requested. For scenarios (A), A-IoT NAS would not generate any NAS PDU. Considering previous RAN2 agreement, A-IoT MAC could rely on sending SDU length set to 0 and more data indication (MDI) set to 0 to indicate that there is no NAS response. However, this would not help the network to differentiate/know when the failure scenarios (A) happened. On other hand, an A-IoT device (in MAC) is currently not allowed response again to a paging for the same transaction ID when it has already been received (i.e. the transaction ID is already stored in the A-IoT MAC of the device). In our understanding, RAN2 should discuss whether any action may be required in the AS layer for these failure scenarios (A); whether the failure happening in the upper layers (e.g., to authenticate or to parse/decode the NAS message) needs to be available (visible) to the A-IoT MAC of the device and/or to the Reader. It is important to highlight that the A-IoT MAC has a valid D2R scheduling configuration (to be used) and could potentially be used if required to inform of the failure to the Reader. Moreover, the A-IoT MAC of the device could allow future retries for the same transaction ID in case of the failure (e.g., by releasing the stored transaction ID and AS ID).

When the A-IoT NAS does not respond to an inventory or command due to a failure in the upper layers, the A-IoT MAC of the device still has a valid D2R scheduling information to be used (or otherwise wasted).

- For the scenarios in which A-IoT NAS does not respond (i.e., ignores the received A-IoT NAS message), RAN2 should discuss whether the A-IoT MAC needs to release the stored transaction/AS ID and/or to inform the reader (via a MAC kind of indication) of the upper layer failure or decision of not responding.

- For the scenarios in which A-IoT NAS responds with a failure cause information, to discuss whether the A-IoT MAC in UE and Reader needs be aware of (e.g., to release the stored transaction ID and allow a re-access to the same transaction).

# Conclusion

The observations captured are the following:

Observation 1.	CT1 TS 24.369 defines a group of scenarios, described in this document as scenarios (A), in which the A-IoT NAS does not respond (i.e., ignores). Scenarios (A) includes a failure of the integrity protection check, other error scenarios (e.g., message being too short, unknown/unforeseen IE in the non-imperative message part, or non-imperative message part errors).

Observation 2.	For a failure of the integrity protection check or detecting a NAS message being too short (two out of the 4 cases of the scenario (A)), CT1 TS 24.369 captures that the AIoT device (NAS) provides an indication to the lower layers that the message was ignored at the NAS layer.

Observation 3.	CT1 TS 24.369 defines a group of cases, described in this document as scenarios (B), in which the A-IoT NAS responds including a cause value information in a NAS response message (as a reject command for write or for read, or as a status). Scenarios (B) includes not supporting a command, not being able to successfully execute a command, or not being able to implement a command, and other errors (e.g., unknown/unforeseen message type, non-semantical mandatory information element errors).

Observation 4.	When the A-IoT NAS does not respond to an inventory or command due to a failure in the upper layers, the A-IoT MAC of the device still has a valid D2R scheduling information to be used (or otherwise wasted).

The proposals captured are the following:

Proposal 1.	For the scenarios in which A-IoT NAS does not respond (i.e., ignores the received A-IoT NAS message), RAN2 should discuss whether the A-IoT MAC needs to release the stored transaction/AS ID and/or to inform the reader (via a MAC kind of indication) of the upper layer failure or decision of not responding.

Proposal 2.	For the scenarios in which A-IoT NAS responds with a failure cause information, to discuss whether the A-IoT MAC in UE and Reader needs be aware of (e.g., to release the stored transaction ID and allow a re-access to the same transaction).

# Reference

- R2-250xxxx, Summary of A-IoT MAC open issues (outcome of [POST131bis][017][AIoT] MAC CR), November 2025.

# Annex

References to failure scenarios and corresponding action described in TS 24.369 (v1.1.0):

### 4.2.3	Integrity protection of AIoT NAS signalling messages

…

In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIoT device shall discard the AIoT NAS message and provide an indication to the lower layers.

In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIOTF shall discard the AIoT NAS message.

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #1 "Command type specific parameters invalid".

#### 5.3.2.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The read command is unsuccessfully executed (e.g. user memory operation error).

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #2 "Error, unspecified".

Editor's note:	It is FFS the command and inventory procedure collision case.

### 5.4.1	Status procedure

#### 5.4.1.1	General

The purpose of the sending of the STATUS message is to report certain error conditions detected in a received AIoT NAS message by the AIoT device (see example in figure 5.4.1.1.1).

Editor's note: Ciphering and integrity protection for STATUS is FFS.

Figure 5.4.1.1.1: Status procedure

#### 5.4.1.2	Status procedure initiation

The AIoT device initiates the status procedure by sending a STATUS message to the AIOTF as described in clause 6.

#### 5.4.1.3	Status procedure completion by the AIOTF

If the AIOTF receives a STATUS message the AIOTF shall take different actions depending on the received cause value:

#97	message type non-existent or not implemented.

The AIOTF shall abort any ongoing AIoT NAS procedure related to the AIoT device and stop any related timer.

NOTE:	If the read command procedure or the write command procedure was aborted due to reception of the STATUS message with the cause #97 "message type non-existent or not implemented", the network can take this into account to avoid initiating further command procedures of the same command type towards the AIoT device, in order to save unnecessary signalling wasting radio resources.

The local actions to be taken by the AIOTF on receipt of a STATUS message with any other cause values are implementation dependent.

### 6.2	Message too short or too long

### 6.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3], and the AIoT device shall provide an indication to the lower layers that the message was ignored.

### 6.2.2	Message too long

Editor's note:	Maximum AIoT NAS message length is FFS and to be defined by RAN3.

### 6.3	Unknown or unforeseen message type

If the network receives an AIoT NAS message with a message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message.

If the AIoT device receives an AIoT NAS message with a message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall return a STATUS message with cause #97 "message type non-existent or not implemented".

NOTE:	A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

### 6.4	Non-semantical mandatory information element errors

### 6.4.1	Common procedures

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error;

is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall:

-	if the message is not one of the messages listed in clause 6.4.2, ignore the message and return a STATUS message with cause #96 "invalid mandatory information"; and the network shall either:

a)	try to treat the message (the exact further actions are implementation dependent); or

b)	ignore the message.

### 6.4.2	Specific AIoT procedures

The following procedures shall apply when the AIoT device encounters an error with a mandatory information element in an AIoT NAS message:

a)	If the message is a READ COMMAND, then a READ COMMAND REJECT message with cause #96 "invalid mandatory information", shall be returned; and

b)	if the message is a WRITE COMMAND, then a WRITE COMMAND REJECT message with cause #96 "invalid mandatory information", shall be returned.

### 6.5	Unknown and unforeseen IEs in the non-imperative message part

### 6.5.1	IEIs unknown in the message

The AIoT device shall ignore all IEs with unknown IEI in a message.

The network shall take the same approach.

### 6.5.2	Out of sequence IEs

The AIoT device shall ignore all out of sequence IEs in a message.

The network should take the same approach.

### 6.5.3	Repeated IEs

If an information element with format T, TV or TLV is repeated in a message, the AIoT device shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element.

The network should follow the same procedures.

### 6.6	Non-imperative message part errors

### 6.6.1	General

This category includes syntactically incorrect optional IEs.

### 6.6.2	Syntactically incorrect optional IEs

The AIoT device shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

The network shall take the same approach.

### 6.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the AIoT device shall perform the foreseen reactions of the procedural part of clause 5. If, however no such reactions are specified, the AIoT device shall ignore the message.

The network should follow the same procedure.

### 7.1.5	Read command reject

#### 7.1.5.1	Message definition

The READ COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of reading the AIoT data from the memory.

See table 7.1.5.1-1.

Message type:	READ COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.5.1-1: READ COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.3 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.9 | M | V | 4 |
|  | Read command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Read cause | Cause 7.2.8 | M | V | 1 |

### 7.1.8	Write command reject

#### 7.1.8.1	Message definition

The WRITE COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of writing the AIoT data from the memory.

See table 7.1.8.1-1.

Message type:	WRITE COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.8.1-1: WRITE COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.3 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.9 | M | V | 4 |
|  | Write command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Write cause | Cause 7.2.8 | M | V | 1 |

x

### 7.1.11	STATUS

#### 7.1.11.1	Message definition

The STATUS message is sent by the AIoT device to the AIOTF to report certain error conditions listed in clause 6.

See table 7.1.11.1-1.

Message type:	STATUS

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.11.1-1: STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Status message identity | Message type 7.2.2 | M | V | 1 |
|  | Status cause | Cause 7.2.8 | M | V | 1 |

### 7.2.8	Cause

The purpose of the cause information element is to indicate the cause of the failure of the read or write operation.

The cause information element is coded as shown in figure 7.2.8-1 and table 7.2.8-1.

The cause is a type 3 information element with length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.8-1: Cause information element

Table 7.2.8-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | Command type specific parameters invalid |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | Error, unspecified |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | Low energy |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| All other values are spare and if received shall be interpreted as "Error, unspecified". |  |  |  |  |  |  |  |  |  |

#### A.1	Causes related to command procedure

Cause #x1 – TBD

This cause is used to indicate that the requested read or write command is not supported by the AIoT device.

Cause #1 – Command type specific parameters invalid

This cause is used to indicate that the command type specific parameters (e.g. the offset to read application data from the AIoT device user memory, the length of the application data to read, the offset where to write the application data, or the length of the application data to write) is invalid.

Cause #2 – Error, unspecified

This cause is used to indicate that the requested read or write command was not executed successfully.

Cause #3 – Low energy

This cause is used to indicate that the write command procedure cannot be implemented because the energy will be run out.

Cause #96 – Invalid mandatory information

This cause is used to indicate that the equipment sending this cause has received a message with a non-semantical mandatory IE error.

Cause #97 – Message type non-existent or not implemented

This cause is used to indicate that the equipment sending this cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this cause