3GPP TSG-RAN WG2 Meeting #132	Tdoc R2-2508414

Dallas, USA, Nov. 17th-21st

Source:	Panasonic

Title:	RRC Signaling Framework with more close integration with the slices

Agenda item:	10.3.2.2	RRC Structure and (re)configuration

Document for:	Discussion and Decision

- Introduction:

Lessons learned from 5G show that generalized signaling introduces inefficiencies and complexity. 6G must support diverse scenarios (URLLC, XR, NTN, IoT) with a lean, modular, and forward-compatible signaling framework that minimizes overhead while enabling fast service adaptation.

The slice was supported from 5G but there are some areas for the improvement. Here we provide RRC Signaling Framework with more close integration with the slices considering the above lessons learned from 5G.

- Discussion:

Limitations of 5G RRC Signaling
Current 5G RRC signaling design is based on a generalized parameter set applied across all services. While this approach simplifies initial standardization, it introduces several inefficiencies and limitations when adapting to diverse service requirements in 6G:

- Redundant Signaling and Overhead
Example: During RRCSetup or RRCReconfiguration, the network transmits full parameter sets (e.g., DRX, mobility, HARQ, security) even when most values are standard defaults. For Industrial IoT, only a few parameters differ, yet the entire configuration is signalled.

- Lack of Scenario-Specific Optimization
Example: URLLC would require ultra-low latency and high reliability and XR would require specific configuration, but initial configuration uses generic DRX and timer settings. Additional RRCReconfiguration messages are often needed to meet scenario specific requirements, increasing latency and signaling overhead.

- Inefficient Handling of Multiple Services
Example: When a UE switches from eMBB to XR or URLLC, a full RRCReconfiguration is required instead of applying a pre-defined scenario profile, leading to unnecessary signaling and delay.

- No Label-Based Parameter Association
Example: A UE requesting eMBB slice cannot simply indicate “apply eMBB profile.” The same can be said for URLLC. Instead, the network must explicitly signal all eMBB or URLLC-specific parameters.

- Limited Flexibility for Future Scenarios
Example: New use cases such as XR, holographic telepresence would require additional parameters and signaling procedures, making the system complex and less scalable.

These limitations highlight the need for a more flexible and efficient RRC signaling framework in 6G. A unified yet modular signaling structure is needed to avoid excessive options and enable efficient scenario-specific configuration.

Proposed Framework

To address these challenges, we propose a scenario-oriented RRC signaling framework that introduces the following concepts:

- Modular signaling structure composed of:

- Generic part: Common baseline configuration applicable across scenarios.

- Scenario-oriented part: Additional or overriding parameters tailored to specific scenarios.

- Parameter set association with scenario identifiers
Scenario-oriented parameter sets can be linked to identifiers such as Slice/Service Type (SST), QoS profiles, or other scenario-indicative information. This enables efficient signaling by applying predefined parameter sets rather than transmitting all parameters individually.

- Hierarchical overwrite mechanism
Generic part values can be overridden by scenario-oriented values, and scenario-oriented values can be further refined by dedicated signaling when necessary. This layered approach minimizes redundant signaling while maintaining flexibility.

- Label-based signaling
Using SST as a label for scenario-oriented parts allows efficient mapping between service requirements and RRC configurations. The standardised labels like SST ensures consistent interpretation across networks and devices.

The potential advantages of the proposed approach are as follows.

Advantages of the Proposed Approach

- Reduced Signaling Overhead: Predefined parameter sets, and scenario labels minimize repetitive transmission of full configurations.

- Faster Service Adaptation: Switching between scenarios (e.g., eMBB → URLLC) can be achieved by referencing a scenario profile instead of sending complete RRCReconfiguration messages.

- Improved Energy Efficiency: Less signaling reduces UE processing and radio activity, which is critical for energy-constrained devices.

- Enhanced Flexibility and Scalability: Supports future scenarios (e.g., XR, NTN) without major redesign, as new parameter sets can be added without impacting the generic structure.

- Better Interoperability: Using standardized labels like SST ensures consistent interpretation across networks and devices.

- Conclusion:

Observations 1: 6G will support diverse scenarios (URLLC, XR, NTN, IoT), which require more flexible and efficient RRC signaling than the current 5G design.

Observations 2: The existing generalized RRC configuration approach in 5G leads to inefficiencies such as redundant signaling, lack of scenario-specific optimization, and increased overhead when adapting to diverse service requirements.

Observations 3: Introducing a scenario-oriented signaling framework, leveraging predefined parameter sets and scenario labels (e.g., SST), can significantly reduce signaling overhead, improve adaptability, and enable faster configuration for diverse use cases.

Proposal 1: RAN2 should study and discuss a scenario-oriented RRC signaling approach that enables efficient configuration tailored to diverse 6G use cases.

Proposal 2: RAN2 should consider an RRC signaling structure composed of a “Generic part” and a “Scenario-oriented part”, allowing common baseline settings with scenario-specific extensions.

Proposal 3: RAN2 should evaluate the use of Slice/Service Type (SST) as a label for the scenario-oriented part of RRC signaling, facilitating association between parameter sets and service requirements.