3GPP TSG-RAN WG2 Meeting #132	R2-2508416

Dallas, USA, Nov. 17th-21st

Agenda Item:	10.4

Source:	Amazon

Title:	Overview of 6GR NTN mobility

Document for:	Discussion and Decision

# Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting (RP-251881) [1]. The main objectives are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

This document provides an overview of 6GR NTN mobility scenarios and requirements.

# NTN Mobility Scenarios and Requirements

The 6G study item should consider different mobility scenarios for NTN, including intra- and inter-frequency handovers, mobility between NTN and TN networks, and even the possibility of multi-connectivity between NTN and TN networks for the same device when coverage overlaps or partially overlaps as shown in the figure below.

Figure 1. NTN/TN overlapping and non-overlapping coverage scenarios

It is desirable to harmonize NTN and TN mobility procedures where possible to simplify UE and network implementations, including the use of measurement reports, time/location-based triggers, and cell-selection criteria.

Proposal 1: As part of the 6GR study, consider how to harmonize NTN and TN mobility procedures where possible to simplify UE and network implementations, including the use of measurement reports, time/location-based triggers, and cell-selection criteria.

Mobility Procedures for 6GR NTN

Mobility procedures for NTN can be based on the same framework as for TN mobility, however there are several key differences between NTN and TN systems, since in the context of NTN an additional node is introduced, the NTN Gateway (GW), which is a Transport Network Layer node. The satellite and the NTN GW communicate over the feeder link using the Satellite Radio Interface (SRI). This introduces the question of where the different radio access network functions should be deployed as shown in the figure below.

Figure 2. NTN Mobility Example

In one extreme, all of the radio access and network functions may be deployed on the ground where the access link is forwarded by the NTN GW and satellite to the user (e.g. a transparent architecture). At the other extreme, all of the network and radio access functions are deployed on the satellite (e.g. a regenerative architecture).

The choice of architecture will dictate how radio-related functions for one or multiple cells on the ground are managed, including admission control, connection establishment, user-plane and control-plane routing between the 5G-RAN and the 5G core, and radio resource management (RRM) functions such as mobility for CONNECTED UEs and IDLE mode cell (re)selection.

As a result, both L3 triggered mobility and lower-layer (e.g. L1/L2) mobility procedures should be considered for NTN including cases where the same or different cell ID is used across the source and target satellites.

Proposal 2: The 6GR study should consider L3 triggered and lower-layer (e.g. L1/L2) triggered NTN mobility procedures.

Additionally, the mobility procedure can be triggered based on different measurements/conditions/events evaluated at the UE and/or network. Which measurements or conditions are to be used in the system can be determined based on the specific mobility scenario and deployment architecture and the 6GR design should support network configuration of these parameters (e.g. in a broadcast and/or dedicated manner).

Proposal 3: The 6GR study should consider NTN mobility procedures which are at least based on:

- RRM measurement-based events

- Time-based trigger conditions

- Location-based trigger conditions

In addition, the 6GR study should consider support for conditional handovers which can be performed in a RACH-less manner. This is advantageous for NTN systems to trigger the handover procedure at the UE with minimal signaling overhead, by assuming sufficiently accurate timing/location information is available (e.g. via GNSS).

Proposal 4: The 6GR study should consider RACH-less and conditional handover procedures.

UE Assistance Information for 6GR NTN Mobility

It is important to consider necessary enhancements for NTN UEs due to the specific challenges of NTN networks, such as very large propagation delays, Doppler variations across source and target satellites, discontinuous coverage etc. This can be mitigated through the use of assistance information (e.g. satellite ephemeris, propagation delay, timing offset parameters) provided in dedicated or system broadcast messages which can be used for handovers, intra-RAT and inter-RAT cell selection (e.g. to/from 5G or 6G), or mobility to/from NTN and TN networks.

Proposal 5: As part of the 6GR study, consider necessary UE assistance information (e.g. satellite ephemeris, propagation delay, timing offset parameters) provided in dedicated or system broadcast messages which can be used for handovers, intra-RAT and inter-RAT cell selection (e.g. to/from 5G or 6G), or mobility to/from NTN and TN networks.

# Conclusion

In this contribution, we made the following proposals:

Proposal 1: As part of the 6GR study, consider how to harmonize NTN and TN mobility procedures where possible to simplify UE and network implementations, including the use of measurement reports, time/location-based triggers, and cell-selection criteria.

Proposal 2: The 6GR study should consider L3 triggered and lower-layer (e.g. L1/L2) triggered NTN mobility procedures.

Proposal 3: The 6GR study should consider NTN mobility procedures which are at least based on:

- RRM measurement-based events

- Time-based trigger conditions

- Location-based trigger conditions

Proposal 4: The 6GR study should consider RACH-less and conditional handover procedures.

Proposal 5: As part of the 6GR study, consider necessary UE assistance information (e.g. satellite ephemeris, propagation delay, timing offset parameters) provided in dedicated or system broadcast messages which can be used for handovers, intra-RAT and inter-RAT cell selection (e.g. to/from 5G or 6G), or mobility to/from NTN and TN networks.