3GPP TSG-RAN2 #132	R2-2508439

# Dallas, USA, Nov. 13 – Nov. 17, 2025

Agenda item:	10.3.1.3 (FS_6G_Radio)

Source:	LG Electronics

Title: 	Consideration on Scheduling, retransmissions and uplink scheduling information in 6G

Document for:	Discussion and decision

1.	Introduction

In RAN2#131bis meeting [1], followings are agreed related to scheduling, retransmission operation in 6G Radio.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This document further discusses our views on 6G scheduling and retransmission.

2.	Discussion

2.1	UL grant for 6G

In 5G NR, the UL grant reception and the LCP procedure to allocate the UL-SCH resources is performed per MAC entity. In detail, when the MAC entity receives a UL grant, LCP procedure is performed among all the selected logical channels in the MAC entity. In the LCP procedure in NR, the MAC entity allocates the UL-SCH resources of the received uplink grant to transmit the data of logical channels, ensuring the PBR for each logical channel. In Rel-19 XR, adjustment of logical channel priority is introduced in order to ensure the transmission of delay-critical data for XR traffic.

On the other hand, as studied in TR 38.835 [2] and TR 38.838 [3], one XR service may have multiple traffic streams conveying different types of data (e.g., pose/control, scene, video, audio, etc), which can be mapped to difference QoS flows based on QoS requirements of each stream (e.g., reliabilities, delay budget). In order to support the various traffic streams with different traffic characteristics (e.g., periodicity, jitter, PDB), the network allocates the uplink resource to meet the QoS requirements of each stream based on the traffic characteristics for the corresponding stream.

However, even though the network allocates an uplink to serve the data of the traffic stream, current LCP procedure performed per MAC entity does not ensure the transmission of the specific QoS flow. Specifically, even though the network knows the amount of data for a specific data stream and allocates an uplink grant to meet the QoS requirement for the specific stream, the allocated uplink grant would be used to transmit the data of other traffic streams as well, since the LCP procedure is processed for all selected logical channels in the MAC entity. Then, the transmission of data from the corresponding traffic streams is not guaranteed, which cause possible degradation of XR service due to failure of transmission of data within the delay budget of the corresponding data.

Observation 1. LCP procedure in 5G NR is performed among all the selected logical channels in the MAC entity, causing inflexibility to schedule the uplink grant for each traffic streams in XR service.

Given that XR service is one of killer applications in 6G as an immersive communication, the various characteristics of the XR traffic should be carefully considered in 6G L2 scheduling. Therefore, in order to satisfy QoS requirements of the various traffic streams in an XR service, the 6G L2 scheduling should have more flexibility to allocate an uplink grant with fine control of each stream, e.g., per-LCG UL grant or per-LCH UL grant.

Fig. 2.1.1. Example of fine-grained scheduling; per-LCG UL grant

Proposal 1. Study UL grant enhancement with finer granularity, e.g., per-LCG UL grant, per-LCH UL grant

Further, in 5G NR, when the MAC entity receives a UL grant, the MAC entity performs the LCP procedure to allocate the UL-SCH resource to transmit the corresponding MAC CE and user data, which causes the HOL (Head-of-Line) blocking issue between the MAC CE and user data. In detail, as described in our companion paper [4], the delay-critical data transmission may be blocked by the MAC CE transmission, since most of MAC CEs are prioritized over the user data during LCP procedure. To avoid such problem, it is possible to adjust logical channel priority for logical channel carrying delay-critical data to be transmitted prior to MAC CEs, as discussed in Rel-19 XR. However, in this case, important MAC CE may be blocked by the delay-critical data, and the important MAC CE may not be delivered to the network in time.

In order to allow the network controllability to meet the QoS requirements for XR traffic and to ensure the transmission of MAC CEs, the separation of UL grant for MAC CE and user data can also be considered. If UL grants dedicated to MAC CE or user data are provided, this will offer a knob to schedule either urgent control signal or time-critical user data on the transport channel and the physical layer.

Proposal 2. Study separated UL grant for MAC CE and L2 user data in 6G, e.g.

- UL grant dedicated to MAC CE

- UL grant dedicated to user data

2.2	CG enhancements

In RAN2#131bis meeting [1], it was agreed to support the dynamic uplink grant and configured uplink grant in 6G, and further discuss whether the UL latency issue should be addressed in 6G.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

In 5G NR, the UE triggers BSR/DSR based on the UL data arrival and the remaining time of the delay budget. After triggering BSR/DSR, if there is no UL-SCH resource to transmit the corresponding DSR/BSR MAC CE, the UE triggers SR to request the uplink resource for transmission. Even though the SR is successfully transmitted, the network does not know the amount of UL data, so the network only allocates the UL resource to transmit the DSR/BSR MAC CE. On the other hand, if there is no SR resource or if the SR transmission is failed, Random Access procedure is initiated to transmit the BSR/DSR MAC CE, causing additional delay to transmit the UL data.

As a result, at least initial SR transmission delay and several round-trip times are needed to transmit the UL data, due to SR  DSR/BSR  data transmission procedure. This long latency for UL transmission procedure causes user experience degradation in XR service with delay-sensitive traffic, since the user data may not be transmitted within the delay budget if SR is triggered to transmit DSR/BSR.

Fig. 2.2.1. Example of UL scheduling delay due to SR/BSR/DSR latency

Observation 2. UL scheduling based on SR/DSR/BSR procedure causes long latency, which causes degradation of user experience for use cases requiring low latency transmission.

Although the configured grant can be used to resolve the UL scheduling delay caused by the SR/DSR/BSR procedure, it is not efficient for network resource utilization, since the network needs to pre-allocate the UL resource for the specific UE. In addition, this pre-allocation of UL grant is not suitable for dominant services in 6G (e.g., immersive communication, generative AI traffic), where the data burst arrives with irregular periodicities and/or various data volumes. Therefore, in order to address the UL scheduling latency issue for XR/AI traffic, configured grant dedicated for one UE is not sufficient and new mechanism is needed to reduce the BSR/SR/RA latency.

Observation 3. Configured grant in 5G NR does not resolve the UL latency issues, since it is not suitable for dominant services in 6G (e.g., immersive communication, generative AI traffic) with variable data burst size and irregular periodicity.

During the RAN2#131bis meeting [1], allocating contention-based UL resource (e.g., contention-based CG) was discussed, as one method to resolve the UL scheduling delay due to BSR/SR/RA procedure. In our understanding, it would be beneficial to reduce the UL latency due to SR  BSR procedure while keeping the UL resource efficiency from the network side. Moreover, it can be used not only in the connected mode but also in inactive mode, e.g., for fast RRC state transition (e.g., RACH-less transition) or data transfer (e.g., CG-SDT).

Proposal 3. Study contention-based CG in order to address the UL latency due to BSR/SR/RA procedure.

2.3	Enhancements on scheduling information

In 5G NR, Buffer Status Report (BSR) is transmitted by a UE in order to provide the information about UL data volume in the MAC entity. In the BSR, the amount of data of all logical channels for each LCG is reported. In addition, based on the normative work in Rel-18 XR WI, Delay Status Report (DSR) is introduced in order to provide the delay information of UL data, in order to support the XR traffic with stringent delay requirement.

However, current BSR and DSR procedure in 5G NR is performed separately, causing inefficiency issue. In detail, the DSR includes the information on UL data volume associated with the remaining time threshold, which can be duplicated by the BSR procedure. Since the information included in BSR and DSR is quite overlapped, the duplicated transmission of DSR and BSR causes resource inefficiency. Further, given that the BSR MAC CE has higher priority than the user data, if the network allocates the uplink grant for delay-critical data after transmitting DSR MAC CE, duplicated transmission of BSR MAC CE would cause HOL blocking issue of user data, which may degrade the user experience of XR service.

During the Rel-18 XR, it was discussed on whether DSR MAC CE can be defined as an extension of BSR MAC CE, but it was not agreed due to the complexity of mixing the existing BSR and the newly defined DSR mechanism. However, given that there is no complexity issue for 6G study, it can be considered to combine the DSR and BSR to report the data volume information and delay information in one MAC CE.

Observation 4. Since the information in BSR and DSR are quite overlapped, duplicated transmission of DSR and BSR in NR causes resource inefficiency.

In addition, current BSR and DSR cannot sufficiently handle the various characteristics of XR traffics. Specifically, while XR service may include different types of QoS flows (e.g., pose/control, scene, video, audio, and other data), but the network cannot know the amount of data or the amount of delay-critical data for the specific QoS flow since the BSR and DSR in 5G NR reports the amount of data for all logical channels in the LCG.

Observation 5. In NR, UL data volume in DSR and BSR is reported per LCG, which causes inefficiencies for network to schedule the uplink resource for a specific data steam.

As mentioned above, the various characteristics of the XR traffic should be carefully considered in 6G L2 scheduling, since it will be one of the dominant applications in 6G. In this sense, both BSR and DSR should be supported with single MAC CE, and the data volume information should be reported with finer granularity in order to allow the uplink grant scheduling based on the knowledge of the amount of data of each QoS flow.

Proposal 4. Study combined BSR/DSR for XR traffic with finer granularity, e.g. per-QoS flow or per-LCH.

In 5G NR, the data volume information in BSR and DSR is reported using BS table index with exponential function. Although this exponential code point for BS table provides same accuracy ratio for all code points, the absolute step size is different for each code point. This BS table indexing causes large quantization error for large-sized data, which cause inefficiency in XR traffic with large-sized data with stringent delay requirement.

In Rel-18 XR, in order to reduce the quantization error for XR traffic, the additional BS table is introduced to supports up to 750KB based on the ratio between maximum link rate (60Mbps) and minimum frame rate (15 fps). However, given that higher display resolution is expected in the future XR applications and other use cases also needs to be considered for immersive communication (e.g., metaverse [5]), BS table defined in 5G NR would not be sufficient to cover all traffics for XR/immersive communication expected in 6G.

In our understanding, defining fixed additional BS table(s) does not ensure to report all the possible amount of data of traffic in various use cases of XR/immersive communication with small quantization error, since the data rate will be further increased based on the display resolution and frame rate, depending on use cases. Therefore, in order to support the reporting of the larger data volume for traffics with reasonable quantization error, the additional mechanism to indicate the buffer size in a BS table should be studied considering the various data rates in different use cases for immersive communications.

Proposal 5. Study a mechanism to indicate buffer size with small quantization error

2.4	Fast MAC PDU generation/processing

In 5G NR, when the UE receives a UL grant from the network, the UE performs the LCP procedure to construct a MAC PDU to fit the size of the UL grant. During the LCP procedure, the UE allocates UL resources to logical channels and MAC CEs based on a predefined priority. Based on the allocated UL resource for each component, the UE constructs a MAC subPDU for each component, and each MAC subPDU consists of a MAC subheader and a MAC SDU, MAC CE, or padding.

As explained above, since the UE constructs a MAC PDU after receiving the UL grant, it takes much time to allocate UL resources to each logical channel and construct a MAC PDU according to the allocated UL resource due to the various restrictions and priorities. It means that the network should schedule the UL grant considering this significant processing delay. In addition, if the MAC CE security is introduced in 6G, the processing delay would be even more deteriorated.

In 6G, one of the key requirements is HRLLC (Hyper-Reliable and Low-Latency Communication), and the HRLLC traffic may have a latency requirement up to 0.1ms. However, even in 6G, to support the fairness and prioritization among the logical channels, the LCP procedure would need to be supported. Thus, since LCP is a real-time operation with very tight processing timeline, the latency requirement for HRLLC may not be satisfied.

In order to support the HRLLC traffic in 6G, a possible way is that the UE pre-generates multiple MAC PDUs based on the expectation, and indicates the list of expected MAC PDU size to the network. When the network receives the list of expected MAC PDU size, the network selects one of the expected MAC PDU size based on the network resource and radio condition, and provides UL grant to the UE where the size of UL grant is fit to the selected MAC PDU size.

Fig. 2.4.1. Example of fast MAC PDU generation; based on UE expectation

In other way, the UE can pre-generate a MAC PDU based on the network assistance information. For example, the UE receives a UL grant containing the next UL grant size information. In this case, the UE generates a first MAC PDU transmitted on the UL grant, and the UE pre-generates a second MAC PDU based on the next UL grant size information.

Fig. 2.4.2. Example of fast MAC PDU generation; based on network assistance information

Proposal 6. Study a mechanism to pre-generate a MAC PDU before receiving the UL grant based on the UE expectation or the network assistance information.

2.5	Fast ARQ retransmission

In RAN2#131bis meeting [1], it was discussed on ARQ latency in 5G NR and agreed to further study how to reduce the ARQ retransmission delay, e.g., with the coordination with HARQ status.

Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

As in our companion paper [4], error correction through ARQ procedure is still needed for lossless transmission, e.g., when the high reliability is required. However, ARQ retransmission procedure defined in 5G NR causes additional latency, since the STATUS PDU to trigger ARQ retransmission is transmitted based on the t-Reassembly timer expiry or polling procedure. This ARQ latency is not suitable for many use cases for immersive communication (e.g., XR services), where the high reliability with low latency transmission is required. Although remaining time-based retransmission and polling procedure is introduced in Rel-19 XR, it is not sufficient since it triggers only one RLC retransmission for delay-critical RLC SDU.

Observation 6. ARQ retransmission procedure in 5G NR delays the recovery of transmission failure, which is not suitable for use cases requiring low latency transmission with high reliability.

As discussed in last meeting, ARQ retransmission procedure can be improved based on HARQ transmission status. In detail, if the UE detects HARQ failure for a specific SDU or SDU segment, the corresponding SDU or SDU segment can be retransmitted via ARQ procedure, without waiting STATUS PDU from peer L2 entity. In our understanding, utilizing HARQ status for ARQ retransmission procedure enables the fast and efficient recovery of the SDU transmission failure without STATUS PDU, ensuring the lossless transmission with low latency in XR/immersive communication.

Regarding the details on the coordinating ARQ and HARQ procedure, separated discussion is needed for UL and DL. For UL, the UE may not know about the HARQ status for UL data since gNB does not provide any feedback on HARQ decoding status in NR. Instead, the gNB allocates an uplink grant for new transmission or for retransmission. In other words, even though gNB failed to decode the UL data, the gNB may give up the HARQ procedure for this MAC PDU and choose to allocate an uplink grant for new transmission for this HARQ process ID. In this case, the gNB may provide an explicit indication on HARQ decoding failure in order to allow the ARQ retransmission based on HARQ failure.

For DL, based on the HARQ feedback information provide from UE side, the gNB may perform HARQ retransmission or trigger ARQ retransmission of SDU or SDU segment of corresponding MAC PDU. One issue for ARQ retransmission based on HARQ status for DL transmission is whether/how to handle NACK-to-ACK error, since it is possible that gNB determines that HARQ transmission is successful, even though the actual HARQ decoding is failed at the UE side. However, in our view, it depends on reliability of HARQ feedback, so it is too early to decide whether NACK-to-ACK error case should also be handled for fast ARQ retransmission in RAN2.

Proposal 7. Support fast ARQ retransmission based on HARQ failure information.

3. Conclusion

Based on the above discussion, we present the following observations and proposals:

Observation 1. LCP procedure in 5G NR is performed among all the selected logical channels in the MAC entity, causing inflexibility to schedule the uplink grant for each traffic streams in XR service.

Proposal 1. Study UL grant enhancement with finer granularity, e.g., per-LCG UL grant, per-LCH UL grant

Proposal 2. Study separated UL grant for MAC CE and L2 user data in 6G, e.g.

- UL grant dedicated to MAC CE

- UL grant dedicated to user data

Observation 2. UL scheduling based on SR/DSR/BSR procedure causes long latency, which causes degradation of user experience for use cases requiring low latency transmission.

Observation 3. Configured grant in 5G NR does not resolve the UL latency issues, since it is not suitable for dominant services in 6G (e.g., immersive communication, generative AI traffic) with variable data burst size and irregular periodicity.

Proposal 3. Study contention-based CG in order to address the UL latency due to BSR/SR/RA procedure.

Observation 4. Since the information in BSR and DSR are quite overlapped, duplicated transmission of DSR and BSR in NR causes resource inefficiency.

Observation 5. In NR, UL data volume in DSR and BSR is reported per LCG, which causes inefficiencies for network to schedule the uplink resource for a specific data steam.

Proposal 4. Study combined BSR/DSR for XR traffic with finer granularity, e.g. per-QoS flow or per-LCH.

Proposal 5. Study a mechanism to indicate buffer size with small quantization error

Proposal 6. Study a mechanism to pre-generate a MAC PDU before receiving the UL grant based on the UE expectation or the network assistance information.

Observation 6. ARQ retransmission procedure in 5G NR delays the recovery of transmission failure, which is not suitable for use cases requiring low latency transmission with high reliability.

Proposal 7. Support fast ARQ retransmission based on HARQ failure information.

4. Reference

[1] Draft Report of 3GPP TSG RAN WG2 meeting #131bis, Prague, Czech Republic, 13 - 17 October, 2025

[2] TR 38.835 v.18.0.1

[3] TR 38.838 v.17.0.0

[4] R2-2508432 Considerations on required functionalities and design objectives for 6G protocol stack, LG Electronics.

[5] TS 22.156 v.19.1.0