3GPP TSG-RAN WG2#132	R2-2508459

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.2.1

Title:	Discussion on UE capability aspects in 6G

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

In RAN2#131bis meeting, some agreements for the next meeting were made on UE capability aspects as follows:

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities. Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this contribution, we would like to provide our views on improvements for UE capability signalling and reporting.

# Discussion

## Dynamic UE capability change in connected mode

### Use case of dynamic UE capability change

In 5G NR, UAI (UE assistance information) message can inform the NW of assistance information for appropriate reconfiguration when the UE itself realizes to resolve certain conditions like overheating, MUSIM, IDC, power saving and so on. After UAI reporting, the UE is expected to receive the reconfiguration message according to the indicated information.

Observation 1:	In NR, UAI can be used to inform of a recommended configuration from UE perspective.

However, in RAN2#131bis meeting, several companies proposed it is beneficial to update the reported UE capability without leaving RRC connected mode, instead of UAI reporting. Because UAI is not defined as a request but just indication, thus it might be ignored by the NW. In addition, in 6G CP session, to avoid the RRC establishment, it was proposed that the UE is allowed to keep/apply the good configuration (namely UE-decided RRC configuration) if a part of configuration from the NW is considered as invalid/inappropriate. These proposals are related to each other, and it looks similar functions. For example, if the UE in connected mode realizes and indicates the necessity of capability change according to the situations like overheating, power saving or IDC problem, the NW initiates RRC reconfiguration procedure immediately based on the reported UE capability. While the NW wants to reconfigure the current UE configuration, however, it is not suitable from the UE perspective, the UE may keep/apply a proper configuration regardless of NW configurations.

UAI can also include additional assistance information which cannot be indicated by the UE capability itself. For example, UE preferred DRX cycle for power saving purpose, affected frequency for IDC purpose or preferred RRC state for MUSIM purpose is not included in the UE capability message. Hence, even if dynamic UE capability change is adopted, UAI-like mechanism is still needed to indicate additional assistance information for multiple purposes.

Observation 2:	Understand UAI, dynamic UE capability change in connected mode and UE-decided RRC configuration aim to avoid an inappropriate RRC (re)configuration.

Observation 3:	Even if dynamic UE capability change is adopted, it is expected that UAI-like mechanism is needed to indicate additional assistance information.

Proposal 1:	For dynamic UE capability change, RAN2 to study how the UE indicates a preferred configuration which is not defined in the UE capability message.

### Expected NW impacts on dynamic UE capability change

If the dynamic UE capability update is allowed in 6G, there are several issues on the NW side. In our understanding, the UE capability is temporally changed therefore it should not be stored in the CN. Otherwise, the UE has to remember the last reported UE capability and indicate the latest one during the RRC setup procedure if temporary reason has already been resolved or changed. It may cause to increase the UE complexity and a signalling overhead in Uu and NG interfaces.

Observation 4:	In typical, the UE capability update in connected mode is temporally used.

Proposal 2:	Even if the dynamic UE capability is adopted, the stored UE capability in the CN should not be replaced.

Considering the current UAI procedure, the UE would repeat to transmit the UAI message to the new gNB after successful HO, if the UAI has been sent during the last 1 second. Because the target gNB may be unaware if the source gNB receives the UAI message after initiating HO preparation. Thus, it would be natural to reuse the same principle.

Proposal 3:	If the dynamic UE capability is adopted, for HO, RAN2 to consider the similar concepts are applied to the inter-node RRC procedures:

- The reported dynamic UE capability is delivered to the target gNB during HO preparation procedure

- The RRCReconfiguration message in HO command is generated by the target gNB based on the reported dynamic UE capability

Proposal 4:	If the dynamic UE capability is adopted, the UE may repeat to send the dynamic UE capability change message to the target gNB after successful HO as in UAI.

# Conclusion

In this paper we presented some views on UE capability signalling and reporting aspects and have come up with the following observations and proposals:

Observations:

Observation 1:	In NR, UAI can be used to inform of a recommended configuration from UE perspective.

Observation 2:	Understand UAI, dynamic UE capability change in connected mode and UE-decided RRC configuration aim to avoid an inappropriate RRC (re)configuration.

Observation 3:	Even if dynamic UE capability change is adopted, it is expected that UAI-like mechanism is needed to indicate additional assistance information.

Observation 4:	In typical, the UE capability update in connected mode is temporally used.

Proposals:

Proposal 1:	For dynamic UE capability change, RAN2 to study how the UE indicates a preferred configuration which is not defined in the UE capability message.

Proposal 2:	Even if the dynamic UE capability is adopted, the stored UE capability in the CN should not be replaced.

Proposal 3:	If the dynamic UE capability is adopted, for HO, RAN2 to consider the similar concepts are applied to the inter-node RRC procedures:

- The reported dynamic UE capability is delivered to the target gNB during HO preparation procedure

- The RRCReconfiguration message in HO command is generated by the target gNB based on the reported dynamic UE capability

Proposal 4:	If the dynamic UE capability is adopted, the UE may repeat to send the dynamic UE capability change message to the target gNB after successful HO as in UAI.

# References

- RP-252912, Revised SID: Study on 6G Radio