3GPP TSG-RAN WG2#132	R2-2508460

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.2.2

Title:	Discussion on 6G TN/NTN integration

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

In the RAN2 #131bis meeting [2], the following guideline was indicated regarding TN/NTN integration:

⇒ For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution, we would like to provide our views on TN/NTN integration.

# Discussion

## 6G NTN essential requirements

### Background - 5G NTN features

In 5G NTN, several NTN specific features have been discussed and specified to access/connect to the satellites from Rel-17 to Rel-19. Table 1 shows the major features specified in NR NTN.

Table 1 – 5G NTN features

| Main categories | Subcategories |
| --- | --- |
| Compensation for long propagation delay | UL timing alignment (Kmac) |
|  | TA reporting |
|  | Disabling HARQ |
|  | Autonomous timing adjustment (RA, DRX) |
|  | Offset for UL/DL scheduling (Koffset) |
| Idle mode mobility | Multiple TAC |
|  | NTN specific cell barring |
|  | TN/NTN area assistance information (This applies to measurements/connected mode mobility) |
| Measurements | SMTC timing adjustments |
|  | Time-based measurements |
|  | Location-based measurements |
| Connected mode mobility | Conditional Event A4 |
|  | Time-based L3 mobility (Event Tx) |
|  | Location-based L3 mobility (Event Dx) |
|  | Satellite switch with resync |
|  | RACH-less HO |
| Coverage | UL/DL repetition |
|  | PUSCH DMRS bundling |

### Applicable scenarios

To identify the essential requirements, it is important what scenarios should be supported for 6G NTN. In NR-NTN, as a satellite type, both NGSO and GSO have been supported, since the current satellite operators are operating different satellite types according to the different services. Therefore, it doesn’t make sense to select either one of them for a baseline of 6G NTN architecture. In this sense, 6G NTN should support both GSO and NGSO satellites.

For payload type, both transparent and regenerative payload (full on board) are already supported in 5G NR-NTN. The transparent payload does not require installing a base station on the satellite. On the other hand, the regenerative payload may reduce propagation delay between the base station and the UE compared to the transparent payload. The transparent payload may be operated for low-end devices. The regenerative payload may be adopted to support higher data rate communication for non-TN covered area. From RAN2 perspective, it is possible to support both of payload types because it is transparent from UE.

Observation 1:	The following NTN architectures are assumed in 6G NTN in Day1:

- both GSO and NGSO satellites.

- both transparent and regenerative payload types.

The main difference between TN and NTN is the propagation delay. In TN, the ISD of Uma deployment is up to 500 meters, even for Rma deployment, the maximum ISD is 5,000 meters, which are far shorter than the distance between UE and a satellite, therefore, it is not easy to use a common solution to accommodate the delay to both TN and NTN cells. In NR NTN, the almost features shown in table 1 are specified to resolve the long propagation delay to/from NTN. It means the features to compensate for the long propagation delay should also be key issues in 6G NTN.

Observation 2:	The features to compensate for the long propagation delay to/from the NTN are necessary as in NR.

Figure 1 indicates three 6G NTN and TN scenarios, i.e., a) 6G NTN cell is overlapped with 6G TN cells, b) 6G NTN cell is not overlapped with 6G TN cells and c) 6G NTN cell is partially overlapped with 6G TN cells.

| a) 6G NTN cell is overlapped with 6G TN cells | b) 6G NTN cell is not overlapped with 6G TN cells |
| --- | --- |
| c) 6G NTN cell is partially overlapped with 6G TN cells |  |

Figure 1 - NTN cell deployment models in 6G

Option a) illustrates a deployment model where 6G NTN cell is fully overlapped with 6G TN cells. In option a), handheld device (like smartphone) may not use NTN link, but sensing/monitoring devices may connect to the NTN cell even in the TN cell coverage. Also, NTN link can be used as a backup link when TN link is unavailable due to e.g., disaster. Option b) illustrates 6G NTN cell is isolated. This option can be used in case of non-TN coverage area, like maritime and/or island. Option c) illustrates 6G NTN/TN cells form the coverage area complementarily. In options a) and c), NTN-TN mobility may be required to continue the communication services. As indicated in Figure 1, NTN-TN coexisting scenarios should be assumed in 6G NTN.

Observation 3: NTN-TN cell coexisting scenarios is a baseline in 6G NTN study.

Considering NTN-TN cell are coexisting scenarios, a UE connected to NTN cell may need to switch to TN cell to perform data transmission, while a UE camped in a TN cell may also transfer to an NTN cell to receive an MBS service or may be switched to NTN cell by network to balance cell load. To maximize the user experience, on-going service should be also continued in NTN-TN mobility with minimum service interruption as several companies required seamless service continuity for 6G NTN [3] in RAN#109. Therefore, the seamless mobility study in 6G should take NTN specific aspects into account. It should aim to achieve less interruption time compared with NR NTN.

Observation 4: A seamless mobility should take NTN specific aspects into account.

Based on the above observations, we propose the following are essential requirements in 6G NTN.

Proposal 1: RAN2 to consider at least the following requirements for 6G NTN:

- to enable an access to NTN cells under long propagation delay

- to achieve a seamless mobility between NTN-TN and NTN-NTN cells

Thus, the related features should be considered as NTN-specific functions, i.e., UL timing alignment, TA reporting, disabling HARQ, autonomous timing adjustment, and offset for UL/DL scheduling shown in table 1.

Proposal 2:	RAN2 to consider at least the following functions to enable an access to NTN cells under long propagation delay:

- UL timing alignment

- TA reporting

- disabling HARQ

- autonomous timing adjustment

- offset for UL/DL scheduling

### Required functions for NTN mobility

Different from TN deployment scenario, the NTN payloads equipped over satellites over different orbits are moving or located based on the predefined satellite ephemeris, and a specific satellite may not always provide coverage for the same area. To tackle the issue, satellite information is required, e.g., satellite ID, ephemeris, the coverage dwell time, to assist the decision of whether and when to switch to the corresponding NTN cell. Such satellite information is required to achieve the seamless mobility.

Observation 5:	Satellite information including potential satellite orbits are useful functions for the seamless mobility.

Furthermore, in TN, the handover is mainly triggered due to UE mobility, when moving from cell center to cell edge, the signal quality of serving cell may gradually decrease, while the signal strength of the neighboring cell is increasing, therefore the signal strength or quality can be accounted for the mobility between cells in TN. However in NTN, strength or quality of the signal received by a UE at cell center can be quite similar with those of the signal received at cell edge, since the propagation delay and channel characteristics are comparable, therefore cell/satellite switch within NTN considers the distance to satellite or the serving time of the satellite, and location based and time based (conditional) handover are applied for NTN network. In 6G, to implement seamless mobility with NTN cell, location-based mobility and time-based mobility are supported regardless of UE RRC state.

Observation 6:	Location-based mobility and time-based mobility and related measurement mechanisms are useful functions for the seamless mobility.

In RAN1#122bis meeting [4], the following agreements were reached:

Agreement - The aspects to consider for supporting NTN include, but not limited to - Initial access, including cell search and SSB periodicity - Coverage - Duplexing - Capacity - Signalling overhead - GNSS-less/resilient/based operation - Large/varying doppler and propagation delay - Beamforming / beam management / beam hopping

In this RAN1 agreement, almost all aspects have been considered in NR NTN. However, GNSS-less operation is not yet supported in NR therefore it is likely a significantly different aspect. In the last RAN2 meeting, several companies also pointed out the vulnerabilities of GNSS and proposed to study GNSS-less operation. Given that RAN1 has agreed that this should be considered as one of the aspects, RAN2 also needs to consider this operation.

Observation 7:	GNSS-less operation for NTN would be considered in 6G Day1.

Regarding location based mobility, UE uses UE location depending on GNSS in NR NTN. On the other hand, considering the GNSS potential vulnerability, UE may not able to acquire accurate UE location via GNSS, and without the accurate UE location, mobility between NTN-TN and NTN-NTN will be far from achieving seamless, thus UE’s location acquisition without GNSS should also be studied in 6G NTN in addition to GNSS-based UE positioning. Regarding UE’s location with fine granularity without GNSS, as an example, it is useful to apply Multi-RTT (Round Trip Time) positioning, which was introduced in Rel-18. In addition, to support positioning of IDLE/INACTIVE UEs, it is necessary to study UE location acquisition considering NTN characteristics.

From the above, UE location acquisition should be considered as NTN specific features, with or without GNSS.

Proposal 3:	RAN2 to consider at least the following functions to achieve a seamless mobility between NTN-TN and NTN-NTN cells:

- Satellite information acquisition including potential satellite orbits

- Location-based mobility and time-based mobility and related measurement mechanisms

- UE location acquisition with and without GNSS

# Conclusion

In this paper we presented some views on TN/NTN integration and have come up with the following observations and proposals:

Observations:

Observation 1:	The following NTN architectures are assumed in 6G NTN in Day1:

- both GSO and NGSO satellites.

- both transparent and regenerative payload types.

Observation 2:	The features to compensate for the long propagation delay to/from the NTN are necessary as in NR.

Observation 3: NTN-TN cell coexisting scenarios is a baseline in 6G NTN study.

Observation 4: A seamless mobility should take NTN specific aspects into account.

Observation 5:	Satellite information including potential satellite orbits are useful functions for the seamless mobility.

Observation 6:	Location-based mobility and time-based mobility and related measurement mechanisms are useful functions for the seamless mobility.

Observation 7:	GNSS-less operation for NTN would be considered in 6G Day1.

Proposals:

Proposal 1: RAN2 to consider at least the following requirements for 6G NTN:

- to enable an access to NTN cells under long propagation delay

- to achieve a seamless mobility between NTN-TN and NTN-NTN cells

Proposal 2:	RAN2 to consider at least the following functions to enable an access to NTN cells under long propagation delay:

- UL timing alignment

- TA reporting

- disabling HARQ

- autonomous timing adjustment

- offset for UL/DL scheduling

Proposal 3:	RAN2 to consider at least the following functions to achieve a seamless mobility between NTN-TN and NTN-NTN cells:

- Satellite information acquisition including potential satellite orbits

- Location-based mobility and time-based mobility and related measurement mechanisms

- UE location acquisition with and without GNSS

# References

- RP-252912, Revised SID: Study on 6G Radio

- RAN2#131bis report

- RP-252943, Moderator's summary for RAN led 6G SI: Requirements of new and existing services

- R1-2508198, FL summary#5 on overview of 6GR air interface