3GPP TSG-RAN WG2 Meeting #132													     	R2- 2508473

Dallas, USA, November 17th - 21st, 2025

Agenda Item:		10.3.3.2

Source:	Samsung

Title:	Considerations for Energy Saving in 6GR

Document for:		Discussion

1. Introduction

In RAN#108, a study item on the next generation of non-backward compatible Radio Access Network (RAN), i.e., 6G Radio (6GR), was approved [1]. And RAN2 has discussed the initial round of energy savings in RAN2#131bis meeting, with the following agreements:

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

This contribution provides a summary of Samsung’s views on UE and network energy saving in 6GR.

2. Discussion

2.1 Energy-Saving Technologies in 5G

5G has introduced a range of advanced energy-saving technologies designed to optimize energy efficiency across both user equipment (UE) and network infrastructure (NG-RAN). These innovations represent a significant step forward in reducing energy consumption while maintaining high performance. Figure 1 illustrates the introduced 5G energy saving technologies in each release.

Figure 1 5G UE and Network Energy Saving Technologies

Despite significant advancements, 5G networks face multiple challenges in implementing energy-efficient solutions:

- First, the fundamental trade-off between energy efficiency and network performance. Operators are required to carefully balance power savings with critical QoS requirements including low latency and reliable coverage, while simultaneously managing different parameters across various UEs in different operational states.

- Second, periodic and always-on signaling requirements. Essential network operations such as synchronization signals, reference signals, and system information updates prevent both UEs and gNBs from entering deeper, more efficient sleep states, creating inherent limitations in power saving capabilities.

- Third, the lack of harmonization between different power-saving technologies. Energy-efficient solutions for UEs and networks have been developed separately without proper coordination, resulting in limited system-level efficiency.

- Fourth, the increasing network complexity. Modern deployments face coordination challenges in mixed macro/small cell scenarios, coexistence issues in unlicensed spectrum bands, and unique synchronization requirements when integrating NTN and diverse IoT devices - all of which complicate energy optimization efforts.

- Fifth, the gradual development of new features creates challenges in adaptation. The gradual rollout of energy-saving technologies across different 5G releases creates compatibility gaps, preventing early-generation devices from utilizing newer power-saving features.

These challenges collectively impact the real-world effectiveness of 5G energy-saving technologies and highlight areas requiring further research and development. In contrast, 6G should aim to overcome these limitations through innovative and integrated approaches.

Observation 1: 3GPP NR studied various energy saving mechanisms including:

- UE energy saving: DRX, BW adaptation, Measurement relaxation, DCP, UAI, PEI, and LP-WUS.

- Network energy saving: cell DTX/DRX, on-demand SSB, SIB1, and adaptive PRACH.

Observation 2: 5G networks face inherent limitations in energy efficiency due to fundamental trade-offs between power savings and QoS requirements, always-on signaling, and lack of harmonization between UE and network power-saving technologies.

Observation 3: 5G networks also encountered challenges from increasing network complexity due to mixed deployments and heterogeneous device integration, as well as compatibility gaps arising from the gradual development of new features.

2.2 Cell DTX/DRX and UE DRX

In 3GPP 5G NR, Cell DTX (Discontinuous Transmission) and Cell DRX (Discontinuous Reception) were introduced to enhance network energy efficiency, specifically in the RRC Connected state. These techniques allow base stations to minimize power consumption by dynamically suspending transmission and reception during inactive periods. However, there exists a significant limitation that these mechanisms were not extended to the Idle and Inactive Modes, leaving untapped potential for energy savings in scenarios with low or no user activity.

Observation 4: In 5G NR, Cell DTX/DRX were introduced to enhance network energy efficiency in the RRC Connected state but were not extended to Idle and Inactive Modes, leaving untapped potential for energy savings.

Furthermore, while Release 19 explored on-demand SSB (Synchronization Signal Block) and SIB1 (System Information Block 1) transmission, the implementation of on-demand SSB was confined to SCell operations. This restriction limited their broader applicability in PCell (Primary Cell) and standalone deployments, preventing more widespread energy efficiency improvements.

To address these shortcomings in the 6G era, a more comprehensive approach is needed. One key proposal is to extend Cell DTX/DRX functionality to Idle and Inactive Modes, enabling base stations to enter low-power states during inactivity while ensuring rapid reactivation when needed. Another critical advancement would be the implementation of dynamic, on-demand signaling across all cell types, including PCells and standalone cells, rather than restricting it to SCells.

Proposal 1: Study a unified Cell DTX/DRX mechanism that works in RRC Connected, Idle and Inactive Modes.

Building on the cell DRX/DRX and UE DRX, allowing coordination among cell and UEs to adjust their cell DTX/DRX and UE DRX patterns may enable simultaneous energy savings on both UEs and 6G sNBs. This coordination could align the active durations and inactive durations of all the components in each cell and it could significantly enhance the net energy consumption.

Proposal 2: Study a coordinated/joint operation of cell DTX/DRX and UE DRX.

Figure 2  Coordinated DTX/DRX and 6G Dynamic Cell On/Off with Carrier/Cell Separation

2.3 Carrier/Cell Separation

A more sophisticated approach to enhancing network energy efficiency involves the strategic separation of coverage carriers from capacity carriers, enabling their independent operation, as in Figure 2. This architectural innovation allows for dynamic activation and deactivation of capacity carriers based on real-time traffic demands, significantly reducing unnecessary power consumption while maintaining essential network coverage. By implementing this technology, both network and UE can maximize their deep sleep periods, minimizing energy waste during low-traffic intervals. The 6G system should support both network-initiated and UE-assisted strategies for dynamic cell management (e.g., dynamic cell on/off), offering flexibility in operational approaches. However, this separation may require adjustments to UE initial access procedures to ensure seamless connectivity. Given these potential benefits, RAN2 should prioritize studying carrier separation from the earliest stages of 6GR development, focusing on network deployment strategies, UE initial access procedures, dynamic resource allocation mechanisms, and power management optimization to establish it as a foundational element of the 6G network architecture.

Proposal 3: Study carrier/cell separation architecture from 6GR's initial phase, focusing on

- Network deployment strategies with limited number of carriers/cells having always-on signals/channels, and UE initial access procedures associated with such separated carriers/cells

2.4 Wake Up Radio/Signal (WUR/WUS)

Low UE power consumption is not just a desirable feature for 6GR networks but a foundational design objective. It plays a critical role in meeting global sustainability goals, enhancing user experience through extended battery life, and supporting diverse applications—from ultra-low-power IoT devices to high-performance XR/AR applications. While NR introduced iterative power-saving enhancements, 6GR must adopt a holistic and unified approach from the outset to avoid fragmentation and inefficiencies.

6GR should treat UE power saving as a foundational technology, integrating existing NR mechanisms cohesively while eliminating redundancy. Key aspects include:

- Unified Framework: Avoid multiple overlapping mechanisms (e.g., LP-WUS, PEI, DCI 2_6) by designing a single, optimized wake-up signal;

- Backward Compatibility-Free Design: Since 6GR does not need to support legacy LTE/NR constraints, it can adopt more efficient signaling structures;

- Cross-Working Group Coordination: Collaborate with RAN WG4 to evaluate system-level impacts (e.g., sync raster sparsity on cell search efficiency).

For the first point, to illustrate the design methodology, we consider the 6GR wake-up signal (WUS) design prioritizing OFDM sequence-based WUS, which does not requiring separate Low Power Radio (LR). Unlike the NR approach, which introduced multiple WUS designs (e.g., LP-WUS, DCP, and PEI) across different UE states, 6GR should prioritize a unified WUS design from the outset to avoid redundancies. Specifically, we propose embedding LP-WUR functionality within the Single Radio architecture, where the WUS can be monitored using the same receiver components as the main radio, with selective power-down of non-essential modules during WUS reception. This approach would reduce UE complexity by eliminating the need for a separate low-power radio (LR) while maintaining power-saving benefits. Additionally, the design should account for joint network and UE energy efficiency by minimizing resource overhead and latency. For RRM measurements during WUS monitoring, SSB can serve as a reference with relaxed periodicity for both serving and neighboring cells. To further optimize power consumption, the UE could perform neighboring cell measurements during WUS monitoring, ensuring consistent coverage with other downlink signals like SSB.

Observation 5: 5G NR introduced multiple fragmented wake-up signal (WUS) designs (e.g., LP-WUS, DCP, PEI) across different UE states, resulting in implementation complexity and suboptimal power efficiency due to lack of a unified approach.

So we propose to study the integration of LP-WUR prioritizing the OFDM sequence-based WUS to enable efficient WUS monitoring without requiring a separate receiver. This design should focus on minimizing resource overhead, enabling joint energy consumption, and reducing UE complexity while maintaining compatibility with existing downlink signals.

Proposal 4: Study the WUR/WUS in 6GR prioritizing the OFDM sequence-based WUS.

2.5 Uplink Assistance Information (UAI)

While 5G NR introduced various power-saving techniques leveraging Uplink Assistance Information (UAI) to enhance UE energy efficiency, many of these mechanisms have not been adopted in real-world deployments. Despite their theoretical benefits, practical implementation challenges—including network complexity, backward compatibility requirements, and the required UE side information —have hindered their widespread adoption. For instance, features such as UE-assisted BWP switching and early RRC release based on traffic patterns could significantly reduce power consumption but remain underutilized.

To overcome these limitations, the advancement of UAI in 6G must focus on three key aspects: simplicity, interoperability, and intelligent automation. By enabling UEs to dynamically report their operational states and traffic characteristics, UAI can facilitate more adaptive and context-aware power-saving strategies. This includes optimizing sleep cycles, resource allocation, and signaling overhead based on real-time UE behavior while maintaining performance levels.

The table below demonstrates some examples of potential energy savings achievable through UAI-driven optimizations, based on study from 3GPP NR:

Table 1 UAI driven power saving techniques in NR

| Technique | Applicable Traffic Patterns | Expected gain [2] |
| --- | --- | --- |
| Fast RRC release | Bulk download with following idle periods (YouTube, Netflix, web browsing, …) | 95% ~ 99% reduction by transition to RRC_Idle/Inactive |
| BWP adaptation | Low-latency persistent traffic (gaming, live streams, ..) | 16% ~ 45% reduction by bandwidth-reduction |
| SCell release | High-volume data download for sustained periods (FTP, video download, …) | 12% ~ 57.75% reduction by carrier component(CC) reduction |

Observation 6: 5G's UAI-based power-saving features faced low adoption due to implementation complexity and backward compatibility issues.

Proposal 5: Study enhanced UAI frameworks that can be practically implemented and effectively utilized by operators, networks, and UEs.

3. Conclusion

With lessons learned from NR, the following observations and proposals are derived:

Observation 1: 3GPP NR studied various energy saving mechanisms including:

- UE energy saving: DRX, BW adaptation, Measurement relaxation, DCP, UAI, PEI, and LP-WUS.

- Network energy saving: cell DTX/DRX, on-demand SSB, SIB1, and adaptive PRACH.

Observation 2: 5G networks face inherent limitations in energy efficiency due to fundamental trade-offs between power savings and QoS requirements, always-on signaling, and lack of harmonization between UE and network power-saving technologies.

Observation 3: 5G networks also encountered challenges from increasing network complexity due to mixed deployments and heterogeneous device integration, as well as compatibility gaps arising from the gradual development of new features.

Observation 4: In 5G NR, Cell DTX/DRX were introduced to enhance network energy efficiency in the RRC Connected state but were not extended to Idle and Inactive Modes, leaving untapped potential for energy savings.

Observation 5: 5G NR introduced multiple fragmented wake-up signal (WUS) designs (e.g., LP-WUS, DCP, PEI) across different UE states, resulting in implementation complexity and suboptimal power efficiency due to lack of a unified approach.

Observation 6: 5G's UAI-based power-saving features faced low adoption due to implementation complexity and backward compatibility issues.

Proposal 1: Study a unified Cell DTX/DRX mechanism that works in RRC Connected, Idle and Inactive Modes.

Proposal 2: Study a coordinated/joint operation of cell DTX/DRX and UE DRX.

Proposal 3: Study carrier/cell separation architecture from 6GR's initial phase, focusing on

-	Network deployment strategies with limited number of carriers/cells having always-on signals/channels, 
and UE initial access procedures associated with such separated carriers/cells

Proposal 4: Study the WUR/WUS in 6GR prioritizing the OFDM sequence-based WUS.

Proposal 5: Study enhanced UAI frameworks that can be practically implemented and effectively utilized by operators, networks, and UEs.

4. Reference

- RP-251881	New SID: Study on 6G Radio	NTT DOCOMO	RAN#108

- TS 38.840 	Study on User Equipment (UE) power saving in NR. (Release 16) 	3GPP