3GPP TSG-WG2 Meeting #132	R2-2508474

Dallas, USA, November 17 – 21, 2025

Agenda Item:	10.3.1.2

Source:	Ericsson, Deutsche Telekom

Title:	RAN Aspects for QoS framework and Service-Awareness

Document for:	Discussion, Decision

# 1	Introduction

RAN2 discussed different aspects of QoS and framework in the last meeting where the first step was to understand the issues and shortcomings with the current NR QoS framework; while looking into potential benefits for application/service-awareness in the RAN. Further, it was also agreed to look at a general framework rather than bespoke solutions.

Furthering these discussions, in this contribution we provide some insights into the 5G QoS framework in terms of traffic characterisation, service awareness in RAN as introduced in 5G and discuss the way forward with the 6G QoS framework highlighting the ToR across WGs.

# 2	Discussion

## 2.1	5G QoS Framework

The 5G QoS framework provides a suite of functionalities to identify and map IP packets onto QoS flows. Each of the QoS flows can be characterized, among others, by a set of QoS parameters. These flows would then be assigned a QoS Flow Identity (QFI).

The above figure summarizes how the traffic of a PDU Session can receive differentiated QoS treatment. In the DL, the UPF uses Packet Detection Rules (PDRs) provided by e.g. the SMF to identify and classify IP packets into their corresponding QFIs. In the UL, the UE uses IP packet filter sets (QoS rules) to identify and map IP packets. The RAN then configures the UE (AS filters) to establish an appropriate mapping between QFIs and DRBs. This mapping functionality is handled by the SDAP layer.

The 5G CN is capable of traffic identification and has been further enhanced as traffic has evolved with additional filters and, for example, APIs in the exposure functionality to provide even greater application awareness and already has the business logic in place to facilitate these functions. Furthermore, 5GS supports slicing that relies on 'URSP' (User Equipment (UE) Route Selection Policy, see TS 23.503). USRP enables the UE to support PDU Session based network-controlled steering of application specific data flows onto a QoS flow with the appropriate packet treatment. Additional QoS Flows can also be established in these PDU Sessions if further differentiation is needed.

In RAN, each DRB is configured to fulfil specific QoS requirements. The RAN independently configures each DRB by setting appropriate parameters at various protocol layers, for example each DRB can have its own scheduling configuration. Together, these configurations and scheduling treatment ensure that the QoS flows mapped to a DRB meet the required QoS requirements for that flow.

DRBs form the RAN’s fundamental mechanism for service differentiation: by configuring per-DRB parameters (at multiple protocol layers) and applying appropriate scheduling, the RAN enforces priority, latency, reliability, and throughput characteristics for each QoS flow.

5G QoS framework provides a baseline for service and traffic differentiation.

Based on information from the CN, DRBs are configured to meet the required QoS requirements for a QoS flow.

## 2.2	Service Awareness in RAN in 5G

Service Awareness in RAN has been introduced during the XR Study Item in Release 17 and Work Item in Release 18. It had the scope to improve capacity and the perceived quality of experience by becoming aware of the PDU Sets. In short, a PDU Set is defined as the set of packets which, together, are meaningful for an application. In the context of XR or video, a PDU Set consists of a set of packets which define, for example, one image (e.g. I-frame, or P-frame). All these packets would need to be delivered within the associated PDU Set Delay Budget (PSDB).

This functionality is introduced as follows:

- It relies on UPF traffic identification and PDR matching procedure. In addition, based on SMF instructions, UPF can perform further analysis and detect information, e.g., contained in the RTP/IP header extensions of each packet where the application can include this information. The application can mark the packets which belong to a PDU Set and UPF can be instructed to acquire this information.

- The information acquired by the UPF would then be passed to the RAN which could make use of it for different purposes such as scheduling, discarding packets that do not meet the latency requirements, or configuring suitable DRX configuration, among others.

Service Awareness in RAN has not modified but makes use of the basics of the 5G QoS framework. Packets are still distributed into DRBs based on their QFI.

Service Awareness in RAN in 5G is based on the 5G QoS framework.

Core Network identifies and characterizes the traffic.

DRBs are the baseline for service differentiation in RAN.

RAN consumes the information provided by the CN and UE to enhance capacity and application experience.

## 2.3	6G QoS Framework and Service Awareness in RAN

The overall 6G QoS framework is the responsibility of SA2; therefore, RAN2 should not lead these high-level discussions. As a baseline, RAN2 should assume the existing 5G QoS framework unless SA2 provides alternative directions. Under that baseline, the RAN will obtain for each packet the QoS-related and PDU Set information already available from the 5G framework.

Along the time, traffic has evolved and consequently SA2 has had to enhance the packet filters/traffic identification capabilities. For example, SA2 introduced enhancements in Release 19 that added additional filtering/identification capabilities focused on RTP traffic. It is, therefore, SA2 that needs to revisit whether the current filters are sufficient and whether any changes are needed to address any weaknesses and to continue to identify and characterize services and traffic. Further, this is already in scope of the 6G QoS framework study in SA2. Furthermore, they will also study aspects like dynamic QoS requirements.

Service and traffic identification/characterization is being discussed in SA2 as part of the 6G QoS framework study in S2-2509800 as aligned with the ToR.

Assuming the 5G QoS framework serves as the baseline for 6G, it naturally follows that the primary mechanism for service and traffic differentiation continues to be the classification of packets into DRBs. In 5G, the QFI derived from packet filters is the sole parameter used to determine which DRB a packet is mapped into. While it is worth considering whether additional inputs could enhance the DRB mapping decision, this should be first studied and anchored with the work in SA2.

Another key aspect, of service and traffic differentiation in the uplink, is the management of buffers associated with each DRB. This involves prioritizing these buffers and controlling the amount of data transmitted from each, a process commonly referred to as logical channel prioritization. These procedures rely on network-configured parameters such as priorities, Prioritized Bit Rates (PBR), and Bucket Size Durations (BSD). Because these parameters are not easily adjustable and it is not possible to find a set of values that fits in all conditions, by the UE becomes more important, especially when multiple services and traffic types compete for limited resources. Furthermore, the network needs accurate and relevant information about buffer status and traffic characteristics. This feedback allows the network to make informed decisions when issuing grants, optimizing resource allocation and maintaining QoS between different users. For more detailed insights, you can refer to the linked contribution on scheduling aspects including Logical Channel Prioritization (LCP) [3].

As a baseline for QoS discussions:

  - Assume the existing 5G QoS framework unless SA2 provides alternative directions.

  - Service/traffic differentiation in RAN is based on DRBs differentiation i.e. packets are routed to an adequate DRB

# 3	Conclusion

In this paper, we have reached these observations:

Observation 1	5G QoS framework provides a baseline for service and traffic differentiation.

Observation 2	Based on information from the CN, DRBs are configured to meet the required QoS requirements for a QoS flow.

Observation 3	Service Awareness in RAN in 5G is based on the 5G QoS framework.

Observation 4	Core Network identifies and characterizes the traffic.

Observation 5	DRBs are the baseline for service differentiation in RAN.

Observation 6	RAN consumes the information provided by the CN and UE to enhance capacity and application experience.

Observation 7	Service and traffic identification/characterization is being discussed in SA2 as part of the 6G QoS framework study in S2-2509800 as aligned with the ToR.

We propose:

Proposal 1	As a baseline for QoS discussions:

a.	Assume the existing 5G QoS framework unless SA2 provides alternative directions.

b.	Service/traffic differentiation in RAN is based on DRBs differentiation i.e. packets are routed to an adequate DRB

# 4	References

RP-251881, New SID: Study on 6G Radio, 3GPP RAN#108, Prague, Czech Republic, June 9-13, 2025 [online]

Internet Engineering Task Force (IETF), RFC 9330, “Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture”, December 2023

R2-2508344, Scheduling and retransmissions, Ericsson, Dallas, USA, November 17 – 21, 2025