3GPP TSG-RAN WG2 Meeting #132	 R2-2508523

Dallas, USA, November 17-21, 2025

Source:	KDDI Corporation

Title:	Discussion on 6GR User plane.

Tdoc Type:	Discussion

Agenda Item:	10.3.1.3

Document for:  Discussion

Release:       Rel-20

1	Introduction

In RAN#108 meeting, following Radio Interface protocol architecture and procedures for 6GR objective was introduced [1]:

One of the detailed objectives of the study are:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This paper is a contribution regarding the 6G user plane.

2	Discussion

2.1 L2 retransmission efficiency

Currently, user plane retransmission control is performed independently at multiple layers: HARQ (MAC layer), L2 ARQ (RLC layer), and transport layer (e.g., TCP). This multi-layered approach often results in redundant retransmissions and increased latency, which is particularly problematic for emerging 6G services requiring ultra-low latency and high reliability (e.g., XR).

Proposal 1: RAN2 study methods to optimize and retransmission operations across protocol layers, aiming to eliminate redundancy and improve overall efficiency in 6G user plane design.

In optimizing retransmission control, it is essential to consider the characteristics of upper-layer transport protocols and applications. As both QUIC-based and non-QUIC-based applications are expected to coexist in 6G, the user plane protocol design must be flexible enough to accommodate the diverse requirements of these use cases.

Proposal 2: RAN2 agree to study user plane protocols that cover use cases for both QUIC-based and non-QUIC-based applications.

In addition, considering the diversity of transport protocols expected in 6G, such as QUIC, it is important to evaluate the appropriate use of RLC modes based on the characteristics of the upper-layer protocol. QUIC, being a UDP-based protocol with built-in retransmission control and fast connection setup, is designed to reduce latency and improve responsiveness. These features may influence the necessity of retransmission at the L2 layer. Therefore, it should be studied whether RLC-AM or RLC-UM is more suitable when QUIC is used. If RLC-UM is adopted, potential concerns such as reduced reliability due to the lack of retransmission at the RLC layer should be clearly identified and evaluated.

Proposal 3: RAN2 study the appropriate use of RLC modes (AM vs. UM) in relation to upper-layer transport protocols such as QUIC, and identify any challenges associated with adopting RLC-UM.

3	Conclusion

We make the following observations and proposals:

Proposal 1: RAN2 study methods to optimize and retransmission operations across protocol layers, aiming to eliminate redundancy and improve overall efficiency in 6G user plane design.

Proposal 2: RAN2 agree to study user plane protocols that cover use cases for both QUIC-based and non-QUIC-based applications.

Proposal 3: RAN2 study the appropriate use of RLC modes (AM vs. UM) in relation to upper-layer transport protocols such as QUIC, and identify any challenges associated with adopting RLC-UM.

3	　Reference

[1] RP-251881, NTT Docomo (Moderator), "Study on 6G Radio"