3GPP TSG-RAN WG2 Meeting #132	                                                             R2-2508532

Dallas, USA, Nov. 17th - 21st, 2025

Source:	TURKCELL, CATT

Title:	Functionality management for RRM measurement prediction

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

For RRM measurement prediction of UE sided model, the following objectives are listed for further RAN2 work [1]:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

In this contribution, we further discuss the applicability reporting of RRM measurement prediction for UE sided model.

- Discussion

As per TR 38.744 [2], the applicability reporting procedure is described as follows:

Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message. If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release it. When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported. For an inapplicable full inference configuration, UE continues to perform the inference. It is up to network implementation what to do with reported result after UE indicates the corresponding full inference configuration is inapplicable. If a full inference configuration is applicable, UE applies it. No dynamic lower layer signalling is needed to active a full inference configuration. Applicability can be updated via UAI. A flag in OtherConfig is introduced to indicate whether applicability reporting via UAI is enabled or not. When an inference configuration becomes inapplicable UE shall report its inapplicability via UAI.

We think that the above description reached in the SI phase should also be confirmed for the WI. The technical foundations described above, which were established during the SI, should form the basis for the subsequent work in the WI.

For applicability reporting via RRCReconfigurationComplete message, we propose:

Proposal 1: RAN2 confirms that upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message.

Proposal 2: RAN2 confirms that if an inference configuration is inapplicable, UE may include a flag to indicate its preference to release it, and when UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

Proposal 3: RAN2 confirms that upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

Proposal 4: RAN2 confirms that if a full inference configuration is applicable, UE applies it. No dynamic lower layer signalling is needed to active a full inference configuration.

For applicability reporting via UAI, we propose:

Proposal 5: RAN2 confirms that a flag is introduced in OtherConfig to indicate whether applicability reporting via UAI is enabled or disabled.

Proposal 6: RAN2 confirms that when an inference configuration becomes inapplicable UE shall report its inapplicability via UAI.

In the Rel-19 RAN2#131 meeting [3], following agreements have been achieved on applicability/inapplicability of AI/ML PHY:

Agreements on applicability/inapplicability

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

For AI/ML mobility, the same procedure should be supported for RRM measurement prediction use case.

Proposal 7: The following agreements on applicability/inapplicability in AI/ML PHY are also applied to RRM measurement prediction:

RRCReconfigurationComplete shall include applicability/inapplicability status for:

- All inference configurations included in the immediately preceding RRCReconfiguration message, and

- Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

- Conclusion

In this contribution, we provide our views on applicability reporting for RRM measurement prediction, and the proposals are summarized as follows:

Proposal 1: RAN2 confirms that upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message.

Proposal 2: RAN2 confirms that if an inference configuration is inapplicable, UE may include a flag to indicate its preference to release it, and when UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

Proposal 3: RAN2 confirms that upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

Proposal 4: RAN2 confirms that if a full inference configuration is applicable, UE applies it. No dynamic lower layer signalling is needed to active a full inference configuration.

Proposal 5: RAN2 confirms that a flag is introduced in OtherConfig to indicate whether applicability reporting via UAI is enabled or disabled.

Proposal 6: RAN2 confirms that when an inference configuration becomes inapplicable UE shall report its inapplicability via UAI.

Proposal 7: The following agreements on applicability/inapplicability in AI/ML PHY are also applied to RRM measurement prediction:

RRCReconfigurationComplete shall include applicability/inapplicability status for:

- All inference configurations included in the immediately preceding RRCReconfiguration message, and

- Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR (Release 19), 3GPP

- R2-2506702, Report of 3GPP TSG RAN WG2 meeting #RAN2 131