3GPP TSG-RAN WG2 Meeting #132	R2-2508546

Dallas, U.S, Nov 17-21, 2025

Agenda item:	10.3.2.3

Source:	Sony

Title:	Discussion on 6GR initial and system access aspects

Document for:	Discussion

# 1 Introduction

At RAN2#131bis, the following agreements were made on control plane system information and paging [1].

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Agreements

Paging design should consider energy efficiency and simplity for both UE and Network.

In this paper, we discussed the control plane aspects e.g., initial access, BWP management, system information and paging to be addressed in RAN2 6GR study.

# 2 Discussion

## 2.1 Common initial access for all device types

Support of different device types requires the base station to know the capabilities of these devices earlier in the process of initial access procedure. This has led to a common solution of RACH resource partitioning and broadcasting these partitioning of resources. RACH partitioning was introduced for features like Redcap, slicing, SBFD, SDT etc. Resource partitioning is not a good practice from resource utilisation and feature implementation point of view and should be avoided in our view.

In order to avoid any early recognition of UE capabilities during initial access, common initial access procedure should be designed, and all device types should be able to perform initial access to any base station. UE capability for such diverse device types should be signalled later. UE may be able to indicate its capability either in MSG3 or MSG5 or later.

Proposal 1	RAN2 to agree that common initial access procedure should be supported for different device types. This may be achieved without RACH resource partitioning.

## 2.2 BWP management

In terms of resources, initial access should be performed on initial BWP. We think that initial and dedicated BWPs should be supported for power saving purposes. For TDD with SBFD, a simple configuration and switching is necessary at the UE such as treating the subband as either UL BWP or DL BWP during transmission or reception. The need for configuring 4 BWPs should be discussed further and any simplification of ASN.1 should be studied for BWP related parts. Also, the need for simultaneous activation of more than one BWP should be discussed as this was never used in NR.

Proposal 2	Initial and dedicated BWP should be supported. Need for multiple dedicated BWPs and simultaneous activation of more than one dedicated BWP should be studied further.

## 2.3 System information

In Rel-15 NR, Other SI which includes all SIBs not broadcast in the Minimum SI can be broadcast on-demand on DL-SCH upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED. In Rel-19 NR, on-demand SIB1 was introduced, where network can broadcast SIB1 after receiving OD-SIB1 request from UEs in RRC_IDLE, RRC_INACTIVE or some RRC_CONNECTED UEs. The reduction of always-on SIs broadcast in the downlink data channel would be beneficial to the network energy saving performance, therefore NR on-demand SI/SIB1 should be the baseline for 6GR system information handling.

Proposal 3	NR on-demand SI/SIB1 is the baseline for 6GR system information handling.

In Rel-19 NES OD-SIB1, it is required to evaluate the impact to the legacy UEs and the negative impact to the legacy UEs should be minimized. That’s one of the reasons that in Rel-19 on-demand SIB1, only Case 2 where UE obtains OD-SIB1 request configuration from Cell A and transmits OD-SIB1 request to NES Cell and receives OD-SIB1 from NES Cell, is specified. But the limitation because of the co-existence with legacy UEs is not applicable to 6GR anymore. We support to further study the other cases which has been discussed during Rel-19 NES OD-SIB1 SI phase in 6GR, as we see the needs to reduce the signalling exchanges among different cells as well as to enable more flexible NES cell deployment. Furthermore, in Rel-19 OD-SIB1, no modification of SSB is assumed. The design of SSB may be discussed in 6GR and its impact to OD-SIB1 as well as the potential combination of SSB modification and OD-SIB1 should be further studied in RAN2 after RAN1 makes any progress on the study of SSB design.

Proposal 4	Other on-demand SIB1 cases should be further studied in 6GR in order to reduce the signalling exchange among cells as well as to enable more flexible NES cell deployment.

In RAN2#131bis meeting, it was agreed to study how to improve flexibility and extensibility of scheduling SI design. Regarding SI scheduling, we see issues in NR as follows to be addressed in 6G NR.

- In NR, the SI scheduling is very complicated, with various schemes mixed up which make the implementation difficult. In each release, a new SI scheduling scheme was added as some new SIBs (and some have subsequent SIBs) were introduced. 6G SI scheduling design should consider the forward compatibility and target to fit in any newly introduced SIBs in the future.

- There are some restrictions on NR SI scheduling, e.g., Only SIBs or posSIBs having the same periodicity can be mapped to the same SI message (this can allow the SI message transmitted in a pre-defined periodicity in a simpler way), SI-windows of different SI messages do not overlap (for the sake of simpler SI window design). The efficient usage of the radio resources was somehow compromised in order to pursuit the simplicity of SI scheduling. In 6G, simple yet efficient SI scheduling should be one of the design goals.

- Due to the above design restrictions in NR, when a network needs to broadcast a series of SIBs (especially with different periodicity which is quite common) or should broadcast with a relatively longer period (depends on the SIBs/SIs), the network has to be in the ON state without the opportunity to go to an energy saving state, and thus leading to a compromised energy saving performance. Therefore, 6G SI scheduling design should consider to maximise the network energy saving gain while on the other hand, to minimise the impact to the UE power saving.

Proposal 5	6G NR SI scheduling should target to design a simple yet efficient scheme, to maximise the network energy saving gain while on the other hand, to minimise the impact to the UE power saving.

## 2.4 Paging

Paging as performed in NR RRC_IDLE and RRC_INACTIVE states is still power consuming and complex. We think the paging concept can be reviewed and improved when entering 6GR to support better energy efficiency in the UE.

Paging or paging indication using the LP-WUS signaling should be studied in RAN as this can improve the power consumption performance for all UEs. The power consumption of a LP-WUR receiver in the UE is substantially lower than the normal UE receiver, as the receiver does not need accurate synchronization and the active time of the UE could also be reduced. When the paging can be performed using the LP-WUR, it should also be studied whether the LP-WUR can be used for mobility measurements in RRC_IDLE state.

Proposal 6	Study usage and support of using LP-WUR for mobility and reception of paging indications.

# 3 Conclusion

In this contribution, our proposals are as follows:

Proposal 1	RAN2 to agree that common initial access procedure should be supported for different device types. This may be achieved without RACH resource partitioning.

Proposal 2	Initial and dedicated BWP should be supported. Need for multiple dedicated BWPs and simultaneous activation of more than one dedicated BWP should be studied further.

Proposal 3	NR on-demand SI/SIB1 is the baseline for 6GR system information handling.

Proposal 4	Other on-demand SIB1 cases should be further studied in 6GR in order to reduce the signalling exchange among cells as well as to enable more flexible NES cell deployment.

Proposal 5	6G NR SI scheduling should target to design a simple yet efficient scheme, to maximise the network energy saving gain while on the other hand, to minimise the impact to the UE power saving.

Proposal 6	Study usage and support of using LP-WUR for mobility and reception of paging indications.

# 4 References

- Minutes RAN2#131bis, Prague.