3GPP TSG-RAN WG2 Meeting #132	R2-2508570

Dallas, USA, 17th-21st November, 2025

Agenda item:	8.19.1

Source: 	Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC, Samsung, Nokia

Title: 	Support early CSI acquisition for L3 Handover [EarlyCSI_L3HO]

Document for:	Discussion and Decision

# Introduction

RAN2 discussed supporting early CSI acquisition for L3 handover during RAN2-131 [1] and sent an LS to RAN1 [2]:

R2-2506504 LS on early CSI acquisition for L3 handover RAN2 LS out Rel-19 TEI19 To: RAN1 Cc: RAN4 LS is approved R2-2506450 Support early CSI acquisition for L3 handover [EarlyCSI_L3HO] Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC CR Rel-19 38.331 18.6.0 5426 1 B TEI19 R2-2505604 The CR is technically correct RAN2 assumes there is no RAN4 work The CR is postponed pending RAN1 confirmation and will be discussed next meeting

RAN1 discussed the LS in RAN1-122b and sent reply LS to RAN2 [3] stating that there is no problem to support early CSI acquisition for L3 handover by re-using the early CSI acquisition framework for LTM. RAN1 has also sent updated feature list [4] and these are implemented in UE capabilities mega CR [6].

We discuss the progress in RAN1 including their reply LS in this contribution and propose changes to RAN2 specs.

# Discussion

## Background

Following the LTM solution framework, the early CSI acquisition and CSI reporting for L3 handover involves mainly 3 steps that are described below:

- CSI-RS configuration: Unlike in LTM, there is no need for pre-configuration of CSI-RS for L3 handover. UE can be provided with CSI-RS information along with handover command.

- CSI-RS measurement: UE performs the CSI-RS measurement upon reception of handover command (same as in LTM famework).

- CSI reporting:  Re-uses LTM framework. CSI report is conveyed by UCI and further multiplexed into the 1st PUSCH after handover or PUSCH scheduled by RAR, etc. towards target cell, and the lowest CQI index will be set if the CSI report is not ready at that time.

The proposed solution is illustrated in Figure 1.

Figure 1: Proposed CSI acquisition solution for L3 handover

## RAN1’s suggested changes to RRC

RAN1 has discussed the LS from RAN2 and confirmed that there is no problem to support early CSI acquisition for L3 handover by re-using the early CSI acquisition framework. RAN1 made the following agreements:

Agreement - Draft LS R1-2507925 is endorsed in principle. - Adopt the following TP for TS38.214 Section 5.2.4a and TS 38.212, section 6.3.1.2.1 - Note: No dedicated issue to support early CSI acquisition for L3 handover to be discussed in RAN1 before RAN#111.

RAN1 also suggested the following additional change to the RRC change proposed in R2-2506450 to align with LTM’s CSI acquisition framework:

RAN1 thanks RAN2 for the LS R1-2506716 (R2-2506504) on early CSI acquisition for L3 handover. From RAN1 perspective, there is no problem to support early CSI acquisition for L3 handover by re-using the early CSI acquisition framework for LTM. On the configuration of channel measurement resources and interference measurement resources in RAN2 RRC CR (R2-2506450), RAN1 suggests to directly use NZP CSI-RS resource set and CSI-IM resource set, where the corresponding changes on EarlyCSI-Acquisition-r19 are as follows: EarlyCSI-Acquisition-r19 ::= SEQUENCE { early-NZP-CSI-RS-ResourceSet-r19 NZP-CSI-RS-ResourceSetId, early-CSI-IM-ResourceSet-r19 CSI-IM-ResourceSetId OPTIONAL, -- Need R reportQuantity-r19 ENUMERATED {cri-RI-PMI-CQI, spare}, cqi-Table-r19 ENUMERATED {table1, table2, table3, table4-r17}, codebookConfig-EarlyCSI-r19 CodeBookConfig-EarlyCSI-r19 }

So, we directly use NZP CSI-RS resource set and CSI-IM resource set in EarlyCSI-Acquisition-r19 in RRC Reconfiguration.

Proposal 1: Directly use NZP CSI-RS resource set and CSI-IM resource set in EarlyCSI-Acquisition-r19 in RRC Reconfiguration.

RAN1 also endorsed the CRs to TS 38.212 [7] and TS 38.214 [8] that introduced early CSI acquisition for L3 handover. These are provided in Appendix.

## UE capabilities

RAN1’s feature list [4] contains features to support early CSI acquisition for LTM. Below two UE capabilities are relevant to support early CSI acquisition for L3 handover.

| 63. NR_Mob_Ph4 | 63-6 | CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS and CSI-IM resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell | RAN2 FG for LTM in Rel-18 | Yes | No | Periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-10 | CSI-RS measurement and CSI reporting without CSI-IM reception | 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration | 63-6 or 63-6a or 63-7 or 63-7a | Yes | No | CSI -RS measurement and CSI reporting without CSI-IM reception is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |

Proposal 2: Define the following UE capabilities to support early CSI acquisition for L3 handover:

- Optional per-band UE capability to indicate support of CSI-RS and CSI-IM measurement and CSI reporting

- Optional per-band UE capability to indicate support of CSI-RS and CSI reporting without CSI-IM resource configuration

The CRs to TS 38.331 and TS 38.306 are provided in [9] and [10].

# Conclusions

In this contribution, we propose the following:

Proposal 1: Directly use NZP CSI-RS resource set and CSI-IM resource set in EarlyCSI-Acquisition-r19 in RRC Reconfiguration.

Proposal 2: Define the following UE capabilities to support early CSI acquisition for L3 handover:

- Optional per-band UE capability to indicate support of CSI-RS and CSI-IM measurement and CSI reporting

- Optional per-band UE capability to indicate support of CSI-RS and CSI reporting without CSI-IM resource configuration

# References

[1] R2-2505598 	Support early CSI acquisition for L3 Handover, Huawei, et al.

[2] R2-2506504 LS on early CSI acquisition for L3 handover

[3] R1-2508168 Reply LS on early CSI acquisition for L3 handover [EarlyCSI_L3HO]

[4] R1-2507979 Updated RAN1 UE features list for Rel-19 NR after RAN1 #122bis

[5] RAN1-122b 	Chairman Notes https://www.3gpp.org/FTP/tsg_ran/WG1_RL1/TSGR1_122b/Inbox/Chair_notes

[6] UE Capabilities mega CR, https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23131bis%5D/%5BPOST131bis%5D%5B004%5D%5BUE%20caps%5D%20UE%20capability%20CR%20(Xiaomi)

[7] R1-2508217 Introduction of Rel-19 early CSI acquisition for L3 handover to TS 38.212 [EarlyCSI_L3HO]

[8] R1-2508257 Corrections on NR mobility measurements phase 4, 38.214

[9] R2-2508573 Introduction of early CSI acquisition for L3 handover [EarlyCSI_L3HO]

[10] R2-2508574 Introduction of UE capability to support early CSI acquisition for L3 handover [EarlyCSI_L3HO]

# Appendix - Endorsed changes in RAN1

## TS 38.212

### 6.3.2.1.2 CSI If cqi-BitsPerSubband is configured, this Clause 6.3.2.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4. <Unchanged part omitted> The mapping order of CSI fields of one report for CSI reporting for L1/L2-triggered mobility and handover as defined in Clause 5.2.4a of [6, TS 38.214] is provided in Table 6.3.1.1.2-7 by taking only Tables 6.3.1.1.2-1/3 for the determination of the bitwidth of a CSI field. The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8A. The mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8B. The mapping order of CSI fields of one report for SSBRI/RSRP or CRI/RSRP reporting for L1/L2-triggered mobility is provided in Table 6.3.1.1.2-8C. The mapping order of CSI fields of one report for MRI/CLI-RSSI is provided in Table 6.3.1.1.2-8D. The mapping order of CSI fields of one report for predicted CRI/RSRP or predicted SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8E. The mapping order of CSI fields of one report for time instance indicator/predicted CRI/predicted RSRP or time instance indicator/predicted SSBRI/predicted RSRP reporting is provided in Table 6.3.1.1.2-8F. The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP if nrofReportedRS is configured is provided in Table 6.3.1.1.2-8G. The mapping order of CSI fields of one report for RS-PAI is provided in Table 6.3.1.1.2-8H. The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP, SSBRI/RSRP, predicted CRI/RSRP, predicted SSBRI/RSRP, time instance indicator/predicted CRI/predicted RSRP, time instance indicator/predicted SSBRI/predicted RSRP, RS-PAI, CRI/SINR, SSBRI/SINR, MRI/SRS-RSRP, MRI/CLI-RSSI reporting, CSI reporting for L1/L2-triggered mobility, CSI reporting for handover, TDCP reporting, delay offset reporting, frequency offset reporting, both delay offset and frequency offset reporting, and phase offset reporting. <Unchanged part omitted>

## TS 38.214

### 5.2.4a CSI Reporting for LTM and handover A UE configured with LTM-Config can be provided configurations for CSI acquisition, by up to one Reporting Setting, ltm-CSI-ReportConfig, for a candidate cell. A UE can be provided configuration for CSI acquisition, by one Reporting Setting, earlyCSI-Acquisition in ReconfigurationWithSync, for a target cell. Each Reporting Setting ltm-CSI-ReportConfig or earlyCSI-Acquisition is associated with either one or two Resource Settings - When one Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement or early-NZP-CSI-RS-ResourceSet) is configured, it provides a list of NZP CSI-RS resources for both channel and interference measurements. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting. - When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement or early-NZP-CSI-RS-ResourceSet) provides a list of NZP CSI-RS resources for channel measurement, and the second Resource Setting (given by higher layer parameter ltm-ResourceForInterferenceMeasurements or early-CSI-IM-ResourceSet), provides a list of CSI-IM resources for interference measurement. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting. The Resource Setting given by higher layer parameter ltm-ResourcesForChannelMeasurement, LTM-CSI-ResourceConfig, contains configuration of a ltm-NZP-CSI-RS-ResourceSet which comprises of a list of Z ≥ 1 NZP CSI-RS resource indices (given by ltm-CSI-RS-ResourceList) and a list of Z LTM-CandidateIds (given by ltm-CandidateIdList) referring to candidate cells associated with the NZP CSI-RS resource indices. For CSI acquisition associated with a Reporting Setting, ltm-CSI-ReportConfig, the UE is expected to measure the NZP-CSI-RS resources in ltm-CSI-RS-ResourceList associated with the LTM-CandidateId that is equal to the LTM-CandidateId of the LTM-Candidate under which the Reporting Setting is configured. The UE shall expect the following configuration provided by ltm-CSI-ReportConfig or earlyCSI-Acquisition: - For the frequency granularity of the CSI report, the CQI format indicator is Wideband CQI. - For the frequency granularity of the CSI report, the PMI format indicator is Wideband PMI. - The codebook type is typeI-SinglePanel. - The reportQuantity is set to ‘cri-RI-PMI-CQI’. After a UE receives an LTM Cell Switch Command MAC CE [10, TS 38.321] providing a candidate cell (given by Target Configuration ID field), and a ltm-CSI-ReportConfig is configured for the candidate cell, the UE can measure corresponding NZP CSI-RS resources and CSI-IM resources if configured, and shall transmit a CSI report to the candidate cell. After a UE receives a ReconfigurationWithSync configured with earlyCSI-Acquisition, the UE can measure corresponding NZP CSI-RS resources and CSI-IM resources if configured, and shall transmit a CSI report to the target cell. For RACH-less LTM cell switch or RACH-less handover [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant [6, TS 38.213]. For RACH-based LTM cell switch or RACH-based handover using a contention-free random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the PUSCH scheduled by the RAR UL grant or MsgA PUSCH. For RACH-based LTM cell switch or RACH-based handover using a contention-based random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission corresponding to MsgB, or the first PUSCH scheduled by a PDCCH transmission addressed to the C-RNTI, received in response to Msg3. For the PUSCH determined for CSI reporting, when the UE applies PUSCH repetition Type A or Type B, the CSI report is transmitted only on the first repetition for PUSCH repetition Type A, or on the first actual repetition for PUSCH repetition Type B. If a valid CSI is not available, the UE shall transmit a CSI report which contains a CQI corresponding to the lowest CQI index. Depending on the UE capability, the UE may measure NZP CSI-RS resources and CSI-IM resources if configured corresponding to a ltm-CSI-ReportConfig before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]. <omitted text>