3GPP TSG-RAN WG2 Meeting #132					      R2-2508576

Dallas, USA, 17 - 21 November, 2025

Title: 	Discussion on data transfer and AI/ML framework in 6G

Source: 	Huawei, HiSilicon

Agenda item:	10.3.3.1

Document for:	Discussion/Decision

- Introduction

In RAN2#131b meeting, RAN2 discussed data transfer/collection and AI/ML framework, and some agreements were made. In this contribution, we further discuss data collection/management framework, including analysis of different data types, functions, requirements and potential directions. We also discuss AI/ML model transfer and LCM framework, including functions and model management in 6G.

- Data collection/management framework

Analysis of different data types

In the following Table 1, we provide a summary on data requirements for diverse types of data. For dataset/model parameters delivery for two-side model and UE-side data collection for training in 5G-A R20, RAN#110 (this December) plans to decide on solutions. So, we suggest to wait for that progress before discussing data requirements of these data types in 6G.

Here we have the following assumptions:

- For the metric of QoS requirements: we mainly analyze the latency, and latency requirements refer to RAN1 definition in the LS [1].

a) Relaxed (e.g., minutes, hours, days, or no latency requirement)

b) Near-real-time (e.g., several tens of msecs to a few seconds)

c) Time-critical (e.g., a few msecs)

Table 1: Analysis on data requirements for diverse types of data (Note: Data content, data size for UE-RAN data)

| Data Types | Data content | Applicable Use case(s) | Producer(s)/Consumer(s); Data path | Data size per report | Data rate | QoS requirements (e.g. latency) |
| --- | --- | --- | --- | --- | --- | --- |
| Sensing data | Raw data (delay-doppler-angle profile) | Case 1: target detection Case 2: Environment reconstruction & assist communication | Case 1: UE / CN Data path: 1) UE ->CN; 2) UE->RAN->CN Case 2: UE / RAN & CN Data path: 1) UE->RAN; 2) UE->RAN->CN Case 3: RAN / CN Data path: RAN->CN | Case 1: ~10Mbits Case 2: ~Mbits | Case 1/2: Large | Case 1: Near-real-time Case 2: Relaxed or near-real-time Case 3: Near-real-time |
|  | Multi-path component | Case 1: target detection Case 2: Environment reconstruction & assist communication |  | Case 1/2: ~Kbits | Case 1/2: Small |  |
|  | Points | Case 1: target detection Case 2: Environment reconstruction & assist communication Case 3: RAN-based target detection |  | Case 1/2: ~Kbits Case 3: ~60Kbits | Case 1/2: Small Case 3: Large |  |
| AI/ML data (Note 1) | Offline training data | Case 4 - low overhead CSI-RS with NW-side model Case 5 - low overhead DMRS with AI/ML receiver Other cases, for NW-side model and UE-side model training | UE / RAN; UE->RAN UE / UE server; 1) UE->RAN->OAM->UE server 2) UE->CN->UE server | Case 4: ~64Kbits per sample Case 5: ~20Kbits per sample | Large | Relaxed |
|  | Online finetuning data | Case 4 - low overhead CSI-RS with NW-side model Case 5 - low overhead DMRS with AI/ML receiver | UE / RAN; UE->RAN | Case 4: ~64Kbits per sample Case 5: ~20Kbits per sample | Large | Near-real-time or time-critical |
|  | Monitoring | Case 4 - low overhead CSI-RS with NW-side model Case 5 - low overhead DMRS with AI/ML receiver | UE / RAN; UE->RAN | Case 4: ~64Kbits Case 5: ~1Kbits | Large or small | Near-real-time |
| MDT data | M1-M9, logged data | Immediate/Logged MDT | UE / OAM; UE->RAN->OAM | <9kB | Small | Near-real-time/relaxed |
| SON data | L3 measurements | RLF report and others | UE / RAN&OAM; UE->RAN, UE->RAN->OAM | <9kB | Small | Near-real-time/relaxed |
| QoE data | QoE measurements | Container based QoE, RAN visible QoE | UE / RAN&OAM; UE->RAN, UE->RAN->OAM | <9kB | Large/Small | Near-real-time/relaxed |

Note 1: Data transfer of other types of AIML data between UE, RAN, CN, and OAM depends on further progress in other groups, e.g., SA2, SA5, RAN3 and RAN1.

In the following sub-sections we elaborate further on the use cases captured in Table 1.

Sensing related data

For sensing, RAN#109 plenary meeting agreed to study the use cases of detection and tracking for 6GR and will further discuss other use cases. One of the use cases mentioned in TR 22.870 is digital twin and environment object reconstruction, which can be used to enhance communication performance, or enable new services such as collaborative robots, smart city, etc.

To further understand the data transfer requirements for sensing, we do an analysis on the data content and data size based on the urban grid scenario that was proposed to be added into TR 38.914 by more than 10 companies considering the evaluation for ISAC (Integrated Sensing and Communication) in RAN #109. We analyze the data types and size for two potential ISAC use cases, i.e. the target detection, and the environment reconstruction and assisted communication.

For the use case mentioned above the data producer is a UE which performs a sensing measurement which needs to be reported to the network.

For the target detection use case, the CN can be the data consumer and utilize the collected sensing data from UE for target detection. In TR 22.870, several use cases are defined for the object detection and tracking service, in which the latencies range from 0.1 s to 1s for “Safe and economic UAV transport” and “UAV detection, classification and counting” service respectively, as shown by use case “Target detection” in Table 1. In this case, the data transfer path can be from the UE to the CN, as illustrated by the first data path option the table.

For the environment reconstruction and assisted communication use case, the RAN can collect and use the sensing data to reconstruct cell-level environment and further utilize it to assist communication via e.g. CSI feedback enhancement, beam prediction, etc. In this case, RAN is the data consumer, as illustrated by the first data path option of use case “Environment reconstruction and assist communication” in Table 1. In addition, RAN may further transmit sensing data to CN if there is a sensing service request from the CN for more accurate and larger environment reconstruction. In this example, both RAN and CN are data consumers, and the data transfer path can be UE -> RAN -> CN as shown by the second data path in the table.

For the above two sensing use cases, three potential data types are used to analyze the corresponding data sizes, and the results are summarized in Table 1. The detailed calculation can be found in the Annex. It should be noted that the latency requirement is referenced from TR 22.870 from the service perspective. The RAN-level QoS requirement for latency may be lower than the specified values. It can be observed that there are diverse service requirements.

The characteristics of each sensing data type, along with a comparison of data size and data rate between the two use cases are discussed as follows:

- Raw data: In this case, the sensing data corresponds to the amplitude and phase profiles in the delay-Doppler-angle domain. This level preserves the most information, but also results in the largest data size. The data size for environment reconstruction is smaller than that for target detection, primarily because moving target detection require a longer accumulation time to ensure accurate velocity estimation.

- Multi-Path Component (MPC) data: Here, the sensing data consists of multi-path component information, extracted from the initial delay-Doppler-angle profile. It is a sparse subset of raw data, retaining only significant paths that are typically sparsely distributed across time, space, and Doppler domains. The resulting data size is moderate compared to raw data. The data size for environment reconstruction is larger than that for target detection, because in the urban grid scenario, static surfaces of the wall tend to generate more multi-paths than moving targets.

- Point data: Here, the sensing data is derived from MPC data through further processing. Each point is described by its position, velocity (not required for environment reconstruction), and power. This type yields the smallest data size among the three. The data size for environment reconstruction is larger than that for target detection, since the reconstruction needs more points to represent the shape of the object than the binary results for detection.

We can see that the sensing data content shows a clear trade-off between information completeness and data efficiency across different data types.

Detailed analysis on data types can be found in section 7.1.

Furthermore, already in 5G-A sensing data transfer between RAN and CN is being discussed in RAN3 working group where RAN node can transfer sensing data to CN sensing function (SF).

AI/ML related data

New use cases in 6G

In RAN1#122bis, 12 observations have been obtained with 20 use cases being identified [2]. To discuss the different data types for AI data, we choose low overhead CSI-RS with NW-side model and low overhead DMRS with AI/ML receiver as two examples. The typical data types for these use cases include at least training data, inference data and monitoring data. For training data, both offline training data and online finetuning data should be considered since they may be different from each other in terms of data size and refreshing rate. When it comes to inference data of these two use cases, we assume L1 signalling will be used for this purpose (i.e. same as non-AI based reporting), so it is not analyzed in the context of data collection.

For low overhead CSI-RS with NW-side model, the input of the AI/ML model is sparse channel measurement and output is the full channel matrix,

- Training data: the model can be trained based on full channel matrix data. Sparse elements can be picked up from the full channel matrix data to serve as model input, and the full channel matrix data serve as the training label. For online finetuning, the data samples used will be much less than those of offline training. However, online finetuning will be conducted more frequent than offline training.

- Monitoring data: several metrics, such as SGCS and throughput, can be used for monitoring. Take SGCS as an example, to calculate it, the ground-truth full channel matrix should be obtained, which can be compared with the mode inference results to check the performance of the model.

Assuming a 256x8 MIMO configuration which is agreed for 7GHz carrier frequency, the analysis on data for training dataset (both for offline training and online fine-tuning), inference data and monitoring data is summarized in Table 1, where float 16 is used to represent a real number.

For low overhead DMRS with AI/ML receiver, a two-sided model solution can be considered to for superimposed pilot or DMRS free sub-use cases. At the transmitter side, AI/ML model is used to mapped the modulated symbols onto resource elements with DMRS or without DMRS. At the receiver side, AI/ML model is used to estimate the channel for further processing (i.e., equalization and demodulation) or directly reconstruct LLRs from the received signals.

- Training data: If to train the transmitter side of the two-side model, a data sample should contain modulated symbols and the transmitted symbols. If to train the receiver side, the received symbols and ground truth coded bits should be used. And it is worth noting again the difference between offline training and online finetuning.

- Monitoring data: usually, BLER is the metric to check the demodulation performance of UE data transmission. To calculate the BLER, the ground truth information bits before coding should be provided.

Assuming one resource, the analysis on data for training dataset (both for offline training and online fine-tuning), inference data and monitoring data is summarized in Table 1, where float 16 is used to represent a real number.

Detailed analysis on data types can be found in section 7.2.

Use cases following 5G-A

In 5G-A R18, RAN1 started study on AI for air interface, and the study outcome has been captured in TR 38.843 [3]. In R19, RAN1, RAN2 and RAN4 have finalized work item work according to the WID [4]. In R20, more enhancements on AI for air interface can be found in the WID [5].

In 5G-A R19, RAN2 started study on AI for mobility, and the study outcome has been captured in TR 38.744 [6]. In R20, RAN2 will start work item work according to the WID [7].

For all above topics, some data types have been identified and defined.

For network-side data collection for NW-side model training, RAN2 agreed on UE logging mechanism, and RRC signaling is used to do the data collection configuration and reporting. For inference and monitoring data collection, L1 reporting mechanism is used.

For UE-side data collection for UE-side model training, [3] has listed some candidate options in 7.2.1.3.2, such as 1a, 2 (UE via CN to UE server), 3 (UE via OAM to UE server). This is still under feasibility study in some WGs, and a check point at RAN#110 will check whether/how to do the data collection via 3GPP NW.

SON (Self-Organizing Networks) related data

SON features were introduced since Rel-16. For SON features, UE is data provider and RAN is data consumer, and there may be signalling exchange between different RAN nodes for some SON features.

The collected data is used by RAN for RAN optimization purpose. In addition, according to TS 32.422 (SA5 spec), if OAM is also interested in using the collected data, RAN can transfer the reports to OAM based on the requests from OAM.

MDT (Minimization of Drive Test) related data

MDT features were introduced since Rel-16. This feature can utilize user devices to collect network measurements, reducing the need for traditional, costly manual drive tests. This feature allows for more efficient network optimization, faster identification of issues, improved network performance, and lower operational costs.

There are two ways of triggering MDT procedures:

- Management based MDT: the MDT task is initiated by OAM trace function without targeting a specific UE.

- Signalling based MDT: the MDT task is initiated towards a specific UE by the signalling trace activation messages from CN nodes,

Furthermore, there are two modes for the MDT measurements: Logged MDT and Immediate MDT. The definitions can be found as below (from TS 37.320 [8])

Immediate MDT: MDT functionality involving measurements performed by the UE in CONNECTED state and reporting of the measurements to RAN available at the time of reporting condition as well as measurements by the network for MDT purposes.

Logged MDT: MDT functionality involving measurement logging by UE in IDLE mode, INACTIVE state, CELL_PCH, URA_PCH states and CELL_FACH state when second DRX cycle is used (when UE is in UTRA) for reporting to eNB/RNC/gNB at a later point in time, and logging of MBSFN measurements by E-UTRA UE in IDLE and CONNECTED modes.

In [8], measurements for immediate MDT and logged MDT are listed. In order to support Immediate MDT, the existing RRC measurement configuration and reporting procedures apply. Some extensions are used to carry location information.

QoE (Quality of Experience) related data

QoE features were introduced since Rel-17. This feature can enable collection of application layer measurements from the UE, such as DASH streaming services, MTSI services, VR services, MBS communication service. There are two types of QoE related Data:

(1) Container based QoE

The intention of container based QoE is to let OAM collect QoE measurements via RAN from the UE, and this allows for a better assessment of the network's ability to meet user experience requirements and optimize the network accordingly. The QoE measurements are defined in SA4 and are transferred using RRC in a RAN-transparent containers.

(2) RAN visible QoE

The intention of RAN visible QoE is to let RAN collect QoE measurements from the UE, and then QoE measurements (e.g. application layer buffer level list, play out delay, PDU session ID list) can be utilized by the RAN for network optimization, such as RRM scheduling optimization.

RAN visible QoE measurements are configured by RAN, and the QoE measurements are generated from UE application layer. Once the UE AS gets these QoE measurements from UE application layer, it will report them to the gNB using a standardized format (ASN.1) via RRC signalling.

Summary of analysis of different data types

Based on Table 1, we see that different types of data need to be collected in 6G to support different services/use cases, and there are different data producer and data consumer (e.g. RAN, CN, OAM). For these data types, there are different data sizes and different QoS requirements (e.g. latency), which can be categorized as follows:

- High-throughput with relaxed latency (High-throughput = large data rate)

- High-throughput with Near-real-time or time-critical

- Low-throughput with relaxed latency (Low-throughput = small data rate)

- Low-throughput with Near-real-time or time-critical

The summary table is listed as below:

Table 2: Summary of data types according to data path and QoS requirements

| Producer(s)/Consumer(s) Data path | Data types | Examples of applicable use cases | Summary on data/QoS requirements |
| --- | --- | --- | --- |
| UE / OAM UE->RAN->OAM | QoE data | Container based QoE | High-throughput with relaxed latency |
|  | MDT data (M1, M2, M6, M8, M9) | Immediate MDT | Low-throughput with Near-real-time or time-critical |
|  | MDT data | Logged MDT | High-throughput with relaxed latency |
|  | SON data | RLF/CEF/RACH/MHI/SHR/SPCR report | Low-throughput or high-throughput depending on the number of SON reports. Relaxed latency. |
|  | AI for air interface (training) | Network-side data collection for network-side model (e.g. BM). | High-throughput with relaxed latency |
| UE / UE server UE->RAN->OAM->UE server UE->CN->UE server | AI for air interface (training) | UE data collection for UE-sided model training | High-throughput with relaxed latency |
| UE / CN UE->RAN->CN | Sensing data | Case 1: target detection Case 2: Environment reconstruction & assist communication | Low-throughput or high-throughput depending on data content For case 1: near-real-time or time-critical For case 2: relaxed latency or near-real-time |
| RAN / CN RAN->CN | Sensing data | RAN-based target detection | Low-throughput or high-throughput depending on data content Near-real-time or time-critical |
| UE / RAN UE->RAN | QoE data | RAN visible QoE | Low-throughput with near-real-time or time-critical |
|  | SON data | ANR | Low-throughput with near-real-time or time-critical |
|  | SON data | RLF/CEF/RACH/MHI/SHR/SPCR report | Low-throughput or high-throughput depending on the number of SON reports. Relaxed latency |
|  | AI for air interface (training) | Network-side data collection for network-side model (such as BM, low overhead CSI-RS, low overhead DMRS with AI/ML receiver). | High-throughput with relaxed latency |
|  | AI for Online finetuning | Low overhead CSI-RS Low overhead DMRS with AI/ML receiver | High-throughput with near-real-time or time-critical |
|  | AI for air interface (monitoring) | Monitoring data reporting for UE-side/NW-side models (such as BM, low overhead CSI-RS) | High-throughput or low-throughput; Near-real-time or time-critical |
|  | Sensing data | Environment reconstruction & assist communication | Low-throughput or high-throughput depending on data content; Relaxed latency or near-real-time latency |

Observation 1: Different types of data need to be collected in 6G to support different services/use cases, with different data producers and data consumers (e.g. RAN, CN, OAM), different data size, different latency requirements, etc.

Observation 2: For the same data type and data producer, there can be multiple data consumers for different services/use cases. For example, the data consumer of SON can be RAN and OAM, the data consumer of NW-sided training data can be RAN or OAM, and the data consumer of sensing data can be RAN and CN.

Proposal 1: RAN2 to capture characteristics and requirements of different types of data that needs to be collected in 6GR in the TR. Table 2 can be used as a starting point.

Data collection/management framework functions

Even though we usually speak of "data collection" or "data transfer", there are more functions and aspects related to data management which need to be considered when designing a framework to handle the data collected through 6G network. The following functions need to be accounted for:

- Data Collection: This includes aspects such as network determining and configuring the selected UEs with what measurements/information need to be collected, in what format etc. In addition, different reporting schemes should be configurable, e.g. some data may be first logged at the UE and reported to the network in an on-demand manner while other data may need to be reported in “real-time” with a certain periodicity.

- Data Transfer: A process of transferring data from a data provider to a data consumer. The protocol stack used for transmitting the data needs to be flexible to support various QoS requirements of different types of data, as analyzed in section 2.1. RAN2 should focus on studying the data transfer over air interface between the UE and RAN.

- Data Utilization: As shown in section 2.1, the collected data can be used not only to facilitate network management but also to support new services and applications, such as AI/ML model training and inference, digital twin construction, etc. Furthermore, in some cases, the same data can be utilized for multiple purposes.

- Data Routing: This includes determining the consumer of the data and also routing the data in the intermediate nodes placed between the data provider and its consumer. This function should also consider that the same data may need to be available/visible to multiple nodes, so that duplicate transmission of the same data is avoided.

- Data Monitoring: Monitoring operations are performed on data. 6G involves more types of data, requiring data monitoring to ensure data quality and guarantee performance of different tasks or services utilizing this data. For instance, for sensing data, there may be some monitoring parameters for sensing data quality, such as the accuracy of the sensing results which refers to how closely the measured or estimated parameters match their true values.

- Data Compression: To achieve an efficient transmission, especially over the air interface, overhead of transmission needs to be considered. Therefore, data compression of the data transferred over air interface needs to be studied.

Observation 3: The study of data collection/management framework needs to consider such functions and aspects as data collection, data transfer, data utilization, data routing, data monitoring and data compression.

Data collection/management framework requirements

During RAN2#131bis, RAN2 agreed an initial set of the requirements for the data collection framework which includes aspects such as security of the collected data, respecting user privacy and user consent, data visibility/controllability by the MNO, extendibility of the framework and minimizing an impact to the UE. The agreement was captured as applicable “at least for AI/ML’ and with an FFS on whether there is a difference between the requirements applicability to UE-sided and NW-sided data collection. In our view, there is no reason to differentiate the requirements related to AIML and other use case as long as they require collection of data from the UE. When it comes to UE-side data collection, one of the options discussed in 5G-A was a transparent transmission of UE-side data between the UE and OTT server. Since transmission of UE-side data due to this option has no standard impact, the requirements discussion is not applicable for it. On the other hand, in case a standardized solution for UE-side data collection transfer is developed in 6G, it should follow the same requirements as NW-side data collection.

Proposal 2: Clarify the agreement on the data collection framework requirements from RAN2#131bis as follows:

- Transparent transmission of UE-side data between the UE and OTT server should still be possible in 6G

- In principle (i.e. unless otherwise stated), the requirements apply equally to NW-side data collection, standardized UE-side data collection and other studied use cases (i.e. not only to AI/ML data collection)

On top of the requirements which were captured during RAN2#131bis, companies were tasked to analyze additional requirements, considering multiple data types and use cases, including AI/ML, sensing, QoE and SON/MDT. As can be seen in section 2.1 and in Table 1, the requirements depend not only on the use cases, but also on the type of data and intended data usage within one use case. In particular, there are different requirements when it comes to the size of data transmitted during a single data collection process or in a single report, various latency requirements ranging from tens of milliseconds to tens of seconds. The reporting frequency or periodicity may also be different. Furthermore, the same data may be useful for different nodes, e.g. sensing data can be used by RAN for assisted communications and also by the CN for environment reconstruction, QoE data may also be utilized by RAN for real-time communications adjustment and by OAM for long-term network fine-tuning. Considering the volume of data that needs to be collected, it is crucial to consider how to ensure an efficient data transmission over the air interface, e.g. by avoiding duplicate data transmission, data compression, transmission continuity after HO etc.

Proposal 3: Data management/collection mechanism in 6GR should meet the following requirements:

- Support of different types/formats of data, including AIML data, sensing data, SON/MDT, QoE

- Flexible configuration of data reporting QoS, e.g. latency, priority, bit rate, reporting periodicity

- Possibility to transfer the collected data to different/multiple nodes, e.g. RAN, CN, OAM

- Efficient transmission over the air interface, e.g. configurable data resolution, data compression, transfer continuity after HO, and avoiding a collection of the same data multiple times by different nodes (e.g. RAN, CN, OAM)

Potential directions for data transfer

Based on analysis of different data types in section 2.1, it can be seen that there are mainly the following data transfer paths:

(1) UE -> RAN

(2) UE -> RAN -> OAM

(3) UE -> RAN -> CN

(4) UE -> CN/OAM -> UE server

For (1), (2), (3), and (4), the data transfer between UE and RAN can be commonly analyzed. For the data transfer between RAN and OAM, it assumed to be SA5 area and RAN2 may co-ordinate with SA5 later. For the data transfer between RAN and CN, it assumed to be SA2/RAN3 area and RAN2 may co-ordinate with SA2/RAN3 later. Hence in RAN2, we can focus on how the data is transferred between the UE and RAN regardless of who the final consumer of data is.

For data types shown in Table 1, data volume/data rate emerge as a key factor. From RAN2’s perspective, the data volume size will impact the data transfer mechanism, especially in the case the size becomes quite large. Therefore, we suggest to study how to support potentially large data transfer between UE and RAN. The data transmission latency should also be an evaluation criterion, because it will have an impact on the designed solution.

At RAN2#131bis meeting, RAN2 has agreed on MNO visibility and controllability for data collection framework. We suggest to study how to ensure data visibility and controllability by the MNO during the data transfer.

5G-A utilizes RRC-based option for transferring SON/MDT data, QoE data, and other data types, incorporating QoS differentiation mechanisms. For instance:

- SRB2 has a lower priority than SRB1 and UEInformationResponse shall use SRB2 when logged MDT is to be transferred

- SRB4 is dedicated to QoE data transfer with a configurable priority (typically set as the lowest among SRBs/DRBs)

- A new SRB has been defined in Rel-19 for NW-side data collection for AI based beam management. This SRB also has a configurable priority which should be configured lower than SRB1 not to impact real-time network operation.

For 6G, we think it is important to study how to differentiate the data for data transfer and other data (such as control plane or user plane data).

Table 3: Limitations of 5G-A to support data transfer between UE and RAN

| Data transfer mechanisms | The currently supported data types | Potential improvement directions to support diverse data types |
| --- | --- | --- |
| CP-based (Control Plane): RRC-based | RRC-based: 1) Network-side data collection for AI for communication performance enhancement 2) SON/MDT data collection 3) QoE data collection | Scalability: For high-throughput data types with relaxed latency requirements, CP-based option lacks scalability. QoS flexibility: Current SRB design shows insufficient QoS adaptability, and it requires definition of new SRB types for emerging data types. |
| UP-based (User Plane): PDU session-based | Currently PDU session is mainly for transferring user application data, so it may be irrelevant to data transfer. In addition, if UP-based solution is used, it means the data needs to be transferred from UE via RAN to CN, and then CN needs to transfer data back to RAN. | Path inefficiency: UE-to-CN path may lead to redundant data transfer when the RAN is data consumer. |

Based on the limitation of CP-based and UP-based solutions listed in the above table, we think enhanced CP-based, and enhanced UP-based approaches should be studied, in order to address the listed limitations.

For enhanced CP-based way, it may be challenging as RRC may need to be largely enhanced by considering scalability and QoS flexibility requirements.

In comparison, UP-based solution can meet most of the requirements. However, the collected data is not visible to RAN, and there is path inefficiency issue which leads to redundant data transfer.

In this case, in order to meet the requirements that we mentioned before, we think that RAN2 can further study pros/cons of using IP (e.g. UP-based solution) vs a dedicated data plane on top of Layer 2 UP, between UE and RAN.

For (4), in 5G-A, it is only for UE-side data collection. For the transmission path UE->OAM->UE server, the data transfer between UE and RAN may be common with (1) and (2). For the transmission path UE->CN->UE server, RAN2 has studied Option 2 (including CP tunnel and UP tunnel solutions), and then SA2 is studying the feasibility of Option 2-UP. Details can be found in the TR 38.843.

For the data transfer between UE and CN, similar as above, the data volume size and data transmission latency should be considered, and it is up to SA2 on the data transfer solution discussion. From RAN2’s perspective, we could analyze data types together with applicable use cases.

Observation 4: There are limitations of using either CP-based data transfer (e.g. not friendly to large data size, no flexible QoS differentiation support), or UP-based data transfer (e.g. data not visible to RAN).

Proposal 4: For data transfer between UE and RAN/CN/OAM, RAN2 should study enhanced CP-based, enhanced UP-based approaches and potentially new options (e.g. a dedicated data plane between UE and RAN) according to the agreed requirements.

As discussed before, one of the more important requirements to consider in 6G is for the solution to avoid data duplication and allow for the same data to be used in different network nodes. Therefore, we propose the following:

Proposal 5: Study an approach where RAN collects the data from the UE first and then may forward it to other nodes, e.g., CN or OAM, if needed.

- AI/ML model transfer

In 5G-A R19 study phase, both RAN1 and RAN2 studied model transfer for one-side UE model, and TR 38.843 captures the analysis as well as some conclusions.

For RAN1, the necessity of model transfer for one-side UE model has been discussed. According to the section 8 Conclusions in TR 38.843 [3], some model transfer/delivery cases are deprioritized and here are the conclusions as below:

-	From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons:

-	Risk of proprietary design disclosure

-	Burden of offline cross-vendor collaboration

-	From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y):

-	No much benefit compared to Case y

-	Risk of proprietary design disclosure

-	Large burden of offline cross-vendor collaboration

-	Additional burden on model storage within in 3GPP network

-	From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19.

In this case, we understand that in 5G-A R19, model transfer for one-side UE model is left to UE implementation i.e. OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). In 6G, new AI/ML use cases may be discussed in RAN1/RAN2 or other WGs, and the situation in 5G-A R19 can be kept unless a requirement in RAN is identified.

Observation 5: In 5G-A R19, model transfer for UE-side model is left to UE implementation.

Proposal 6: For the one-side UE model, the model transfer can be left to UE implementation unless a requirement in RAN is identified.

- AI/ML LCM framework

6G AI/ML LCM framework functions

During RAN2#131bis, RAN2 agreed the LCM functions which should be supported by 6G. Even though they may appear as the same as those used in 5G-A, there will be some additional requirements which need to be considered to cater for the continuous development of AIML techniques that is happening currently. As noted in the 6G SID, 6G system is expected to provide extensible AI/ML enablers to support various AI/ML use cases. At the same time, it is noted that “6GR and RAN design shall ensure that the 6G System can also operate without AI/ML.” Therefore, the functions in 6G need to meet certain requirements, as described below. We believe that in particular model management can be improved in comparison to the baseline that was specified in 5G-A, which is explained in more detail in section 4.2.

Functionality management

As a principle, in 6G it should be the network which is in control of whether a certain functionality is enabled or not, e.g. depending on the current needs, preferences, network conditions etc. UE fallback or switching between AI/ML-based and non-AI/ML based functions does not have to be precluded, but its necessity should be carefully studied and its usage should always be under network control.

Model management

Offline model training was assumed in 5G-A discussion. Furthermore, trained models were assumed to be applicable only in certain cells sharing common radio characteristics which was denoted with a so-called associated ID. However, considering the number of different network configurations and deployments one may come across in the real life, it is inefficient to train a model dedicated to, e.g. each network cell configuration separately. Therefore, techniques like online model training/adjustment will have to be investigated closer, as discussed in more detail in section 4.2.

Model performance monitoring

In 6G, similar principles can be followed for performance monitoring as those developed in 5G-A. In particular, as a baseline a common performance monitoring framework should be utilized while allowing a certain degree of flexibility to cater for use case specific requirements and metrics. In addition, with more introduced use cases, AI/ML functionalities may be coupled or dependent on each other (e.g. output of one AI/ML functionality may be the input for another AI/ML functionality). This calls for consideration of more efficient and low overhead/complexity/power consuming monitoring mechanisms.

Model management in 6G

As mentioned shortly above, in 5G-A, an offline model training was assumed and a notion of associated ID was developed. In practice, this may lead to a situation where the UE needs to train a separate model for each cell or for small areas in the network which have exactly the same radio configuration. The model storage capacity at the UE is usually very limited, so there may be a need to download a new model after UE performs a handover to another cell/area. Furthermore, also the model storage capacity at the OTT server itself is limited, so a need to retrain a full model can happen very often which in practice can make applying AIML techniques difficult or at least leads to inference discontinuity and large data collection efforts.

Observation 6: To achieve inference continuity during UE mobility, it should be possible for the UE to reuse the same/similar model in multiple cells, even with different cell configurations.

Therefore, online training should be considered to achieve more flexible model update. As defined in TR 38.843 [3], online training is an AI/ML training process where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. With the consideration of complexity, online training is usually conducted based on a previous (potentially outdated) version of the model rather than re-training it from scratch, i.e., low overhead online finetuning is more desirable. Another important factor that needs to be considered is UE complexity, and hence two methods can be envisaged:

- One choice of online training is to transmit the newly collected data back to the model training server, where the model is updated and sent back to the device (UE or BS) for further usage. This method is suitable for the case when the computing capability of the device is limited and some model update delay is acceptable.

- For the case when the timeliness of the model is important, the online training procedure can be conducted on a device. Processing capability and power consumption are required for the device, however, advanced training methods such as Low-Rank Adaptation (LoRA) can help with alleviating this issue.

The online training procedure can be triggered by events such as performance degradation, and it can be conducted along with the changing of data distribution. Continuous learning is important in this case to allow the model to stay up to date.

Furthermore, to train a robust model, data from multiple sources needs to be gathered and processed at the training entity. This data should have a proper distribution, e.g. the training entity may require data only from certain parts of the cell which are underrepresented in the collected data. Therefore, it is also important in these situations to allow the network to control the UE selection and type/range of data that needs to be collected. In addition, there is also a use case that is being studied by RAN1 which is based on distributed models and distributed inference, e.g., AI/ML-enabled RAN digital twin with distributed model, where AI/ML models deployed across multiple UEs can help to improve the precision and completeness. In this use case, each single UE can only generate sparse sensing data which reflects the local environmental information around the UE. Based on the inference with the sparse environmental information as input, the AI/ML model can output more precise and complete environmental information. The UE-part model input is in forms of sparse sensing data, e.g., multi-path information, angle information, point cloud information, etc. UE-part model is adopted to convert the raw data to latent space and outputs latent space information for compressing. the NW-part model takes the latent space information reported from multiple UEs as input for aggregation and builds the precise and complete global environment information, e.g. point cloud information of global environment, as inference output. The NW-part model works with one-on-multiple manner to various UE-part models. Distributed inference takes place when inference results from multiple UEs are used as an input to the model at the network side to generate a final inference result. RAN2 should keep these use cases and techniques in mind when developing the AIML LCM framework for 6G.

Proposal 7: RAN2 should study necessary specifications changes for how to support enhanced AIML model handling, i.e.:

- Online training and model finetuning

- Distributed training and inference

The study of data collection framework should consider the requirements for the data collection and reporting for online and distributed training/model finetuning.

- Conclusion

Data framework requirements

Observation 1: Different types of data need to be collected in 6G to support different services/use cases, with different data producers and data consumers (e.g. RAN, CN, OAM), different data size, different latency requirements, etc.

Observation 2: For the same data type and data producer, there can be multiple data consumers for different services/use cases. For example, the data consumer of SON can be RAN and OAM, the data consumer of NW-sided training data can be RAN or OAM, and the data consumer of sensing data can be RAN and CN.

Proposal 1: RAN2 to capture characteristics and requirements of different types of data that needs to be collected in 6GR in the TR. Table 2 can be used as a starting point.

Observation 3: The study of data collection/management framework needs to consider such functions and aspects as data collection, data transfer, data utilization, data routing, data monitoring and data compression.

Proposal 2: Clarify the agreement on the data collection framework requirements from RAN2#131bis as follows:

- Transparent transmission of UE-side data between the UE and OTT server should still be possible in 6G

- In principle (i.e. unless otherwise stated), the requirements apply equally to NW-side data collection, standardized UE-side data collection and other studied use cases (i.e. not only to AI/ML data collection)

Proposal 3: Data management/collection mechanism in 6GR should meet the following requirements:

- Support of different types/formats of data, including AIML data, sensing data, SON/MDT, QoE

- Flexible configuration of data reporting QoS, e.g. latency, priority, bit rate, reporting periodicity

- Possibility to transfer the collected data to different/multiple nodes, e.g. RAN, CN, OAM

- Efficient transmission over the air interface, e.g. configurable data resolution, data compression, transfer continuity after HO, and avoiding a collection of the same data multiple times by different nodes (e.g. RAN, CN, OAM)

Data transfer solution directions

Observation 4: There are limitations of using either CP-based data transfer (e.g. not friendly to large data size, no flexible QoS differentiation support), or UP-based data transfer (e.g. data not visible to RAN).

Proposal 4: For data transfer between UE and RAN/CN/OAM, RAN2 should study enhanced CP-based, enhanced UP-based approaches and potentially new options (e.g. a dedicated data plane between UE and RAN) according to the agreed requirements.

Proposal 5: Study an approach where RAN collects the data from the UE first and then may forward it to other nodes, e.g., CN or OAM, if needed.

AIML model transfer

Observation 5: In 5G-A R19, model transfer for UE-side model is left to UE implementation.

Proposal 6: For the one-side UE model, the model transfer can be left to UE implementation unless a requirement in RAN is identified.

AIML LCM framework

Observation 6: To achieve inference continuity during UE mobility, it should be possible for the UE to reuse the same/similar model in multiple cells, even with different cell configurations.

Proposal 7: RAN2 should study necessary specifications changes for how to support enhanced AIML model handling, i.e.:

- Online training and model finetuning

- Distributed training and inference

The study of data collection framework should consider the requirements for the data collection and reporting for online and distributed training/model finetuning.

- References

- R2-2311720, Reply LS on Data Collection Requirements and Assumptions, Source: RAN1, To: RAN2

- Chair Notes for RAN1#122bis.

- TR 38.843 v19.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-250792, R19 WID on AI/ML for NR air interface

- RP-252445, R20 WID on AI/ML for NR air interface PH2

- TR 38.744 v19.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR

- RP-252899, R20 WID on AI/ML for mobility

- TS 37.320, v19.0.0, Radio measurement collection for Minimization of Drive Tests (MDT); Overall description

- R1-2508485, Performance metric, methodologies, and initial evaluation results for ISAC, Huawei, HiSilicon, Dallas, USA, November 17-21, 2025

- Annex for detailed analysis of data types

Analysis of sensing data types

The calculation instances of the data size for different sensing use cases listed in Table 1 are interpreted in this section. For case 1 and case 2 the examples with typical assumptions are calculated as follow. For case 3, the example is referred in [9]. The parameterized assumptions can be further discussed based on RAN1 inputs.

- Level 1: raw data

At Level 1, the sensing data corresponds to the amplitude and phase profile in the delay-Doppler-angle domain. The data size is calculated based on general assumptions and configuration parameters as follows:

- Number of FFT points in the frequency domain: NF = 4096

- Number of OFDM symbols used for sensing

  - For target detection: NT = 64

  - For environment reconstruction: NT = 4

- Number of receiving antenna ports: NS = 8

- Number of TRPs:  NTrp = 1

- Number of UEs: Nue = 1

- Quantization: 16 bits for both real and imaginary parts of each complex value

The data size can be calculated using the following formula:

Raw data size = NF × NT × NS × NTrp × Nue × 16 × 2 (bits)

By applying the parameters above, the data size is:

- Target detection: ~10Mbits

- Environment reconstruction & assist communication: ~several Mbits

- Level 2: multi-path component (MPC) data

At Level 2, the sensing data corresponds to multi-path component information, extracted from the initial range-Doppler-angle profile. This represents a sparse subset of Level 1 data, where only significant paths are retained, typically distributed sparsely across the time, spatial, and Doppler domains. The data size is calculated based on general assumptions and configuration parameters as follows:

- Number of sensing targets:

  - Target detection: NTarget = 3

  - Environment reconstruction: NTarget = 10

- Number of points per target:  Npoint_per_target = 2

- Number of paths per point: Npath_per_point = 2

- Total number of paths from the targets: Npath = NTarget × Npoint_per_target × Npath_per_point

- Number of background paths: Nbackgound_path = 10

- Total number of paths: Ntotal_path = Npath + Nbackgound_path

- Number of parameters per path:

  - For target detection: Npara_per_path =6 (delay, doppler, azimuth angle, zenith angle, power, phase)

  - For environment reconstruction: Npara_per_path =5 (delay, azimuth angle, zenith angle, power, phase)

- Number of TRPs: NTrp = 1

- Number of UEs: Nue = 1

- Quantization: 16 bits per parameter

The data size can be calculated using the following formula:

MPC data size = Ntotal_path × Npara_per_path × NTrp × Nue × 16(bits)

By applying the parameters above, the data size is:

- Target detection: ~several Kbits

- Environment reconstruction & assist communication: ~several Kbits

- Level 3: point data: position, velocity and power per detected point

At Level 3, the sensing data corresponds to point-level information, which is derived from the multi-path component (MPC) data at Level 2 through further processing. Each point can be represented by its position, velocity (not required for environment reconstruction), and power. The data size is calculated based on general assumptions and configuration parameters as follows:

- Number of sensing targets:

  - Target detection: NTarget = 3

  - Environment reconstruction: NTarget = 10

- Number of points per target: Npoint_per_target = 2

- Total number of paths from the targets: Npoint = NTarget × Npoint_per_target

- Number of background points: Nbackgound_point = 3

- Total number of points: Ntotal_point = Npoint + Nbackgound_point

- Number of parameters per point:

  - For target detection: Npara_per_point =7 (x, y, z, vx, vy, vz, power)

  - For environment reconstruction: Npara_per_point =4 (x, y, z, power)

- Number of TRPs: NTrp = 1

- Number of UEs: Nue = 1

- Quantization: 16 bits per parameter

The data size can be calculated using the following formula:

point data size = Ntotal_point × Npara_per_point × NTrp × Nue × 16(bits)

By applying the parameters above, the data size is:

- Target detection: ~several Kbits

- Environment reconstruction & assist communication: ~several Kbits

Analysis of new AI/ML data types in 6G

For the AI/ML related data of new use cases in 6G, the calculation instances of low overhead CSI-RS with NW-side model and low overhead DMRS with AI/ML receiver are chosen as two examples. The parameterized assumptions can be further discussed based on RAN1 inputs:

- low overhead CSI-RS with NW-side model

For low overhead CSI-RS with NW-side model, the data size is calculated based on general assumptions and configuration parameters as follows:

- Number of Tx Ports:  NTx= 256

- Number of measured Tx Ports:  N Tx_meas= 64

- Number of Rx Ports:  NRx= 8

- Quantization: 16 bits for both real and imaginary parts of each complex value

The data size can be calculated using the following formula:

for offline training data / online finetuning data / monitoring data:

data size = NTx × NRx ×2(complex) x 16(bits) x #samples

By applying the parameters above, the data size is:

- Offline training data / online finetuning/ monitoring data → ~64Kbit x #samples

Note that for monitoring data, the number of samples will be much smaller than that of offline training data / online finetuning data.

- low overhead DMRS with AI/ML receiver

For low overhead DMRS with AI/ML receiver, the data size is calculated based on general assumptions and configuration parameters as follows:

- Number of subcarriers in a RB:  Nsc= 12

- Number of symbols in a slot:  Nsym = 14

- Modulation order:  NQAMbits= 6 (64QAM)

- Quantization: 16 bits for both real and imaginary parts of each complex value

The data size can be calculated using the following formula:

for offline training data / online finetuning data:

data size = 2(model input/output) x Nsc ×Nsym ×2(complex) x 16(bits) x #layer x #samples

for monitoring data:

data size = Nsc ×Nsym ×NQAMbits x codeRate x #layer x #samples

By applying the parameters above, the data size is:

- Offline training data / Online finetuning → ~10Kbit x #layers x #samples

- Monitoring data → ~1Kbit x codeRate x #layer x #samples

Note that for monitoring data, the number of samples will be much smaller than that of offline training data / online finetuning data.