3GPP RAN WG2 Meeting #132    	                                                                                R2-2508592

Dallas, USA, Nov. 17th-21st , 2025

Agenda Item:	10.4

Source:	HONOR

Title:	Discussion on Mobility management for 6GR

Document for:	Discussion and Decision

- Introduction

RAN plenary #108 meeting discussed the objectives for 6GR and provide the initial version for the 6GR SID which is revised in RAN plenary #109 meeting [1]. As shown in the following, RAN2 would lead the study of mobility for 6GR and come to the following agreements in RAN2#131-bis meeting.

Agreements

1    Study mobility with the following requirements in 6G mobility design:

-     Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-     Robustness of mobility procedures

-     Energy efficiency for both UE and NW

2    Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this contribution, we will provide our initial thinking for the mobility management for 6GR for different RRC states and RRM.

- Discussion

Mobility is a key function for the cellular network to provide a continuous service, similar to 5G, the study of mobility management aims to copy with the UE with different RRC state and different phase of mobility management (cell switch/beam switch, measurement, failure detection and handling, etc.).

In 5G, in addition to the baseline cell switch mechanism, different mobility enhancements are introduced to copy with different scenarios or meet different requirements. For example, to support seamless service during mobility, DAPS is introduced to enable the UE to communicate with both source and target during handover. To improve the robustness due to failed signalling exchange, handover execution could be triggered by UE based on the NW-preconfigured configuration and event based on the conditional handover mechanism. To reduce the latency caused by RRC signalling and the interruption caused by L2/L3 reset/reconfiguration, LTM is introduced as a mechanism triggered by MAC CE based on the L1 measurement report. Furthermore, LTM is also extended to a conditional mechanism based on preconfigured event to further reduce the latency caused by the signalling interaction.

Observation 1: The 5G cell switch mechanisms triggered by different node (i.e., NW triggered, or UE triggered) based on cross layer mechanism (e.g., L3 HO or L1/L2 based cell switch) aim to different scenarios and requirements.

For 6G Radio Access Technology, to achieve a transformative leap in capability, efficiency, and simplicity, the study of mobility should consider the anticipated diverse service classes and try to a unified yet flexible framework for all RRC state and different phase (e.g., RRM). We would like to discuss the mobility for different RRC states and the measurement related issues.

  - Mobility for UEs in connected RRC state

NW based mechanism is the baseline to switch the UE to the target cell which NW takes account in the information (e.g., measurement result) reported from the UE and the NW status (e.g., load balance, energy efficiency). However, the signalling may be lost due to the fast channel quality degradation. Compared with NW based mechanism, the UE triggered mechanism could be more applicable in some scenarios. In 5G, separate configurations are provided to the UE and the configuration for different cell switch mechanism are prepared via parallel Xn procedures with signalling between gNBs. However, for the same target/candidate cell, part of the configuration (e.g., CFRA configurations, SIBs, etc) is the same for different cell switch mechanism. During the study in the Release 19, RAN2 agreed to reuse the configuration for CLTM by sending the LTM SCS MAC CE. Thus, at least this part could be the reference configuration to reduce the signalling burden for the NW triggered and UE triggered cell switch mechanism.

Proposal 1: A unified configuration framework could be used for different cell switch mechanism (e.g., UE triggered and NW signalling based switch mechanism).

In 5G, when the UE moves along, the target cell and source cell could be intra-DU, inter-DU intra-CU and inter CU. Based on the relationship between the target cell and the source cell, the UE could perform different level of protocol reset. Different level of protocol reset could be discuss based on the NW architecture to further reduce the interruption caused by cell switch. Furthermore, the lower layer signalling could further reduce the latency caused by signalling interactions but only limited information could be indicated to the UE upon cell switch. Higher layer signalling (e.g., RRC) could carry the complete configuration but signalling interaction and message decoding would cause larger latency. For the measurement report, the L1 measurement and report could provide prompt information while L3 measurement could provide more stable results. Cross layer design could utilize the advantages for different layer.

Proposal 2: Introduce a cell switch mechanism for cross layer switch (e.g., L1/L2/L3 cell switch) based on a unified framework.

In 5G, beam management is to switch the serving beam within the serving cell based on the L1 measurement reported from the UE. However, when introducing LTM in the later release of 5G, UE could perform the L1 measurement for the candidate cells and report the L1 results to the serving cell. MAC CE is used by the source DU to trigger the UE to apply the preconfigured L3 configuration of the target candidate cell and indicate the target beam for accessing the target cell. Beam specific operation and L1 measurement are not limited within the serving cell. The cell is covered by multiple beams which may be transmitted by multiple TRPs. Based on our understanding, beam switch could be triggered if the UE is moving within a limited area (e.g., the serving cell) and the cell switch is only triggered when the UE leaving the previous limited area. In another word, when the LTM candidate cell becomes the serving cell of the UE, it could be switched to the preconfigured beam when it moves within this candidate cell and could be switched to the preconfigured beam of another candidate cell when it moves out of this candidate cell. Similarly, for the study of 6G, the cell switch could be target to a specific beam and the beam management could be a cross cell operation. A unified design for the cell switch and beam management may be beneficial to reduce the redundant configurations and options for the same functionality.

Proposal 3: Consider the unified the design for the cell switch and beam management. Inter-WG work with RAN1 is required.

Early DL/UL synchronization is one of the key functions to enable minimize interruption for mobility. During RAN2#131bis meeting, RAN2 agreed to study the aspect for early DL/UL synchronization. In 5G, the UL sync is introduced in LTM with a new RACH procedure. During previous discussion, RAN2 already confirmed that we would study the conditional handover. We still need to discuss the early synchronization of conditional handover is triggered by the NW or the UE based on measurement result. On one hand, we could reuse the NW triggered early TA requisition defined for TA adjustment with less specification impacts. But it is possible that UE is hard to received NW signalling before it triggers the execution of the LTM based on the measurement results. On the other hand, the early synchronization is also triggered by the UE based on the preconfigured condition. For example, UE could trigger the TA acquisition with a lower threshold.  This enables that the early synchronization could be obtained earlier than the conditional HO by performing synchronization upon the satisfaction of an easily satisfied condition and executing the cell switch upon a stricter condition.

Proposal 4: Introduce both NW triggered and UE triggered Early DL/UL synchronization.

  - Mobility for UEs in non-connected RRC state

Mobility management for non-connected UEs is primarily implemented through cell selection and reselection. In 5G, cell reselection process mainly includes configuring measurement and evaluation parameters, detecting cells at different frequencies, measuring cell quality, and evaluating measurement results. When camped on a cell, non-connected UEs shall regularly search for a better cell and try to select a more suitable cell according to the RRC_IDLE or RRC_INACTIVE state measurement results and the cell reselection criteria.

Compared to connected mobility management, non-connected mobility management only supports UE-triggered reselection procedures. However, both are essentially based on UE measurement results to evaluate cell quality and ultimately help the UE determine the most suitable cell. They also share some similarities in the measurement object (e.g. SSB) and measurement parameter types (e.g. cell/frequency offset). The main difference lies in the cell quality evaluation standards, such as CondEvent trigger conditions for connected UEs and cell-ranking criterion for non-connected UEs.

Observation 2: Non-connected mobility management and connected mobility management have many similarities, including their fundamental principles, part of measurement objects, and types of measurement parameters.

In addition, cell selection/reselection involves measurements of sync signals and parameters carried in system information in the serving and neighbour cells. In previous discussions about 6GR, many companies mentioned further reducing always-on signals or transmitting them more sparsely to achieve network energy savings. However, this may affect the cell reselection process (e.g., increasing cell reselection latency) and impact the UE experience.

Observation 3: In 6GR, the design of system information and sync signals to achieve network energy saving may affect the cell selection/reselection.

In summary, as the main method of non-connected UE mobility management, cell reselection should be studied from 6G Day1. It is recommended to consider 5G cell reselection process as a starting point. However, based on the agreements of mobility framework and target in the last meeting, we should try to optimize the cell reselection process, considering the complexity and gains, as well as the energy efficiency of both NW and UE. For example, based on the similarities between non-connected mobility management and connected mobility management, design a unified cell quality evaluation criterion for both connected and non-connected UEs. They can share the same set of basic evaluation parameters, such as cell offset, frequency priority, frequency offset, and threshold conditions. These basic parameters can be broadcast via system information. If NW needs to modify the configuration for certain connected UEs, it can do so via RRC messages. If no modification is required, the connected-mode UE can use the basic parameters from the system information by default.

Proposal 5: Consider 5G cell selection/reselection process as a starting point and try to optimize it for simplicity and energy efficiency:

- Design a unified cell quality evaluation criterion for both connected and non-connected UEs.

- Identify the impact from the design of 6G system information and sync signals on the cell selection/reselection process.

2.3 Measurement

In order to achieve the unified mobility, a unified measurement framework would also be pursued. Generally speaking, a unified measurement framework for 6GR should cover 5G functions of L1 and L3 measurement and report.

In the current measurement framework, the measurement configuration for L1 and L3 measurement are configured independently and are configured in different RRC signalling structure. Since both L1 and L3 measurement can be the results of the same reference signal and the same sample, from this perspective, it is feasible to make a unified configuration. Due to the overlap between the configuration related to L1 measurement and that related to L3 measurement, this will result in larger signalling overhead.  In addition, it might result in duplicate measurement on the same measurement resources that are configured in different structure. Based on these considerations, a unified measurement configuration as the foundation of a unified architecture needs to be considered. For instance, a common resource configuration can be considered for both L1 and L3 measurement.

Proposal 6: A unified RS resource framework can be considered for both L1 and L3 measurement.

In 5G, two separate mechanisms are introduced for L1 and L3 measurement report. Both the configuration for triggering L1 and L3 measurement report are configured independently. In detail, the configuration for triggering measurement report includes the configuration for periodic/aperiodic/semi-persistent report and the configuration for event-triggered report. If both L1 and L3 reporting are triggered, there may be partial duplication between L1 and L3 reporting on some certain RS. Due to the possible redundancy between the L1 reporting and the L3 reporting, the impact of signalling overhead will be further increased. To address the issue caused by the 5G design, unified L1 and L3 measurement report can be considered for unified design.  Based on the above analysis, there are two options for further consideration.

Option 1:  Unified measurement report for both L1 and L3 measurement

In the unified architecture, a common trigger mechanism for L1 and L3 reporting can be considered. If the common trigger mechanism is applied, it is natural that both L1 and L3 reporting will be triggered by the same one condition. To further unified the reporting, a single reporting signalling can be considered to carry L1 and L3 reporting.  This can be regarded as a direct way to solve the problem of partial duplication in L1 and L3 reporting.

Option 2: unified measurement report based on the NW configured parameters merging L1 and L3

Since L1 and L3 measurement might refer to the same sample with the same RS, for the same sample, there is no need to provide the report via two approaches (L1 &L3 signalling) with the redundant information. The network can configure a set of parameters for measurement report based on the NW requirements without differentiating L1 and L3. For example, a filtering factor or the range of filtering factors can be configured to indicate the characteristic for L1 or L3 reporting as in NR.

Comparing these two methods, for option 1, the standard changes are relatively small, but the signalling overhead has not been effectively reduced. However, option 2 can effectively reduce signalling overhead, but the standard changes are relatively significant.

Proposal 7: RAN2 to study the unified measurement report mechanism and further study the follows options:

- Unified measurement report for both L1 and L3 measurement.

- Unified measurement report based on the NW configured parameters merging L1 and L3.

- Conclusions

In this contribution, we discussed mobility related issues and made the following observations and proposals:

Observation 1: The 5G cell switch mechanisms triggered by different node (i.e., NW triggered, or UE triggered) based on cross layer mechanism (e.g., L3 HO or L1/L2 based cell switch) aim to different scenarios and requirements.

Observation 2: Non-connected mobility management and connected mobility management have many similarities, including their fundamental principles, part of measurement objects, and types of measurement parameters.

Observation 3: In 6GR, the design of system information and sync signals to achieve network energy saving may affect the cell selection/reselection.

Proposal 1: A unified configuration framework could be used for different cell switch mechanism (e.g., UE triggered and NW signalling based switch mechanism).

Proposal 2: Introduce a cell switch mechanism for cross layer switch (e.g., L1/L2/L3 cell switch) based on a unified framework.

Proposal 3: Consider the unified the design for the cell switch and beam management. Inter-WG work with RAN1 is required.

Proposal 4: Introduce both NW triggered and UE triggered Early DL/UL synchronization.

Proposal 5: Consider 5G cell selection/reselection process as a starting point and try to optimize it for simplicity and energy efficiency:

- Design a unified cell quality evaluation criterion for both connected and non-connected UEs.

- Identify the impact from the design of 6G system information and sync signals on the cell selection/reselection process.

Proposal 6: A unified RS resource framework can be considered for both L1 and L3 measurement.

Proposal 7: RAN2 to study the unified measurement report mechanism and further study the follows options:

- Unified measurement report for both L1 and L3 measurement.

- Unified measurement report based on the NW configured parameters merging L1 and L3.

- References

- RP-252912 Revised SID: Study on 6G Radio