3GPP RAN WG2 Meeting #132                                                                                R2-2508597

Dallas, US, Nov. 17th – 21st, 2025

Agenda Item:	10.3.2.3

Source:	HONOR

Title:	Discussion on System Information for 6GR

Document for:	Discussion and Decision

- Introduction

In the last meeting, RAN2 has reached an agreement on the basic goals and requirements about system information [1]:

Agreements on System Information

- System information design should consider energy efficiency and low complexity for both network and UE.

- System information design should ensure that the public warning requirements are met.

- Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

- Study how to improve flexibility and extensibility of scheduling SI design

- Study mechanisms to improve SI update mechanism

- Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

- Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

In this contribution, we will analyse the system information related issues in 6GR based on the above starting list.

- Discussion

  - Baseline for System Information

System information contains the essential data required by the UE to access and camp on a cell. In 5G, the design of system information has considered to address UE and network energy consumption concerns and system efficiency, which include the following aspects:

-	System information is categorized into Minimum System Information (MSI) and Other System Information (OSI). MSI is always broadcast periodically, while OSI can be broadcast either periodically or upon UE request.

-	With the change of SIBs, the RAN node will notify the update via SI change indication and update in the next Modification Period. Take DRX into account, the notification and update scheme are beneficial for UE power saving.

-	Receiving the notification or cell change, the UE needs to determine whether the stored system information is valid according to valueTag and areaScope. If it is invalid, the UE needs to update it in time.

In previous discussions regarding 6G, there has been a consensus to adopt a native and joint design approach to achieve energy efficiency and savings for both the network and the UE [2]. Therefore, it is feasible to reuse some of the system information transmission mechanisms from 5G. However, considering that 6G may introduce new scenarios and technologies, such as artificial intelligence (AI) applications, the contents of each system information blocks remain an open issue for discussion.

Proposal 1: System information design of 6G can reuse some 5G system information transmission mechanisms, specifically:

- Inherit the concepts of system information classification: Minimum SI and Other SI. It is FFS to discuss the content of MSI based on 6G scenarios and requirements.

- Support system information broadcast periodically or upon UE request.

- Consider the existing rules of SIB validity across time domain and spatial domain as a baseline.

- Support to update system information based on system information change indication. Details need to be further discussed in normative work.

Considering new traffic, new terminal, new services (e.g. AI/Sensing), RAN Plenary is discussing a limited set of device type to reduce the design complexity of 6GR while meeting different performance requirements in different scenarios [3]. Compared to system information introduced feature by feature in NR, it is necessary to consider the impact of introducing device type on system information broadcast, and how to design an efficient broadcast for System Information.

Proposal 2: Study how to design the system information for various device type.

  - Consideration on SIB1

2.2.1 Content of SIB1 for 6GR

In NR, SIB1 carries a lot of information, including information required for initial access, the scheduling of other system information blocks, and prohibition information required for unified access control. However, the size of SIB1 is limited to 2976 bits, which lead to re-examine the parameters that must be carried in SIB1 in the later release.  Considering more device types and 6G features, the information required for access or camping may increase greatly and it is necessary to study which information could be taken in SIB1.

Observation 1: There is a limit to the maximum message size of SIB1.

From the perspective of change frequency, in 5G, updating OSI will lead to changes in the OSI scheduling information in SIB1, which will result in frequent updates and broadcasts of SIB1. However, apart from OSI scheduling information, the other part in SIB1, such as cell selection or access prohibition information does not actually change frequently. Therefore, it is necessary to study how to avoid frequently broadcasting the unchanged information. For example, taking SIB1 in 5G as the baseline, the content could be divided into two part and the SIB1 for 6GR only includes the information changing infrequently, while OSI scheduling information could be broadcast separately (e.g. included in the other SIB), but this also forces the UE to perform additional information acquisition, increasing UEs’ energy consumption.

Observation 2: Some parts in SIB1 does not change frequently while the other parts changes with OSI update, i.e. SI scheduling information. And the UE should always re-acquire the unchanged part with unnecessary energy wastage.

For 5G OSI, it could be defined as area-level valid which could reduce OSI acquisition if the cell changes. SIB1 could be only cell-level valid, and the UE should acquire SIB1 immediately after cell change. Actually, the parameters in SIB1, e.g. parameters for initial access like barring bits, could also be the same for different cells and areaScope could be considered for these parameters. That is, we could consider dividing the existing SIB1 into a cell-valid part and an area-valid part. This could reduce the repeated acquisition of the complete SIB1 when the UE moves within the region. The transmission modes and cycles of the two parts can also be considered separately to achieve further energy savings.

Observation 3: The parameters in SIB1, e.g., parameters for initial access like barring bits, could be also the same for different cells.

Proposal 3: Study the content of SIB1 for 6GR from different dimensions, such as update frequency, device type, functionality, and valid scope.

2.2.2 Broadcast Mode

- Content of SIB1 for 6GR

It is worth noting that in the initial 5G spec, the SIB1 will broadcast every 20ms and OSI can be broadcast in a UE-triggered manner. In release 19, RAN2 has studied how to improve the network energy saving efficiency with reducing SIB1 broadcasting which include extending the broadcast periodicity or OD-SIB1. Several different scenarios were proposed for OD-SIB1 in R19, but considering time and workload constraints, only case2 was standardized [4]. Since energy efficiency remains an important issue in 6G architecture design, it is recommended to further study how to broadcast SIB1 efficiently, e.g. extend the broadcast period, and OD-SIB1 from 6G Day-1.

Proposal 4: RAN2 should study how to broadcast SIB1 efficiently, e.g. extend the broadcast period and OD-SIB1 from 6G Day-1.

The standardized scenarios in R19 should be considered as a baseline. Further research can be conducted on scenarios proposed in Release 19 but not yet extensively studied, such as standalone NES cell scenarios. New scenarios that combine OD-SIB1 with other features improving system energy efficiency can also be considered, such as combining OD-SIB1 with Cell DRX, combining OD-SIB1 with SSB adaptation, and binding OD-SIB1 and UE-triggered OD-SSB for transmission, etc.

Proposal 5: Keeping R19-standardized OD-SIB1 as a baseline, study extended scenarios for OD-SIB1.

2.3 Notification of System Information Change

In NR, the UE will be notified the change of OSI and then decide whether to acquire the update SIB via checking scheduling information from the latest SIB1. If the UE does not need to acquire the update SIB, it is redundant to acquire the SIB1 in the next MP of SI change notification. This will delay the UE from entering sleep mode and cause unnecessary power consumption.

We believe that system information can be grouped into multiple levels based on device type, service type, features, necessity, etc. OSI changes can be indicated level by level in a more granular manner through the content and/or transmission method of SI change notification, so that the UE can be specifically instructed to acquire the updated OSI scheduling information and OSI only when the system information related to it changes.

Proposal 6: Support fine-grained indication of OSI changes from multiple dimensions, e.g. device type level, feature-level, in SI change notifications.

2.4 Consideration on OSI

In legacy spec, OSI supports periodic broadcasting or broadcasting based on UE requests. Regardless of the broadcasting method, the NW always transmits complete SIBs within the cell. While this ensures that the UE can acquire the complete SIBs whenever it arrives at the cell, it incurs significant signalling overhead and energy consumption. Several companies have proposed using delta configuration and/or delta transmission to reduce redundant transmission during OSI broadcasting and updates, avoiding unnecessary energy wastage. However, there is no consensus on the understanding of delta configuration and/or delta transmission currently. It is recommended that RAN2 first clarify what delta operation for system information is, and then further study whether and how to support delta configuration and/or delta transmission.

Proposal 7: RAN2 to align the understanding of delta operation for system information. FFS: Whether and how to support delta operation for system information.

- Conclusions

In conclusion, we propose the followings for System Information of 6GR.

Observation 1: There is a limit to the maximum message size of SIB1.

Observation 2: Some parts in SIB1 will not changes frequently while the other parts changes with OSI update, i.e. SI scheduling information. And the UE should always re-acquire the unchanged part with unnecessary energy wastage.

Observation 3: The parameters in SIB1, e.g. parameters for initial access like barring bits, could be also the same for different cells.

Proposal 1: System information design of 6G can reuse some 5G system information transmission mechanisms, specifically:

- Inherit the concepts of system information classification: Minimum SI and Other SI. It is FFS to discuss the content of MSI based on 6G scenarios and requirements.

- Support system information broadcast periodically or upon UE request.

- Consider the existing rules of SIB validity across time domain and spatial domain as a baseline.

- Support to update system information based on system information change indication. Details need to be further discussed in normative work.

Proposal 2: Study how to design the system information for various device type.

Proposal 3: Study the content of SIB1 for 6GR from different dimensions, such as update frequency, device type, functionality, and valid scope.

Proposal 4: RAN2 should study how to broadcast SIB1 efficiently, e.g. extend the broadcast period and OD-SIB1 from 6G Day-1.

Proposal 5: Keeping R19-standardized OD-SIB1 as a baseline, study extended scenarios for OD-SIB1.

Proposal 6: Support fine-grained indication of OSI changes from multiple dimensions, e.g. device type level, feature-level, in SI change notifications.

Proposal 7: RAN2 to align the understanding of delta operation for system information. FFS: Whether and how to support delta operation for system information.

- References

- R2_131bis_ChairNotes_25-10-17_13-00

- 6GWS-250238_3GPP_6G_Workshop_Summary

- RP-252873 Summary of 6G diverse device types

- R2-2409210 Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands