3GPP TSG RAN WG2 Meeting #132                                                                   R2-2508627

# Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:	10.3.1.1

Source:	Ofinno

Title:	6G User plane functionalities

Document for:	Discussion and decision

# Introduction

The following agreements were made for user plane in RAN2#131bis meeting:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

Based on the previous agreements, especially on the highlighted ones, this contribution provides our views on the functionalities for 6G user plane.

# Discussion

## SDAP Functionalities

5G SDAP functionalities [1]

Figure 1: 5G SDAP functionalities

- Mapping between QoS flows and RBs: For DL and UL, SDAP maps QoS flows to DRBs.

- Marking QoS flow IDs in packets: SDAP marks the QoS flow ID (QFI) in both DL and UL: packets, which is essential for QoS handling. This is done by adding an SDAP header to the data packet when configured.

- Reflective QoS Flow to DRB Mapping: For UL data, SDAP supports a reflective mapping mechanism. This allows a UE to monitor the QoS flow to DRB mapping used in the DL and apply the same mapping rule for the UL. This function is performed at the UE if a DL SDAP header is configured.

Limitations in 5G SDAP functionalities

A key objective for 6G is to design a leaner and more efficient user plane architecture by removing redundant sublayers and minimizing protocol processing overhead. In the 5G NR user plane, the SDAP was introduced as a new sublayer primarily to handle QoS flow to DRB mapping and to support Reflective QoS. While conceptually enabling flexibility, some limitations have been identified through both field deployment experience and implementation feedback. For example, SDAP’s core functions—QoS flow mapping and QFI marking—could be performed within PDCP without requiring a separate sublayer. The coexistence of SDAP and PDCP introduces duplicated sequence handling, buffering, and PDU processing logic. In NR, one SDAP entity may be associated with multiple PDCP entities. This complicates QoS flow remapping during handover or DRB reconfiguration, making in-order delivery and buffering management more complex.

Study on 6G SDAP functionalities

To address these issues and align with the 6GR goal of a simplified, hardware-friendly, and low-overhead user plane, the study to remove or integrate the SDAP into PDCP is recommended:

- Study Aspects: Evaluate the removal of SDAP as a standalone sublayer. Its essential functions (QoS flow to bearer mapping and QFI marking) could be integrated into other layer e.g., a 6G PDCP layer. If integration is adopted, RAN2 still would require to define a clear QoS to bearer mapping model supporting associations to single and/or multiple PDCP entities.

- Expected benefit:

  - Fewer protocol headers and simplified packet processing.

From a system design perspective, the 5G user plane architecture introduced additional layering complexity with the SDAP sublayer, which has shown limited functionalities and created additional protocol processing overhead.

RAN2 to study whether to remove SDAP and integrate its essential functions (e.g., QoS flow to bearer mapping and QFI marking) into the 6G PDCP layer. The study should also consider how 6G PDCP layer would provide a clear QoS to bearer mapping model supporting associations to single and/or multiple PDCP entities.

## PDCP Functionalities

5G PDCP functionalities [2]

Figure 2: 5G PDCP functionalities

- Security functions:

  - Ciphering and deciphering: PDCP performs encryption and decryption for both user plane data (on DRBs) and control plane data (on SRBs), if configured. This applies to the MAC-I and the data part of the PDCP Data PDU.

  - Integrity protection and verification: This function provides integrity for control plane data on SRBs and can be configured for user plane data on DRBs.

- Header compression: PDCP can compress and decompress packet headers. The protocols supported include:

  - ROHC (RObust Header Compression) for IP packets.

  - EHC (Ethernet Header Compression) for Ethernet packets.

  - UDC (Uplink Data Compression), which is a DEFLATE-based compression for uplink user data.

- Reordering and in-order delivery: The receiving PDCP entity can reorder received packets to ensure they are delivered in sequence to the upper layers. A reordering timer is used to detect lost packets.

- Duplicate discarding: PDCP is responsible for discarding duplicate packets received from lower layers.

- SDU/PDU set Discard: PDCP supports a timer-based SDU discard function, where SDUs that have been buffered for too long are discarded. A PDU set discard operation was introduced in Rel-19 for XR traffic. The UE may discard all packets in a PDU set when one PDU belonging to this PDU set is discarded due to discard timer expiry.

- Routing and PDCP duplication:

  - For split bearers in DC or DAPS bearers, the transmitting PDCP entity performs routing, deciding which path (e.g., MCG or SCG) to send the data on.

  - PDCP also supports packet duplication, where the same PDCP PDU is sent over multiple RLC entities to increase reliability, a feature particularly useful for URLLC services.

- Status reporting: PDCP can generate and process status reports to handle retransmissions and confirm successful delivery of data.

- Data recovery: PDCP supports a data recovery procedure, for instance, after a handover, to retransmit PDCP Data PDUs that were not successfully delivered by the source cell.

Limitations in 5G PDCP functionalities

In 5G NR, the PDCP discard mechanism is primarily static and configuration-based, relying on fixed discard timers defined per bearer. While sufficient for relatively stable traffic patterns, this design presents several limitations when applied to highly dynamic or bursty service types expected in 6G (e.g., interactive XR, AI inference, and real-time sensor fusion):

- Lack of traffic adaptiveness: The discard policy is not dynamically responsive to real-time variations in traffic arrival, service delay tolerance, or buffer occupancy. As a result, PDCP may retain stale packets even when their associated service window has expired.

- Increased memory usage and buffer pressure: Static discard timers lead to accumulation of outdated PDUs, increasing memory footprint and introducing backpressure on RLC and MAC layers, especially under congestion or fluctuating uplink throughput.

- Reordering delay and latency impact: Retention of obsolete PDUs in the buffer can delay in-order delivery of newer packets, especially for delay-sensitive traffic. This induces Head-of-Line (HOL) blocking at the PDCP reordering stage.

- Inflexibility for heterogeneous QoS flows: In NR, the discard mechanism operates uniformly per bearer, without the ability to apply differentiated policies per flow, per PDU set, or per service context, limiting efficiency for multi-service or multi-stream applications.

Study on 6G PDCP functionalities

To address these limitations, the 6G user plane should support adaptive SDU/PDU (set) discard mechanisms in PDCP, allowing more intelligent and context-aware buffer management. For example, enhancing the PDCP discard function to be dynamic and service-aware, enabling the protocol to adjust discard decisions based on real-time traffic conditions, latency budgets, and QoS requirements. The discard behavior should be applicable at both individual SDU/PDU level and PDU-set level (e.g., grouped frames for XR or AI tasks).

#### Study Aspects:

- Adaptive discard control: Allow the discard timer or threshold to dynamically adjust according to buffer occupancy levels, observed delay/jitter tolerance, Service or QoS profile (e.g., survival time, importance indicator), etc.

- Per-service or per-flow discard policy: Support differentiated discard configurations based on service type, PDU-set priority, or latency requirement, rather than a single static timer per bearer.

### Expected Benefits:

- Lower memory usage and reduced buffering delay across PDCP and RLC.

- Reduce latency, particularly for real-time or interactive traffic.

- Avoidance of Head-of-Line blocking due to outdated or expired PDUs.

- Better energy efficiency by reducing unnecessary retransmission or processing of obsolete data.

- Enhanced adaptability for diverse 6G service categories with variable data lifetimes.

The static, configuration-based PDCP discard mechanism in 5G lacks adaptability to real-time traffic dynamics, leading to inefficient buffer management and increased latency. Its uniform per-bearer policy causes stale packet retention, memory pressure, and Head-of-Line blocking, limiting performance for heterogeneous and latency-sensitive 6G services.

RAN2 to study adaptive SDU/PDU (set) discard mechanisms. The study should consider dynamic, service-aware discard control that adjusts to traffic patterns, buffer conditions, and/or QoS requirements.

## RLC Functionalities

5G RLC functionalities [3]

Figure 3: 5G RLC functionalities

- Data transfer modes: The RLC sublayer supports three distinct modes for data transfer:

  - Transparent Mode (TM): Used for logical channels like BCCH, PCCH, and CCCH. It involves no segmentation and adds no RLC headers; it simply passes RLC SDUs to lower layers.

  - Unacknowledged Mode (UM): Used for DTCH, MCCH, MTCH, SCCH, and STCH. It supports segmentation and reassembly of RLC SDUs but does not provide acknowledged delivery.

  - Acknowledged Mode (AM): Used for DCCH, DTCH, SCCH, and STCH. It is the most complex mode, providing reliable data delivery.

- Error correction through ARQ: In AM, RLC performs ARQ. The receiving RLC entity can detect lost PDUs and request retransmissions from its peer entity by sending a STATUS PDU. This is a key function for ensuring reliable data delivery.

- Segmentation, re-segmentation, and reassembly:

  - For UM and AM, the RLC sublayer segments RLC SDUs into smaller RLC PDUs to fit the transmission opportunities indicated by the MAC layer.

  - The receiving entity reassembles these segments back into the original RLC SDU. A reassembly timer is used to detect the loss of PDUs.

  - In AM, RLC can also re-segment an already transmitted RLC SDU segment into smaller segments for retransmission.

- Duplicate detection: In AM, the receiving RLC entity is responsible for detecting and discarding duplicate RLC PDUs.

- RLC SDU discard: Both UM and AM support the discarding of RLC SDUs, which can be triggered by PDCP.

Limitations in 5G RLC functionalities

The RLC AM in NR was designed to ensure reliable delivery through retransmissions and selective repeat ARQ mechanisms. While it achieves high reliability, its design introduces substantial latency penalties that conflict with emerging 6G service requirements for sub-millisecond or deterministic delay.
Key limitations are summarized as follows:

- Contradictory latency and reliability trade-off: In 5G, RLC AM is optimized for reliability, not latency. Once a packet enters the RLC retransmission window, delivery delay becomes unbounded, limited only by PDCP discard timer. This makes it unsuitable for ultra-low-latency or high-rate real-time applications, even when reliability needs are moderate.

- Excessively long ARQ feedback cycle: RLC ARQ feedback and retransmission typically require tens to hundreds of milliseconds, as retransmission is triggered only after the t-Reassembly timer expires or after a polling cycle. This timer is deliberately set long to avoid false retransmissions, but it causes significant and variable delay for recovery.

Study on 6G RLC functionalities

### In 5G NR, RLC reliability is configured per bearer (with AM or UM RLC entity) and cannot adapt dynamically to per packet or per group of packets service requirements. This coarse granularity leads to inefficiency for mixed-content services (e.g., video I/P/B frames, AI sensor streams) where some packets can tolerate loss while others require guaranteed delivery. Allowing packets or a group of packets from the same radio bearer to be (re-)transmitted via different RLC entities and/or via different RLC transmission modes (e.g., AM/UM) within the same RLC entity introduces dynamic, context-aware reliability decisions that improve both latency and efficiency.

### RAN2 to study the introduction of differentiated handling at 6G RLC layer for per packet or per group of packets via different RLC entities and/or via different RLC transmission modes within the same RLC entity (e.g., AM/UM), enabling the network or UE to determine (re-)transmission behavior for each individual PDU/SDU or a group of PDU/SDU. The study should enable  fine-grained, adaptive reliability control within a single bearer and reducing latency for loss-tolerant traffic in 6G without the need to switch bearer configuration.

Study Aspects: Define packet or group of packets transmission via different RLC entities and/or via different RLC transmission modes (e.g., AM/UM) within the same RLC entity, e.g., based on the requirement of the packet is reliable (e.g., lossless) or loss-tolerant (e.g., no retransmission).

Expected Benefits:

- Granular reliability control at packet level or a group of packets level, enabling optimal treatment for diverse traffic within one radio bearer.

- Reduced average latency by skipping retransmissions for non-critical packets.

- Improved spectral and energy efficiency through elimination of unnecessary retransmissions.

In 5G, the RLC layer’s reliability configuration is fixed per bearer, preventing adaptive handling of mixed-content traffic with varying reliability needs. This coarse granularity results in inefficiencies for heterogeneous services, as loss-tolerant and delay-sensitive packets are treated uniformly. A more dynamic 6G RLC design is needed to enable per-packet or per-group packet reliability control, improving both latency and resource efficiency without requiring separate bearers.

RAN2 to study whether/how to support packet or group of packets from the same radio bearer to be (re-)transmitted via different RLC entities and/or via different RLC transmission mode (e.g., AM/UM) within the same RLC entity.

## MAC Functionalities

5G MAC functionalities [4]

Figure 4: 5G main functions of MAC

- Random Access (RA) procedure: MAC initiates and manages the RA procedure, which is triggered for different events, e.g., initial access, re-establishment, handover, and when UL synchronization is lost, etc.

- Scheduling and resource allocation:

  - MAC handles logical channel prioritization (LCP), deciding which data to send first based on priority and token-bucket algorithm. Logical channel mapping restrictions can also be configured to control which resources a logical channel can use.

  - MAC supports Semi-Persistent Scheduling (SPS) for downlink and Configured Grants (CG) (Type 1 and Type 2) for uplink, allowing for pre-allocated resources without dynamic grants.

- Data multiplexing: MAC multiplexes MAC SDUs from various logical channels into TBs to be sent to the physical layer. In the reverse direction, it demultiplexes TBs into MAC SDUs for the correct logical channels.

- Error correction through HARQ: MAC performs HARQ for error correction on the DL-SCH and UL-SCH. This involves managing HARQ processes for retransmissions.

- Information reporting: MAC triggers Scheduling Request (SR), Buffer Status Reports (BSR), Delay Status Report (DSR), Power Headroom Reports (PHR), and Timing Advance Reporting (TAR) from the UE to assist the network.

- Power saving: MAC manages DRX to save UE power, controlling when the UE monitors the PDCCH.

- BWP/Cell adaptation: MAC preforms BWP switch and SCell activation/deactivation to adapt the resources in frequency/spatial domain.

Limitations in 5G MAC functionalities

In 5G, BSR and DSR procedures are generated and transmitted separately. However, a large portion of the information contained in BSR and DSR is overlapping, causing an additional burden for the UE.

The MAC status reporting (BSR/DSR) is typically limited to LCG granularity. This coarse granularity means the network cannot know the amount of data or delay-critical data for a specific QoS flow. This limits dynamic, cross-LCH information needed for multi-modal, tokenized, or immersive services.

Study on 6G MAC functionalities

Study aspects: RAN2 to study how to combine BSR and DSR into a single BSR/DSR procedure. In the single BSR/DSR procedure, study what’s the essential content of a single BSR/DSR MAC CE and how to design the formant. In 5G, the triggering conditions of the BSR are different from the DSR. We should also study how to trigger this single BSR/DSR procedure by considering the 5G triggering conditions. In 5G, finer granularity of the BSR/DSR reporting can be studied, e.g., the report can be indicated per LCH or per QoS flow.

Expected Benefits:

- Reduced MAC CE overhead and improved control efficiency.

- Lower UE processing load and simplified MAC CE design.

- More accurate uplink scheduling decisions through real-time buffer and delay visibility.

The separation of BSR and DSR procedures in 5G leads to redundant reporting and additional UE overhead, while their course LCG-level granularity prevents the network from accurately identifying delay-critical or flow-specific data. This lack of unified and fine-grained status visibility limits the scheduler’s ability to optimize uplink resources for emerging 6G multi-modal and immersive service types.

RAN2 to study whether/how to combine BSR/DSR functionalities (e.g., including procedure, content/format, triggering conditions, and graduality of the reporting).

# Conclusion

Based on the analysis above, we have the following observations and proposals:

- From a system design perspective, the 5G user plane architecture introduced additional layering complexity with the SDAP sublayer, which has shown limited functionalities and created additional protocol processing overhead.

Proposal 1	RAN2 to study whether to remove SDAP and integrate its essential functions (e.g., QoS flow to bearer mapping and QFI marking) into the 6G PDCP layer. The study should also consider how 6G PDCP layer would provide a clear QoS to bearer mapping model supporting associations to single and/or multiple PDCP entities.

The static, configuration-based PDCP discard mechanism in 5G lacks adaptability to real-time traffic dynamics, leading to inefficient buffer management and increased latency. Its uniform per-bearer policy causes stale packet retention, memory pressure, and Head-of-Line blocking, limiting performance for heterogeneous and latency-sensitive 6G services.

Proposal 2	RAN2 to study adaptive SDU/PDU (set) discard mechanisms. The study should consider dynamic, service-aware discard control that adjusts to traffic patterns, buffer conditions, and/or QoS requirements.

In 5G, the RLC layer’s reliability configuration is fixed per bearer, preventing adaptive handling of mixed-content traffic with varying reliability needs. This coarse granularity results in inefficiencies for heterogeneous services, as loss-tolerant and delay-sensitive packets are treated uniformly. A more dynamic 6G RLC design is needed to enable per-packet or per-group packet reliability control, improving both latency and resource efficiency without requiring separate bearers.

Proposal 3	RAN2 to study whether/how to support packet or group of packets from the same radio bearer to be (re-)transmitted via different RLC entities and/or via different RLC transmission mode (e.g., AM/UM) within the same RLC entity.

The separation of BSR and DSR procedures in 5G leads to redundant reporting and additional UE overhead, while their course LCG-level granularity prevents the network from accurately identifying delay-critical or flow-specific data. This lack of unified and fine-grained status visibility limits the scheduler’s ability to optimize uplink resources for emerging 6G multi-modal and immersive service types.

Proposal 4	RAN2 to study whether/how to combine BSR/DSR functionalities (e.g., including procedure, content/format, triggering conditions, and graduality of the reporting).

# Reference

3GPP TS 37.324: “NR; Service Data Adaptation Protocol (SDAP) specification.”

3GPP TS 38.323: “NR; Packet Data Convergence Protocol (PDCP) specification.”

3GPP TS 38.322: “NR; Radio Link Control (RLC) protocol specification.”

3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification.”