3GPP TSG-RAN WG2 Meeting #132	R2-2508664

Dallas, USA, 17 - 21 November 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	LTM corrections

Document for: Discussion and Decision

# 1	Introduction

This contribution discusses several issues on MAC and RRC for LTM.

# 2	MAC issues

Clause 5.32.5 is specified as follows:

### 5.35.2	Performing measurement

An RRC_CONNECTED UE obtains L1 beam level measurement results by measuring one or multiple RSs as configured by the network as specified in TS 38.214 [7] for the LTM candidate cell(s) with the candidate ID configured in ltm-CandidateReportConfigList for evaluation of reporting criteria or of execution condition. For each L1 beam level measurement result in RRC_CONNECTED, the UE applies the layer 1 filtering by implementation, before using the measured results for evauation of reporting criteria and measurement reporting or of execution condition. When the UE has two indicated TCI-states, the UE uses the best beam of serving cell is used for LTM event evaluation. It is up to the UE implementation how to choose the best beam. The MAC entity performs the evaluation of reporting criteria as specified in 5.35.3 or of execution condition as specified in 5.36.2 based on the L1 measurement results from lower layer. For the LTM candidate cell(s) with the candidate ID not configured in ltm-CandidateReportConfigList, the UE is not required to perform and the event evaluation in 5.35.3 on the RSs belonging to the candidate ID for the corresponding event.

For L1 beam level event triggered measurements report, the network can configure SS/PBCH block(s) or CSI-RS as event evaluation RS type, and L1-RSRP as trigger quantity. Reporting quantity is the same as the trigger quantity.

TS 38.214 only specifies measurement and reporting "For a report setting ltm-CSI-ReportConfig configured with ltm-ReportConfigType set to ‘periodic’ or ‘semiPersistentOnPUCCH’ or ‘semiPersistentOnPUSCH’ or ‘aperiodic’" i.e., it does not cover the case of L1 event-triggered measurements or L1 execution conditions.

Proposal 1: Remove the reference to TS 38.214 in TS 38.321 clause 5.32.2 because TS 38.214 does not cover measurements for L1 event-triggered reports or L1 execution conditions.

5.35.2 includes considerations on:

-	L1 filtering;

-	determination of the current beam when two TCI states are indicated;

- 	which cell to measure for event-triggered evaluation in 5.35.3;

-	performing evaluation of reporting criteria in 5.35.3 or of execution condition in 5.36.

However, 5.35.3.1 determine the "applicable RS", which are exactly the ones to be measured, but 5.35.2 repeats that, in a different language, and with English language mistakes. Moreover, in 5.32.2, it is not specified for which LTM-CSI-ReportConfig this is applicable, while this is specified in 5.35.3.1:

The MAC entity shall:

1>	for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event triggered measurement report:

Proposal 2: Remove from 5.32.2 all considerations of which cell to measure (because this is redundant with 5.35.3.1) and add in 5.35.3.1 to derive L1 measurement results for applicable RS, following the indications in 5.32.2.

### 5.35.2	Performing measurement

For L1 measurements performed for the evaluation of reporting criteria as specified in 5.35.3 or of execution condition as specified in 5.36.2, the MAC entity shall:

-	 apply layer 1 filtering by implementation, before using the measured results for evaluation of reporting criteria and measurement reporting or of execution condition;

-	when the UE has two indicated TCI-states, use the best beam of the serving cell. It is up to the UE implementation how to choose the best beam;

And in 5.36.3 for conditional LTM evaluation:

#### 5.35.3.1	General

[...]

The MAC entity shall:

The MAC entity shall:

1>	for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event triggered measurement report:

2>	if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration:

3>	if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if the associated LTM-CSI-ResourceConfig includes ltm-NZP-CSI-RS-ResourceSet:

5>	consider the current beam of serving cell, i.e. the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

4>	else:

5>	consider the current beam of serving cell, i.e. the SSB QCLed with the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

3>	if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if ltm-CandidateReportConfigList is configured:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed in ltm-CandidateReportConfigList, to be applicable.

4>	else:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed, to be applicable.

2>	derive L1 measurement results for the applicable RS, as specified in clause 5.35.2;

On which cell/RS the UE shall perform measurements is entirely determined by the LTM-CSI-ReportConfig for

As mentioned above, for which RS to perform measurement for execution condition evaluation is not specified in 5.35.2, but 5.36.2 specifies the "applicable beams", instead of RS. Also, the wording is extremely obfuscated.

Proposal 3: Specify in 5.36.2 that the UE shall derive L1 measurement results for applicable RS as specified in 5.35.2 (to make the consideration on "best beam" apply to execution conditions).

Proposal 4: Change in 5.36.2 "beam" to "RS" and simplify the terribly obfuscated text.

### 5.36.2	L1 measurement based Conditional LTM triggering condition evaluation

Upon indication from upper layers to initiate the LTM cell switch conditions evaluation based on L1 measurements, the MAC entity shall:

1>	for each entry within the VarLTM-ExecutionConditionList with executionCondition set to l1-Conditions:

2>	consider the RS of the ltm-CandidateId that are in the LTM-CSI-ResourceConfig indicated by ltm-ResourcesForChannelMeasurement of the ltm-CSI-ReportConfigId in this entry to be applicable;

2>	derive L1 measurement results for the applicable RS, as specified in clause 5.35.2;

2>	if the entry condition for the event associated with the ltm-CSI-ReportConfigId in this entry is fulfilled for TTT for one or more applicable RS:

3>	consider the event associated with LTM-CSI-ReportConfigId to be fulfilled for the ltm-CandidateId associated with LTM-CSI-ReportConfigId;

3>	perform the CLTM execution procedure for the LTM candidate configuration associated with ltm-CandidateId according to the procedure specified in 5.36.3.

# 2.2	RRC issues from ASN.1 review

To be added in a later revision (after RRC rapporteur has provided feedback).

# 3	Conclusion

In this contribution, we have listed necessary corrections for MAC and RRC and provided TPs.

MAC corrections -see TPs in the discussion part

Proposal 1: Remove the reference to TS 38.214 in TS 38.321 clause 5.32.2 because TS 38.214 does not cover measurements for L1 event-triggered reports or L1 execution conditions.

Proposal 2: Remove from 5.32.2 all considerations of which cell to measure (because this is redundant with 5.35.3.1) and add in 5.35.3.1 to derive L1 measurement results for applicable RS, following the indications in 5.32.2.

Proposal 3: Specify in 5.36.2 that the UE shall derive L1 measurement results for applicable RS as specified in 5.35.2 (to make the consideration on "best beam" apply to execution conditions).

Proposal 4: Change in 5.36.2 "beam" to "RS" and simplify the terribly obfuscated text (see TP).

RRC corrections