3GPP RAN WG2 Meeting #132	R2-2508683

Dallas, USA,  November 17th – 21st, 2025

Agenda Item:	10.3.2.3

Source:	InterDigital, Inc.

Title:	Initial and System Access and Others for 6GR.

Document for:	Discussion

# Introduction

The study on 6G contains the following initial and system access and the spectrum aggregation related aspects.

#### 10.3.2.3		Initial and System Access and Others

Including contributions on initial and system access, including system information, paging, random access, etc.   Contributions can include aspects related on on-demand SIB, SSBs, SI update mechanism, SIB1 size, area specific SIBs, etc.  and understanding of problems to address.

This contribution identifies initial and system access issues for 6G control plane. The aim is to highlight where NR design created inefficiency or complexity, and where 6G can deliver simplification, scalability, and improved energy efficiency. In addition, this contribution includes the discussion on the spectrum aggregation, which was discussed in the last RAN2 meeting [2].

# Discussion

## 2.1	6G CP Design Considerations for initial access

Control plane design for 6G should be lean and scalable, avoiding the fragmentation and inefficiencies observed in NR. The following drawbacks from NR should be considered as lessons learned, and the following design targets should guide 6G.

The following drawbacks from NR should be considered for 6G initial and system access:

- Always-on SSB/SIB1 created high network and UE energy consumption, especially in multi-carrier deployments.

- System Information (SI) is static (periodically sent even if it doesn’t change), redundant, and unprotected, creating overhead and spoofing risks.

- Idle/Inactive states overlap, with Inactive adoption low due to limited benefit vs. Idle.

- UE capability signalling has become excessively large, with poor support for dynamic changes.

- Access control is coarse and not service- or device-aware, limiting vertical support.

6G offers an opportunity to streamline design for wider adoption and the following design targets should be considered to achieve that:

- Simplification: Reduce signalling overhead, remove redundancy, and streamline RRC state handling.

- Energy efficiency: Enable native support for network and UE energy saving from Day-1.

- Low latency: Provide fast transitions between states, lean reconfiguration, and minimized attach time.

- Service-awareness: Adapt control plane procedures to traffic/service type, device capability, and QoS.

- Scalability: Ensure a single framework supports IoT, XR, NTN, and eMBB devices efficiently.

Observation 1: NR control plane suffers from high overhead, overlapping states, and complex signalling. 6G should use single framework that minimizes complexity while being scalable, energy-efficient, and service-aware to deliver low-latency and robust operation.

## 2.2	Initial Access

In NR, SSB and SIB1 are transmitted periodically and always-on. This is a major source of UE and network power consumption, especially in multi-carrier deployments where each carrier duplicates the overhead. In Rel-19 SIB1-less operation was introduced, however due to backwards compatibility with legacy devices the applicability is somewhat limited.

Always-on design does not scale to diverse devices (IoT, XR, eMBB) or dense deployments. It also prevents efficient spectrum utilization. More importantly, it does not provide sufficient network energy saving (NES), where cells or beams may periodically switch off or operate with reduced signalling. On-demand transmissions cut always on transmission overhead allowing for network power saving. Multi-carrier-aware initial access enables flexible use of coverage and capacity layers. Access mechanisms should also account for cases where SSB may be transmitted with different periodicity or not transmitted from some cells/carriers at a specific time.

Initial access mechanisms should therefore support:

- SSB transmission without mandatory essential SIB (e.g. SIB1) on every carrier, decoupling coverage and capacity layers.

- On-demand SSB

- On-demand SIB1, reducing broadcast overhead and NW power consumption.

- Cases where SSB is transmitted at longer periodicities or temporarily absent on some carriers to enable NES.

Proposal 1: Support on-demand System Information, including reference signal transmission without essential SIB (e.g. on-demand SIB1-less SSB), and on-demand SSB

## 2.3	Spectrum aggregation

In the last RAN2 meeting (RAN2#131-bis [2]), the following agreements were made for the support of spectrum aggregation for 6G.

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Some companies proposed to redefine a cell with multi-carrier radio resources while the others proposed the legacy multi-cell operation had no issue and we could reuse the legacy concept for 6G.

In legacy, there were concepts of an access cell (i.e. PCell) and data delivery/capacity cells (i.e. SCells) and both the access and the capacity cells transmitted reference signals (e.g. SSBs in NR) so that UE can measure the radio quality of the capacity cells and get synchronized with the capacity cells.

For 6G, the access and the capacity multi-carrier deployment may remain and the signalling of the capacity carriers may be improved. For example, if the access and capacity carriers are co-located, then the UE can obtain the timing of the capacity carriers by the access carrier and the UE can estimate the radio quality of the capacity carriers. Thus, it makes sense to support:

- Multi-carrier aware access, where coverage/initial access carriers provide full attach information while capacity carriers may provide lighter or conditional signalling.

Proposal 2: Support multi-carrier aware access, and cases where SSB may not always be present on some carriers.

In legacy, paging signalling imposed some resource wastage because the network did not know the exact UE location and needed to page in a number of cells belonging to the same registration area or RAN area.

For 6G, as mentioned above, spectrum aggregation may be considered and in that case, it makes sense to be aware of the multi-carrier for paging so that the network can page on a specific carrier or only a subset of carriers.

Proposal 3: Support multi-carrier aware paging, where one or more carrier sends paging signals while the other carriers do not.

## 2.4	System Information

In NR, any initial access related information is commonly delivered to the UE via MIB and SIB1 and MIB and SIB1 are always broadcast (though Rel-19 introduces SIB-less operation), while other SIBs are provided on-demand. This reduces overhead compared to legacy LTE but still requires frequent broadcast of SIB1.

6G will support an even wider range of services (eMBB, IoT, NTN, sensing), increasing the diversity of SI. A flexible framework is needed that goes beyond NR’s SIB1/on-demand split and provides scalable, secure SI delivery. The following aspects need to be considered:

- Service-based SI delivery: For example, device type specific SIBs ensures UEs only receive relevant information, lowering power consumption. Rather than every UE receiving the same set of SIBs, the network provides different SI content per service, slice, or device type.
Fine-grained SI granularity: per PLMN/slice, service (or device type), enabling flexible sharing and tailored configurations.

Proposal 4: Support scalable SI delivery for 6G, with fine-grained granularity (different SI content per-PLMN/slice/service/device type) enabling flexible sharing and tailored configurations.

In NR, area specific SIB was introduced and that feature allows UE to utilise the stored area specific SIBs received at a different cell than the current serving cell as long as the new serving cell is within the area scope of the stored area specific SIBs. This allows UE to avoid reacquiring the area specific SIBs after cell reselections while the UE stays in the same system information area. Besides, in legacy, each SIB have own version number so that UEs can be aware of SIB update and the UEs re-acquire only the updated SIBs. The SIB version was controlled by a value tag.

For 6G, the concept of the area specific SIB should be valid as it’s very likely to have a scenario that   exactly the same configuration parameters in some SIBs would be valid across multiple cells. In addition, the value tag mechanism is also useful in 6G.

Proposal 5: Support area specific SIB and value tag for optimal SI acquisition.

## 2.5	Access Control

In NR, access control is relatively static and coarse, mainly distinguishing between broad categories such as MO-Data vs emergency. This approach lacks flexibility and does not adapt to service, slice, or device diversity.

With 6G expected to support IoT, XR, NTN, public safety, and vertical services, static AC is insufficient for several reasons:

- Lack of service or data differentiation: All MO-Data is treated the same, even though some traffic (e.g. XR or safety-critical) requires higher priority.

- Device diversity: IoT or RedCap devices should not compete with XR headsets or smartphones under identical barring conditions.

- Slice and PLMN awareness: Static AC cannot scope barring to a specific slice or PLMN, so barring one service may unintentionally affect others.

- Energy efficiency: Uniform AC leads to repeated retries and wasted signalling, whereas adaptive AC can defer or relax access attempts intelligently.

For 6G, access control should evolve towards an adaptive framework that is service-aware, device-aware, and slice-aware, while minimizing signalling updates through validity/scope mechanisms.

Proposal 6: Support adaptive access control with per-PLMN/slice/service/device-type granularity with reduced AC parameter signalling overhead..

In NR, multiple barring flags were introduced for multiple features, e.g. one for regular UEs, one for RedCap, one for NTN, one for NES and so on… and the spec needed to specify that a specific UE ignores existing barring flags and only considers a specific barring flag applicable for the UE (e.g. NTN capable UE ignores the Rel-15 barring flag in MIB but checks the NTN specific barring flag in SIB1) and so it was redundantly complicated.

For 6G, it’s good idea that the barring mechanism takes into account multiple slices, services and/or device-types from the beginning.

Proposal 7: Support barring mechanism per-PLMN/slice/service/device-type granularity on Day 1

# Conclusion

This contribution outlines initial views on control plane in 6G, and makes the following observation and proposals:

Observation 1: NR control plane suffers from high overhead, overlapping states, and complex signalling. 6G should use single framework that minimizes complexity while being scalable, energy-efficient, and service-aware to deliver low-latency and robust operation.

Proposal 1: Support on-demand System Information, including reference signal transmission without essential SIB (e.g. on-demand SIB1-less SSB), and on-demand SSB.

Proposal 2: Support multi-carrier aware access, and cases where SSB may not always be present on some carriers.

Proposal 3: Support multi-carrier aware paging, where one or more carrier sends paging signals while the other carriers do not.

Proposal 4: Support scalable SI delivery for 6G, with fine-grained granularity (different SI content per-PLMN/slice/service/device type) enabling flexible sharing and tailored configurations.

Proposal 5: Support area specific SIB and value tag for optimal SI acquisition.

Proposal 6: Support adaptive access control with per-PLMN/slice/service/device-type, with reduced AC parameter signalling overhead.

Proposal 7: Support barring mechanism per-PLMN/slice/service/device-type granularity on Day 1.

# References

- RP-251881 "Study on 6G Radio"

- R2-2508002 "RAN2#131-bis Meeting Report", MCC