3GPP TSG-RAN WG2 Meeting #131	R2-2508705

Dallas, USA, 17th - 21st, November 2025

Agenda item:	8.9.2

Source:	Sateliot, Novamint, Nordic, Thales

Title:	On the need for a NAS indication for deciding S&F Cell Suitability

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#131, with the help of illustrative examples, [1] tried to explain how the lack of AS-NAS coordination can lead to suboptimal S&F operation of an NB-IoT device. In particular, there could be situations in which a UE may remain camped on a cell which is considered suitable as per AS conditions but renders useless as per NAS conditions (i.e. NAS procedures will not be initiated in the cell), preventing the UE from considering other potential candidate cells.

This emanates from the fact that AS has not all the information needed to make a decision about the suitability of a cell when operating in S&F mode but needs to rely on NAS layer decision, which can determine whether NAS procedures can be initiated based on:

- Whether NAS currently runs the T3451 timer and the corresponding normative behaviour associated to this; and

- Whether the S&F cell is operated by a satellite whose ID is in the NAS-configured S&F monitoring list.

Both AS and NAS need coordination to process information individually available with them. For instance, NAS requires to know from SIBs whether a cell operates in S&F mode and the satellite ID of the cell. With these two pieces of information, NAS can decide whether a detected S&F cell can be used for any NAS procedure based on whether this satisfies T3451 timer and S&F monitoring list criteria.:

With this background, the objective of this paper is to justify and specify how a simple, but a vital NAS indication or notification can help AS decide on S&F cell suitability.

# 2	Discussion

## 2.1 Cell (Re)Selection enhancements

With the help of illustrative examples as presented in clause 2.1 of [1], it has been sufficiently explained as to how the lack of AS-NAS coordination can easily result in AS selecting a cell that is suitable from AS perspectives – but may not be useful from NAS perspective for the reasons mentioned earlier. In other words, NAS is not able to use an S&F cell or generate any NAS procedure in that S&F cell due to, at least, the following:

- Because of conditions associated with timer T3451, which are normative/mandatory.

- Because of the use of the S&F Monitoring list. Although the use of the S&F Monitoring list has been left to UE implementation, its use could still result in situations where the NAS layer decides not to initiate NAS procedures in a S&F cell of a satellite not belonging to the list, as captured in the following note in clause 4.13.9.1 of [6].

NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

As a result, the UE may remain camped on an S&F cell which is considered suitable as per AS conditions but renders useless as per NAS conditions, preventing the UE from considering other potential candidate cells.

Observation 1: Lack of AS-NAS coordination in the context of S&F can lead to suboptimal operations.

To address the above-mentioned lack of coordination or to avoid a situation where a UE remain camped on a “useless” S&F cell where NAS cannot initiate any NAS procedure, there exist 2 options:

- Option A: letting NAS provide raw information such as the S&F monitoring list and the T3451 for AS to consider these two pieces of information to decide on an S&F cell suitability, or

- Option B: letting NAS simply provide an indication whether an S&F cell is usable from NAS perspectives.

With recent RAN2#131 agreements and a positive response from CT1 in terms of the need of satellite ID as indicated in a new LS reply [2], the current possibility is that the NAS is able to check its criteria in terms of whether a camped on S&F cell is useable from NAS perspectives and indicate to AS so that AS can use it as an extra criterion to decide on an S&F cell suitability for camping on. Further, at the moment AS and NAS specifications are not clear in terms of whether NAS can provide the above-mentioned vital pieces of information such as the S&F monitoring list and/or whether NAS currently runs T3451 to AS.

Observation 2: Currently AS and NAS specifications are not clear in terms of whether NAS provides the S&F monitoring list and/or whether NAS currently runs T3451 to AS.

As it was mentioned earlier, in order for AS to decide on an S&F cell suitability, NAS needs to provide at least two pieces of information and not just the S&F monitoring list. Further, it is inappropriate to send NAS timer information to AS.

Observation 3: For AS to check cell suitability from NAS perspectives, NAS passing the S&F monitoring list on to AS is not enough.

Observation 4: For AS to check cell suitability from NAS perspectives, NAS passing a NAS timer (e.g., T3451) information is inappropriate.

For the above-mentioned reason, Option 1 is not desirable. Hence, a priority has to be given to Option 2.

Proposal 1: RAN2 is respectfully requested to pursue Option B to allow optimal S&F satellite operation.

## 2.2 Cell Suitability

A suitable cell is defined in [3] as a cell on which the UE may camp on to obtain normal service meaning that any NAS procedure should be possible. Given that S&F satellite operation introduces some new NAS behaviours that may render a legacy suitable cell unsuitable, an additional criterion has to be added, for example, in clauses 4.3 and 5.2.4.4 of [3]:

- For an S&F Satellite operation capable UE, a cell operating in S&F mode may not be considered suitable if upper layers indicate that NAS procedures cannot be initiated in this cell (e.g., because of the timer T3451, because of the use of the S&F monitoring list).

Proposal 2: RAN2 is respectfully requested to add a stage-3 description in clauses 4.3 and 5.2.4.4 of TS 36.304 such that AS considers a NAS indication as an extra criterion to decide on S&F cell suitability (related TP provided in the Annex).

# 3	Conclusion

This paper elaborated that how lack of coordination hitherto existing between AS and NAS in terms of selecting a suitable cell in the context of S&F mode operation adversely impacts UE energy consumption while unnecessarily prolonging communication time. After outlining the recent RAN2 decisions to expose required information from AS for the NAS in order to address the escalated cell (re)selection issue, the paper makes the following observations and proposals:

Observation 1: Lack of AS-NAS coordination in the context of S&F can lead to suboptimal operations.

Observation 2: Currently AS and NAS specifications are not clear in terms of whether NAS provides the S&F monitoring list and/or whether NAS currently runs T3451 to AS.

Observation 3: For AS to check cell suitability from NAS perspectives, NAS passing the S&F monitoring list on to AS is not enough.

Observation 4: For AS to check cell suitability from NAS perspectives, NAS passing a NAS timer (e.g., T3451) information is inappropriate.

Proposal 1: RAN2 is respectfully requested to pursue Option B to allow optimal S&F satellite operation.

Proposal 2: RAN2 is respectfully requested to add a stage-3 description in clauses 4.3 and 5.2.4.4 of TS 36.304 such that AS considers a NAS indication as an extra criterion to decide on S&F cell suitability (related TP provided in the Annex).

# Reference

- R2-2506151, On cell (re)selection and idle mode task relaxation for S&F Satellite operation, Sateliot, Thales, Novamint, RAN2#131, Bangalore, India, Aug. 2025.

- R2-2508004, Reply LS on S&F Operation, Dallas, USA, Nov. 2025.

- TS 36.304, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode, V19.0.0, (2025-10).

- TS 24.301, Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3, V19.4.0 (2025-09).

- TS 23.122, Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode, V19.4.0 (2025-09).

- TS 23.401, General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access, V19.4.0 (2025-09).

# Annex: TP for TS 36.304

* * * First Change * * *

## 4.3	Service types in Idle Mode

This clause defines the level of service that may be provided by the network to a UE in Idle mode.

The action of camping on a cell is necessary to get access to some services. Three levels of services are defined for UE:

-	Limited service (emergency calls, ETWS and CMAS on an acceptable cell). It is not applicable to RRC_INACTIVE state.

-	Normal service (for public use on a suitable cell)

-	Operator service (for operators only on a reserved cell)

Furthermore, the cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications), and it is not applicable to RRC_INACTIVE state. Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in a E-UTRAN network:

-	The cell is not barred, see clause 5.3.1;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements.

-	The cell is part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

-	For a CSG cell, the cell is a CSG member cell for the UE;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" TS 22.011 [4], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

-	Except for NB-IoT, if the UE supports authorization of coverage enhancements and upper layers indicated that use of coverage enhancements is not authorized for the selected PLMN:

-	the cell selection criterion S in normal coverage shall be fulfilled;

-	If the UE supports CE mode B and upper layers indicated that CE mode B is restricted:

-	the cell selection criterion S in normal coverage based on values Qrxlevmin and Qqualmin or in enhanced coverage based on values Qrxlevmin_CE and Qqualmin_CE shall be fulfilled.

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

For an S&F Satellite operation capable UE, a cell operating in S&F mode may not be considered as a suitable cell if upper layers indicate that NAS procedures cannot be initiated in this cell.

barred cell:

A cell is barred if it is so indicated in the system information TS 36.331 [3].

reserved cell:

A cell is reserved if it is so indicated in system information TS 36.331 [3].

* * * Next Change * * *

#### 5.2.4.4	Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable for a CN type due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for the CN type for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the TS 25.304 [8]. In case of NR further requirements are defined in the TS 38.304 [38]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

For an S&F Satellite operation capable UE, if the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell operating in S&F mode which is not suitable due to upper layers indicating that NAS procedures cannot be initiated in this cell, the UE may not consider this cell as candidate for reselection as long as the upper layers indication applies.

* * * End of Change * * *