3GPP TSG-RAN WG2 #132	R2-2508742

Dallas, US, November 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | 0147 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on MDT for slicing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of Rel-19 SONMDT features logged MDT is enhanced to support logging information concerning the scenarios that UE is not able to camp on the frequency with highest ranked NSAG. According to RAN2 agreements the UE logs NSAG ID as well as the cell UE camps on upon such failure in camping in highest ranked NSAG. However, the stage 2 specification does not capture what is enabled in logged MDT report according to the RRC TS. |  |  |  |  |  |  |  |  |
| q |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 2 description for logged MDT is corrected to capture that upon UE failing to re-select to a suitable cell that supports the NSAG ID with the highest priority and selecting to a different cell, following information are logged in logged MDT report: - The NSAG ID with the highest priority; - Cell ID of the re-selected cell; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, stage 2 description of the logged MDT feature does not include the UE support on logging NSAG information in the logged MDT report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.4.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is revision 1 of R2-2507838 which was in principle agreed CR in the meeting RAN2#131bis. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

AICH	Acquisition Indicator CHannel

BLER	Block Error Rate

BSSID	Basic Service Set Identifier

CA	Carrier Aggregation

CDMA	Code Division Multiple Access

CHO	Conditional Handover

CN	Core Network

CPICH	Common Pilot CHannel

DAPS	Dual Active Protocol Stack

DCH	Dedicated CHannel

DL	Downlink

DRX	Discontinuous Reception

ECGI	E-UTRAN Cell Global Identifier

E-CID	Enhanced Cell-ID (positioning method)

E-DCH	Enhanced Uplink DCH

EDGE	Enhanced Data rates for GSM Evolution

E-RUCCH	E-DCH Random Access Uplink Control CHannel

eNB	Evolved NodeB

EPLMN	Equivalent PLMN

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FACH	Forward Access CHannel

FDD	Frequency Division Duplex

FIFO	First Input First Output

FPACH	Fast Physical Access CHannel

GERAN	GSM EDGE Radio Access Network

gNB	Next Generation Node B

GNSS	Global Navigation Satellite System

HESSID	Homogenous Extended Service Set Identifier

HOF	Handover Failure

IMEI-SV	International Mobile Equipment Identity Software Version

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

ISCP	Interference on Signal Code Power

LA	Location Area

LTE	Long Term Evolution

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBSFN	MBMS Single Frequency Network

MDT	Minimization of Drive-Tests

NG-RAN	Next Generation RAN

NPN	Non-Public Network

NR	New Radio

NSAG	Network Slice Access Group

OAM	Operation and Maintenance

P-CCPCH	Primary Physical Common Control CHannel

PCH	Paging CHannel

PCI	Physical Cell Id

PDCP	Packet Data Convergence Protocol

PH	Power Headroom

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

PS	Packet Switched

QCI	QoS Class Identifier

QoS	Quality of Service

RA	Routing Area

RAB	Radio Access Bearer

RAT	Radio Access Technology

RB	Radio Bearer

RF	Radio Frequency

RLC	Radio Link Control

RLF	Radio Link Failure

RNC	Radio Network Controller

RPLMN	Registered PLMN

RRC	Radio Resource Control

RRM	Radio Resource Management

RSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RTT	Round Trip Time

RTWP	Received Total Wideband Power

SCell	Secondary Cell

SIR	Signal to Interference Ratio

SINR	Signal to Noise plus Interference Ratio

SNPN	Stand-alone Non-Public Network

SNR	Signal to Noise Ratio

SON	Self Organizing/Optimizing Network

SRB	Signalling Radio Bearer

SRNC	Serving RNC

SSB	Synchronization Signal Block

SSID	Service Set Identifier

TA	Tracking Area

TCE	Trace Collection Entity

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunication System

UPH	Uplink PH

URA	UTRAN Registration Area

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

Next Change

#### 5.4.2.X	Logging of NSAG related information

For NR, upon UE failing to re-select to a suitable cell that supports the NSAG ID with the highest priority and selecting to a different cell, following information are logged:

- The NSAG ID with the highest priority;

- Cell ID of the re-selected cell;

End of changes