3GPP TSG-RAN WG2 Meeting #131bis	R2-2508746

Dallas, USA, Nov. 17th - 21st 2025

Agenda item:		10.3.2.3 (Initial and System Access and Others)

Source:		Lenovo, Aumovio

Title:		Observations for initial and system access

Document for:		Discussion and Decision

# Introduction

In Prague, RAN2#131bis was convened and first and very useful agreements could be realized. This document build on these and is focused on following highlighted Agenda:

10.3.2.3		Initial and System Access and Others

Including contributions on initial and system access, including system information, paging, and aspects related to spectrum aggregation etc.   Contributions can include aspects related on on-demand SIB, SSBs, SI update mechanism, SIB1 size, area specific SIBs, etc. and understanding of problems to address.

# Discussion

The following main aspects should be considered from the beginning for 6G system and control plane design:

# SSB, Cell detection and System Access

In a 5G NR system, using initial access a User Equipment (UE) establishes a connection with the network for the first time. This involves cell search, synchronization with the Synchronization Signal Blocks (SSBs), acquiring Master Information Block (MIB), at least System Information Blocks Type 1(SIB1), and performing random access to request resources. The initial access enables the UE to register with the CN (and thereby receive Paging) and maintain connectivity, including authentication, security setup etc. The following aspects are interesting here when we design for 6G:

- Particularly in MRSS (5G-6G Multi-RAT Spectrum Sharing for migration) scenario, which is one of the Migration solution agreed to be studied in the SID, for the cell search and initial camping procedure, whether it is feasible to minimize duplication of potentially common DL transmission (synchronization with the Synchronization Signal Blocks (SSBs)) should be studied. 5G UEs should not be affected as these are already in field. This encompasses PSS, SSS and also MIB/ PBCH information. SSB duplication minimization can save a significant part of otherwise about 3.5% SSB overhead in FR2, 100 MHz BW @ 120 kHz SCS with 64 SSB transmission at 20 ms periodicity.

- Intended cell coverage area(s) and Msg3 size: This is fundamental to understand what Msg3/ MsgA sizes are realizable given the approved deployment scenarios in [2]. This will affect many aspects of our fundamental system access design including how fast the network gets to know the UE (type, identity, establishment cause etc.). RAN2 should ask for RAN1 opinion on this aspect.

- Access Class and Barring simplification: While SA groups will study the Access classes and Access Categories, RAN2 should focus on cell barring aspects. In 5G NR, release wise we have introduced too many barring bits (cellBarredATG, cellBarred-eRedCap1Rx, Barred-eRedCap2Rx, cellBarredNES, cellBarredNTN, cellBarred-RedCap1Rx, cellBarred-RedCap2Rx). This led to specification complexity and worse to keep out the legacy UEs (since these are not the intended feature specific UEs) the MIB barring was relied upon – which has different implications based on local regulation and devoid legacy UEs of making even emergency calls! For this reason, since we start in 6G with a clean slate, we should collect/ shortlist useful features and implement these from first release of 6G. At the same time, we should develop future compatibility to avoid such issues in 6G.

Observation 1a: SSB duplication minimization in MRSS can save significant overhead (e.g., about 3.5% SSB overhead in FR2, 100 MHz BW @ 120 kHz SCS with 64 SSB transmission at 20 ms periodicity).

Observation 1b: To keep out the legacy UEs (since these are not the intended feature specific UEs) the MIB barring was relied upon in 5G – which has local regulation specific implications and prevent legacy UEs from making even emergency calls!

Proposal 1: For initial access RAN2 should study a) SSB duplication minimization in MRSS (to be checked with RAN1) b) check for plausible Msg3 sizes from RAN1 and c) means for achieving forward compatibility (minimize a “future” feature deployment requiring to bar a legacy UE).

# System Information

NR (with no ubiquitous CRS) minimizes always-on transmissions (SSB+SI) compared with LTE. The on-demand SI provisioning further reduces broadcast. Simulation results on how much Network Energy Saving it generates are available in Ch. 6.1.5.2 of 3gpp TR 38.864. One source revealed that with on-demand SIB1 at empty load with baseline of 20ms SSB/SIB1 periodicity, 5.8%~8.6% BS energy savings can be achieved at SIB1 transmission rate of 20%~5% for one SSB beam, and the gains can increase to 32.1%~38.8% for 8 beams case for a same SIB1 transmission rate range.

Observation 2a: BS Energy saving gains of about 30+% can be achieved using on-demand SI provisioning at empty load with baseline of 20ms SSB/SIB1 periodicity for 8 beams case.

So, do we need any further optimization?

From the cited TR, the contribution of the SSB/ SIB1/ Other SI transmission to the overall “operational” energy consumption (including energy dissipation, light bulbs, cooling system if any etc.) of a base station is not available as the intention of the study was only comparative to the full (radio) load situation. It is possible that these gains are insignificant compared with the total power consumption used to run the BS operations. But the sheer number of the Base Stations already deployed today is estimated upwards of 20 million [3] and if a fully deployed 6G System is to eventually replace (or be upgraded to) most of these in years to come, even a much smaller (further) saving can make significant impact.

Observation 2b: About 20 million Base Station deployed today may be replaced by or upgraded to a 6G base station in fully deployed 6G System. Further work on minimizing broadcast might bring significant ecological gain even for a meagre 1 or 2% further NES saving.

Where will the further enhancements come from?

The on-demand system information in 5G NR provides the entire requested SIB and in fact the SI Message containing the SIB. Our initial study of the field logs suspects that rate of change if some System Information is slow – hinting that system information is rather stable. But it is better to study this further and if found true, 6G can design SI distribution which minimizes provisioning the same information repeatedly along with a fraction of changes SI. If much of the system information is stable, there could be other designs minimizing broadcasting it.

Observation 2c: The fraction of SI change is likely (very) low. The complete SI broadcasting should be minimized, even for on-demand SI provisioning.

Can’t gNB just provide only the updates IE(s) of any SIB?

An update of System information is not very frequent in the field and when updates occur it is typically a few parameters rather that the value of each parameter of the SIB (not all SIBs in a SI-Message may change at the same time). If we envisage a SIB to be a tree with several branches, sometimes the update might be made to only one leaf of the entire tree. This requires a delta signalling approach to System Information.

However, the RRC message encoding based on ASN.1 (with unaligned PER, which RRC uses in 5G NR) doesn’t carry field names over the air and doesn’t have a built-in “delta” or “diff” possibility. A decoder interprets a bitstream strictly by the ASN.1 definition and the field order/optionality defined in the ASN.1 definitions of RRC specification. So, a UE can’t receive a free-floating “IE_Name1” and merge it into its stored SIB.

Observation 2d: Leaf only (i.e., changed IE only) signalling can’t be done using ASN.1 with UPER encoding.

So, since an absence of SI change can’t be ruled out completely, RAN2 should strive to provisioning only leaves (changed IEs) – after initial study if it is established that SI changes do not necessarily mean changing every/ most IEs of any SIB. Next, RAN2 should also study workaround for the ASN.1 UPER limitation to provide only changed leaves (IEs).

Proposal 2: RAN2 kindly study rate and fraction of SI changes in real 5G deployments.

Proposal 3: RAN2 should study workarounds to the ASN.1 UPER limitation so that only the modified leaf IEs are provided.

SIB1 Size

In the last meeting in Prague, as well as in the beginning phase of 5G NR (and likely also for the previous technologies), the SIB1 content are discussed enthusiastically with proposals even to split it. The SIB1 fills a sweet spot of compromises – not all cell selection assistance information can be included in PBCH/ MIB since SSB transmission has an even bigger cost – not all cell access information can be delayed further to a later, less periodic (or on-demand) SIB. On the other hand, if we can minimize SI update as discussed previously, and can realize on-demand even for SIB1 delivery, as in Rel. 19, the need for splitting SIB1 reduces significantly.

Proposal 4: A study for SIB1 splitting should be justified with evaluation assuming SIB1 on-demand provisioning and keeping in mind plausible PBCH load restrictions for intended cell sizes.

# Paging

In 5G NR, the paging procedure allows the network to reach idle or inactive UEs for incoming services or system information updates. The gNB transmits paging messages via the Paging Control Channel (PCCH), mapped onto PDSCH, with notifications indicated by the PDCCH. Paging occasions and groups are derived from UE identities and DRX configurations to balance energy saving and latency. Enhancements like Paging early indication (PEI) and paging adaptation from Rel. 19 reduce false alarm for the UE and minimize network paging transmission by reducing the number of POs, beyond what was possible until rel. 18.

The proponent believe that while Rel. 19 could be used as baseline for paging design in 6G, RAN2 should further study Real Paging load and required paging latency – and thereby avoid too much over-dimensioning: UE as well as network energy saving potential based on realistic requirements.

Following are the observations from R2-2406890.

Observation#1a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#1b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Besides, the following aspects can be considered:

- IoT/ Redcap and commercial UEs should be considered separately to see if anything separately needs to be done for these e.g., to help with efficient software download, application pings etc.

- Only one simple schemes for UE power saving can be sufficient to reduce false alarm/ minimize wakeup of main radio i.e., both PEI and LP-WUS may not be necessary. Operator input on deciding the best scheme to adopt in 6G would be very helpful.

- Paging areas/ policy/ propagation: Should check if potential impacts to RAN2

We can start the 6G study with the following:

Observation#5a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#5b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Proposal 5: Study real Paging load in a cell and required paging latency to avoid too much over-dimensioning of Paging delivery.

# Spectrum Aggregation

In the last meeting following agreements were reached:

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

One company proposed to study a single framework for spectrum aggregation based on carrier aggregation, enhanced with fast setup of additional carriers, flexible combining of UL/DL carriers and improved resiliency. Another company requested to study single cell with multi-carriers (SCMC) to aggregate multiple carriers in the same or different bands as a single cell, with the assumption of same/diverse coverage and co-located/non-co-located deployment among the carriers.

A third company emphasized that, the single cell concept with fragmented carriers is supported. In their proposal, the N-carrier Single Cell (NSC) consists of DL/UL anchor carriers and one or more DL/UL non-anchor carrier(s). The initial access is performed on the anchor carrier of the NSC.

Fragmented spectrum utilization and PDCCH minimization are both very useful and practical aims. But, at the same time we need to ensure that we design a simple deployable system. A system design replicating dual connectivity in another terminology, or complicating initial access – camping on carrier A, paging reception on carrier B and responding (MT) in carrier C could lead to potential complications including access delays, complicated load balancing and DL/ UL timing issues. It is possible that such solution(s) work, but we should set some basic guidelines to evaluate these.

- A “cell” definition is unclear now for 6G and even the need for one might be questioned (though we believe a “cell” must be a defined minimum entity of a “cellular” network). Still, it is imperative to understand, how many schedulers are there in a proposed “cell”? Just assuming ideal backhaul isn’t good enough, experience shows otherwise!

- Are the multi carrier definitions restricted to any RRC states e.g., for RRC Idle or connected only – or can these be applied equally well e.g., system information is broadcasted in carrier-1 to RRC Idle UEs but provided dedicatedly to RRC Connected UEs for which carrier-1 is not available.

- RRC Idle UEs must not be required to announce their presence on the component carrier.

- Any potential RAN4 performance issues with intra/ inter-band independent UL/ DL carrier?

On the other hand, Dual Stack/ steering is another means for spectrum aggregation and could make AS design simpler if the splitting, steering and switching rules are transparently handled at application (e.g., at/ below Quick) with rules under network control.

Proposal 6: The spectrum aggregation means including CA (enhancements), SCMC and Dual Stack should be studied, keeping in mind: the scheduler count, backhaul; RRC state-agnostic multi-carrier handling; no Idle CC presence signaling; assessing RAN4 intra/inter-band UL/DL impacts.

# Conclusion

In this contribution, we discussed some fundamental System Access aspects. Based on the discussion, we have the following observation and proposals:

Initial Access

Observation 1a: SSB duplication minimization in MRSS can save significant overhead (e.g., about 3.5% SSB overhead in FR2, 100 MHz BW @ 120 kHz SCS with 64 SSB transmission at 20 ms periodicity).

Observation 1b: To keep out the legacy UEs (since these are not the intended feature specific UEs) the MIB barring was relied upon in 5G – which has local regulation specific implications and prevent legacy UEs from making even emergency calls!

Proposal 1: For initial access RAN2 should study a) SSB duplication minimization in MRSS (to be checked with RAN1) b) check for plausible Msg3 sizes from RAN1 and c) means for achieving forward compatibility (minimize a “future” feature deployment requiring to bar a legacy UE).

System Information

Observation 2a: BS Energy saving gains of about 30+% can be achieved using on-demand SI provisioning at empty load with baseline of 20ms SSB/SIB1 periodicity for 8 beams case.

Observation 2b: About 20 million Base Station deployed today may be replaced by or upgraded to a 6G base station in fully deployed 6G System. Further work on minimizing broadcast might bring significant ecological gain even for a meagre 1 or 2% further NES saving.

Observation 2c: The fraction of SI change is likely (very) low. The complete SI broadcasting should be minimized, even for on-demand SI provisioning.

Observation 2d: Leaf only (i.e., changed IE only) signalling can’t be done using ASN.1 with UPER encoding.

Proposal 2: RAN2 kindly study rate and fraction of SI changes in real 5G deployments.

Proposal 3: RAN2 should study workarounds to the ASN.1 UPER limitation so that only the modified leaf IEs are provided.

SIB1 Size

Proposal 4: A study for SIB1 splitting should be justified with evaluation assuming SIB1 on-demand provisioning and keeping in mind plausible PBCH load restrictions for intended cell sizes.

Paging

Observation#5a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#5b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Proposal 5: Study real Paging load in a cell and required paging latency to avoid too much over-dimensioning of Paging delivery.

Spectrum Aggregation

Proposal 6: The spectrum aggregation means including CA (enhancements), SCMC and Dual Stack should be studied, keeping in mind: the scheduler count, backhaul; RRC state-agnostic multi-carrier handling; no Idle CC presence signaling; assessing RAN4 intra/inter-band UL/DL impacts.

# References

RP-252912_6G_WG_SID_cl

RP-252888 Deployment scenario

[3] https://www.mobileworldlive.com/china-mobile/china-5g-base-stations-approach-4-5m/