# 3GPP TSG- Meeting #132	R2-2508758

# Dallas, USA, Nov 17-21, 2025

Agenda Item:	10.3.2.2

Study Item:	FS_6G_Radio

Source: 	Qualcomm Incorporated

Title: 	Views on 6G RRC structure and (re)configuration

Document for:	Discussion/Decision

# Introduction

RAN#108 approved new SI for 6G Radio (6GR) which was further revised by RAN#109 [1]. One of the RAN2-led objectives is as follows (highlight added):

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

RAN2#131bis discussed encoding/decoding, signalling structure, pain points of one-size-fits-all RRC model used up until 5G, and captured the following agreements:

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

This document provides our initial views on RRC configuration improvements, modularization of RRC structure, and some thoughts on keeping the signalling size small (as per the Chair’s guidance for the agenda item).

# Discussion

## RRC Configuration Improvement: Minimize RRC reestablishment during (re)configuration

As captured in the 4G and 5G specifications, RRC reconfiguration procedure is used to modify an RRC connection, e.g. to establish/modify/release RBs/BH RLC channels/Uu Relay RLC channels/PC5 Relay RLC channels, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, conditional reconfiguration configuration, LTM configuration, multi-path configuration. NAS dedicated information may be transferred from the Network to the UE as part of this procedure.

| NR RRC reconfiguration, successful | NR RRC reconfiguration, failure |
| --- | --- |

Figure  NR RRC Reconfiguration

Although we believe NW to be quite good at providing the right configurations, in the real deployments we have observed a tangible portion of reconfigurations are such that the UE cannot apply the whole configuration.

If a UE cannot apply some (part) of the NW-provided configuration, while a majority of the configuration is good, generally the whole configuration is useless. When problematic configurations are encountered, or when reconfiguration failure occurs, currently (in NR and LTE), the UE initiates connection re-establishment procedure (e.g. see TS 38.331 section 5.3.5.8.2).

In 5G, if a UE cannot apply some (part) of the NW-provided configuration, while a majority of the configuration is good, generally the whole configuration is useless. When problematic configurations are encountered, or when reconfiguration failure occurs, currently the UE initiates connection re-establishment procedure.

Even after the UE does connection re-establishment, there is no guarantee that the NW provided new configurations can be successfully applied.

In 6G, the design principle should be such that UE should be allowed to keep/apply the good (part of) configuration. This is to enable the UE to avoid a re-establishment procedure (while applying the partial (good) configuration). The primary purpose of partial reconfiguration would be to allow the UE and network to continue functioning without triggering a call drop or re-establishment when certain configurations are considered “invalid”.

RAN2#131bis briefly discussed this issue.

Some companies argued that the dynamic UE capability update or UAI could solve these problems. However, the problem may be more dynamic than UE capability update (note it takes some time to update the NW with the new/restricted capabilities)​. UAI has its own baggage (UE needs to declare the capability for UAI, network has to configure UE to allow sending the UAI for specific cases, UE sends UAI, and ultimately NW is not obliged to act upon it in any case).

Some companies commented that these are bad network configurations which should be handled by the NW implementation. However, even with best intentions from the NW side, we observe a tangible number of problematic reconfigurations. And such problems are more prevalent when the technology is just being deployed and not mature enough, especially in new markets where configurations take time to stabilize and the UE must implement targeted workarounds to enable these markets.

After some initial discussion, RAN2#131bis captured the following:

=> For next meeting, can study the reasons why these failures happens and understand the root cause of the problem.

Based on the above guideline, we provide here some actual problems we have observed, followed by one example which is not observed yet but has been brought up during the discussions.

Example Scenario 1: SUL/NUL BWP mismatch for RACH

One example of scenario we observed is as follows: gNB configures BWP-1 on SUL with RACH resources. gNB didn't configure BWP-1 on associated downlink. RACH using BWP-1 of SUL is not possible. gNB also switches UE to active BWP = 1 for SUL. In terms of  validity of signalling - ASN.1 wise, all signalling is valid; UE capability wise, everything is valid.  So, at high level, all configuration seems valid. However, when the UE would attempt to RACH on BWP-1 of SUL, in practice it won't succeed.

Possible Resolution:

Neither dynamic UE capability updates nor UAI would solve this problem. Only possible resolution without needing UE reestablishment would involve UE trimming out BWP-1 of SUL and letting gNB know about current active BWP. Then this could fix the problem immediately.

Example Scenario 2: UL CA SRS / UL Tx Combo Issue

gNB leaves a 2-port SRS configured when adding a second UL (instead of switching to 1-port); or gNB adds concurrent UL MIMO on one cell + UL on another CC without enabling UL Tx switching. Consequently, even though ASN.1 wise all signalling is valid, the RRC config exceeds UE capability. Although there is general expectation that NW should respect UE capability, in practice such configurations are regularly encountered. As a result, UE cannot apply UL config and reconfiguration fails resulting in RLF.

Possible Resolution:

Neither dynamic UE capability updates nor UAI could wolve this problem. On the other hand, if allowed, UE could apply partial config (e.g. not accepting UL on SCC while keeping PCC UL to be MIMO).

Example Scenario 3: Fold/unfold phone state

Based on phone folding/unfolding (by the user) which may happen frequently, the configuration may have to adjust, e.g. certain SCC/CA combo supported or not supported, number of layers may add or drop.

Possible Resolution:

Dynamic UE capability update or UAI could be too much signalling overhead. Allowing UE to downgrade via applying partial RRC Reconfiguration could help solve the problem.

The above are some representative examples of problems we observe when the technology is being deployed and not mature enough, especially in new markets where configurations take time to stabilize.

We have observed a tangible portion of reconfigurations are such that the UE cannot apply the whole configuration, and such problems are more prevalent when the technology is just being deployed and not mature enough, especially in new markets where configurations take time to stabilize and the UE must implement targeted workarounds to enable such markets.

Neither dynamic capability updates nor UAI can effectively solve such issues, which results in unnecessary reestablishment procedures, service interruptions, and increased RLF statistics.

So, it is important that we study and have a proper solution for this issue in 6G so that we encounter less of the similar issues during early rollouts of 6G. Therefore, we propose:

6G design will allow the UE to keep/apply the good (part of) configuration in order to minimize the number of re-establishment procedures.

Obviously, we need to discuss scenarios and situations where this is possible. So, to enable this, further discussion and design would be needed e.g. for the UE to indicate to the NW that the configuration was partially invalid and couldn’t be applied while the rest of the configuration was ok.

### Need for synchronization of applied reconfiguration between the UE and NW

It is duly noted that it is very important to have proper synchronization of the (re)configuration between the UE and the NW. Additionally, in 5G, there is no efficient mechanism to indicate to the network which part of the configuration is problematic resulting into the RLF.

It is very important to have proper synchronization of the (re)configuration between the UE and the NW.

In 5G, there is no efficient mechanism to indicate to the network which part of the configuration could or could not be applied by the UE.

Today, the UE transmits RRC Reconfiguration Complete message using the new radio resource configuration as the result of received RRC Reconfiguration message. This means that the network is likely waiting for the complete message based on the full reconfiguration of the radio resource. To allow the UE to apply a part of reconfiguration, and transmit the (partial) reconfiguration complete message, RAN2 design must make sure the inter-operability between the UE applying the partial reconfiguration and the network applying the full reconfiguration. This would likely mean that there would need to be differentiation of parameters that are ‘critical’ that must be applied (or else reestablishment failure shall be triggered) and not-so-critical parameters that are best if applied successfully but in the worst case possible to be ‘downgraded’ without interrupting the connection.

For example, assume RRC Reconfiguration message reconfigures SR resource. If the UE does not apply the invalid reconfiguration and the network anyhow applies it, the connection may get stuck without UE being able to get any UL grant, hence unable to convey SR resource reconfiguration was not successful in the first place. In such case, it may be better to initiate re-establishment procedure. Take another example, if the reconfiguration involves addition of an SCell, it may be possible for the UE to notify to the NW that SCell additional was not successful, without initiating re-establishment procedure.

Study how to keep synchronization of (re)configurations between the UE and the NW while allowing the UE to keep/apply partial (re)configuration.

## Modularization of RRC

As noted above in section 1, RAN2#131bis agreed RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

In the current RRC model, a UE would have to understand/parse the whole ASN.1 schema, even though it doesn’t support many features. Historically, new features were added in later releases, not from day one, e.g. eMTC, NTN, IAB, RedCap, eRedCap, XR etc. All these types of devices would be configured using the same RRCReconfiguration message to indicate the (re)configuration parameters. The same main RRC message is extended over for future use cases, even though they belong to very diverse types of devices. E.g. the same message carries the configurations for Normal MBB UE, NTN UE, ATG UE, IAB Node, BAP entity, UAV UE, SL L2 Relay UE, SL L2 Remote UE, RedCap UE, XR, etc.

Many of the fields would never be present in the dedicated signalling, however the signalling would need to encode the ‘absence’ at the transmitter, and the receiver would need to decode assuming the whole schema (and consider error case if invalid fields are included). In case of broadcast messages (e.g. SIB), the UE would need to parse everything and discard the irrelevant information. E.g. IoT UE would need to parse and discard IAB information elements.

For 6G Radio, to enable creating/implementing a tailored-RRC for different types of devices corresponding to different use cases, RAN2 should aim to design RRC such that different type of devices can efficiently implement and support only the minimum required RRC for the supported use case(s). We think RAN2 should aim to model the 6G RRC as a Baseline RRC module with additional use case specific extensions as separate modules. Obviously, these names are used just for reference.

To enable creating/implementing a tailored-RRC for different types of devices corresponding to different use cases, RAN2 should aim to design RRC such that different type of devices can efficiently implement and support only the minimum required RRC for the supported use case(s).

XR module

RedCap module

NTN module

Baseline RRC module

MBB module

…

+

6G RRC

Figure 2 Modular RRC design concept for different types of use cases in 6G

With such goal in mind, all UEs need to support the baseline RRC module. Other module(s) are only supported by capable UEs. This means that for individual UEs, there would be no need to parse/decode the whole RRC messages and implement the whole RRC procedures. The optional modules can adapt to future use cases, while the baseline module do not need to change drastically. This would potentially help in minimizing the minimum required processing capabilities and memory footprint for lower capability devices.

The design philosophy could for example be as follows.

- Parameters/configurations common to all UEs would be part of the baseline module.

- Use case specific parameters/configurations would be specified as part of the use case specific module i.e., future extensions common to all UE types continue to be included in the baseline module.

- Future extensions of same/similar features would be continued in the same module.

- Future features that are specific to a different device type or use case may be specified as a new module in a future release.

- Only certain device types supporting the specific use cases would implement/load/execute the use case specific RRC module(s).

RAN2 will study RRC modularization consisting of a baseline RRC module and optional use case specific RRC modules.

Now, RAN2 can further study how to achieve the above goal as there can be many ways. For example, for dedicated RRC signalling, instead of extending the same RRC message to make it applicable for all the use cases and new features/verticals, RAN2 should consider introducing new RRC message classes (and RRC messages) for different use cases or verticals. This way, the UEs supporting one use case doesn’t need to worry about the (extensions of) RRC messages that are not applicable for it. The different message classes and messages can be differentiated based on the logical channel identity of the corresponding control channel carrying the message.

For dedicated RRC, consider introducing new RRC message classes (and RRC messages) for different use cases/verticals.

On the other hand, for broadcast signalling (SIB), today one SI message can include multiple SI blocks, and currently there is no requirement on which SIBs can be included in which SI message. In addition, there is no differentiation of maximum SI size limit for different use cases, i.e. eMBB vs NTN vs IoT which may have different limits on the TBS they can support.

To enable modularization, on a high level, the SIBs may be vertical or use case specific SIBs (in addition to common SIBs for all use cases). Then, it could be made sure that only the SIBs corresponding to specific use case or vertical are broadcasted in one SI message, so that UE implementation does not need to implement receiving, decoding and performing actions corresponding to other (uninterested) SI messages. Such design principle also automatically addresses the problem of different SI size limits for different use cases as well and further enables limiting the memory footprint for low capability devices.

For broadcast RRC, consider separating SIBs for different use cases/verticals, and make sure only the SIBs corresponding to specific use case or vertical are broadcasted in one SI message.

Our companion paper on RRC states also covers some details on UE context modularization. Interested readers are referred to R2-2508942 [2].

## Reducing the signalling size

Another aspect covered under this agenda item relates to keeping signalling size small.

One of the well-known concepts in RRC ASN.1 signalling in RAN2 is ‘Parallel List’ which is used for extension of fields within the elements of an original list. ​

In the so-called parallel lists, the same number of elements must be included, and in the same order. In terms of encoding, header is always included again to indicate the size of the extended list, adding unneeded redundancy.​

In case only a few elements need to be signalled with extended parameters (and a large number of elements in the original list do not need extended parameters), we still need to include a very large list with the full header and many empty elements.​ It should be noted that indicating ‘absence’ of optional elements is not same as omitting the element altogether. ​In other words, a list of size 100 with only 2 elements ‘present’ and 98 elements ‘absent’ is NOT the same as a list of size 2.​ That is because even the ‘absence’ of OPTIONAL fields need to be encoded in the signalling.​

It is typical to see examples of very long original lists but only a very few elements needing extensions to be signalled in e.g. band combination signalling in UE capabilities. This is illustrated in the following snippet from TS 38.331:

Note that the Max size of BandCombinationList is 65k+.​ In some cases, original BandCombinationList can be several 1000x of elements, but extensions may apply only to a very limited subset.​ Each empty element in BandCombination-v1610 occupies 7 bits (to indicate ‘absence’ of all 7 subfields in BandCombindationList-v1610).

RAN2 should aim to reduce redundancy and improve the way we define extension lists such as parallel lists.  Although the above example is for UE capability, this should be considered in both directions: NW configuration as well as UE capability signalling reduction.​

For very long original lists with extensions based on parallel lists, there can be a lot of redundancy and encoding overhead even though only ‘empty’ elements are indicated.

One way to tackle this is by introducing variable sized parallel lists to enable signalling only the needed elements in case only a few elements in a large list need to signal extended values.​ ASN.1 Parameterized Type macros can be defined for better human readability of the ASN.1 for this purpose​.

Therefore, we propose:

Aim to reduce redundancy and improve the way we define extension lists such as parallel lists, by e.g. by enabling variable sized parallel lists.

# Summary

Based on the discussion above, we have following observations and proposals:

RRC configuration improvements

Observation 1.	In 5G, if a UE cannot apply some (part) of the NW-provided configuration, while a majority of the configuration is good, generally the whole configuration is useless. When problematic configurations are encountered, or when reconfiguration failure occurs, currently the UE initiates connection re-establishment procedure.

Observation 2.	We have observed a tangible portion of reconfigurations are such that the UE cannot apply the whole configuration, and such problems are more prevalent when the technology is just being deployed and not mature enough, especially in new markets where configurations take time to stabilize and the UE must implement targeted workarounds to enable such markets.

Observation 3.	Neither dynamic capability updates nor UAI can effectively solve such issues, which results in unnecessary reestablishment procedures, service interruptions, and increased RLF statistics.

Proposal 1:	6G design will allow the UE to keep/apply the good (part of) configuration in order to minimize the number of re-establishment procedures.

Observation 4.	It is very important to have proper synchronization of the (re)configuration between the UE and the NW.

Observation 5.	In 5G, there is no efficient mechanism to indicate to the network which part of the configuration could or could not be applied by the UE.

Proposal 2:	Study how to keep synchronization of (re)configurations between the UE and the NW while allowing the UE to keep/apply partial (re)configuration.

Modularization of RRC

Observation 6.	To enable creating/implementing a tailored-RRC for different types of devices corresponding to different use cases, RAN2 should aim to design RRC such that different type of devices can efficiently implement and support only the minimum required RRC for the supported use case(s).

Proposal 3:	RAN2 will study RRC modularization consisting of a baseline RRC module and optional use case specific RRC modules.

Proposal 4:	For dedicated RRC, consider introducing new RRC message classes (and RRC messages) for different use cases/verticals.

Proposal 5:	For broadcast RRC, consider separating SIBs for different use cases/verticals, and make sure only the SIBs corresponding to specific use case or vertical are broadcasted in one SI message.

Reducing the signalling size

Observation 7.	For very long original lists with extensions based on parallel lists, there can be a lot of redundancy and encoding overhead even though only ‘empty’ elements are indicated.

Proposal 6:	Aim to reduce redundancy and improve the way we define extension lists such as parallel lists, by e.g. by enabling variable sized parallel lists.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, RAN#109, Sep 2025

[2] R2-2508942, Discussion on 6G RRC states, Qualcomm Incorporated, RAN2#132, Nov 2025.