3GPP TSG RAN WG2 #131-bis	R2-2508769

Prague, Czechia, 13th – 17th October 2025

Agenda item	: 10.4

Source	: Jio Platforms Limited

Title	: Cell-Pair Specific Inter-RAT Mobility Configuration

Document for	: Discussion and Decision

# Introduction

The new SID “Study on 6G Radio” [1] has been agreed in RAN#108 and revised [2] in RAN#109 meeting and objective 7 of the study item is as shown below:

(7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

(a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

(b)Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]

RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

(b)Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Mobility of UE between 5G NR and 6GR will be of vital importance because for 6GR is likely to be deployed initially in newer spectrums whereas 5G NR radio access network will continue to operate on existing spectrum bands.

Both 5G and 6G radio access networks will have a mix of different types of cells – Macro cells for wide area coverage and small cells for local area coverage. Thus, a 5G small cell can be a neighbour to 6G macro cell and similarly a 6G small cell can also be a neighbour to 5G macro cell. To address such heterogeneous deployment scenarios, it is important to build greater control for Inter-RAT mobility on a per neighbour basis.

This submission describes the lessons learnt from 4G to 5G mobility and how the learnings can be incorporated early into 6GR specifications.

# Discussion

It is customary to use cell specific offset, also referred to as Cell Individual Offset (CIO) over and above the mobility events like A3 to control intra-frequency, intra-RAT mobility with a specific neighbour cell where UE mobility is different compared to other neighbours. For example, a the CIO for a neighbour serving a train route or a highway can be configured such that handover to this neighbour happens faster compared to the other neighbours which are serving residential areas.

The same parameter CIO can also be used to control inter-RAT mobility to a specific inter-RAT neighbour. However, in LTE Release 15, 3GPP Specification 36.331 does not support CIO (cell individual offset) for 5G neighbours. This lack of standardization can result in cases of ping-pong handovers between LTE Macro cell and NR small cells as have been reported during the deployment of 5G networks. This same problem has been discussed in 3GPP TSG-RAN2 meeting#121 [3] and corrected in LTE Release 16 [4].

Ping pong inter RAT handover issues between LTE and 5G-NR cells have been reported in a multi-vendor scenario because LTE Release 15 did not support the cell individual offset for 5G neighbours.

To illustrate this issue, consider the following scenario consisting of an LTE macro cell, an NR macro cell, and an NR outdoor small cell (ODSC).

Figure 1. Different classes of 5G HetNet Cells with differing thresholds for 5G to 4G HO

Figure 1 illustrates that the Threshold 2 (B2 T2) is a common configuration in LTE base stations for all 5G cells. The value is selected to enable a 5dB difference with Threshold 1 (B2 T1) for one of the most common classes of 5G NR base stations, providing a sufficient margin to avoid successive ping-pong handovers between LTE and NR cells. This is demonstrated using the LTE and NR macro cells in Figure 1.

However, another class of 5G base station exists, shown as the 5G small cell in Figure 1. The threshold 1 (B2 T1) for 5G small cells is higher than the threshold 2 (B2 T2) configured for NR in LTE base stations. Therefore, when a multi-RAT UE moves out of the 5G small cell coverage, it enters a zone where the measurement quality of its primary cell drops below the threshold (B2 T1), resulting in a handover initiated to the LTE macro cell for connected UEs in this zone.

However, the LTE base station observes that the measurement quality of the inter-RAT neighbour NR cell is still better than the common threshold 2 (B2 T2) value and thus initiates a handover back to the 5G small cell. This creates a perpetual set of inter-RAT HOs between LTE macro & NR small cell.

The above illustrated inter-RAT ping-pong HO between LTE & NR cells can be handled correctly at the LTE base station by configuring different CIO for different types of 5G neighbours as shown in Figure 2. It should be noted that the CIO for LTE measurement objects is already specified in NR RRC specifications.

Figure 2. Different classes of 5G HetNet Cells with Cell Individual offset configuration

RAN2 should discuss whether cell-pair specific mobility parameter configuration needs to be introduced in 6GR mobility for 5G neighbors.

If the cell-pair specific mobility parameter configuration for 6G neighbours in 5G specifications is considered from the very beginning of 6G specifications, the issue of ping-pong handovers described in observation 1 can be avoided when 6G is deployed.

# Conclusion

Observation 1:	 Ping pong inter RAT handover issues between LTE and 5G-NR cells have been reported in a multi-vendor scenario because LTE Release 15 did not support the cell individual offset for 5G neighbours.

Observation 2:	 The above illustrated inter-RAT ping-pong HO between LTE & NR cells can be handled correctly at the LTE base station by configuring different CIO for different types of 5G neighbours as shown in Figure 2. It should be noted that the CIO for LTE measurement objects is already specified in NR RRC specifications.

Proposal 1:	 RAN2 should discuss whether cell-pair specific mobility parameter configuration needs to be introduced in 6GR mobility for 5G neighbors.

# References

3GPP RAN#108, RP-251881, New SID: Study on 6G Radio

3GPP RAN#109, RP-252912, Revised SID: Study on 6G Radio

3GPP RAN#121, R2-2302248, Report of [AT121][203][LTE] CIO for Inter-RAT HO from LTE to NR

3GPP RAN#121, R2-2301928, Introduction of cell-specific offset for inter-RAT measurement in LTE for NR neighbors