3GPP TSG RAN WG2 #132	R2-2508775

Dallas, USA, 17th – 21st November 2025

Agenda item	: 10.3.3.1

Source	: Jio Platforms Limited

Title	: Model Transfer and Feedback Procedure for 6G AI/ML Framework

Document for	: Discussion and Decision

# Introduction

During the 3GPP TSG-RAN WG2 Meeting #131bis, held in Prague, the following items were discussed.

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

This contribution proposes a Model Transfer and Feedback procedure for 6G AI/ML. It aims to provide a unified mechanism for delivering AI models to UEs, monitoring their performance, reporting applicability, updating models dynamically, and transporting AI‑annotated data. The procedures are illustrated with diagrams and described step by step to facilitate implementation.

# Discussion

## Performance monitoring and feedback (UE → gNB)

AI models deployed at the UE or in the gNB may drift over time due to changes in the propagation environment, mobility patterns, or service mix. If an AI‑based feature performs poorly, it can degrade throughput, latency or reliability. To maintain baseline performance, it is important to monitor AI model effectiveness and adapt the model when necessary and we need a unified standardized mechanism for reporting model performance or triggering model updates in 6G.

AI/ML model performance may degrade in operation, but current specifications lack a standardized feedback mechanism for UEs to report model performance and for the network to trigger model updates or fallback to non‑AI methods.

Proposed procedure for model performance feedback

To address this gap, the following explicit signaling procedure is proposed for UE → gNB model performance feedback. The procedure ensures that model drift is detected early and that remedial action (delta update or fallback) is taken swiftly.

- Performance metric configuration: When an AI feature is activated, the gNB configures the UE with one or more performance metrics (e.g. prediction error, success rate, mean squared error) and associated thresholds. These parameters may be provided in the RRC Reconfiguration alongside the model ID. The thresholds reflect the minimum acceptable performance relative to the baseline (non‑AI) method.

- Local monitoring at the UE: The UE continuously evaluates the configured metric as it runs the AI algorithm. For example, in an AI‑based beam selection feature, the UE compares the predicted best beam to the actual beam used and calculates a success rate. The UE keeps a moving window of results to smooth out variability.

- Event detection: If the performance metric falls below the configured threshold for N consecutive evaluations (where N is configured via RRC), the UE determines that the model is underperforming. To avoid excessive signaling, the UE employs a prohibit timer: after sending a performance report, it will not send another report for a configured period unless performance degrades further. This aligns with the generic concept of event‑triggered and rate‑limited reporting.

- Model Performance Report: The UE sends a Model Performance Report to the gNB. This can be carried in an existing UE Assistance Information message extended with new Information Elements: Model ID, Performance Metric Value, and an optional Cause Indicator (e.g. “Model drift detected” or “Insufficient training data”). The report may also include a confidence flag indicating whether the UE suggests fallback or can accept a delta update.

- Aggregation and decision at the gNB: The gNB collects reports from multiple UEs using the same model. If a single UE reports degradation, the network may treat it as a local anomaly; however, if multiple UEs report poor performance within a short period, the gNB concludes that the model has broadly drifted. The gNB may also combine UE feedback with other data (e.g., NWDAF insights, network statistics) to decide on a remedy.

- Triggering an update: Based on the aggregated feedback, the gNB chooses one of the following actions:

- Delta update: Instruct the model server to generate a small delta update (set of weight adjustments) that adapts the model to the new conditions. The gNB then distributes this delta to affected UEs via an RRC message or using the data plane.

- Model switch: If another model variant (e.g. a version trained for indoor environments) exists, the gNB instructs the UE to switch to that model by sending a new Model ID.

- Fallback to non‑AI: If no suitable update is available or the feature is deemed unreliable, the gNB instructs the UE to fall back to the baseline algorithm (non‑AI) until a new model is prepared.

- Update delivery and acknowledgment: The UE receives the update or fallback command, applies it, and acknowledges via an RRC Reconfiguration Complete or a dedicated confirmation message. The UE then resumes monitoring with the new model or algorithm.

The feedback loop is illustrated in Figure 1.

Figure 1 Al Model Performance Feedback and Update Procedure

RAN2 should study a standardized performance monitoring and feedback mechanism for AI/ML models in 6G with the following aspects:

- UEs should monitor specified performance metrics and report degradation via a standardized message.

- The network should interpret these reports, aggregate them across UEs, and decide whether to send incremental model updates, switch models, or instruct fallback to legacy procedures.

- UE reports should be rate‑controlled to limit overhead, and network decisions should consider multiple inputs to avoid unnecessary updates.

## Model update transfer and activation

The AI/ML models are provisioned statically or dynamically. In static provisioning, the network sends a list of supported models (identified by Model IDs) and activates one of them at the UE. Herein, the offline training will be used, and model updates will be infrequent. For 6G, this paradigm is insufficient. AI‑enabled features will be more numerous and varied (e.g. beam management, channel prediction, sensing), and UEs will not be able to store all potential models. Moreover, some models may be proprietary or shared only on a per‑feature basis. Therefore, the network must be able to provide a model on demand when a UE requires a new capability.

Due to limited UE storage and the large number of potential AI/ML models, UEs may not have the appropriate model pre‑installed for every feature. A standardized procedure is needed for the network to deliver AI models to the UE (when the AI/ML model performance degrade), including model negotiation, secure transfer, and activation.

- Proposed procedure for model transfer

- Capability and model negotiation: During connection setup or when an AI‑enabled feature is invoked, the UE advertises the AI functionalities it supports and the models it already has (by Model ID and version). The gNB selects the required model; if the UE lacks it, the gNB responds with a Model ID and an instruction to download. The negotiation may reuse existing UE Capability Information messages with new fields for AI/ML capabilities and model lists.

- Model delivery: The gNB obtains the model from a training server (OAM/NWDAF) and sends it to the UE. Large models may be transferred over a dedicated data plane bearer with appropriate QoS (low priority since model transfer is not latency critical), while small patches or delta updates can be sent via RRC signaling. The transfer is integrity‑protected and versioned to prevent replay or corruption.

- Activation: After successful download, the UE acknowledges and activates the model. The gNB records the UE’s active model version for the corresponding feature so that subsequent signaling (e.g. delta updates) can be targeted correctly. Activation may be done via an RRC Reconfiguration message that includes the Model ID and configuration parameters.

The procedure is illustrated in Figure 2.

Figure 2 Al Model Transfer Procedure for 6G AI/ML

RAN2 should study standardized model transfer (e.g., for model update) and activation procedure for 6G AI/ML where the procedure should include:

- model negotiation (using Model IDs),

- model delivery over an appropriate bearer (data plane or control plane),

- activation signaling.

Security and integrity of model files should be ensured, and the mechanism should support both full model downloads and incremental updates.

## Applicability reporting with dynamic delta update / fallback

Some AI/ML features may not be applicable in all situations, for example, a beam prediction model trained for macro cells may not work in small cells, or a sensing function may be unavailable due to missing hardware. Applicability report mechanism, i.e., UE indicates whether a feature is applicable for a given cell, is not sufficient as it only provides a binary yes/no status. For 6G, more nuanced information is needed: the UE should report why a feature is not usable and enable the network to respond appropriately (e.g. by sending a delta update or switching to another model).

AI/ML feature applicability varies across cells and scenarios. A UE may determine that an AI model is not applicable or not useful. Current applicability reporting does not provide cause information or support dynamic updates; therefore, a more detailed reporting and update mechanism is required.

- Proposed procedure for applicability reporting and dynamic update

The following procedure extends the applicability reporting concept to enable dynamic delta updates and fallback based on the UE’s feedback:

- Applicability check: For each AI feature, the UE evaluates whether the feature can be applied. This involves checking conditions such as the presence of required reference signals, measurement quality, and the suitability of the current model for the channel conditions (e.g. frequency band or deployment type). If the feature is deemed Not Applicable, the UE prepares an Applicability Report.

- Extended Applicability Report: The UE sends an Extended Applicability Report to the gNB. In addition to the binary applicability flag, this report includes:

- Cause and metrics: A short cause code (e.g. “No reference signal for model training”, “Environment mismatch”, “Insufficient sensing accuracy”) and, if available, a metric indicating how far the conditions are from the model’s expected range. This helps the network decide whether a small adjustment might make the feature usable.

- Suggested remedy indicator: A flag indicating whether the UE prefers a delta update (if minor adjustment could enable applicability) or suggests to temporarily disable the feature (fallback) until a better model is provided.

- Network evaluation: Upon receiving the report, the gNB evaluates the cause and metric. If the cause is due to missing configuration (e.g. an RS or CSI-RS not configured), the gNB may simply reconfigure the radio resources. Otherwise, it contacts the model server/OAM to fetch a delta update or a new model tailored to the UE’s environment.

- Delta update or fallback: - If the model server can generate a delta update, it sends it to the gNB. The gNB forwards the delta to the UE via the appropriate bearer. The UE applies the delta and re‑evaluates applicability. If successful, the feature becomes usable without a full model download.

- If no suitable update exists, the gNB instructs the UE to fall back to non‑AI operation for that feature. This avoids wasting resources on a non‑usable AI function.

- Acknowledgment and monitoring: The UE acknowledges the delta update or fallback instruction and resumes operation. If a delta update was applied, the UE continues to monitor applicability and performance as per Sections 2.2 and 2.3.

This procedure is depicted in Figure 3.

Figure 3 Al Model Applicability - Delta Update or Fallback

RAN2 should study enhanced applicability reporting mechanism for AI/ML features where the UEs should be able to report why a feature is not applicable and provide metrics that help the network decide whether a delta update can enable applicability.

Based on the reports sent by the UEs, the network should either send a delta update from the model server or instruct the UE to fall back to non‑AI mode to ensure efficient use of AI features and minimize unnecessary model downloads or misconfigurations.

## AI‑annotated data flows and dedicated bearers

AI/ML functions generate and consume various types of data: training samples, inference results, performance metrics, model parameters, etc. These data flows have diverse QoS needs—some may be delay‑tolerant bulk data (e.g. training logs), others may be low‑priority control information, and a few might require moderate latency (e.g. small delta updates). Overloading the existing Signaling Radio Bearers (SRBs) or user data radio bearers (DRBs) with AI data can impact essential services. Therefore, a mechanism to classify and separate AI‑annotated flows is needed.

The AI/ML data plane must support various data types with different QoS requirements. Existing NR bearers (SRB/DRB) are optimized for either control or user traffic; a dedicated bearer for AI data would allow flexible scheduling and protect user traffic.

Proposed procedure for applicability reporting and dynamic update

To handle AI data efficiently, the following bearer signaling procedure is proposed:

- Classification of AI data: The network defines categories of AI data, for example:

- AI Control Data: Small messages related to model management (e.g. performance reports, applicability reports, update acknowledgments). These need reliability but not high throughput.

- AI Training Data: Bulk uploads of logs or sensor samples for model retraining. These are delay tolerant and can use low priority.

- AI Model Data: Model parameters, delta updates or complete models. These can be medium priority depending on the size and urgency.

- AI Inference Results: If the network supports Network for AI services, inference outputs from UEs may need medium priority and moderate latency.

- Dedicated AI Data Bearer: The gNB establishes a dedicated AI Data Bearer (AI‑DRB) between the UE and gNB. This bearer uses a low‑priority QoS flow identifier with appropriate scheduling parameters (e.g. low guaranteed bit rate or delay budget). AI Control Data may still use SRB if infrequent; AI Training Data and Model Data use the AI‑DRB. The AI Data Bearer ensures that user traffic on other DRBs is not impacted.

- Signaling AI Data Bearer setup: When an AI feature is activated or when an AI data transfer is needed, the gNB sends an RRC Reconfiguration to set up the AI‑DRB. The configuration includes QoS parameters such as 5QI and ARP (Allocation and Retention Priority) to reflect the low priority of AI training data. If multiple AI features share the bearer, a higher‑layer protocol (e.g. SDAP) can multiplex flows using different QFIs.

- Bearer usage: - Control messages (performance reports, applicability reports, model IDs) are sent over SRB1 or SRB2, extended to carry AI information as described in Sections 2.2 and 2.3.

- Model transfer and delta updates that are too large for SRB are sent over the AI‑DRB. Because these transfers are not delay sensitive, they can be scheduled in low‑priority time/frequency resources.

- Training data uploads use the AI‑DRB. The network controls the timing and rate (e.g. only allow uploads during low cell load) to avoid congestion.

- Bearer release: Once AI data transfer is complete or an AI feature is disabled, the AI‑DRB can be released via a standard RRC procedure. This frees resources and avoids unnecessary overhead.

Figure 4 illustrates the bearer structure.

Figure 4 Bearer Signaling for Al-Annotated Data Flows

RAN2 should study a dedicated AI Data Bearer for 6G to carry AI‑annotated data flows where the bearer would provide low‑priority, delay‑tolerant transport for training data and model updates, ensuring that user traffic is not impacted.

Control messages for AI (reports, IDs) can be carried over SRBs with extended information elements.

## Overall architecture and signaling flows

To support the above procedures, a holistic architecture is needed that connects the UE, gNB, central training facilities, and analytics functions. Figure 5 provides a high‑level view of the model endpoints, traffic splits, and signaling paths across the system.

Figure 5 6G AI/ML architecture and signaling paths

Description of the Architecture

- UE: The UE hosts AI models for functions such as beam prediction, channel estimation, sensing, etc. It runs inference locally, monitors model performance and applicability, and uploads training data or reports via dedicated bearers.

- gNB: The gNB orchestrates AI functions in the RAN. It negotiates model IDs with the UE, delivers models, collects performance and applicability feedback, and aggregates information from multiple UEs. It can perform lightweight training or adaptation but primarily serves as a relay between UEs and higher‑level AI functions.

- Central Training / Model Server: A central training server (which may be co‑located with the gNB or in an edge cloud) aggregates training data from many UEs and gNBs, performs offline or online training, and generates new model versions or delta updates. It may also host a database of models and an OAM/Model Server interface for management.

- NWDAF / Edge Analytics: The Network Data Analytics Function (NWDAF) or edge analytics node collects performance data and KPIs from the gNB and UEs, derives insights (e.g. cell load, mobility patterns), and may recommend policies or adjustments to model selection. It also assists in deciding when to trigger model updates or fallback.

- Signaling paths:

- - Model transfer: From the central training / model server to the UE via the gNB, using the procedure in Section 2.1.

- - Performance feedback: From the UE to the gNB, then to the central training or NWDAF if retraining is needed, as described in Section 2.2.

- - Training data: From the UE or gNB to the central training server, transported via the dedicated AI Data Bearer (Section 2.4).

- - Insights and policies: From the NWDAF or model server to the gNB, guiding how models are chosen, updated or disabled.

The architecture allows flexible deployment of AI functions (UE‑based, gNB‑based, or cloud‑based) and ensures that data flows are properly routed and protected. It also highlights that model transfer and performance feedback are not purely local: the gNB interacts with both the model server and analytics functions to maintain model quality and adapt to network conditions.

A holistic architecture is required to connect UE, gNB, central training, NWDAF and OAM for model management.

RAN2 should adopt the architectural framework illustrated in Figure 5 as a baseline for AI/ML in 6G. which clarifies the roles of UE, gNB, central training, NWDAF, and OAM functions, and defines the signaling paths for model transfer, performance feedback, data upload and policy distribution.

This will ensure a coherent approach across working groups and simplify the integration of AI/ML features in 6G.

# Conclusion

The observations and proposals are as listed below:

Observation 1:	 AI/ML model performance may degrade in operation, but current specifications lack a standardized feedback mechanism for UEs to report model performance and for the network to trigger model updates or fallback to non‑AI methods.

Proposal 1:	 RAN2 should study a standardized performance monitoring and feedback mechanism for AI/ML models in 6G with the following aspects:

- UEs should monitor specified performance metrics and report degradation via a standardized message.

- The network should interpret these reports, aggregate them across UEs, and decide whether to send incremental model updates, switch models, or instruct fallback to legacy procedures.

- UE reports should be rate‑controlled to limit overhead, and network decisions should consider multiple inputs to avoid unnecessary updates.

Observation 2:	 Due to limited UE storage and the large number of potential AI/ML models, UEs may not have the appropriate model pre‑installed for every feature. A standardized procedure is needed for the network to deliver AI models to the UE (when the AI/ML model performance degrade), including model negotiation, secure transfer, and activation.

Proposal 2:	 RAN2 should study standardized model transfer (e.g., for model update) and activation procedure for 6G AI/ML where the procedure should include:

- model negotiation (using Model IDs),

- model delivery over an appropriate bearer (data plane or control plane),

- activation signaling.

Proposal 3:	 Security and integrity of model files should be ensured, and the mechanism should support both full model downloads and incremental updates.

Observation 3:	 AI/ML feature applicability varies across cells and scenarios. A UE may determine that an AI model is not applicable or not useful. Current applicability reporting does not provide cause information or support dynamic updates; therefore, a more detailed reporting and update mechanism is required.

Proposal 4:	 RAN2 should study enhanced applicability reporting mechanism for AI/ML features where the UEs should be able to report why a feature is not applicable and provide metrics that help the network decide whether a delta update can enable applicability.

Proposal 5:	 Based on the reports sent by the UEs, the network should either send a delta update from the model server or instruct the UE to fall back to non‑AI mode to ensure efficient use of AI features and minimize unnecessary model downloads or misconfigurations.

Observation 4:	 The AI/ML data plane must support various data types with different QoS requirements. Existing NR bearers (SRB/DRB) are optimized for either control or user traffic; a dedicated bearer for AI data would allow flexible scheduling and protect user traffic.

Proposal 6:	 RAN2 should study a dedicated AI Data Bearer for 6G to carry AI‑annotated data flows where the bearer would provide low‑priority, delay‑tolerant transport for training data and model updates, ensuring that user traffic is not impacted.

Proposal 7:	 Control messages for AI (reports, IDs) can be carried over SRBs with extended information elements.

Observation 5: 	A holistic architecture is required to connect UE, gNB, central training, NWDAF and OAM for model management.

Proposal 8:	 RAN2 should adopt the architectural framework illustrated in Figure 5 as a baseline for AI/ML in 6G. which clarifies the roles of UE, gNB, central training, NWDAF, and OAM functions, and defines the signaling paths for model transfer, performance feedback, data upload and policy distribution.