3GPP TSG-RAN WG2 Meeting #132	R2-2508785

Dallas, USA, Nov 17th- 21st, 2025

Agenda item:		10.3.1.1

Source:	Rakuten

Title:	Discussion on Radio Protocol Architecture – User Plane

Document for:		Discussion and Decision

# Introduction

The SID of Study on 6G Radio in RAN #108 includes the following objectives (1)

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

During the 5G Study, the Rel-14 RAN2-led TR 38.804, the user plane discussion of the air interface in RAN2 agreed to consider the following aspects under UP protocols functions and bearer types.

- Overview of L2 functions and L2 data flow (impacting RAN1/RAN3/SA)

- Services and functions provided by each UP layer (including MAC/RLC/PDCP/SDAP) (impacting RAN1/RAN3/SA)

- Description of bearer types (e.g. split/SCG bearer) (impacting RAN3)

- Numerologies and scheduling durations (e.g. SCS and symbol/slot duration according to the inputs from RAN1)

- ARQ and HARQ (impacting RAN1)

- Scheduling schemes (e.g. DL/UL dynamic/configured scheduling) (impacting RAN1)

- QoS control (e.g. QoS flow to DRB mapping) (impacting SA)

- Random access procedure (impacting RAN1/RAN4)

- UE power saving (e.g. DRX) (impacting RAN1/RAN4)

In this contribution, we present our analysis and views regarding some of these aspects including the User plane architecture and protocol design for 6GR.

# Discussion

## User Plane functions

The 5G RAN user plane protocol architecture is provided below

Figure 1: 5G User plane protocol architecture

The main services and functions of the MAC sublayer include:

-	Mapping between logical channels and transport channels.

-	Multiplexing/demultiplexing of MAC SDUs (RLC PDUs) belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels.

-	Scheduling (Dynamic Resource Allocation): Decide which UE or logical channel gets which time-frequency resources.

-	HARQ (Hybrid ARQ): Perform fast retransmissions with soft combining for error correction.

-	Buffer Status Reporting, Scheduling Request (SR): Procedures for UE to request uplink resources from gNB.

-  Random Access (RACH) Coordination: Manage initial access and contention resolution.

-  Timing Alignment Maintenance: Adjust UE uplink timing for OFDMA alignment

-	Logical Channel Prioritization (LCP): Priority handling between logical channels of one UE.

-	Priority handling between overlapping resources of one UE.

-	Padding and Fragmentation.

The main services and functions of the RLC sublayer include:

-	Segmentation and re-assembly: Splits PDCP SDUs into smaller RLC PDUs (segmentation) and reassembles them at the receiver

-	Retransmission (ARQ): Detects missing PDUs using sequence numbers and retransmits (AM only)

-  In-sequence Delivery (UM and AM).

-  Re-ordering: Reorders out-of-sequence PDUs using RLC sequence numbers

-  Concatenation: Combines multiple smaller SDUs into one RLC PDU

-	Error Correction (ARQ) - AM only.

-	Duplicate Detection (AM only).

-  Buffer management: Manages transmission and reception buffers

-  Status Reporting: Sends RLC STATUS PDUs (ACK/NACK info)

-	Protocol error detection (AM only).

The main services and functions of the PDCP sublayer include:

-	Sequence Numbering: Assigns and manages PDCP SNs to each PDU

-	Header (de)compression.

-	Compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only.

-	Security (Ciphering & Integrity Protection).

-	Reordering & In-sequence Delivery: Ensures packets are delivered in correct order

-  Duplicate Detection & Elimination: Removes duplicate PDCP PDUs (e.g. during carrier duplication or HO)

-	Re-establishment & HO support: Transfer or re-establish PDCP state between cells.

-	Duplication of control signalling and/or user data.

-	Discard Timer Handling: Discard PDUs that cannot be transmitted.

Based on the list of functionalities performed by each protocol layer, it is clear that all the above listed functions are essential in 6GR also, as the services required to be supported by user plane functions in 5G and 6GR remain the same. However, the new services that were introduced late in 5G which are probably going to be predominant in 6GR (eg: XR/AI/Sensing/HRLLC etc) require more efficient UP processing and higher data rates. Latency and implementation complexity are some more factors that need scrutiny.

In 5G, the Radio Link Control (RLC) layer is a crucial part of the protocol stack, sitting between the MAC (Medium Access Control) and PDCP (Packet Data Convergence Protocol) layers. Its main functions are listed above. However, we observe that there are some redundant functions that exist among different layers of 5G UP protocol stack.

- PDCP and AM RLC, both support the packet retransmissions with a corresponding status report. This requires two separate SNs (Sequence Number) allocated in both PDCP and RLC respectively.

- In-sequence delivery: Because there are two SNs, in-order delivery is performed at both layers.

- The ARQ in PDCP is triggered only for specific use cases (e.g. handover and bearer type change), and the ARQ in RLC is used for the AM RLC. There is also HARQ in MAC which also manages physical layer re-transmissions. Therefore, there are two ARQ and one HARQ based re-transmission functions in the 5G UP stack.

- Duplicate detection is another function which is performed at both RLC and PDCP.

Some user plane functions are duplicated in different layers of the 5G User plane stack.

Because of these redundancies, there is an associated signalling overhead (eg: two SNs) and more cross-layer interactions. This is in addition to the latency, implementation complexity and processing overhead incurred.

Buffers at PDCP, MAC and RLC layers are overhead on memory requirements, because the PDUs have to be stored in the RLC buffers till they are acknowledged. They need to be stored in PDCP buffers till they are re-ordered, which supersedes RLC. This is an overhead because RLC PDUs and MAC PDUs don’t have a 1-1 mapping and MAC maintains its own priority queues.

In summary, considering the duplicated functions and the tight coordination required between different layers, we think that there is a scope to eliminate the duplicated functions and design an optimal and more efficient UP protocol stack for 6GR. One solution in that direction could be first classify the required and redundant functions of RLC, remove the redundant functions of RLC and move the necessary functions to either PDCP or MAC. For eg: there could be only one ARQ based re-transmission, Reordering & In-sequence Delivery function in the entire stack. PDCP layer could be empowered with Segmentation functionality, leading to a lean, efficient, less complex, less overhead and fast processing protocol stack.

We propose to study and design an enhanced User plane protocol architecture for 6GR by eliminating the duplicate functions and merging the required functions of RLC layer with either PDCP or MAC.

Study and identify the duplicate functions in the 5G user plane stack to design a lean, efficient, less complex, less overhead and fast processing protocol stack for 6GR.

Design an efficient and lean User plane protocol stack for 6GR by eliminating the duplicate functions and merging the required functions of RLC layer with either PDCP or MAC.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:

Observation 1	Some user plane functions are duplicated in different layers of the 5G User plane stack.

Proposal 1		Study and identify the duplicate functions in the 5G user plane stack to design a lean, efficient, less complex, less overhead and fast processing protocol stack for 6GR.

Proposal 2		Design an efficient and lean User plane protocol stack for 6GR by eliminating the duplicate functions and merging the required functions of RLC layer with either PDCP or MAC.

# References

      - RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.