3GPP TSG-RAN WG2 Meeting #132                                    	R2-2508809

Dallas, USA, November 17th-21st, 2025

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	Remaining issues on PWS support

Document for:	Discussion and Decision

- Introduction

In the RAN2#131bis meeting, no conclusion could be made on clarifying support of acceptable cell specially in TN networks. In this document we provide details on solutions of acceptable cell in TN networks.

- Discussion

In NB-IoT, cell reselection is based on ranking regardless of frequency priorities. This includes the existing NB-IoT RATs which are not updated to support PWS notification/broadcasting. We have already agreed to introduce UE capability signaling to indicate support of PWS for network awareness and obviously this would be only for new updated network.

It is also agreed that NB-IoT UE will support PWS in TN network. NB-IoT TN network are already widely deployed and current NB-IoT RATs do not support PWS. It is also agreed that PWS capable UE can also camp in an acceptable cell. Therefore, if we assume the acceptable cell functionality is supported in TN networks, then this would be an issue.

If a UE is PWS capable and camps in a cell that does not support PWS while there are neighbor cells supporting PWS, then UE implementing such important feature will not be able to take benefit and is going to miss the warning message. Therefore, we think the UE needs to be aware of the PWS capability of the cell to prioritize cell search and PLMN search. In addition, while searching for the suitable cell, the NB-IoT UE may be able to monitor the paging for emergency message in an acceptable cell. Such indication can be added in SIB1. But to avoid reading SIB1 and speed up the search, such indication can be added in MIB as there are enough spare bits available.

- Confirm NB-IoT UE supports acceptable cell functionality in TN networks.

- If proposal 1 is confirmed, introduce network indication of support of PWS to prevent UEs camping on cells that will not support PWS broadcasting.

In TS 36.306, it is not clarified whether the acceptable cell functionality is mandatory to support, conditional mandatory to support or optional feature. We think some clarification would be needed as acceptable cell functionality support is Rel-19 feature for NB-IoT UE. We think it is fine to make support of acceptable cell conditional on the support of pws-Support-r19.

Clarify in TS 36.306 that support of acceptable cell is conditional on the support of pws-Support-r19.

- Conclusion

Following observation and proposals are made:

Proposal 1	Confirm NB-IoT UE supports acceptable cell functionality in TN networks.

Proposal 2	If proposal 1 is confirmed, introduce network indication of support of PWS to prevent UEs camping on cells that will not support PWS broadcasting.

Proposal 3	Clarify in TS 36.306 that support of acceptable cell is conditional on the support of pws-Support-r19.