3GPP TSG-RAN WG2 Meeting #132                                    	R2-2508812

Dallas, USA, November 17th-21st, 2025

Agenda item:	9.7.2

Source:	Qualcomm Incorporated

Title:	Support of voice over NB-IoT

Document for:	Discussion and Decision

- Introduction

In [1], following objectives are specified.

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2]

  - Coordination with SA is required

  - The objective can be updated with further details based on the outcome of the above down-selection

- Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1]

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2]

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

- Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4]

- Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements

- Note: Coordination with SA4 is expected

In RAN2#131bis, an LS reply [2] is sent to SA2 informing them that UP solution is preferable. In this document, we provide details for support voice in UP solution.

- Discussion

Currently, NB-IoT UE supports only 1 DRB when it supports user plane solution. Additionally, the NB-IoT UE can indicate “multipleDRB-r13” whether it supports additionally 1 more DRB, i.e., whether it supports multiple DRBs.

For the support of voice service, UE may need to establish simultaneous multiple EPS bearers with different QCI, for example, QCI-5- IMS signaling, QCI-9- supplementary service call forward/transfer, QCI-1- voice. In case of mobility from LTE to NB-IoT due to UE being out of TN coverage, multiple EPS bearer (or multiple PDN connections) may need to be continued. This may require UE to support at least 4 DRBs as there has to be 1:1 mapping between EPS bearer and DRB. Considering MMS/SMS traffic and internet traffic, the number of DRBs needed could be 8. However, at least from PHY data rate point of view, transmission of data from more EPS bearers, for example, 8 DRBs, would be challenging due low data rate.

The maximum number of DRBs that needs to be supported simultaneously can be discussed based on SA1/SA2 solution requirements and use case. However, current L2 buffer size does not have to be extended as multiplexing of data from multiple bearers would need to be avoided due to smaller TBS size (i.e., low PHY data rate) and voice DRB would need RLC UM mode operation over GEO channel.

For UP solution, current total L2 buffer size is not extended for NB-IoT UEs supporting voice over GEO.

For UP solution, wait for SA2 progress on the maximum number of DRBs (e.g., 4 or 8) that need to be supported simultaneously.

For QoS and GBR configuration of DRB, it is possible that only prioritization of voice traffic, IMS signaling and non-voice traffic could be defined for the use of DRBs. It also depends on SA2 solutions and design of new QCI. Therefore, we suggest waiting SA2 progress for QoS and GBR for DRBs.

- Wait for SA2 progress on discussion of QoS and GBR for DRBs.

For UP solution, the UE has to indicate it wants to initiate user plane, i.e., DRB such that network is sure connection is not for CP solution by default. In addition, the network has to perform RRC reconfiguration according to the UE’s intention to initiate a voice call. Therefore, a new establishment cause should be introduced if the UE supports voice call and intends to initiate voice call as in LTE and NR.

Introduce mo-Voice-r20 as an establishment cause.

SPS:

Due to nature of very long RTT over GEO channel, UL scheduling based on scheduling requests or buffer status report would be very inefficient. This incurs large scheduling delays and older voice packets may need to be discarded as they become obsolete. Retransmission may be disabled over GEO channel.

NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to periodic UL voice traffic and DL voice traffic. Network may not be able to allocate further UL resources in time domain to UEs for transmitting other data such TA report etc as shown in Figure 1.

Figure 1 For HD NB-IoT, UL and DL voice traffic alone can overwhelm the resources in time domain

To solve this problem, semi-persistence scheduling would be very useful for both UL and DL voice traffic. For UL SPS, it is possible to prioritize the voice traffic from the DRB (may be configured for voice) to use the SPS. This can be done without introducing prioritisedBitRate and bucketSizeDuration, which are not supported currently in NB-IoT.

UL SPS and DL SPS are introduced to carry data from DRBs associated with voice. FFS on details of logical channel prioritization for UL SPS.

As clarified below from TS 36.321 section 5.7, it is almost not feasible to support voice over GEO with HARQ RTT timer running for NB-IoT. However, HARQ feedback enable/disable or UL HARQ mode is configured per HARQ process and can apply to any traffic, i.e., voice or non-voice. One option is that the HARQ RTT timer (both in UL and DL) are not started for SPS associated with voice traffic.

UL SPS and DL SPS corresponding to voice traffic will not use HARQ RTT timer regardless of HARQ process ID.

In voice call, silent period may be very common. In this case the TBS size of the voice traffic may change as the size of silent packet may be smaller. In addition, UE might have other traffic that may need to be transmitted periodically such as TA report, BSR, other non-voice traffics or internet traffic. This traffic can have lower priority than voice traffic. It is to note that NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to UL voice traffic and DL voice traffic in time domain. For a TBS size of 144 bits, a minimum of 64 repetitions of NPUSCH transmission may be needed at worst coverage level. Network may not be able to allocate further UL resource to UE as shown in figure 1.

In UP IP solution, the use of ROHC is even demanding varying TBS sizes due to variant size of RoHC headers and RoHC feedback. Therefore, there should be a mechanism for UE to be able transmit additional data periodically to accommodate any additional data other than voice data as shown in Figure 2.

Figure 2 Additional frequency domain resource

For NB-IoT operating in HD, the time domain resource may be exhausted but network still be able to provide additional frequency domain resources based on need. As shown in figure 2, when there is a need to transmit more data, network may disable smaller TBS size resource and activate the larger TBS size resource, for example,

- based on SR, network can deactivate the SPS and provide larger TBS UL grant.

- It is also possible for network to configure multiple SPS one with smaller periodicity and other with larger periodicity. When they overlap in time domain, the larger TBS SPS can be assumed active.

However, there might be some challenges in transmitting RLC AM data (e.g., non-voice), RLC UM data (e.g., voice data) and MAC CEs. They can be handled with proper logical channel prioritization. But this can be further discussed.

On top of a SPS, introduce a mechanism to provide additional resource in frequency domain periodically or based on demand (i.e., SR/BSR). FFS how to do.

UP IP vs UP non-IP

Whether only one solution between UP IP and UP non-IP or both solutions would be pursued will depend on the outcome of SA2 progress. For UP IP solution, RoHC may become a headache due to its nature of operating in different stages with different RoHC header sizes and frequent RoHC feedback for synchronization between compressor and decompressor over the NLOS GEO channel. NB-IoT currently also does not support the RoHC profile with RTP traffic. We think discussion focussing on UP IP can wait to see outcome of SA2 discussion.

Wait for SA2 decision to start working on specific to UP IP solution or UP non-IP solution.

- Conclusion

Following proposals are made:

Proposal 1	For UP solution, current total L2 buffer size is not extended for NB-IoT UEs supporting voice over GEO.

Proposal 2	For UP solution, wait for SA2 progress on the maximum number of DRBs (e.g., 4 or 8) that need to be supported simultaneously.

Proposal 1	Wait for SA2 progress on discussion of QoS and GBR for DRBs.

Proposal 2	Introduce mo-Voice-r20 as an establishment cause.

Proposal 3	UL SPS and DL SPS are introduced to carry data from DRBs associated with voice. FFS on details of logical channel prioritization for UL SPS.

Proposal 4	UL SPS and DL SPS corresponding to voice traffic will not use HARQ RTT timer regardless of HARQ process ID.

Proposal 5	On top of a SPS, introduce a mechanism to provide additional resource in frequency domain periodically or based on demand (i.e., SR/BSR). FFS how to do.

Proposal 6	Wait for SA2 decision to start working on specific to UP IP solution or UP non-IP solution.

- References

[1] RP-251867, “New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4”, Prague, Czech Republic, June 9-13, 2025.

[2] R2-2507784, “Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC”, Prague, Czech Republic, October 13th – 17th, 2025.