3GPP TSG-RAN WG2 Meeting #132	R2-2508827

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:		10.3.2.3

Source:		Futurewei

Title:		Discussions on 6G initial and system access

Document for:		Discussion and Decision

# Introduction

During RAN#108 meeting, a new study item on “6G Radio” was approved [1]. The SID is further revised in RAN#109. According to the revised SID [2], the following key objectives of the study are related to “Initial and System Access for 6GR”:

The detailed objectives of the study are: ………. - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] ……….

In addition, during RAN2#131bis, the following agreements related to 6GR Initial and System Access were reached [3]:

In this paper, we discuss on-demand SIB, SSBs, paging, and aspects related to spectrum aggregation mainly from energy efficiency perspective.

# Discussions

## Energy efficiency for SSB transmissions

In 5G NR, periodic transmissions such as synchronization signal blocks (SSBs) are essential for functions like initial access, RRM measurement, and beam or CSI acquisition. However, these always-on transmissions cause continuous energy consumption, reducing overall network energy efficiency. To mitigate this, 5G introduced Network Energy Saving (NES) solutions that aim to reduce or replace periodic signaling with on-demand mechanisms. For example, Release 19 defined techniques, such as reduced periodic transmission of common signals, SSB transmission time adaptation, and on-demand SSB delivery to balance functionality with lower energy use. Specifically, SSB transmission time adaptation enables dynamic adjustment of SSB burst periodicity based on traffic load, even though it is limited to non-CD-SSB. The SSB-less SCell concept, first supported in Release 15 for intra-band contiguous FR1, was extended in later releases to inter-band FR1 co-located scenarios, with defined methods for obtaining necessary synchronization and measurements. Furthermore, on-demand SSB was introduced for connected-mode UEs with carrier aggregation, allowing temporary SSB transmission via NW signaling in both intra- and inter-band CA cases.

The NES features introduced in 5G Release 19, such as on-demand SSB, and SSB transmission time adaptation, were designed with constraints to maintain backward compatibility and minimize specification complexity. However, these limitations reduced their potential energy-saving benefits. For example, time adaptation of SSB periodicity was not supported for CD-SSB to avoid affecting initial access latency for legacy UEs. For 6G, these features can be extended by replacing frequent periodic SSB transmissions (e.g., every 20 ms) with infrequent periodic transmissions (e.g., every 160 ms) for both CD-SSB and NCD-SSB types, allowing base stations to enter low-power states. This approach can be complemented by on-demand SSB transmissions in any cell and for UEs in any RRC state, triggered by events such as traffic arrival, uplink WUS, or cell activation, thereby enabling substantial energy savings while preserving acceptable access latency.

In addition, a light/simplified Sync signal (e.g., PSS/SSS, or sequence only structure), or a lightweight Sync signal block (e.g., PSS/SSS+ minimum/light MIB) may also be used to compensate for the longer Sync signal (+PBCH) periodicity and maintain the cell search complexity while providing network energy saving gain. Under this paradigm, frequent periodic transmissions with high per-transmission payload (i.e., full SSB size) are replaced with a combination of the frequent periodic light Sync signal transmissions (e.g., synchronization signals only, such as PSS and/or SSS) or the frequent lightweight SSB transmissions and infrequent periodic (or on-demand) transmissions of MIB (e.g., PBCH), i.e., separation of synchronization and PBCH transmissions (or part of PBCH). The light Sync signal transmissions or the lightweight SSB transmissions may be supported (up to implementation) by a low power transmitter ensuring that the BS can still benefit from energy saving by allowing the higher power radio to remain in a sleep state for longer periods.

Therefore, based on the above analysis and discussion, we have the following observation and proposals:

Observation 1: Replacing frequent SSB transmissions with infrequent periodic transmissions for any SSB type (CD-SSB and NCD-SSB) can improve network energy saving. Support of on-demand synchronization (SSB) can minimize the impact of the infrequent periodic SSB transmission on UE access latency.

Proposal 1: RAN2 to study infrequent periodic transmission of SSB for network energy saving and further combining with on-demand SSB to minimize impacts on UE access latency.

Proposal 2: RAN2 to study frequent periodic transmission of lightweight SSB (or light Sync signal) and further combining with on-demand (or infrequent periodic) MIB to minimize impacts on UE access latency.

## Energy efficiency with on-demand SIB1

System Information Block Type 1 (SIB1) in 5G NR provides essential broadcast information required by UEs for cell access, including scheduling of other SIBs, PLMN identity, and cell selection parameters. In conventional designs, SIB1 is periodically transmitted in every active cell regardless of user presence, ensuring immediate accessibility, but leading to significant energy inefficiency, particularly in low-traffic or dormant cells. To mitigate this, 5G NR in R19 introduced the on-demand SIB1 feature as part of the NES enhancements, complementing other features such as on-demand SSB and SSB transmission time adaptation. The main objective is to reduce unnecessary SIB1 broadcasting while preserving acceptable access latency and ensuring reliable initial access procedures.

The on-demand SIB1 mechanism allows the network to suspend periodic SIB1 transmission and instead activate it only, when necessary, triggered by specific events such as UE access attempts, UL WUS, or cell reactivation. Under this framework, a UE initially acquires minimal system information (via MIB and SSB) and then requests SIB1 through a defined on-demand signaling procedure. NW dynamically controls SIB1 transmission based on network load, UE requests, or energy-saving policies. This event-driven approach for on-demand SIB1 provides substantial energy savings at the cell and network levels, while the flexible scheduling and activation mechanisms introduced in Release 19 ensure that access latency remains within acceptable limits. For example, On-demand SIB1 was defined for IDLE and INACTIVE UEs, where UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell. However, the assumption that the NES cell was always under the coverage of an assisting cell (Cell A) was required for the on-demand SIB1 feature, which may not always be satisfied in real deployments. Another assumption for the on-demand SIB1 feature was the availability of an always-on SSB in the NES cell. The later assumption limited the potential combination of the on-demand SIB1 feature with other features, such as the on-demand SSB, which itself was limited to SCell operation for UE in connected mode with CA.

Looking forward to 6G, the on-demand SIB1 transmission mechanisms specified in 5G NR can also provide a foundation for adaptive and dynamic system information management in 6G. These mechanisms should be considered and further enhanced in 6G. For example, certain limitations and assumptions imposed in the current on-demand SIB1 design, as previously discussed, should be revisited and relaxed to fully unlock their potential in 6G networks. Moreover, future systems in 6G are expected to incorporate context-aware, AI-assisted control of system information transmission, where SIB1 and other SIBs are transmitted only when justified by event driven demand. Such evolution toward a fully on-demand and context-driven system information framework will enable fine-grained trade-offs between energy efficiency, latency, and user experience, supporting sustainable and self-optimizing operation in 6G networks.

Based on the above discussions, we have the following observation and proposals:

Observation 2: This event-driven approach for on-demand SIB1 provides substantial energy savings at the cell and network levels, while the flexible scheduling and activation mechanisms introduced in Release 19 ensure that access latency remains within acceptable limits.

Proposal 3: RAN2 to consider the on-demand SIB1 transmission mechanisms specified in 5G NR as a baseline for adaptive and dynamic system information management in 6G and further study their corresponding enhancements.

Proposal 4: RAN2 to study an event-driven approach for on-demand SIB1, incorporating context-aware and AI-assisted control of system information transmission, where SIB1 and other SIBs are transmitted only when justified by event driven demand.

## Energy efficiency for paging transmissions

In 5G NR, several mechanisms are introduced to improve energy efficiency during the paging procedure. These include Discontinuous Reception (DRX) cycles, allowing the UE to periodically wake up for paging occasions, and paging grouping mechanisms, which distribute UEs across different paging frames or occasions to reduce unnecessary monitoring. Additionally, UE-specific DRX configurations, coverage-based paging repetition, and Paging Early Indication (PEI) enable reduced idle-mode power consumption while maintaining reliable reachability. In Release 18 and beyond, features such as Low Power Wake-Up Signal (LP-WUS) and paging assistance information have been further introduced to enable more selective and event-driven paging, minimizing the active monitoring time of UEs.

Regarding 6GR, all the above energy efficiency mechanisms of 5G NR can be considered and revisited/enhanced. For example, multiple consecutive legacy (i.e., 5G NR) paging frames (PFs) can be clustered/combined in time domain, e.g., a single or successive frames, for a joint or aligned design of cell DTX/DRX and UE duty cycled operations (i.e., IDRX, eDRX, and/or cDRX). The paging probability per legacy PF, and corresponding UE energy consumption, can be maintained by combining with the LP-WUS feature, where the LP-WUS may further indicate/provide grouping information instead (or in addition to) subgrouping information. In addition, paging procedures of 6GR are also expected to evolve into context-aware and AI-assisted frameworks that dynamically adjust paging timing, repetition, and beam configurations based on network load, user mobility, and service type. Event-driven or predictive paging, where paging is triggered by traffic demand or user context (e.g., mobility trajectory or sensor signals), could significantly reduce unnecessary paging broadcasts. Furthermore, the integration of cross-layer wake-up control and multi-RAT coordination would enable energy-efficient paging across heterogeneous cells and carriers in 6G multi-layer networks.

Therefore, based on the above analysis and discussion, we have the following observation and proposals:

Observation 3: LP-WUS support of grouping and/or subgrouping information indication and combining with paging frames (PFs) clustering/grouping can control the paging probability and corresponding UE energy consumption.

Proposal 5: RAN2 to consider paging frames (PFs) clustering/grouping combined with grouping information indication by LP-WUS as an approach for aligned Cell DTX/DRX and UE DRX.

Proposal 6: RAN2 to study event-driven or predictive paging, where paging is triggered by traffic demand or user context.

## Aspects related to spectrum aggregation

In 5G NR, the concept of a Component Carrier (CC) was introduced to support Carrier Aggregation (CA), where multiple contiguous frequency blocks are combined to provide higher bandwidth to UE. Each carrier corresponds to a single cell, with the Primary Cell (PCell) linked to the Primary Component Carrier (PCC) handling main control and data signaling, and Secondary Cells (SCells) linked to Secondary Component Carriers (SCCs) providing additional capacity. Although CA increases flexibility and throughput, it also introduces challenges such as managing different numerologies, scheduling complexity, uplink synchronization, and increased power consumption due to varying carrier conditions. To address these, several energy-saving mechanisms were developed, including SSB-less SCell operation, on-demand SSB transmission (Rel-19), cross-carrier scheduling, and the dormant BWP concept. All aimed at reducing energy use by allowing SCells and UEs to remain in low-power states when possible.

In 6GR, these energy-efficiency techniques should be extended and enhanced to account for more advanced CA scenarios. For instance, for the scenarios when SSB of the PCell cannot be reused in the SCell, an on-demand SSB operation for SCell was defined in Rel 19. When supported, the feature allows transmission of on-demand SSB only during the activation of SCell, which reduces the network energy consumption. Moreover, the current 5G design enforces a one-to-one mapping between cells and carriers, resulting in service interruption when the PCell link fails, since all associated SCells are consequently regarded as lost. A promising evolution for 6G would be to allow one cell to comprise multiple carriers, eliminating redundant periodic signaling (SSB/SIBx) across carriers and enabling SSB/SIBx-less or on-demand operations. In this structure, as long as at least one carrier within the cell remains in good condition, the cell is no longer prone to failure as in previous 5G designs. This design would not only enhance reliability but also yield significant energy savings for both the UE and the network.

Therefore, based on the above analysis and discussion, we have the following proposal:

Proposal 7: RAN2 to study energy efficiency for CA in 6GR in the context of both single cell multiple carriers and one carrier one cell.

# Conclusions

Observation 1: Replacing frequent SSB transmissions with infrequent periodic transmissions for any SSB type (CD-SSB and NCD-SSB) can improve network energy saving. Support of on-demand synchronization (SSB) can minimize the impact of the infrequent periodic SSB transmission on UE access latency.

Observation 2: This event-driven approach for on-demand SIB1 provides substantial energy savings at the cell and network levels, while the flexible scheduling and activation mechanisms introduced in Release 19 ensure that access latency remains within acceptable limits.

Observation 3: LP-WUS support of grouping and/or subgrouping information indication and combining with paging frames (PFs) clustering/grouping can control the paging probability and corresponding UE energy consumption.

Based on these observations, we have the following proposals:

Proposal 1: RAN2 to study infrequent periodic transmission of SSB for network energy saving and further combining with on-demand SSB to minimize impacts on UE access latency.

Proposal 2: RAN2 to study frequent periodic transmission of lightweight SSB (or light Sync signal) and further combining with on-demand (or infrequent periodic) MIB to minimize impacts on UE access latency.

Proposal 3: RAN2 to consider the on-demand SIB1 transmission mechanisms specified in 5G NR as a baseline for adaptive and dynamic system information management in 6G and further study their corresponding enhancements.

Proposal 4: RAN2 to study an event-driven approach for on-demand SIB1, incorporating context-aware and AI-assisted control of system information transmission, where SIB1 and other SIBs are transmitted only when justified by event driven demand.

Proposal 5: RAN2 to consider paging frames (PFs) clustering/grouping combined with grouping information indication by LP-WUS as an approach for aligned Cell DTX/DRX and UE DRX.

Proposal 6: RAN2 to study event-driven or predictive paging, where paging is triggered by traffic demand or user context.

Proposal 7: RAN2 to study energy efficiency for CA in 6GR in the context of both single cell multiple carriers and one carrier one cell.

# References

RP-251881	 New SID: Study on 6G Radio,	NTT DOCOMO, RAN#108.

RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

R2_131bis_ChairNotes_25-10-17_17-00_final.zip.