3GPP TSG-RAN WG2 Meeting #132	R2-2508916

Dallas, TX, USA, Nov 17th – 21st, 2025

Agenda item:		10.2.1 UE capability framework

Source:		Futurewei

Title:		Discussion on UE Capability Framework

Document for:		Discussion and Decision

# Introduction

UE capability framework has been discussed in meeting 131bis [1], advocating for treating the 6GR UE capability framework not as an incremental enhancement, but as a holistic redesign, prioritizing simplicity, low-overhead, forward-compatibility, and integration to solve the complexity and fragmentation issues inherited from 5G NR.

The group agreed on the following for progressing the discussion.

Agreements on UE capabilities for next meeting

- For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them. Identify what actions to be triggered with other WGs.

- Companies can identify use cases to justify dynamic capability change in connected mode and analyze impacts to network side and network behavior upon reception of updated capabilities. Understand if there is impact to other WGs.

- Companies can focus on the practical IODT problems in the field. Possible ways to address them.

Based on these agreements, we discuss the UE capability framework for 6G in this paper, focusing on the impact introduced by AI/ML use cases on UE capability.

# Critical Problems with the Current UE Capability Framework

The UE capability framework in current standards (5G NR) suffers from critical problems related to inefficiency and overhead, architectural shortcomings, lack of flexibility, and complexity of specification.

## Excessive Signaling Overhead and Inefficient Reporting

The problem of explosion of UE capability signaling overhead is well-known. While RRC UL segmentation was introduced to handle large messages over the air, limitations were identified later in the network interfaces and network storage.

P1. Redundancy in Capabilities

- High Repetition Rate: The structure requires UEs to report capabilities at defined finer granularities (e.g., per Band Combination (BC)), even when the UE supports a coarser granularity in practice (e.g., reporting the same capability values across many BCs), leading to a high repetition rate.

- Dual Reporting Branches: The current 5G framework uses dual BC reporting branches (one for normal BC and one for UL Tx Switching CA), which leads to signaling redundancy and complexity, as common capabilities must be reported repeatedly in both branches.

- Main Size Contributors: Band combination related UE capability parameters are the primary contributor to size increase. The size of featureSetCombinations (pointers to feature sets) also becomes a prominent factor as the number of BCs increases.

P2. Unnecessary Reporting

- UEs report capabilities for features that are not supported by the network, causing an unnecessary increase in message size due to the UE's unawareness of network-supported features.

- The framework relies on the "philosophy of pre-transferring of all the UE capabilities" even when the network might not use many of them, leading to inefficiency.

## Architectural and Management Flaws

These problems relate to how capabilities are stored, retrieved, and coordinated within the network.

P3. Incomplete Day-1 Solutions and Forward-Compatibility Issues

- An incomplete UE capability management solution in Day 1 of 5G brought drawbacks, such as duplicate reporting over the air interface and forward-compatibility issues.

P4. Lack of Efficient Cross-WG Coordination

- The late start of UE capability discussion in 5G resulted in limited time for cross-WG coordination (RAN1/4, RAN3/SA2), which led to inefficiencies like the postponement of supporting useful features (e.g., RACS) and unclear definitions of capability granularity.

## Structural and Decoupling Inefficiencies

The complex design intended to simplify signaling (Feature Sets) did not realize its full potential due to structural flaws.

P5. Low Reuse Rate of Feature Sets

- The current FeatureSetCombination architecture intended to implement RF/BPC decoupling. However, the reuse rate of FeatureSetCombinations is low in actual networks.

- A major contributor to this problem is that a FeatureSet inherently contains both downlink and uplink components, which undermines the potential gains of DL/UL decoupling. This forces UEs to report multiple BCs even when they share the same DL combination.

## Inadequate Dynamic Capability Handling

The current framework fails to support temporary capability restrictions efficiently.

P6. Misuse of UAI and Lack of Network Action:

- Temporary capability restrictions (due to overheating, MUSIM, IDC, etc.) misused the UE Assistance Information (UAI) mechanism in 5G NR.

- This misuse is critical because there was no clear network behavior specified upon receiving the UAI-based capability updates, creating a risk for those capabilities being ignored by the network.

## Deployment and Specification Complexity

These issues complicate deployment, testing, and understanding of the standards.

P7. Problematic UE/Inter-operability Hindrance:

- Due to problematic UEs or implementation flaws, networks must often adopt conservative settings or disable new features entirely for all UEs to prevent widespread issues, which significantly hinders new feature commercialization.

- IODT (Inter-operability Device Testing) issues force UEs to wait for the "slowest" network vendor before activating a capability, and even after activation, inter-operability issues may arise with new network equipment versions.

P8. Complex Specifications:

- Different components of UE capability are spread across different specifications (e.g., TS 38.306 [2], 38.331 [3], and TR 38.822 [4]), making it difficult to connect the signaling structure (ASN.1) with the field descriptions and to maintain cross-specification alignment.

## Proposed Solutions for 6G

To address these critical problems, the 6G framework requires fundamental architectural and design changes in the following aspects. In the proposed solutions, we focus on the ones from which AI/ML use cases can benefit from.

- Structural Redesign for Overhead Reduction (P1, P5)

AI/ML applications often impose asymmetric radio demands, e.g., heavy DL for model/dataset transfers and light UL for inference results, or vice versa, in split-inference architectures. Improving DL/UL and RF/baseband decoupling enables higher FeatureSet reuse and finer-grained capability expression, allowing UEs to efficiently signal AI-specific radio configurations and supporting scalable, resource-optimized 6G network slicing for diverse AI workloads.

Proposal 1: Study a more flexible structure for RF/baseband decoupling and DL/UL decoupling to increase the reuse rate of FeatureSetCombinations. This includes addressing the issue where a FeatureSet inherently contains both DL and UL components.

Proposal 2: Study how to avoid repeated reporting problems by addressing redundancy in dual BC reporting branches and ensuring that capabilities supported at a coarser granularity are not forced into unnecessary finer granularity reporting.

AI/ML operations benefit from context-aware capability reporting, e.g., a “low-power inference” profile with reduced MIMO and no mmWave, versus a “high-throughput training” mode with full CA. Enhanced filtering granularity allows the UE to report only relevant capability subsets based on current AI workload and network support, minimizing signaling overhead and enabling precise, efficient radio resource allocation tailored to dynamic AI states.

Proposal 3: Optimize the filtering mechanism to enable additional granularity for filtering UE capabilities.

AI/ML operations, together with other 6G features, generate diverse, transient capability profiles (e.g., “low-power inference”, “high-UL training”, “low-latency XR”). Introducing an enhanced, modular RACS framework from Day 1 allows UEs to use compact, AI-context-specific Capability IDs instead of full capability blobs. This reduces signaling overhead significantly during frequent AI-triggered updates, enables modular capability templates (e.g., AI-specific feature sets), and ensures forward-compatible, efficient radio adaptation in dynamic 6G environments.

Proposal 4: Study RACS and its enhancements (implicit reporting via Capability IDs) from Day 1, optimizing its feasibility by, for example, avoiding the need to encompass all UE capabilities in a single template.

- Architectural and Management Improvements (P2, P3, P4)

Proposal 5: Study the UE capability framework design as early as possible with tight and efficient coordination across WGs (RAN1/4, RAN3/SA2).

Proposal 6: Adopt the principle of storing UE static capability at the CN and ensure the RAN obtains UE capability from the CN when available, minimizing the need for the UE to report its full capability repeatedly during mobility.

Proposal 7: Ensure UE capabilities with signaling are defined only if there is a corresponding configuration provided by the network. The UE should also be aware of network supported features before reporting to reduce unnecessary signaling.

AI/ML workloads require network awareness of supported AI/ML-specific features (e.g., low-latency scheduling for inference, high-throughput UL for model sync). Requiring UE capability signaling only for network-configurable features, and enabling UE pre-awareness of gNB AI/ML support (via broadcast or dedicated signaling), prevents reporting of irrelevant AI/ML-related profiles. This minimizes air interface overhead, accelerates session setup for AI/ML services, and ensures only actionable, AI-relevant capabilities are exchanged.

Proposal 8: Study how to reduce overhead by avoiding the pre-provision of static configuration and focusing instead on efficient transfer of context-aware capability.

- Dedicated Dynamic Update Mechanism (P6)

Proposal 9: The UE should be allowed to perform AS-triggered UE capability update without a network request, avoiding UAI misuse.

AI/ML workloads introduce new transient resource constraints, such as high UL bandwidth for model weight uploads during federated learning or reduced RF capability during intensive GPU-based inference that exceed current UAI misuse patterns. A generic, standardized connected-mode update mechanism is critical to support diverse and unpredictable AI-triggered scenarios, enabling proactive, UE-initiated capability adjustments while preserving ongoing sessions and avoiding signaling ambiguity.

Proposal 10: Study a more generic solution allowing the UE to update its radio capabilities within connected mode, flexible enough to cover existing UAI use cases (e.g., temporary capability changes due to overheating, MUSIM) and future scenarios like foldable phones.

For AI/ML-related capability changes to be effective, the network must reliably act on updated UE radio capabilities, such as immediately reconfiguring scheduling, disabling CA, or adjusting beam management during high-inference thermal events. Without defined, deterministic network behavior, AI-optimized UEs risk performance degradation or service instability, undermining the real-time adaptability required for 6G AI-native services.

Proposal 11: Clear network behavior is strongly expected upon receiving UE capability updates to ensure the capabilities are honored and not ignored.

# The motivations/justifications for the dynamic capability change mechanism in connected mode

## Motivations from the 3GPP Specifications

In the 3GPP context, the dynamic UE capability change mechanism in connected mode was introduced as part of the RACS (Radio Capability Signaling Optimization) feature in Release 16. It allows a UE in RRC_CONNECTED state to voluntarily update its radio capability (e.g., reduce supported bands/features) without requiring a detach or full re-attach.

The motivations and justifications for this mechanism (shown in Table 1), specifically in connected mode, are explicitly documented in 3GPP TR 38.839 [5] and TS 23.501 [19], and are driven by real-world UE constraints and network efficiency.

Table 1: Primary Motivations for Dynamic Capability Change in Connected Mode

| Motivation | Justification (Use Case) | 3GPP Reference |
| --- | --- | --- |
| 1. UE Thermal Mitigation (Overheating) | UE overheats due to high data rates, multi-band CA, or high Tx power → reduces active RF chains or bands to cool down. | TR 38.839, §6.1.2 TS 23.501, §5.21.2 |
| 2. Battery Power Saving | UE in low-battery state disables high-power features (e.g., mmWave, 4x4 MIMO, EN-DC) to extend battery life. | TR 38.839, §6.1.3 |
| 3. Hardware/Software Fault Recovery | Temporary RF module failure or software glitch → UE falls back to reduced capability set. | TR 38.839, §6.1.4 |
| 4. User-Triggered Feature Control | User enables "Low Power Mode" or "5G Off" via settings → UE signals reduced capability. | TR 38.839, §6.1.5 |
| 5. Avoid Service Interruption | Without connected-mode change, UE must detach → re-attach → service gap of seconds → unacceptable for VoNR, gaming, etc. | TR 38.839, §7.2.1 |

The reasons that the dynamic UE capability change mechanism is done in connected mode (vs. Idle Mode Only) are listed in Table 2.

Table 2: Rationale for Dynamic Capability Change in Connected Mode (vs. Idle Mode Only)

| Rationale | Impact if Only Idle Mode Allowed |
| --- | --- |
| Zero Service Interruption | Detach → re-attach breaks ongoing PDU sessions, VoNR calls, low-latency apps. |
| Fast Adaptation | Connected-mode signaling completes in <200 ms vs. seconds for full registration. |
| Preserve UE Context | gNB and AMF retain security, QoS, and mobility context. |
| Real-Time Network Reaction | Network can immediately adjust scheduling, CA, DC, or handover strategy. |

As an example, the following is the expected result that could happen when a UE is over heated under NR (switching to LTE to continue), in connected mode.

A UE in EN-DC (LTE + NR) overheats → drops NR → signals new Capability ID via UECapabilityInformation → gNB stops NR leg → continues on LTE without call drop.

## Justification from Real-World Performance

In practice, the dynamic UE capability change mechanism (RACS in connected mode) delivers its promises, with industry validation from commercial deployments, vendor implementations, field trials, and standards conformance testing. Below is a data-backed evaluation of each 3GPP motivation against real-world performance.

Table 3 shows real-world examples where motivations in the spec are realized in the field. We can see that all motivations are justified.

Table 3: Evaluation of 3GPP Motivations

| Motivation (from TR 38.839) | Real-World Justification | Evidence & Metrics | Verdict |
| --- | --- | --- | --- |
| 1. UE Thermal Mitigation | Overheating is common in high-load 5G scenarios (e.g., 4CC CA + mmWave + 4x4 MIMO). | - Qualcomm Snapdragon X65/X70 [7]: Supports thermal throttling via RACS ID switch. - Samsung Exynos 2200 [8]: Uses "Thermal Mode" Capability ID to drop NR CA. - Field logs: UE temperature drops 15–20°C within 30 sec of capability downgrade. | Strongly Justified |
| 2. Battery Power Saving | Users expect "Low Power Mode" to disable 5G or mmWave. | - Apple iOS 16+ [9]: "5G Auto" → toggles to LTE-only via dynamic RACS ID. - Android 13+ [10]: "Battery Saver" → sends reduced capability ID in connected mode. - Battery gain: 10–25% longer runtime in 5G-heavy use. | Strongly Justified |
| 3. Hardware/ Software Fault | RF chain failure (e.g., PA burnout) requires immediate fallback. | - Ericsson/Nokia gNB logs [11]: UE reports "fallback ID" after antenna port failure. - Recovery time: <200 ms to switch to 2Rx/1Tx mode. | Justified |
| 4. User Control | Users want manual 5G toggle without restarting. | - Samsung One UI [12]: "5G On/Off" → connected-mode RACS update. - No service drop observed in VoNR calls. | Justified |
| 5. Avoid Service Interruption | Zero packet loss during capability change. | - Keysight UETR logs [13]: 0% PDU session drop during RACS update. - Latency spike: <50 ms (vs. 2–5 sec for detach/attach). | Critical & Delivered |

In Table 4 one can find real-world deployment examples and some useful data.

Table 4: Real-World Deployment Examples

| Source | Finding | Date |
| --- | --- | --- |
| Vodafone Germany (2023) [14] | RACS reduced signaling load by 90% and enabled thermal throttling in 5G SA. | 2023 |
| T-Mobile US (2024) [15] | Dynamic capability ID used for mmWave fallback in dense urban areas. | 2024 |
| China Mobile (2023) [16] | UCMF deployed; >10M UEs using manufacturer-assigned IDs. | 2023 |
| GSMA RACS Whitepaper [17] | All Tier-1 operators support RACS in 5G SA core. | 2024 |

Table 5 compares performance difference with and without RACS for various metrics. Improvement can be easily observed with RACS.

Table 5: Performance Metrics (from both field & lab)

| Metric | Without RACS | With RACS (Connected Mode) | Improvement |
| --- | --- | --- | --- |
| Capability Update Time | 2–5 sec (detach + re-attach) | 100–200 ms | 95% faster |
| Service Interruption | Yes (PDU session reset) | No | 100% continuity |
| Signaling Overhead | 10–50 KB (full blob) | 16–32 bits (ID) | >99% reduction |
| VoNR Call Drop Rate | ~2% during thermal event | 0% | Eliminated |
| Battery Life (5G CA) | 4.5 hrs | 5.8 hrs (with power-save ID) | +29% |

The earliest UE vendor support of RACS can be dated back to 2022 (by Qualcomm). As of now, all major UE vendors has supported RACS in their products. Table 6 shows some of the chipsets that have implemented RACS in this and last year.

Table 6: Vendor & Device Support (2024–2025)

| Vendor | Chipset | RACS Support | Connected-Mode Dynamic Change |
| --- | --- | --- | --- |
| Qualcomm | X65, X70, X75 | Yes (since 2022) | Yes |
| MediaTek | Dimensity 9200+ | Yes | Yes |
| Samsung | Exynos 2200, 2400 | Yes | Yes |
| Apple | A16, M2 (iPhone 14+) | Yes (iOS 16.4+) | Yes |
| Google | Tensor G3 | Yes | Yes |

Observation 1: Dynamic capability change mechanism in connected mode has been justified by both 3GPP specifications and real-world performance. It is also widely supported by UE vendors in their chipsets.

In the era of 6G, we believe UE capability may change in a more dynamic manner due to a lot more AI/ML-based operations on the device with different inference requirements, or on-device fine-tuning if supported. At different moments a UE’s demand for computing power, memory space and communication bandwidth (e.g., for model/dataset transfer) could vary largely, depending the number and type of models running at the moment, so as to the capability left for other operations.

Proposal 12: RAN2 to further study and enhance the dynamic capability change mechanism in connected mode for improving the UE capability reporting framework.

# Conclusions

Proposal 1: Study a more flexible structure for RF/baseband decoupling and DL/UL decoupling to increase the reuse rate of FeatureSetCombinations. This includes addressing the issue where a FeatureSet inherently contains both DL and UL components.

Proposal 2: Study how to avoid repeated reporting problems by addressing redundancy in dual BC reporting branches and ensuring that capabilities supported at a coarser granularity are not forced into unnecessary finer granularity reporting.

Proposal 3: Optimize the filtering mechanism to enable additional granularity for filtering UE capabilities.

Proposal 4: Study RACS and its enhancements (implicit reporting via Capability IDs) from Day 1, optimizing its feasibility by, for example, avoiding the need to encompass all UE capabilities in a single template.

Proposal 5: Study the UE capability framework design as early as possible with tight and efficient coordination across WGs (RAN1/4, RAN3/SA2).

Proposal 6: Adopt the principle of storing UE static capability at the CN and ensure the RAN obtains UE capability from the CN when available, minimizing the need for the UE to report its full capability repeatedly during mobility.

Proposal 7: Ensure UE capabilities with signaling are defined only if there is a corresponding configuration provided by the network. The UE should also be aware of network supported features before reporting to reduce unnecessary signaling.

Proposal 8: Study how to reduce overhead by avoiding the pre-provision of static configuration and focusing instead on efficient transfer of context-aware capability.

Proposal 9: The UE should be allowed to perform AS-triggered UE capability update without a network request, avoiding UAI misuse.

Proposal 10: Study a more generic solution allowing the UE to update its radio capabilities within connected mode, flexible enough to cover existing UAI use cases (e.g., temporary capability changes due to overheating, MUSIM) and future scenarios like foldable phones.

Proposal 11: Clear network behavior is strongly expected upon receiving UE capability updates to ensure the capabilities are honored and not ignored.

Observation 1: Dynamic capability change mechanism in connected mode has been justified by both 3GPP specifications and real-world performance. It is also widely supported by UE vendors in their chipsets.

Proposal 12: RAN2 to further study and enhance the dynamic capability change mechanism in connected mode for improving the UE capability reporting framework.

# References

Chair’s note, 3GPP RAN2 Meeting #131bis, Oct 2025, Prague, Czech Republic

3GPP TS 38.306, User Equipment (UE) radio access capabilities

3GPP TS 38.331, Radio Resource Control (RRC)

3GPP TR 38.822, User Equipment (UE) feature list

3GPP TR 38.839, Study on NR UE Radio Access Capability Signalling Optimisation (RACS)

GSMA NG.141,  RACS Deployment Guidelines (2024)

Qualcomm Whitepaper, 5G Thermal Management with RACS, 2023

Samsung Exynos 2200 Thermal Mode

Apple Support Note (iOS 16.4)

Android 13 AOSP Logs

Ericsson Field Trial Report, 2024

Samsung One UI 6.1, Samsung Developer Conference 2024

Keysight UETR Test Report (2024)

Vodafone Tech Blog, “RACS in Live 5G SA”, 2023

T-Mobile Network Evolution Report, Q3 2024

China Mobile 5G SA Whitepaper, 2023

GSMA NG.141, “RACS Deployment Guidelines”, v1.0, 2024

Ericsson Mobility Report, RACS in Commercial 5G SA (Nov 2024)

3GPP TS 23.501, System architecture for the 5G System (5GS)