3GPP TSG-RAN WG2 Meeting #132	R2- 2508939

Dallas, USA, November 17h – 21th, 2025

Agenda Item:	10.2.1 UE capability framework

Source:	ETRI

Title:	Consideration on 6G UE capability framework

Document for:	Discussion and Decision

1	Introduction

The RAN2 discussions on 6G Radio Access Network (6GR RAN2) began at the recent RAN2#131 bis meeting. Agreements were made concerning UE capabilities, which are outlined below.

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

This paper highlights the observations and proposals related to UE capability framework evolution in 6G, emphasizing overhead reduction, service-driven grouping, and interoperability improvement.

2	Discussion

The 6G wireless system aims to deliver seamless connectivity, extreme performance, and native AI integration across a diverse set of device types — from traditional smartphones to edge AI modules and sensor networks. A key enabler of this ecosystem is a robust and scalable UE capability framework, which defines how devices communicate their functional capabilities to the network, ensuring proper feature negotiation, optimization, and service continuity. 5G NR provided a flexible mechanism to report UE capabilities; however, the system became too complex and inefficient due to the lack of a unified baseline and excessive optional features.

The 6G framework must therefore address signaling scalability, dynamic adaptability, and cross-layer consistency, while retaining flexibility for future use cases.

Key Lessons from 5G NR

5G NR introduced a flexible UE capability framework to accommodate a wide range of use cases. However, this approach resulted in several challenges:

- Excessive Flexibility and Ambiguity

Defining numerous optional capabilities without sufficient mandatory requirements for specific use cases resulted in ambiguity regarding essential features. This led to uncertainty in interoperability testing and fragmentation between device types, as vendors selectively implemented different subsets.

Networks often struggled to determine whether a UE was fully compatible with a given feature set, which has delayed the deployment and optimization of new features.

- Signaling Overhead

UE capability signaling messages in NR became extremely large, particularly due to band combination and Feature Set Combination reuse limitations.

The number of Band Combination related parameters is to the increase in the UE capability message size. The framework is inefficient as the UE sends its entire capability set upfront, even if the network may not support a part of it. These messages sometimes exceeded RRC message size limits, requiring segmentation, which introduced latency and resource waste.

- Management and Compatibility Issues

The proliferation of optional features across multiple releases (Rel-15 through Rel-18) has created significant versioning and backward-compatibility challenges, requiring networks to implement increasingly complex capability handling logic.

Additionally, inconsistent capability definitions among different vendors have complicated operations in multi-vendor network environments and hindered effective interoperability verification.

5G NR was too flexible, resulting in unclear mandatory requirements, excessive signaling overhead, and increased testing complexity. This hindered interoperability and delayed network optimization and feature deployment.

6G UE Capability Framework Design Principles

In 6G, a classic trade-off in UE capability design is the problem. Allowing all possible combinations leads to combinatorial explosion which increases protocol overhead. This recurring challenge creates technical bottlenecks. When every feature is debated as “mandatory or optional” to limit the combinatorial explosion, progress slows and standardization becomes stuck. To avoid this stagnation, several alternative design strategies can be discussed.

- Define a fundamental Baseline

All 6G devices should follow a core capability baseline that guarantees essential functions such as connectivity, synchronization, mobility, and basic MIMO operation. This baseline can take inspiration from simplified device categories like RedCap (Reduced Capability UE) or IoT devices, which already demonstrate minimal yet sufficient functionality. Alternatively, it could be defined as a “Rel-15/16 Core-Only” baseline, representing the minimal feature set required for universal interoperability.

- Service-Driven Capability

Capabilities should be grouped by service, not by individual feature. For example, instead of declaring hundreds of parameters, a UE can simply state: “support eMBB and URLLC profiles.”
Each profile (e.g., embb-Capability, urllc-Capability) represents a predefined service container with mandatory and optional elements. This reduces signaling overhead and simplifies configuration. This approach enables vertical optimization without burdening unrelated device types. In addition, it can be defining vendor or device-specific extensions as separate capabilities into layers. This prevents mandatory-feature disputes from blocking progress, while preserving flexibility for innovation.

To define a fundamental Core Capability Baseline guaranteeing essential functions for all devices and move towards Service-Driven Capability Grouping (profiles like eMBB/URLLC) to simplify configuration and avoid feature dispute.

- Simplify Overhead Through Configuration

Signaling overhead should be solved mainly in configuration procedures (e.g., RRC Reconfiguration), not in capability signaling. Capability reports remain static (describing what the UE can do) while configurations are dynamic (describing what the UE is currently doing). 6G should leverage existing mechanisms of capability storage in the gNodeB and CN as introduced in 5G via context fetch during handover and UE Capability ID/RACS.

- Cross-WG Coordination and Forward Compatibility

Early collaboration among RAN1, RAN2, RAN3, and SA2 is essential. RAN2 should define the capability granularity and inter-node management rules early, ensuring forward compatibility and avoiding redundant or overlapping definitions.

Overhead can be reduced by storing static capabilities in the CN, minimizing over-the-air reports, and ensuring UEs only report capabilities supported by network configurations.

- RAN2 should study the new UE capability framework with the goal of simplifying network operations, reducing signaling overhead, and ensuring compatibility across diverse device types.

- To define a fundamental Core Capability Baseline guaranteeing essential functions for all devices and move towards Service-Driven Capability Grouping.

Dynamic Capability Update Mechanisms

In 5G, UE Assistance Information (UAI) was used to signal capability restrictions in connected mode, but network behavior was not clearly specified, it’s up to network implementation. A more general and formal mechanism is needed for 6G.

Beyond existing scenarios (overheating, MUSIM, power saving, IDC), new form factors like foldable phones—where MIMO layers and antenna ports change upon folding/unfolding—further increase the need for dynamic updates.

A UE can autonomously initiate a capability update without a network request, and the network can notify explicit notification (e.g., through RRC reconfiguration).

- A UE-initiated capability mechanism can be allowed for dynamic capability updates (e.g., due to overheating or new form factors). The UAI can be studied as a baseline and as a enhancement requiring predictable network behavior.

Fundamental Reduction of Signaling Overhead

Methods to reduce UE capability reporting size from 6G Day 1 are required.

- Clarify Reporting Conditions

UE capabilities should be reported only when the corresponding network configurations exist. The UE must also be aware of network-supported features before reporting, in order to avoid unnecessary signaling.

- Device-Type-Based Grouping

Define a minimum mandatory capability group for each device type. Each UE then reports only additional optional capabilities beyond this baseline, reducing redundant information.

- Two-Stage Reporting

Separate the reporting of static capabilities (rarely changed features) from dynamic capabilities (features that vary depending on RRC configuration). This split reduces message size and update frequency.

- RACS Enhancement and Utilization:

Although the 5G RACS framework faced challenges in commercialization, it still holds strong potential for 6G. By using manufacturer-assigned UE identifiers, many OTA signaling exchanges can be avoided. Research should focus on relaxing RACS constraints. For example, removing the requirement that all capabilities be contained within a single template to enable more flexible and scalable use.

RACS framework retains strong potential for 6G. By leveraging manufacturer-assigned UE identifiers and relaxing template constraints, RACS can reduce the over-the-air signaling and enable scalable, flexible capability management across releases.

Flexible and Efficient Signaling Structure Design

To address 5G structural inefficiencies, a more flexible structure is also needed.

- RF/BPC and DL/UL Decoupling Improvements

Address low reuse rates of FeatureSetCombination and inefficient DL/UL decoupling by researching more flexible structures to minimize reporting duplication.

A more flexible structure should allow dynamic referencing and modular composition, where common RF and baseband parameters are defined once and selectively applied to DL or UL independently, with inheritance mechanisms to avoid duplication. A example is UL-only cell support in carrier aggregation, where a UE may support uplink transmission on a band without any downlink capability on that carrier.

- Reporting Structure Integration

Currently, certain capabilities such as general BC and UL Tx switching CA are reported through separate structures within the same capability IE. This makes UEs send duplicate overlapping parameters (e.g., supported bandwidths, MIMO configurations) across multiple structures. a flexible and unified reporting structure can avoid the redundant transmission of these parameters. While the design of this flexible structure requires joint work with RAN1, RAN2 can begin the process of reviewing the requirements for solving the problem and establishing the principles for an integrated signaling structure for Layer2 parameter.

- 6G should support flexible and unified signaling structures that improve RF/BPC and DL/UL decoupling and enable unified reporting structures to avoid duplicate parameter transmission

Interoperability and Troubleshooting Support

IODT and Interoperability: UEs activate features after passing IODT with specific network vendors, but interoperability issues arise with other vendors’ equipment. For 6G, research solutions enabling feature activation post-IODT with a vendor set while avoiding interoperability problems with others.

Problematic Terminal Identification: To prevent entire networks adopting conservative configurations or disabling new features due to a few problematic UEs, discuss standardized mechanisms for networks to identify UE/chipset vendor information. This can also overcome current limitations where IMEISV transmission is not always guaranteed.

- Success relies on early, continuous coordination across RAN WGs (RAN1/2/3/SA2) for forward compatibility, alongside mechanisms to manage IODT and identify problematic UE/chipset vendors.

3	Conclusions

In the previous section, the following observations are made:

Observation 1	5G NR was too flexible, resulting in unclear mandatory requirements, excessive signaling overhead, and increased testing complexity. This hindered interoperability and delayed network optimization and feature deployment.

Observation 2	To define a fundamental Core Capability Baseline guaranteeing essential functions for all devices and move towards Service-Driven Capability Grouping (profiles like eMBB/URLLC) to simplify configuration and avoid feature dispute.

Observation 3	Overhead can be reduced by storing static capabilities in the CN, minimizing over-the-air reports, and ensuring UEs only report capabilities supported by network configurations.

Observation 4	RACS framework retains strong potential for 6G. By leveraging manufacturer-assigned UE identifiers and relaxing template constraints, RACS can reduce the over-the-air signaling and enable scalable, flexible capability management across releases.

To conclude, we propose the followings:

Proposal 1	RAN2 should study the new UE capability framework with the goal of simplifying network operations, reducing signaling overhead, and ensuring compatibility across diverse device types.

Proposal 2	To define a fundamental Core Capability Baseline guaranteeing essential functions for all devices and move towards Service-Driven Capability Grouping.

Proposal 3	A UE-initiated capability mechanism can be allowed for dynamic capability updates (e.g., due to overheating or new form factors). The UAI can be studied as a baseline and as a enhancement requiring predictable network behavior.

Proposal 4	6G should support flexible and unified signaling structures that improve RF/BPC and DL/UL decoupling and enable unified reporting structures to avoid duplicate parameter transmission

Proposal 5	Success relies on early, continuous coordination across RAN WGs (RAN1/2/3/SA2) for forward compatibility, alongside mechanisms to manage IODT and identify problematic UE/chipset vendors.

References

RP-251395 Revised SID: Study on 6G Scenarios and requirements

RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)