3GPP TSG-RAN WG2 Meeting #132	R2-2508949

Dallas, USA, 17 – 21, Nov, 2025

Agenda Item:	8.1.3

Source:	Xiaomi

Title:	Discussion on NW data collection [C074][S052][H009/Z005]

Document for:	Discussion

Introduction

In this contribution, we discuss the open RILs of NW-side data collection.

Discussion

C074

The description of this issue is as following,

in case of MCG RLF, UE will initiate RRC re-establishment procedure, and the CSI logged measurement configuration will also be released due to “2> release spCellConfig, if configured;”, as it’s a part of spCellConfig. For the similar reason, it’s also unnecessary to add duplicate description of “release loggedDataCollectionAssistanceConfig,” and “discard the logged measurement entries”, since the same contents have been added in RRC re-establishment procedure.

We also support to delete these parts. Firstly, if UE is not configured with CHO/CLTM, UE would perform the release behaviour twice. Secondly, if UE is configured with CHO, UE may perform CHO recovery after RLF. In this case, the logging configuration shall not be released, since the logging configuration may be retained according to the stored conditional handover command. Therefore, UE should not release UAI and data logging configuration upon RLF

Proposal 1: [C074] Delete the UAI and data logging release description from RLF detection section, i.e. 5.3.10.3.

S052

The description of this issue is as following,

Eevent triggered CSI measurement logging can be performed based on L3 events. It is agreed to use CSI framework and not measurement framework and the event is captured as below in RRC CR. CSI-LoggedMeasurementEventTriggerConfig-r19 ::= SEQUENCE { threshold-r19 CHOICE { aboveThreshold-r19 MeasTriggerQuantity, belowThreshold-r19 MeasTriggerQuantity }, hysteresis Hysteresis, timeToTrigger TimeToTrigger, ... } Now it is not clear how does the UE get the quantity configuration for the L3 filtering for deriving L3 measurements for the event evaluation for logging CSI measurements.

Two options are observed,

Option 1: Quantity configuration is provided in CSI measurement framework.

Option 2:	UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO. This means UE should be provided with the measurement configuration when CSI-LoggedMeasurementEventTriggerConfig is received.

Both options can work. But option 1 can provide more flexibility. Therefore, option 1 is preferred.

Proposal 2: [S052] Quantity configuration of logging event is provided in CSI measurement framework.

H009/Z005

The description of [H009] this issue is as following. [Z005] discusses the same issue.

In the current description of text procedure for performing logging, it only describes UE behaviour regrading logging or not logging , please see below: 5.5x.3.2 Initiation The UE shall: 1> for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList: 2> if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer, AIML for network-side data collection is not full: 3> perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present; 2> if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full: 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger: 4> perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present; 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger: 4> stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList; 2> when performing the logging: 3> for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport: 4> set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers; 4> if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured): 5> set the timeGap to true; 2> when the memory reserved for the logged measurement information for data collection becomes full, stop logging; 2> when the memory reserved for the logged measurement information for data collection is no longer full, resume logging. Actually, this procedure need have the inter-layer interaction when UE performing the logging or stop logging: for example, to indicate the lower layers to stop the L1 measurement, or start the L1 measurement, otherwise, PHY layer need always perform the Layer 1 measurement no matter whether higher layer to log the data or not, this is really power-consuming.

We also agree the logging start/stop shall be indicated to physical layer. Because physical layer is not aware whether the radio logging event is fulfilled or not, since it’s based on L3 cell level RSRP. RRC layer shall explicitly indicate the logging start/stop to physical layer.

Proposal 3: [H009/Z005] RRC layer explicitly indicate the logging start/stop to physical layer.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: [C074] Delete the UAI and data logging release description from RLF detection section, i.e. 5.3.10.3.

Proposal 2: [S052] Quantity configuration of logging event is provided in CSI measurement framework..

Proposal 3: [H009/Z005] RRC layer explicitly indicate the logging start/stop to physical layer.