3GPP TSG RAN WG2 Meeting #132	R2-2508951

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	8.9.3

Source:	CMCC

Title:	Discussion on MEC remaining issues for CB-Msg3 EDT

Document for:	Discussion, Discussion

Introduction

RAN2 has completed the basic functions of R19 IoT NTN in RAN2#131-bis meeting.

In this contribution, we will focus on the possibly required corrections for MAC specification, especially for the sub-topic of CB-Msg3-EDT.

Discussion

In the working assumption and previous CR[2] for CB-RNTI of CB-Msg3-EDT, UEs that select the same Msg3 Window will monitor the same CB-RNTI.

However, there may be more than one UE select a same Msg3 Tx window, some UE may transmit Msg3 successfully, while some may fail. Both successful and unsuccessful UEs will monitor the same CB-RNTI and will see the same Backoff Indicator in Msg4.

After the CB-Msg3 transmission, the MAC entity shall: - start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT; - monitor the PDCCH while CB-Msg3ResponseTimer is running; - if notification of a reception of a PDCCH transmission is received from lower layers; and - if the CB-Msg4 is successfully decoded: - if the CB-Msg4 contains a CB Backoff Indicator subheader: - set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used. - else: - set the CB-Msg3-EDT backoff parameter value to 0 ms. - if the CB-Msg4 contains one or more MAC CMRs; and - if there is a UE Contention Resolution Identity in a MAC CMR that matches the first 48 bits of the CCCH SDU transmitted in CB-Msg3(s): - consider CMR reception successful; - discard the CB-RNTI; - stop CB-Msg3ResponseTimer if running; - flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; - if the corresponding CMR contains a Timing Advance Command field: - process the received Timing Advance Command (see clause 5.2). - start the timeAlignmentTimer associated with this TAG; - if the corresponding CMR contains a C-RNTI field: - set the C-RNTI to the value of the C-RNTI field. - finish the disassembly and demultiplexing of the MAC PDU; - consider CB-Msg3 transmission successfully completed; - if the C-RNTI field is absent in the corresponding CMR and no corresponding MAC SDU is present in CB-Msg4: - indicate the successful completion of CB-Msg3-EDT procedure to the upper layers. NOTE: The UE can receive CB-Msg4 after the CB-Msg3ResponseTimer expired if the corresponding PDCCH is received before the CB-Msg3ResponseTimer expired. - if the CB-Msg3ResponseTimer expires: - when MAC entity has finished decoding of all received CB-Msg4 and if CMR reception is not successful: - flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; - increment CB_MSG3_TRANSMISSION_COUNTER_CE by 1; - if CB_MSG3_TRANSMISSION_COUNTER_CE = cb-Msg3-MaxAttemptNum for the corresponding enhanced coverage level + 1: - discard the CB-RNTI; - consider the CB-Msg3 transmission unsuccessfully completed; - indicate the transmission using CB-Msg3-EDT has failed to the upper layers. - else: - based on the CB-Msg3-EDT backoff parameter, select a random backoff time according to a uniform distribution between 0 and the CB-Msg3-EDT backoff Parameter value; - delay the subsequent CB-Msg3 transmission by the backoff time; - proceed to the CB-Msg3 transmission (see clause 5.1x.2).

According to the current MAC spec as above, the UE will firstly store the backoff parameter value indicated by the BI field of the CB Backoff Indicator subheader upon successfully decoding the CB-Msg4, then after the CB-Msg3ResponseTimer expires and if the CB-Msg3-EDT attempt has not yet reached the maximum number, the UE would apply the backoff parameter value and trigger the backoff operation, e.g., waiting for a random time period before re-selecting another resource and re-transmitting CB-Msg3.

However, in such procedure, there maybe some issues can be noticed that when multiple UEs share the same CB-RNTI, even the UE that successfully completes its transmission is affected by this group-common identifier mechanism. Specifically, after the successful UE3 transmits its Msg3 on a dedicated PUSCH resource and is correctly decoded by the gNB, the gNB sends a single CB-Msg4 response addressed to the entire group of UEs identified by that CB-RNTI. While this Msg4 contains the individual identifier that allows UE3 to successfully complete contention resolution, it may also carry a CB Backoff Indicator (BI) intended to instruct other colliding UEs in the group to back off. Consequently, the successful UE3, while finalizing its access procedure, must additionally receive and decode this composite message containing information for other UEs and, following the protocol procedure, update its internal backoff parameter value, even though this parameter serves no practical purpose for its own successful access. This imposes an unnecessary signaling processing overhead on the successful UE3, which can be seen as an inherent cost of the group-RNTI-based response mechanism imposed upon every member of the group, including the successful one.

Considering the above issues, we need make sure that only the failed UEs will be trigger to backoff a certain time period and re-transmit CB-Msg3 and set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader. Above all, it's suggested to add behaviour to set the CB-Msg3-EDT backoff parameter value to 0 ms for success UE as following：

After the CB-Msg3 transmission, the MAC entity shall: - start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT; - monitor the PDCCH while CB-Msg3ResponseTimer is running; - if notification of a reception of a PDCCH transmission is received from lower layers; and - if the CB-Msg4 is successfully decoded: - if the CB-Msg4 contains a CB Backoff Indicator subheader: - set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used. - else: - set the CB-Msg3-EDT backoff parameter value to 0 ms. - if the CB-Msg4 contains one or more MAC CMRs; and - if there is a UE Contention Resolution Identity in a MAC CMR that matches the first 48 bits of the CCCH SDU transmitted in CB-Msg3(s): - consider CMR reception successful; - discard the CB-RNTI; - stop CB-Msg3ResponseTimer if running; - set the CB-Msg3-EDT backoff parameter value to 0 ms. - flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; - if the corresponding CMR contains a Timing Advance Command field: - process the received Timing Advance Command (see clause 5.2). - start the timeAlignmentTimer associated with this TAG; - if the corresponding CMR contains a C-RNTI field: - set the C-RNTI to the value of the C-RNTI field. - finish the disassembly and demultiplexing of the MAC PDU; - consider CB-Msg3 transmission successfully completed; - if the C-RNTI field is absent in the corresponding CMR and no corresponding MAC SDU is present in CB-Msg4: - indicate the successful completion of CB-Msg3-EDT procedure to the upper layers.

Proposal 1: For CB-Msg3 EDT procedure, we suggest to add behaviour to set the CB-Msg3-EDT backoff parameter value to 0 ms for success UE.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: For CB-Msg3 EDT procedure, we suggest to add behaviour to set the CB-Msg3-EDT backoff parameter value to 0 ms for success UE.

Reference

- R2-2506493 Introduction of IoT NTN enhancements phase 3, Mediatek