3GPP TSG-RAN WG2 Meeting # 132	R2-2508970

# Dallas, USA, 17th – 21st November 2025

Source:	Google Inc.

Title:	Discussion on Initial and system access

Agenda Item:	10.3.2.3

Document for:	Discussion and Decision

# 1. Introduction

In previous RAN2 meeting, RAN2 has discussed the baseline to design 6G system information. Based on the agreements last meeting, this contribution further discusses the details of system information handling and initial access.

- Discussion

  - System Information

Based on RAN2 agreement (Study RAN2 aspects of system information designs (e.g. on-demand SIB...)) in the last meeting, it is noted that the 5G model of a minimal broadcast SI with on-demand acquisition for other SIBs is an effective baseline. This structure aligns with 6G goals by preventing simple IoT devices from wasting energy decoding irrelevant SI, such as for Immersive Communication.

Furthermore, according to RAN2's previous discussions on SIB1 and the agreement to "Study aspects related to SIB1 size", we must consider the impacts of extending the on-demand principle to SIB1 itself. On-demand SIB1 provides better network energy savings, but it introduces a direct trade-off with initial access latency. This latency increase could impact the 6G KPI for 20 ms Control Plane Latency, as the UE must first perform a RACH procedure to request SIB1 before initiating a connection establishment.

To resolve the energy efficiency versus latency trade-off, the on-demand SIB1 mechanism should be configurable by the network.  The network should consider periodic broadcast SIB1 or on-demand SIB1 based on deployment. For example, the network should periodically broadcast SIB1 on cells providing coverage. For other cells providing capacity, the network may configure on-demand SIB1 when the load in these cells becomes quite low.

Besides, the network should consider periodic broadcast SIB1 or on-demand SIB1 based on different types of devices. For example, if the network configures a cell or UL/DL BWPs specific for IoT devices, the network may configure on-demand SIB1 transmission on the cell or DL BWP to maximize energy savings, considering infrequent access from IoT devices. Furthermore, the network should consider periodic broadcast SIB1 or on-demand SIB1 based on service. For example, if the network configures a cell providing HRLLC services, the network should configure periodical broadcast SIB1 to guarantee low latency.

Proposal 1: 6G shall support a minimum set of system information (i.e., SIB1) that is broadcast periodically to enable initial cell access and other system information that can be acquired by the UE on an on-demand basis.

Proposal 2: 	6G shall support on-demand acquisition of SIB1. The mechanism should be configurable by the network to balance the trade-off between energy efficiency and the initial access latency required by different services, device types, and deployment scenarios.

  - System information update

In line with the previous RAN2 agreement to "Study mechanisms to improve SI update mechanism", a specific baseline and study path are proposed. In the current 5G mechanism, the network notifies UEs of a pending SI change via a paging message. From our views, this is robust and energy-efficient and should be adopted in 6G. Meanwhile, the current 5G mechanism may not meet 6G's stringent KPIs. For services like HRLLC, waiting for the next modification period (which can be long) to apply new parameters may not be acceptable for the 1ms latency requirements. Therefore, a mechanism to update system information with a low latency is needed.

Proposal 3: 	Study mechanisms to update system information with a low latency.

  - Initial access

The initial access procedure is a critical component for meeting the stringent latency and efficiency goals of 6G. The established requirement for control plane latency is 20 ms for the transition from a battery-efficient state to an active state. While improvements in System Information (SI) acquisition are being discussed, the random access procedure is also a key contributor to the latency. Two types of random access procedure are supported in 5G: 4-step RACH procedure and 2-step RACH procedure.

While the 4-step RACH procedure is robust, its multi-step nature introduces latency and overhead that challenge the performance targets for many 6G use cases. Reducing the number of over-the-air messages from four to two provides universal benefits that are critical for several 6G KPIs:

- Control Plane Latency: The 4-step RACH procedure consumes a significant portion of the 20 ms C-plane latency budget. By making a 2-step RACH the baseline procedure, the over-the-air signaling time is substantially reduced.

- Hyper-Reliable Low-Latency Communication (HRLLC): HRLLC services require a user plane latency of 1 ms. While this is a U-plane KPI, the initial access or transition time is a critical part of the end-to-end latency for applications requiring near-instantaneous connectivity. A faster, more efficient 2-step access procedure is essential to support the demanding requirements of this use case.

- Device Energy Efficiency: A core goal for 6G is to improve device energy efficiency. A 2-step RACH reduces the time the UE's transmitter must be active, directly leading to power savings.

Proposal 4: The 2-step RACH procedure should be considered the baseline for initial access in addition to the 4-step RACH procedure.

For devices operating in deep sleep and waking only to transmit small data payloads, the standard 4-step RACH procedure introduces significant power overhead. While the 2-step RACH procedure offers improved power efficiency, achieving multi-year battery life requires minimizing the total "time on-air." This makes a "single-shot" access mechanism—where the entire access and data transmission are combined—paramount.

Proposal 5: 	Based on 6G requirements such as Massive Communication, a single-shot access mechanism should be supported (e.g., contention-based Msg3).

- Conclusion

In conclusion, we propose the followings for 6G:

Proposal 1: 6G shall support a minimum set of system information (i.e., SIB1) that is broadcast periodically to enable initial cell access and other system information that can be acquired by the UE on an on-demand basis.

Proposal 2: 	6G shall support on-demand acquisition of SIB1. The mechanism should be configurable by the network to balance the trade-off between energy efficiency and the initial access latency required by different services, device types, and deployment scenarios.

Proposal 3: 	Study mechanisms to update system information with a low latency.

Proposal 4: The 2-step RACH procedure should be considered the baseline for initial access in addition to the 4-step RACH procedure.

Proposal 5: 	Based on 6G requirements such as Massive Communication, a single-shot access mechanism should be supported (e.g., contention-based Msg3).

- References

[1] 3GPP TR 38.914 V0.2.0, "Study on 6G Scenarios and Requirements

[2] RAN2#131bis Chair note