3GPP TSG RAN WG2 Meeting #132	R2-2508980

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	8.9.3

Source:	ETRI, Korea University

Title:		Discussion on the paging enhancement in Store and Forward

Document for:	Discussion and Decision

1	Introduction

In this paper, we would like to discuss and present our views on paging enhancement in Store and Forward satellite operation.

2	Discussion

At the meeting in August, i.e., RAN2 #131, RAN2 discussed paging in S&F as follows [1]:

R2-2506288 Report of [AT131][305][R19 IoT NTN] S&F open issues Sateliot discussion IoT_NTN_Ph3-Core Proposal 1: Given that there is no immediate need for having to increase the size of a satellite ID and define any new satellite ID, RAN2 is respectfully requested not to pursue this. - We don’t increase the size of satellite ID and or define any new satellite ID as part of this Rel-19 WI Proposal 2: Given that it is not clear how P1 of [4] can optimize paging, RAN2 is respectfully requested not to pursue this. - HW thinks paging optimization is essential and easy to support. Nokia agrees. Ericsson see the benefit - QC thinks this is not easy and we cannot even discuss this alone without involvement of other groups. - Samsung is not sure how this would work - Toyota thinks this is useful - ZTE thinks in any case the UE would still have to monitoring system information. HW is not sure - Ericson thinks that SI update would not be trigger - Can come back in the next meeting

The proposal referred to as “P1 of [4]” in the above box is as follows [2]:

Proposal 1: An indication is introduced in the System information to indicate that the UE could skip monitoring paging and initiating RRC connection for MT data reception.

The above proposal was made considering that UEs do not need to monitor the paging or initiate RRC connection when feeder link is not available and the NW does not have any buffered MT data.

At the previous meeting in October, i.e., RAN2 #131bis, RAN2 discussed the paging in S&F as follows [3]:

- Paging enhancement in Store and Forward satellite operation R2-2507061 Paging enhancement in Store and Forward satellite operation Huawei, HiSilicon, Apple, Nokia, Ericsson, CENC discussion Rel-19 IoT_NTN_Ph3-Core Proposal: An indication is introduced in the System information to indicate that the UE could skip monitoring paging. - Apple supports having an indication for this - MTK thinks this is enhancement is not needed - Nordic also think we don’t need such enhancement at this stage - Toyota thinks there could be some benefit - ZTE this could be useful but to make it work eNB and MME should be aligned, so some RAN3 work might be needed - Ericsson think we could have an indication in SIB31 “no more DL data in the buffer” without mandating any UE behaviour. Huawei thinks it’s a possible compromise - QC thinks this impacts the system and other groups need to be involved - Come back in the next meeting to check whether we can have an indication (likely in SIB31) without mandating any UE behaviour - Send a LS to SA2 (Cc: RAN3) saying that RAN2 is discussing to have an indication in SIB to inform UEs than no paging is expected (so that the UE may skip paging) and asking them if any paging is expected from MME to RAN when the feeder link is not available (can further work on the LS text in the Post meeting email discussion)

Summarizing the discussions of the previous two meetings, RAN2 has not yet reached an agreement. According to the company proposing the indication, its purpose was to save UE power and the UEs that wants to save power need to follow the UE behaviour associated with the indication. That is, the indication is an optional UE feature for power saving by skipping monitoring paging, which is up to UE implementation.

As per the current spec. (TS 36.304), the existing discontinuous coverage is up to UE implementation:

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.

In our understanding, the indication has fine alignment with the existing discontinuous coverage based on the followings. Firstly, the purpose of the existing discontinuous coverage is to save UE power by relaxing idle mode tasks which include skipping monitoring paging. Secondly, monitoring paging is an optional UE feature and it is up to UE implementation.

Therefore, we think there is no need to introduce a new indication in SIB to optimize paging for S&F satellite operation.

- RAN2 not to pursue introduction of a new indication for skipping paging monitoring in S&F satellite operation.

Looking back at the meeting in May, i.e., RAN2 #130, the following agreement was made [4]:

Agreements: ... 2. In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list. ...

The above agreement is the result of discussions on whether to utilize S&F Monitoring List or S&F Wait Timer for relaxation of idle mode tasks. On the other hand, the proposed new indication for skipping paging monitoring considers the case where NW has an empty buffer for MT data and the feeder link is not available. However, both the agreement and the proposal consider a fundamentally the same issue in that they consider the case where NW has no MT data.

Consequently, we understand that RAN2 has already agreed to leverage the existing discontinuous coverage in order for UEs to relax idle mode tasks when it has no MT data.

RAN2 have already agreed to leverage the existing discontinuous coverage in order for UEs to relax idle mode tasks which include skipping paging monitoring when NW has no MT data.

As we mentioned earlier, the existing discontinuous coverage and the proposal new indication have a fine agreement in that they need to be implemented only for the UEs that wants power savings. The proposed new indication can be justified only when the benefit brought by it or the limitations imposed by leveraging the existing discontinuous coverage are clearly identified. Therefore, instead of introducing a new indication, the existing discontinuous coverage can be leveraged for S&F satellite operation.

- The existing discontinuous coverage in SIB32 is leveraged in order for UEs to skip monitoring paging considering S&F satellite operation.

If the existing mechanism of the discontinuous coverage in SIB32 can be leveraged for relaxation of idle mode tasks, it is necessary to introduce assistance information such as S&F mode for each satellite in SIB32.

Coverage of a satellite operating in S&F mode can be different for UEs whether they support S&F operation or not. For example, UEs supporting S&F operation can determine that it is in-coverage if it is within the footprint of the satellite operating in S&F mode.

However, the current discontinuous coverage does not consider S&F satellite operation. The current SIB32 can have only the discontinuous coverage information of satellites operating in normal mode. Thus, even if UEs supporting S&F operation are within the footprint of a satellite operating in S&F mode and thus they are in-coverage, they have no choice but to determine that it is out-of-coverage.

- S&F mode of each satellite is broadcast for discontinuous coverage in SIB32.

Considering that the availability of the feeder link changes over time, NW needs to broadcast S&F mode transition time together with S&F mode for discontinuous coverage.

- Together with S&F mode, S&F mode transition time of each satellite is broadcast for discontinuous coverage in SIB32

3	Conclusion

In this paper, the following observation is made:

Observation 1	RAN2 have already agreed to leverage the existing discontinuous coverage in order for UEs to relax idle mode tasks which include skipping paging monitoring when NW has no MT data.

In this paper, we propose the followings:

Proposal 1	RAN2 not to pursue introduction of a new indication for skipping paging monitoring in S&F satellite operation.

Proposal 2	The existing discontinuous coverage in SIB32 is leveraged in order for UEs to skip monitoring paging considering S&F satellite operation.

Proposal 3	S&F mode of each satellite is broadcast for discontinuous coverage in SIB32.

Proposal 4	Together with S&F mode, S&F mode transition time of each satellite is broadcast for discontinuous coverage in SIB32

References

R2-2506203, Report from session on NR NTN and IoT NTN, Session chair (ZTE), RAN2 #131, Aug. 2025.

R2-2505437, Further consideration on Store and Forward, Huawei, HiSilicon, China Telecom, RAN2 #131, Aug. 2025.

R2-2507703, Report from session on NES, NR NTN and IoT NTN, Session chair (ZTE), RAN2 #131bis, Oct. 2025.

R2-2504673, Report from session on NR NTN and IoT NTN, Session chair (ZTE), RAN2 #130, May. 2025.