3GPP TSG RAN WG2 #132                                                                             R2-2509024

Dallas, USA, Nov 17th – 21st, 2025

Source:	CEWiT

Title:	Discussion on Topology 2 for Ambient IoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

In Rel-19, Device 1 specification for Deployment Scenario 1 with Topology 1 was completed. For Rel-20, RAN2 will begin work on Topology 2, where the UE functions as an intermediate node. In RAN#109 [1], RAN updated the guidance for RAN2’s work on Topology 2.

Objectives for RAN 2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this contribution, we discuss our views on aspects of D2T2 for A-IoT.

- Discussion

## 2.1 Deployment scenario

According to the current WID, the FR1 licensed FDD spectrum is utilized for topology 2, where the R2D communication takes place in the UL spectrum, and both D2R and CW transmissions also occur in the UL band. This means that, within the A-IoT radio interface, the UE reader must both transmit and receive using the uplink spectrum when interacting with A-IoT devices.

However, there is some ambiguity regarding whether the spectrum resources assigned to the UE reader for D2R and R2D links are in-band with NR or situated in standalone (out-of-band) frequencies. Therefore, RAN2 is requested to clarify the applicable deployment scenarios. From our perspective, two potential configurations can be considered for topology 2: in-band and out-of-band deployment.

In the in-band deployment scenario, both the conventional NR (Uu) interface and the A-IoT interface at the UE reader operate within the same frequency band. As indicated in the WID, simultaneous or time-overlapping transmission of Uu and A-IoT signals by the intermediate UE in the same band is not supported. Consequently, non-overlapping time resources must be used for Uu and A-IoT operations to prevent interference and ensure proper functioning.

Conversely, in the out-of-band deployment scenario, the Uu and A-IoT interfaces at the UE reader operate in different frequency bands. In this case, both time-overlapping and non-overlapping resources can be utilized for Uu and A-IoT operations.

Observation 1: In A-IoT topology 2, both D2R and R2D operate in the FR1 uplink spectrum, requiring the UE reader to both transmit and receive within the UL spectrum for A-IoT communication.

Observation 2: Depending on deployment type:

- In in-band deployment, where Uu and A-IoT share the same band, simultaneous operation is not supported, and non-overlapping time resources must be used.

- In out-of-band deployment, where Uu and A-IoT operate in different bands, both overlapping and non-overlapping resources can be used.

Proposal 1: RAN2 is requested to clarify the following options regarding resource configuration for the UE reader in topology 2:

- Whether the resources allocated for D2R and R2D communication are

  - Option 1: Within the in-band NR spectrum, or

  - Option 2: In standalone (out-of-band) frequency bands, or

  - Option 3: A combination of both in-band and standalone configurations, depending on deployment conditions.

## 2.2 Resource Allocation

Topology 2 requires additional enhancements on the Uu interface to support A-IoT, particularly in terms of gNB-controlled radio resource allocation for UE readers. It has been agreed that resources are dedicated to UE reader, provided via dedicated RRC signaling from the serving gNB, and that such resources are not valid across cells in this release. Since the Rel-19 normative work on Topology 2 was not completed, it should be reaffirmed in Rel-20 that cross-cell resource validity is not supported. A key issue still open is the granularity of resource allocation. Several options have been identified, ranging from per UE reader/service request to per transmission, per procedure, per transmission pair, per paging round, or per access round. For frequent A-IoT services (e.g., inventory use cases with many messages and large numbers of devices), allocating resources per transmission would create excessive signaling overhead on the Uu interface, as the UE Reader would need to request resources for each message. In such cases, allocating resources per UE reader is more efficient, as it reduces signaling and provides flexibility to manage frequent transmissions. Conversely, for light traffic scenarios (e.g., a few devices or command/response exchanges), per transmission or per pair allocation may be more resource-efficient.

Proposal 2: RAN2 to adopt per UE reader resource allocation as the baseline, with gNB assigning a resource set to each UE reader.

Proposal 3: RAN2 to evaluate additional granularities (e.g. per R2D/D2R transmission or per pair) based on the different use cases (e.g. command operations).

In Release 19, two alternatives for A-IoT radio resource allocation were examined during the study phase. The first approach allows the base station (BS) to allocate A-IoT resources to the UE reader upon a request from the reader, while the second involves resource allocation triggered by a service request from the core network (CN). Both mechanisms were discussed in the previous RAN2 meeting [3], with the following agreement

‘UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may    be useful to be provided to gNB’.

and it was concluded that in either case, the BS retains control over the A-IoT resources used between the UE reader and A-IoT devices. The study also noted that the UE reader could provide assistance information to support efficient resource allocation.

Observation 3: Two alternatives for the radio resources were discussed in Rel-19:

- Alt 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

- Alt 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

Both alternatives were found complementary and feasible. The CN-based request (Alt-2) is more suitable for initial resource setup when a new A-IoT service is initiated, whereas the UE reader-based request (Alt-1) is better suited for dynamic reconfiguration, such as when additional resources are needed due to collisions, transmission failures, or procedure completion as BS may not be aware of collisions among devices during random access, while the UE reader being directly involved has better visibility. If collisions occur, the reader might need to retransmit paging messages, and the initially allocated resources could become insufficient and UE reader can request for more resources and also for releasing the existing resources. Similarly, the UE reader can report radio condition-based information and A-IoT traffic information. This exchange of assistance information provides flexibility and efficiency in managing resources across multiple UE readers and A-IoT procedures.

Therefore, it is proposed that RAN2 confirm support for both mechanisms in Release 20, where Alt-2 enables initial configuration and Alt-1 provides flexibility for adaptive resource management.

Proposal 4: RAN2 should support both alternatives for radio resource allocation, with the following applicability:

- Alt-1: Enables flexible on-demand reconfiguration, such as adjusting or releasing resources in response to dynamic conditions (e.g., collision events, D2R failures, or service completion).

- Alt-2: Provides an effective mechanism for initial resource configuration at the time of service establishment.

Proposal 5: RAN2 should agree that the UE Reader should provide the assistance information to the gNB to adapt the pool of resources at the reader UE.

Proposal 6: At least following types of assistance information should be supported

- A-IoT air interface measurements

- A-IoT traffic

- Resource request

- Release request.

The possible ways for A-IoT resource allocation between the gNB and the UE reader are periodic, aperiodic and  one shot resource allocation. In periodic A-IoT resource allocation, the gNB configures recurring resource sets for the UE reader, similar to a configured grant mechanism. These resources are defined by parameters such as periodicity, duration, and time offset, allowing the reader to perform transmissions at regular intervals without frequent reconfiguration. The second approach, continuous A-IoT resource allocation, provides a persistent set of resources that remain active until explicitly released by the gNB. The start and release of this allocation are controlled by the gNB, making it suitable for long-running or continuous A-IoT operations that require stable connectivity. The third approach, one-shot A-IoT resource allocation, involves allocating short-term resources for a single transmission opportunity or a limited duration.

Observation 4: The following possible ways to configure the A-IoT resources using dedicated Uu RRC signaling,

  - Periodic A-IoT resource allocation (similar to CG) with defined periodicity and duration:

  - Continuous A-IoT resource allocation with explicit start and release indications.

  - One-shot A-IoT resource allocation

In the RAN2#131bis meeting [3], the topic of resource validity for the A-IoT air interface was discussed. The discussion focused on how to define the duration of validity for the A-IoT resources allocated by the gNB specifically, whether this should be managed through a timer-based mechanism or by providing an explicit validity indication. This discussion aligns with the two potential solutions already examined in TR 38.769 [2] as part of the Rel-19 studies.

Reader UE performing AIOT operation in RRC_CONNECTED (down-selection or support of both options will be decided in the WI phase):

Option 1: UE receives resources in dedicated signalling.  The resources remain valid until the network releases them explicitly (understanding is this would be supported by RRC reconfiguration)

Option 2: UE receives resources in dedicated signalling along with a time period in which they can be used.  The resources remain valid for that time period.

From a practical perspective, Option 1 is simpler and sufficient in many cases, as the network already knows when the UE reader is expected to complete a service (e.g., following an inventory or command request). In this approach, the BS configures the resources (e.g., via RRC reconfiguration), and the UE reader uses the allocated resources until the service completes.. On the other hand, Option 2 introduces higher specification impact, as it requires defining timer configuration, start/stop conditions, and potentially additional assistance information, which may be challenging since service duration depends on device scheduling, A-IoT radio conditions, and reader implementation details. While more complex, Option 2 could still be useful in specific cases, e.g. temporary out of connection to optimize signaling overhead and resource utilization. In the case when UE readers temporarily lose connection, for instance, during a handover or a radio link failure, device inventory or command execution may still be on going. And in this condition, UE reader should not be forced to terminate these ongoing operations solely due to a temporary loss of connectivity. As long as the previously assigned A-IoT resources remain valid, the reader can continue its operations. Therefore, in such cases option 2 is suitable.

Proposal 7: Support both validity options, with Option 1 (explicit network release) as baseline and Option 2 (time-based validity) for further study in specific use cases such as temporary out of connection cases.

Proposal 8: For A-IoT resource allocation in temporary out-of-connection scenarios, it is proposed to support Option 2, where the allocated resources remain valid for a defined time period.

UE readers differ in their capabilities like some support only Device 1, others support Device 2b, and some support both. Similarly, cells have different deployments some host only Device 1, some only Device 2b, and others both; in some cases the network provides the carrier wave, while in others the UE must generate it. To enable efficient and interoperable Ambient IoT deployments, it is important to standardize mechanisms for UE Readers to report their capabilities (e.g., supported device types, waveforms, carrier wave generation, traffic types) and for the network to communicate cell-specific configurations. This ensures proper resource allocation, prevents operational mismatches, and supports multi-vendor interoperability. Accordingly, RAN2 should consider specifying capability exchange procedures between UE readers and the network to support flexible deployments.

Proposal 9: RAN2 should establish standardized mechanisms for UE Readers to report their capabilities and for the network to communicate cell-specific configurations.

For A-IoT service operations involving multiple A-IoT devices, a key consideration is how the UE reader handles the forwarding of D2R and R2D data. In such scenarios, the UE Reader may collect data or responses from several A-IoT devices and combine or aggregate these results before forwarding them to the network. Therefore, RAN2 discuss the potential support for data aggregation functionality at the UE reader, enabling efficient handling and transmission of information gathered from multiple A-IoT devices during a  service operation.

Proposal 10: RAN2 to discuss whether the UE reader supports data aggregation for different A-IoT devices.

In Rel-19, RAN2 acknowledged the multi-reader scenario in terms of Transaction ID collision but decided up to implementation as below.

RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

However, the decision made in Release 19 should be reconsidered for Topology 2 in Release 20, as handling multi-reader scenarios becomes more complex in this context. In Release 19, the reader function was implemented at the gNB, where fixed reader locations and controlled network planning minimized interference issues. In contrast, in Release 20, the reader is a UE, introducing mobility and resulting in dynamic and unpredictable interference patterns between UE Readers, making the implementation significantly more challenging.

Proposal 11: RAN2 should rediscussed the multi-reader scenario for topology 2 in Rel 20.

- Conclusion

In this contribution, we provide our views we discuss our views on aspects of D2T2 for A-IoT.

The proposals and observations are summarized as follows.

Observation 1: In A-IoT topology 2, both D2R and R2D operate in the FR1 uplink spectrum, requiring the UE reader to both transmit and receive within the UL spectrum for A-IoT communication.

Observation 2: Depending on deployment type:

- In in-band deployment, where Uu and A-IoT share the same band, simultaneous operation is not supported, and non-overlapping time resources must be used.

- In out-of-band deployment, where Uu and A-IoT operate in different bands, both overlapping and non-overlapping resources can be used.

Proposal 1: RAN2 is requested to clarify the following options regarding resource configuration for the UE reader in topology 2:

- Whether the resources allocated for D2R and R2D communication are

  - Option 1: Within the in-band NR spectrum, or

  - Option 2: In standalone (out-of-band) frequency bands, or

  - Option 3: A combination of both in-band and standalone configurations, depending on deployment conditions.

Proposal 2: RAN2 to adopt per UE reader resource allocation as the baseline, with gNB assigning a resource set to each UE reader.

Proposal 3: RAN2 to evaluate additional granularities (e.g. per R2D/D2R transmission or per pair) based on the different use cases (e.g. command operations).

Observation 3: Two alternatives for the radio resources were discussed in Rel-19:

- Alt 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

- Alt 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

Proposal 4: RAN2 should support both alternatives for radio resource allocation, with the following applicability:

- Alt-1: Enables flexible on-demand reconfiguration, such as adjusting or releasing resources in response to dynamic conditions (e.g., collision events, D2R failures, or service completion).

- Alt-2: Provides an effective mechanism for initial resource configuration at the time of service establishment.

Proposal 5: RAN2 should agree that the UE Reader should provide the assistance information to the gNB to adapt the pool of resources at the reader UE

Proposal 6: At least following types of assistance information should be supported

- A-IoT air interface measurements

- A-IoT traffic

- Resource request

- Release request.

Observation 4: The following possible ways to configure the A-IoT resources using dedicated Uu RRC signaling,

      - Periodic A-IoT resource allocation (similar to CG) with defined periodicity and duration:

      - Continuous A-IoT resource allocation with explicit start and release indications.

      - One-shot A-IoT resource allocation

Proposal 7: Support both validity options, with Option 1 (explicit network release) as baseline and Option 2 (time-based validity) for further study in specific use cases such as temporary out of connection cases.

Proposal 8: For A-IoT resource allocation in temporary out-of-connection scenarios, it is proposed to support Option 2, where the allocated resources remain valid for a defined time period.

Proposal 9: RAN2 should establish standardized mechanisms for UE Readers to report their capabilities and for the network to communicate cell-specific configurations.

Proposal 10: RAN2 to discuss whether the UE reader supports data aggregation for different A-IoT devices.

Proposal 11: RAN2 should rediscussed the multi-reader scenario for topology 2 in Rel 20.

# References

[1]   RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2.

[2]   TR 38.769 V2.0.1 (2024-12), Study on solutions for ambient IoT (Internet of Things).

[3]   Chairman notes, RAN2# 131-bis meeting