3GPP TSG-RAN WG2 Meeting #132	R2-2509039

Dallas, U.S.A., 17th – 21st November 2025

Agenda Item:	10.3.1.2

Source:	MediaTek Inc., Meta

Title:	On service awareness in the access stratum

Document for:	Discussion and decision

# Introduction

Based on the agreement below from the previous meeting [1], this document discusses potential benefits associated with the access stratum (AS) being service-aware, and corresponding design considerations.

Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework.

# Discussion

The services that are expected to evolve over 6G’s lifetime are several and varied as evidenced by inputs to the previous meeting on AI and token-based traffic, immersive services, intent-based operations and so on.  For example, OpenAI today provides a video conversation service, which has low latency and high data rate requirements. Google Gemini also provide Gemini AI Studio service, which provides similar functions with similar latency and data rate requirements. Requirements to serve immersive use cases are similar to such AI video conversation services.

However, we need to also acknowledge that these services are rapidly evolving, and their traffic characteristics can change over time. The risk we run with tailoring the 6G design to cater to specific services in their current form is that by the time we standardise such a design, these services evolve to a different set of requirements. To ensure a future-proof 6G design, we need to consider designing a service-agnostic framework that can cater to varied applications in the future as they evolve. For example, a common thread across these use-cases is that if the AS is aware of the service it carries, the end-user experience can be improved.

Proposal 1: The framework for service awareness in the AS is designed in a service-agnostic way to ensure that it can cater to varied services that will evolve over 6G’s lifetime.

Another common thread across these use-cases is the dynamic nature of the traffic being carried. Tokens can have different reliability requirements, XR frames can have different importance, different services can have different priorities that can vary over time based on user intent. It is evident that the static nature of a radio bearer is not well suited to deal with such differences. As we design the service-awareness framework, we need to take the dynamic nature of such traffic into consideration.

Proposal 2: The framework for service awareness in the AS enables dynamic differentiation of data handling at a service-level granularity.

Figure 1: Head of line blocking typically seen in the field with NR

An end user of mobile phone typically uses several services running in parallel. For example, it is common when playing a cooperative game to have a call ongoing with other players, while other applications run in the background. While the radio bearer concept has been part of 3GPP design for a very long time, the predominant scenario in the field is that all services for a user are transferred over the same radio bearer. This bearer is nearly always configured with in-order delivery, as several services (especially those that rely on TCP for transport) are sensitive to out of order data arrival. The consequence of transmitting data from multiple services over this common data pipe as widely seen in deployments today is head-of-line blocking, where the loss of a single packet affects all services as illustrated in Figure 1.

Observation 1: The use of a single radio bearer to transport all services for a UE as is typical in the field, leads to head-of-line blocking of all services at the receiver.

Figure 2: Proposed solution to limit head of line blocking with 6G

The use of a single bearer for several services is likely to be the norm in the 6G timeframe for several reasons such as net neutrality considerations or that these services share the same QoS enforcement. The reason for head-of-line blocking at the NR receiver is because all services in the bearer share the same sequence number (SN) space. Instead, if the AS is aware of the existence of different services and assign them to independent SN spaces, they can be handled separately at the receiver preventing one service from blocking another as illustrated in Figure 2. By doing so, we limit the impact of data loss only to the affected service(s) improving the latency experienced by other services, and we reduce the amount of data needed to be buffered at the receiver. To enable such operation, the AS needs to be service-aware, which needs further discussion over the course of the 6G study. .

Proposal 3: The framework for service awareness in the AS is designed to ensure that one service does not cause head-of-line blocking of other services carried in the same bearer.

Another consideration for service-awareness in the AS is the performance of UEs at the cell edge with respect to service continuity. In NR, several features (e.g., HPUE, repetitions, UE aggregation, coverage enhancements, etc.) were introduced to enhance cell edge UE performance. Some of these features are designed to improve robustness (e.g., repetition), while others increase cell edge throughput (e.g., HPUE, UE aggregation). As new services (e.g., XR and mobile AI) emerge, the demand for UL data rate requirement increases as well. With an AS that is service-aware, we need to additionally explore what improvements can be had to enhance the end-user experience of cell edge UEs.

Proposal 4: RAN2 to study enhancements to improve service robustness and continuity for cell-edge UEs.

# Conclusion

In this contribution we propose that service awareness in the AS is designed with the following considerations.

Proposal 1: The framework for service awareness in the AS is designed in a service-agnostic way to ensure that it can cater to varied services that will evolve over 6G’s lifetime.

Proposal 2: The framework for service awareness in the AS enables dynamic differentiation of data handling at a service-level granularity.

Observation 1: The use of a single radio bearer to transport all services for a UE as is typical in the field, leads to head-of-line blocking of all services at the receiver.

Proposal 3: The framework for service awareness in the AS is designed to ensure that one service does not cause head-of-line blocking of other services carried in the same bearer.

Proposal 4: RAN2 to study enhancements to improve service robustness and continuity for cell-edge UEs.

# References

- R2-2508002 - RAN2#131bis Meeting Report (ETSI MCC)