3GPP TSG RAN WG2 Meeting #132	R2-2509059

Dallas, US, Nov 17th – 21st, 2025

Agenda Item:	8.19.1

Source:	Xiaomi

Title:	Discussion on SFN-DFN in case of MH SL-Relay

Document for:	Discussion and Decision

# Introduction

Based on the discussion triggered at the RAN2#131bis meeting on whether and how to support SFN-DFN in the case of multi-hop sidelink relay, companies held different views on the valid use case and solutions [1]. This Tdoc provides some analysis on this.

# Whether and how to support SFN-DFN for MH SL-relay

During the last meeting, companies discussed whether to support SFN-DFN enhancement for the MH SL relay scenario, but the use case remains unclear. In our view, the controversial points can be split into two parts:

- Which scenario is valid for supporting positioning in the multi-hop SL relay case?

- In the valid scenario, are there any essential enhancements needed to support SFN-DFN offset forwarding for MH SL Relay?

## When is SFN-DFN actually needed?

To investigate the first part, here is some analysis on the possibilities for the in-coverage case and out-of-coverage case (including RAT-dependent positioning and GNSS positioning).

Firstly, let’s review the reason for introducing the "SFN-DFN offset" in the case of single-hop sidelink relay. As referred to in [2], SFN-DFN is introduced because "Downlink positioning methods require knowledge of the system time by the target UE. However, a L2 U2N remote UE may be unable to obtain the system time due to lacking a direct Uu link and potentially being out of Uu coverage." The applicable case directly excludes the "in-coverage" case, as such UEs can directly acquire the network’s SFN timeline from the gNB, making the proposed multi-hop SFN-DFN offset forwarding redundant.

Secondly, even for remote UEs out of coverage, the utility of SFN-DFN offset is limited to specific positioning scenarios. Multiple companies note that positioning in multi-hop relay scenarios is primarily for A-GNSS (network-assisted GNSS) positioning, in which case remote UEs out of coverage may need relay UEs to forward the SFN-DFN offset for GNSS timing reference. For RAT-dependent positioning, PRS is unlikely to be visible for remote UE when it’s out of coverage of gNB which provide both communication and positioning service. Thus, the mainstream use case for supporting SFN-DFN offset forwarding can be limited to the A-GNSS positioning scenario.

Proposal 1: RAN2 to focus on the discussion on SFN-DFN for MH SL-Relay only in the A-GNSS positioning case.

## Whether to support SFN-DFN enhancement for MH SL-relay?

Building on the A-GNSS positioning case, we can discuss whether and how to support SFN-DFN offset enhancement. Companies have put forward different views and comments on this:

View 1: It is necessary to compensate for timing differences (e.g., propagation delay, clock drift) between relay UEs to ensure the accuracy of SFN-DFN offset in MH SL relay scenarios.

View 2: It is unnecessary to introduce additional timing difference compensation even for MH SL relay scenarios. When UEs in SL relay chains are synchronized, the cited timing errors (propagation delay, clock drift) have no impact on the SFN-DFN offset.

View 3: Accurately estimating additional timing references is infeasible. UEs lack knowledge of the precise propagation delay and clock drift of other UEs, making it impossible to compensate for degradation in the accuracy of the SFN-DFN offset.

To summarize, Views 1 and 2 focus on whether additional timing difference compensation is needed, while View 3 focuses on the feasibility of UEs estimating or calculating such additional offsets. In our view, these two issues should be addressed by RAN4 rather than RAN2, as they are influenced by multiple factors such as UE synchronization accuracy, errors and UE capabilities.

Thus, it is proposed that the need for additional timing difference compensation and its feasibility should be confirmed by RAN4.

Proposal 2: If RAN2 confirms the valid use case for supporting SFN-DFN forwarding for MH SL relay, the need for additional timing difference compensation and its feasibility should be confirmed by RAN4.

Proposal 3: RAN2 sends an LS to RAN4 to confirm the need and its feasibility of additional timing difference compensation for MH SL Relay.

# Conclusion

In this paper, the following proposals are discussed.

Proposal 1: RAN2 to focus on the discussion on SFN-DFN for MH SL-Relay only in the A-GNSS positioning case.

Proposal 2: If RAN2 confirms the valid use case for supporting SFN-DFN forwarding for MH SL relay, the need for additional timing difference compensation and its feasibility should be confirmed by RAN4.

Proposal 3: RAN2 sends an LS to RAN4 to confirm the need and its feasibility of additional timing difference compensation for MH SL Relay.

# Reference

- R2_131b_Positioning-Relay-2025-10-16-1035

- R2-2313813	Downlink positioning support and posSIB request for L2 UE-to-network remote UE [PosL2RemoteUE]	MediaTek Inc., CATT, Huawei, HiSilicon, Qualcomm Incorporated, Xiaomi, Intel Corporation, vivo, Ericsson, Samsung, ZTE, Lenovo	CR	Rel-18	38.331	17.6.0	4066	6	C	TEI18	R2-2306839

# Annex-Draft LS to RAN4

# 3GPP TSG-RAN WG2 Meeting #132	R2-250xxxx

Dallas, US, Nov 17th – 21st, 2025

Title:	[Draft] LS on SFN-DFN forwarding for MH SL relay

Response to:

Release:	Rel-19

Work Item:	TEI-19

Source:	Xiaomi [to be RAN2]

To:	RAN4

Cc:

Contact Person:

#### Name:	Shuai Gao

E-mail Address:	gaoshuai3@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

Since SFN-DFN offset forwarding was introduced in Rel-18 for remote UE positioning in single-hop sidelink relay scenario, RAN2 has discussed whether to support SFN-DFN offset forwarding in multi-hop sidelink relay scenario. During RAN2 discussion, questions have arisen regarding whether "additional timing differences" between relay UEs should be introduced to compensate for degradation in the accuracy of the SFN-DFN offset as it travels along a chain of relays.

Thus, RAN2 has identified two issues requiring feedback from RAN4 to advance the discussion, as follows:

Q1: Whether it is necessary to support additional timing differences compensation between UEs?

Q2: Whether it is feasible for a UE to estimate or calculate the additional timing difference?

RAN4 is kindly requested to provide feedback on the above questions so that RAN2 can proceed with discussions on this issue.

2. Actions:

To RAN4:

ACTION: 	RAN2 respectfully asks RAN4 to take the above information from RAN2 into account.

3. Date of Next TSG-RAN2 Meetings:

3GPP TSG RAN WG2#133				 9 - 13 February 2026		Gothenburg, Sweden

3GPP TSG RAN WG2#133bis	      13 - 17 April 2026		Malta, MT