3GPP TSG-RAN WG2 Meeting #132	R2-2509060

Dallas, USA, 17 – 21 November 2025

Agenda item:	9.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM measurement prediction

WID/SID:	NR_AIML_Mob - Release 20

Document for:	Discussion and Decision

# 1	Introduction

In RAN#109 the WI for AIML-assisted Mobility in RRC_CONNECTED mode in NR was approved in RP-252899. RAN2 was asked to work on the following objectives:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

This contribution outlines our views for the RRM measurement prediction framework, considering the different cases identified during the SI and inter-frequency prediction. We also present preliminary considerations for prediction configuration, reporting, and AI-related enhancements.

# 2	Discussion

## 2.1	UE-Side Measurement Prediction (Framework)

Measurement prediction (temporal domain case A and B and frequency domain) can be understood as a predictive filter applied to the 5G measurement framework (TS 38.300).

Figure 2.1-1 RRM Measurement Prediction Sub-Case 2

Figure 2.1-1 shows sub-case 2 as an example. The cell-level L3 measurement C is given as input into the measurement prediction filter, which predicts measurements CPred in the PW or in the target frequency. Reporting criteria may be applied to the predictions to determine if they should be reported. The measurements can be configured and reported as legacy, including the measurements used as input for obtaining the predictions, and the reporting of either measurements and/or predictions should be possible to configure independently.

Observation 1: As shown in Figure 2.1-1, the RRM measurement prediction functionality can be embedded in the 5G measurement framework as an additional predicting filter.

Proposal 1: The reporting of either measurements and/or predictions should be configured independently.

### 2.1.1	Intra-Cell Temporal Domain RRM Measurement Prediction Case A

Let us consider Figure 2.1.1-1, which shows different ways to make predictions based on past information. The yellow squares are actual measurements in the OW. The green squares are predictions made in the PW. Grey squares are predictions made earlier that have now become obsolete. Each example demonstrates how adjusting the prediction settings (PW and PP) changes how many future points are predicted and how far apart they are.

Figure 2.1.1-1 Examples of PW and PP configurations resulting in different scenarios

As can be seen from Figure 2.1.1-1, temporal domain RRM measurement prediction case A can be configured with Prediction Window (PW) and Prediction Period (PP). Three examples of different PW and PP configurations are shown in Figure 2.1.1-1. Prediction period can be expressed, for example, in relation to the measurement or sampling period. The PW length may be defined as a time window or as number of predicted measurements with the specified PP.

Figure 2.1.1-2 Measurement reduction with temporal Domain Case B and Case A

Configuring the PP as a fraction of the measurement period can be used as an alternative method for measurement reduction, which can potentially enable measurement gap reduction or optimizing reference signal scheduling and offers a unified framework for both measurement reduction and mobility optimization goals.

Note that intra-cell temporal domain RRM measurement prediction can be used also to potentially reduce measurement gaps, if the predicted cell is not the serving cell and is in another frequency band than the serving cell. An example is shown in Figure 2.1.1-2, where the white squares represent measurement periods/steps, where e.g. measurement gaps are scheduled in case the predicted cell requires them for making measurements. Configuring a predicted measurement in between each measurement step may enable reducing the frequency of the measurements that need to be configured, in effect reducing measurements and potentially measurement gaps.

Observation 2: Intra-cell temporal domain RRM measurement prediction case A may be used to reduce the required measurements and measurement gaps if the prediction period is configured to be a fraction of the measurement/sampling period.

Observation 3: In temporal domain intra-cell measurement prediction, in addition to the PW, it is necessary to be able to configure also the Prediction Period (PP).

Proposal 2: In temporal domain measurement prediction, both prediction window and prediction period are configured from the NW.

### 2.1.2	Temporal Domain Intra-Cell RRM Measurement Prediction Case B

In temporal domain intra-cell RRM measurement prediction case B, the UE is assumed to be skipping measurements or samples it would be able to make, in which case the exact skipping pattern may not be configured by the network. This does not allow, e.g., measurement gap reduction in cases where measuring the predicted cell requires those.

Observation 4: In temporal domain intra-cell RRM measurement prediction case B, the UE is assumed to be skipping measurements it would be able to make.

The measurement reduction is configured via the MRRT, which configures the maximum allowed measurement reduction ratio in time. It should be discussed, if the network should always be aware if the reported value is a measurement or a prediction.

Proposal 3: MRRT configures the maximum allowed measurement reduction ratio in time.

Proposal 4: Discuss the NW awareness on whether the reported value is a measurement or a prediction.

### 2.1.3	Inter-Frequency RRM Measurement Prediction

Inter-frequency RRM measurement prediction is another method for measurement reduction and for reducing the required measurement gaps and consequently improving the spectral efficiency. Inter-frequency measurement prediction is spatial measurement prediction, where cell-level measurements in one frequency are predicted from cell-level measurements taken in another frequency.

The inter-frequency measurements can be configured using the framework presented in Figure 2.1-1. Event-based reporting of inter-frequency measurement predictions can be configured. Based on the report, the NW may, for example, decide to configure the UE with an updated measurement configuration to measure the target frequency. This allows configuring the inter-frequency measurements, including the required measurement gaps, when there is already indication that connecting to the target frequency would be possible.

Spatial measurement prediction, including inter-frequency measurement prediction, requires defining the set of cells to measure and the set of cells to predict. In the WID, this use case is limited to collocated inter-frequency cells only, i.e., to scenarios where the source and target cells of the prediction are in the same site and sector (same position and azimuth) but have different frequencies. In a collocated scenario, a target cell to be predicted can be defined by defining its collocated cell counterpart in the source frequency and the frequency (band) of the target cell. Especially, if cluster-based method is not used, but the measurements of one target cell are predicted from measurements of one collocated source cell, it can be enough to configure only the frequency of the target cell.

Observation 5: In a collocated scenario, a target cell to be predicted can be defined by defining its collocated cell counterpart in the source frequency and the frequency (band) of the target cell.

Proposal 5: Inter-frequency measurement prediction configuration should include at least the target frequency.

### 2.1.4	RRM Measurement Prediction Configuration

Figure 2.1.4-1 RRM measurement prediction configuration

To provide further details on the framework shown in Figure 2.1-1, let us consider Figure 2.1.4-1. The MeasConfig IE can be extended to include the required configuration for the measurement prediction scenarios and the MeasId IE can be extended with an optional reference to those configurations, which defines the relationship between the measurements used as input, the configuration of the measurement prediction and the reporting configuration for the predictions and/or measurements. If no reference to measurement prediction configuration is given, normal measurements and measurement reporting are configured.

The required configurations depend on which RRM measurement prediction scenario is configured, temporal domain case A or B or frequency domain. The parameters should configure, at least:

- Temporal domain intra-cell case A

  - PW length

  - Prediction period

  - Cells to be predicted (if not all in the measurement object used as input)

- Temporal domain intra-cell case B

  - Maximum allowed MRRT

  - Cells to be predicted (if not all in the measurement object used as input)

- Inter-frequency prediction

  - The frequency to be predicted

Observation 6: The required measurement prediction configurations depend on which RRM measurement prediction scenario is configured, temporal domain case A, B or frequency domain.

Proposal 6: RAN2 to take the measurement prediction configuration framework in Figure 2.1.4-1 as a starting point for measurement prediction configuration. Details related to configuration of measurement predictions and prediction reporting are FFS.

When a measurement prediction is reported by the UE, it is necessary for the NW to know what is reported, e.g., case A, PW, PP etc, to be able to make decisions based on and utilize the reported predictions. For example, predicted measurements can be used to decide on early preparation of a handover, or inter-frequency prediction to update the measurement configuration. As in legacy measurements, the report should be linked to the configuration on which it is based by including the corresponding measurement or inference identifier in the report. In temporal domain measurement prediction, one prediction report may contain more than one time-step of measurements.  FFS if legacy measurement report can be reused and extended to report predictions.

Observation 7: When a measurement prediction is reported by the UE, it is necessary for the NW to know, what is reported, e.g., case A, PW, PP etc.

Proposal 7: In principle, aim to reuse legacy measurement reporting mechanisms as much as possible. Study extensions needed to support reporting the predictions.

Proposal 8: Discuss the linkage between the reported measurement prediction to the configuration based on which they were reported.

## 2.2	AI-related enhancements

### 2.2.1	Applicability Framework

In RAN2#131 it was agreed that:

Agreements 1. Confirm that as a baseline following agreements on applicability reporting in AI-based beam management are applicable for AI mobility: - UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration. - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. - When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). - The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported predicted values after UE indicates that an inference configuration is not applicable. - The UE shall report when an inference configuration becomes non-applicable. - How to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration. - Whether Option A and Option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. - RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). NOTE: these agreements will be aligned with AI PHY agreements at the end of this week and will not be re-discussed

Considering the above agreement and the SI discussions, at least at Stage 2 level, we should have all the components we need to define the feature. Additional enhancements of existing mobility framework can be considered once the baseline procedures are stable.

Observation 8: Progress made during the Rel. 19 SI discussions was sufficient to define the Stage 2 details for applicability exchange.

### 2.2.1	Associated ID

Generalization level of the AI/ML measurement prediction functionality determines, what affects the applicability to different network configurations, areas etc. For example, cell configuration changes such as transmission power or down-tilt changes may affect area-specific models, but the applicability of more generic models may depend on, for example, the type of the site (macro, micro). In the study item, the RRM measurement prediction generalization to UE speeds and different site types and ISDs was evaluated, and it was observed that the models generalized well.

Observation 9: The relevant features to be considered in the applicability framework depend on the generalization level and design of the ML functionality.

If any additional configuration is required to be provided by the network to the UE to assist the UE-side measurement prediction applicability determination, the relevant aspects affecting the applicability depend on the generalization level of the RRM measurement prediction, which can differ between UEs. Therefore, if any additional configuration is required to assess the applicability, the parameters should have clearly defined meaning, so that they can be correctly interpreted by the NW and the UE. This challenge is further complicated in inter-cell mobility compared to, e.g., beam prediction, due to increased complexity of potentially more than one cell being involved.

Observation 10: In inter-cell mobility, assessment of applicability is further complicated due to increased complexity of potentially more than one cell being involved.

Proposal 9: If any additional configuration is required to assess the applicability (e.g., associated ID), the parameters should have clearly defined meaning.

# 3	Conclusion

This document has made the following observations:

Observation 1: As shown in Figure 2.1-1, the RRM measurement prediction functionality can be embedded in the 5G measurement framework as an additional predicting filter.

Observation 2: Intra-cell temporal domain RRM measurement prediction case A may be used to reduce the required measurements and measurement gaps if the prediction period is configured to be a fraction of the measurement/sampling period.

Observation 3: In temporal domain intra-cell measurement prediction, in addition to the PW, it is necessary to be able to configure also the Prediction Period (PP).

Observation 4: In temporal domain intra-cell RRM measurement prediction case B, the UE is assumed to be skipping measurements it would be able to make.

Observation 5: In a collocated scenario, a target cell to be predicted can be defined by defining its collocated cell counterpart in the source frequency and the frequency (band) of the target cell.

Observation 6: The required measurement prediction configurations depend on which RRM measurement prediction scenario is configured, temporal domain case A, B or frequency domain.

Observation 7: When a measurement prediction is reported by the UE, it is necessary for the NW to know, what is reported, e.g., case A, PW, PP etc.

Observation 8: Progress made during the Rel. 19 SI discussions was sufficient to define the Stage 2 details for applicability exchange.

Observation 9: The relevant features to be considered in the applicability framework depend on the generalization level and design of the ML functionality.

Observation 10: In inter-cell mobility, assessment of applicability is further complicated due to increased complexity of potentially more than one cell being involved.

And proposed the following:

Proposal 1: The reporting of either measurements and/or predictions should be configured independently.

Proposal 2: In temporal domain measurement prediction, both prediction window and prediction period are configured from the NW.

Proposal 3: MRRT configures the maximum allowed measurement reduction ratio in time.

Proposal 4: Discuss the NW awareness on whether the reported value is a measurement or a prediction.

Proposal 5: Inter-frequency measurement prediction configuration should include at least the target frequency.

Proposal 6: RAN2 to take the measurement prediction configuration framework in Figure 2.1.4-1 as a starting point for measurement prediction configuration. Details related to configuration of measurement predictions and prediction reporting are FFS.

Proposal 7: In principle, aim to reuse legacy measurement reporting mechanisms as much as possible. Study extensions needed to support reporting the predictions.

Proposal 8: Discuss the linkage between the reported measurement prediction to the configuration based on which they were reported.

Proposal 9: If any additional configuration is required to assess the applicability (e.g., associated ID), the parameters should have clearly defined meaning.