3GPP TSG-RAN WG2 Meeting #132	R2-2509082

Dallas, USA, Nov. 17th - 21th, 2025

Agenda Item:	8.12.2

Source:	OPPO

Title:	Remaining MAC issues of MIMO

Document for:	Discussion, Decision

# Introduction

In RAN2#131bis meeting, RAN2 discussed the UE behaviours for the different cases where for the UE-initiated beam report of mode-B, PUSCH and PUCCH are configured in different TAGs:

- a) The TAT for mode-B PUSCH is expired while the TAT for PUCCH is running:

- b) If the TAT for mode-B PUCCH is expired while the TAT for PUSCH is running

According to the RAN2#131bis meeting discussion, RAN2 made the following agreements:

RAN2#131bis meeting agreement [1]: - For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, and if the Type1 CG is only associated to this PUCCH for mode-B UEI reporting, the UE clears the Type1 CG PUSCH.

According to the email discussion [2] for the MAC remaining issue of MIMO, case 1) was discussed, and company views are quite diverse. In this contribution, we provide our understandings on the MAC remaining issue of the Rel-20 MIMO.

# Discussion

## Issue-1: UE behaviour for the case: the TAT for mode-B PUSCH is expired while the TAT for PUCCH is running

According to the email discussion [2] for the MAC remaining issue of MIMO, for the case that the TAT for mode-B PUSCH is expired while the TAT for PUCCH is running, companies discussed the following options:

- Option 1 (baseline from RAN2#131bis): UE releases the PUCCH if the configured PUCCH resource id is only associated to this type-1 CG PUSCH (i.e., not associated with any other UCI).

- Option 2 (to avoid any impact to other UCI transmission): UE does not release the PUCCH, UE does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.

- Option 3: The UE does not release PUCCH and transmits UEIRI.

For Option 1, companies raised the concern that as the PUCCH for mode-B UEI reporting can be shared by other UCIs (e.g. HARQ-ACK), the release of the PUCCH for mode-B UEI reporting could cause extra complexity at the network for HARQ-ACK feedback decoding.

For Option 2, the concern is that extra UE complexity is needed to suspend the UCI reporting triggered by mode-B UEI reporting.

For Option 3, if the UE keeps sending UEIRI, the PUCCH resource for the UEIRI is wasted, as the CG PUSCH of mode-B UEI is anyway unable to be used.

From our understanding, it seems that there is no perfect solution to handle case when PUCCH and CG PUSCH for mode-B UEI are configured in different TAGs, and one TAG has valid TA while another TAG has no valid TA. As the Rel-20 MIMO item is closed, and companies’ views on the candidate solutions are quite diverse, we think that RAN2 can restrict the configuration by always configuring PUCCH and CG PUSCH for mode-B UEI in the same TAG, so that no extra handling in MAC is needed.

Proposal: By reverting the previous RAN2 agreement, PUCCH and PUSCH for mode-B UEI reporting are always configured in the same TAG.

# Conclusion

According to the above analysis, we have the following proposal.

Proposal: By reverting the previous RAN2 agreement, PUCCH and PUSCH for mode-B UEI reporting are always configured in the same TAG.

# Reference

[1] RAN2#131bis meeting minutes

[2] [Post131bis][215][MIMOevo] CR for TS 38.321 (Samsung)