3GPP TSG-RAN WG2 Meeting #132	R2-2509085

Dallas, USA, Nov. 17th -21st 2025

Agenda Item:	8.9.3

Source:	Ericsson

Title:	Last remaining open issues for Store and Forward and PWS

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we address the last remaining open issues for Store and Forward and PWS marked as come back last meeting: impact of S&F mode in AS procedures (cell re-selection and barring) and support for acceptable cell in TN for NB-IoT PWS

# 2	Discussion

## 2.1	Specification impact of S&F cell de-prioritization

In RAN2#131, the following was agreed:

A Rel-19 UE may deprioritize the neighbor cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review).

Regarding the procedural perspective, during the post email discussion, the agreement was captured as informative procedural text in a note within the section for Cells with cell reservations, access restrictions or unsuitable for normal camping.

NOTE: If the highest ranked cell or best cell according to absolute priority reselection rules is cell operating in store and forward mode the UE may deprioritse or not consider this cell as candidate cell for cell reselection and continue considering other cells not operating in store and forward mode for cell reselection.

According to the 3GPP Drafting Rules (TS 21.801), "Notes are used to provide informative guidance only; they shall not contain requirements or instructions." Placing the de-prioritization behaviour in a NOTE would render it non-binding, allowing different UE implementations to handle S&F cells inconsistently, which could lead to undesirable mobility decisions and negatively impact the user experience in NTN environments.

Furthermore, cell prioritization and de-prioritization rules for idle/inactive mode are consistently defined in normative text. For example, TS 38.304 specifies mandatory rules for cell reselection based on absolute priorities, and TS 38.331 defines how a UE shall handle cells with specific restrictions. Placing the S&F cell handling rule in a NOTE would be a deviation from established specification practice for similar functionalities.

To ensure consistent UE behaviour and prevent reselection to cells where immediate service is unavailable, it is more appropriate to allow the UE to treat such cells as unsuitable and exclude them from the set of candidate cells for cell reselection.

Capture in normative text that a Rel-19 UE can exclude S&F cells as candidate cell for cell reselection.

## 2.2	Cell access upon operation mode transition

According to TS 36.304, when a UE identifies a cell as barred, it must exclude that cell from the cell reselection candidate list for 300 seconds. Depending on the intraFreqReselection parameter configuration, the UE may extend this exclusion to the entire frequency. This mechanism, while robust, presents challenges in certain geographical areas where satellite operation mode frequently transition.

A UE excludes a barred cell (or frequency) from the candidate list for cell re-selection for 300 seconds.

As an illustrative example, consider a scenario where a UE lacking support for S&F finds a cell that is currently operating in S&F mode but is scheduled to transition to non-S&F operation soon. In accordance with the existing procedure, the UE is mandated to exclude this cell (or, depending on parameter configuration, the entire frequency) from its cell reselection candidate list for a duration of 300 seconds. Given that this exclusion period is comparable to, or may even exceed, the typical service time of LEO satellites, the UE may be prevented from re-selecting to the cell even after it becomes available (i.e., the barring bit is deactivated). Consequently, this can lead to missed opportunities for connectivity and suboptimal service continuity in dynamic NTN environments.

During the service time of a LEO satellite, a UE may not re-select to a cell previously determined as barred, even if the satellite changes operation mode (e.g., non-S&F) and the barring bit is deactivated.

To address the identified limitation regarding prolonged cell exclusion in dynamic NTN environments, it is proposed that when a neighbouring cell is barred due to S&F operation prior to its t-ModeSwitching, the UE excludes this cell from the cell reselection candidate list only until the specified operation mode transition time. Upon expiry of the operation mode transition time, the UE can reassess the barring status based on the most recent system information. This approach aims to reduce unnecessary exclusion durations, thereby enhancing the likelihood of timely connectivity and improving both network efficiency and user experience.

The UE excludes a barred neighbouring cell due to S&F operation mode only until the operation mode transition time.

## 2.3	Camping on acceptable cells for NB-IoT PWS reception

The last topic that was raised in the previous meeting concerns an earlier discussion about whether the concept of an acceptable cell for NB-IoT should be introduced in terrestrial networks. Some companies have suggested that a new network indication is needed so that a PWS-capable NB-IoT UE can select a suitable cell—specifically, one that truly supports public warning system (PWS) functions. This may become a real issue if RAN2 or other working groups choose to support acceptable cell for TN, given that a UE may both support NB-IoT TN and NTN and not all NB-IoT cells in TN may support PWS.

From the perspective of RAN2 standards, there is currently no restriction that would prevent PWS support in TN. The eNB is capable of broadcasting PWS-related system information. However, introducing the concept of acceptable cell in TN could have a significant impact on existing deployments and other working groups. Since there is no clear business requirement or market demand for PWS services in TN, and the main use case for this feature comes from NTN, we prefer to limit changes to the specifications by not adding the concept of acceptable cell in TN.

Acceptable cells for the reception of PWS in NB-IoT UE apply only to NTN.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A UE excludes a barred cell (or frequency) from the candidate list for cell re-selection for 300 seconds.

Observation 2	During the service time of a LEO satellite, a UE may not re-select to a cell previously determined as barred, even if the satellite changes operation mode (e.g., non-S&F) and the barring bit is deactivated.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture in normative text that a Rel-19 UE can exclude S&F cells as candidate cell for cell reselection.

Proposal 2	The UE excludes a barred neighbouring cell due to S&F operation mode only until the operation mode transition time.

Proposal 3	Acceptable cells for the reception of PWS in NB-IoT UE apply only to NTN.