3GPP TSG-RAN WG2 Meeting #132	R2-2509109

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.2.1

Source:	OPPO

Title:	Discussion on 6G RRC Connection Management

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to present our consideration regarding RRC modelling.

Discussion

During R2#131bis, R2 reached the conclusion as follows, i.e., the FFS issue is mainly on RRC_INACTIVE state

Agreements Continue study - Idle mode – - Connected mode – - Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g., cell selection/reselection), UE context and identification and energy efficient for both UE and NW. Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

So, R2 has already settle down the work plan, i.e., to discuss the functionality first, and then compare the different modelling, i.e., to model the mode as a separate RRC_INACTIVE state, or a sub-state of RRC_CONNECTED.

- For 6G INACTIVE/Sub-State mode, R2 concluded to discuss the targeted functionality for the mode before decision on modelling (i.e., whether treat it as a separate RRC_INACTIVE state or a sub-state of RRC_CONNECTED).

Before looking into the intended functionality, it is good to re-screen the legacy 5G RRC handling to understand the pros and cons.

Table 1 5G RRC State Handling

| Index | When there is no data ongoing | Upon data arrival | Consumption when no data ongoing | Latency upon data arrival | Mobility support when there is data on-going | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 5G RRC_IDLE | 5G RRC_ CONNECTED | Low | Long | Y (without Xn reliance) | Typical way |
| 2 | 5G RRC_ INACTIVE | 5G RRC_ CONNECTED | Low | Medium (Due to UE context retrieve) | Y (with Xn reliance) | Limited Deployment |
| 3 | 5G RRC_ CONNECTED | 5G RRC_ CONNECTED | Medium if tuning DRX properly (Due to measurement and signalling for mobility) | Low | Y (without Xn reliance for L3 HO, with Xn reliance for LTM in 5G) |  |
| 4 | 5G RRC_ INACTIVE | 5G RRC_ INACTIVE | Low | Medium (Due to UE context retrieve) | N | Limited to small data packet |

Based on this table, there could be several observations.

Firstly, for approach-4, performance-wise it is quite similar to approach-2, in the sense that UE context retrieval is still needed, although RRCResume message is saved. While the key problem here is the incapability to support mobility during data traffic period.

Then, for Approach-3, the drawback is the power / resource consumption level when there is no data.

If there is no mobility, the long DRX cycle and RRM relaxation can help to reduce the UE power / resource consumption to a minimum level. But when there is mobility, the measurement, report and the associated signalling is unavoidable, for 5G RRC_CONNECTED state.

Take LTM procedure as an example,

- Step-1/2/3/4b/5/6/7 includes signalling exchange as consumer for resource, and

- Measurement for step-1/5, sync for step-1/4a, are all consumers for UE power,

- Reception for step-4b/7 are all consumers for network power

Figure 1 5G LTM Procedure

- 5G RRC_CONNECTED state was designed for fast UP delivery, but without sufficient optimization of the power / resource consumption during mobility when there is no data activity.

On the contrary, approach-1/2 solve the concern of power / resource consumption above, but end up with medium/high level of latency upon data arrival.

Specifically, in approach-1, the steps from 1 to 8a are all necessary, covering jobs for NAS to prepare UE AS context, NAS and AS security activation, and SRB/DRB configuration, which is quite long latency.

Figure 2 5G RRC Setup Procedure

While for approach-2, although the NAS/AS UE context preparation steps can be saved, but when the serving RAN node is not the anchor RAN node, the reactive UE context retrieval procedure, which is to solve the consumption due to proactive network-controlled mobility, on the other hand, introduce additional latency (compared with approach-3), which happens between step-1 and step-4. I.e., the resulted latency is still larger (although lower than RRC setup procedure) for service continuity.

Figure 3 5G RRC Resume Procedure

As defined by 38.300

- 9.2.2.1 Overview RRC_INACTIVE is a state where a UE remains in CM-CONNECTED and can move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. In RRC_INACTIVE, the last serving gNB node keeps the UE context and the UE-associated NG connection with the serving AMF and UPF.

5G RRC_INACTIVE was designed to achieve balance between

- 5G RRC_CONNECTED, where the proactive UE context relocation caused higher consumption level when there is no data on-going, and this is changed to reactive UE context retrieval to save power and resource consumption;

- 5G RRC_IDLE, where the state transition effort is high, since the UE context is not maintained at RAN side, and the whole procedure starting from NAS preparation leads to high consumption and latency for state transition;

- 5G RRC_INACTIVE was designed to reach a trade-off between UP resumption latency and UE power consumption during no-data period.

So, if one prefers RRC_INACTIVE than RRC_IDLE, the advantage mainly comes from the CP latency.

However, if we compare the CP latency between transition from RRC_IDLE and RRC_INACTIVE

Table 2 Transition latency for RRC_IDLE and RRC_INACTIVE

| Description | NR (2 symbol TTI, 1 processing delay) |  |
| --- | --- | --- |
|  | RRC_IDLE [ms] | RRC_INACTIVE [ms] |
| Average delay due to RACH scheduling period | 0.428571 | 0.785714 |
| RACH Preamble | 0.214286 | 0.214286 |
| Preamble detection and transmission of RAR (Time between the end RACH transmission and UE's reception of scheduling grant and timing adjustment) | 0.714286 | 0.714286 |
| UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Request) | 2.14 | 2.14 |
| Transmission of RRC Setup Request / RRC Resume request | 1 | 1 |
| Processing delay in gNB (L2 and RRC) | 4 | 4 |
| Transmission of RRC Setup (and UL grant)/ RRC Resume | 1 | 1 |
| Processing delay in the UE (L2 and RRC) | 10 | 6 |
| Transmission of RRC Setup complete (including NAS Service Request)/ RRC Resume Complete | 1 | 1 |
| Processing delay in gNB (Uu –> S1-C / Xn) | 4 | 4 |
| S1-CP Transfer delay / Xn Transfer delay | 3 | 3 |
| AMF Processing Delay (including UE context retrieval of 10ms) | 15 | 10 |
| S1-CP Transfer delay / Xn Transfer delay | 3 | 3 |
| Processing delay in gNB (S1-C / Xn –> Uu) | 4 | 4 |
| Transmission of RRC Security Mode Command | 1 |  |
| Processing delay in UE (L2 and RRC) | 5 |  |
| Transmission of RRC Security Mode Complete | 1 |  |
| Processing delay in gNB (L2 and RRC) | 4 |  |
| Transmission of RRC Reconfiguration | 1 |  |
| Processing delay in UE (L2 and RRC) | 16 |  |
| Transmission of RRC Reconfiguration Complete | 1 |  |
| Processing delay in gNB (L2 and RRC) | 4 |  |
| UE Capabilities and Initial Context Setup request | [80] |  |
| UP Latency [2] | 4 | 4 |
| Sum | 86.50 [166.50] | 44.85 |

It can be observed that the reduction is from 86.5/166.5ms to 44.85ms (note that 166.5ms can be further reduced to 86.5ms if considering capability delivery is only necessary during initial registration step).

Then if we checked the 5QI definition table, to identify the traffic that requires latency lower than 86.5ms but higher than 44.85ms, we can identify the following ones (where the green ones are based on assumption of 166.5ms case).

Table 3 PDB for QoS defined in 5G

| 5QI Value | Packet Delay Budget (NOTE 3) | Example Services |
| --- | --- | --- |
| 1 | 100 ms (NOTE 11, NOTE 13) | Conversational Voice |
| 2 | 150 ms (NOTE 11, NOTE 13) | Conversational Video (Live Streaming) |
| 3 | 50 ms (NOTE 11, NOTE 13) | Real Time Gaming, V2X messages (see TS 23.287 [121]). Electricity distribution – medium voltage, Process automation monitoring |
| 4 | 300 ms (NOTE 11, NOTE 13) | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 9, NOTE 12) | 75 ms (NOTE 7, NOTE 8) | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 (NOTE 12) | 100 ms (NOTE 10, NOTE 13) | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 12) | 100 ms (NOTE 10, NOTE 13) | Mission Critical Video user plane |
| 75 (NOTE 14) | 50 ms (NOTE 13) | V2X messages (see TS 23.287 [121]). A2X messages (see TS 23.256 [136]) |
| 71 | 150 ms (NOTE 11, NOTE 13, NOTE 15) | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 72 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 73 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 74 | 500 ms (NOTE 11, NOTE 15) | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 76 | 500 ms (NOTE 11, NOTE 13, NOTE 15) | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 5 | 100 ms (NOTE 10, NOTE 13) | IMS Signalling |
| 6 | 300 ms (NOTE 10, NOTE 13) | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.), AI/ML model download for image recognition (e.g. for model topology) (see TS 22.261 [2]) |
| 7 | 100 ms (NOTE 10, NOTE 13) | Voice, Video (Live Streaming) Interactive Gaming, AI/ML model download for image recognition (e.g. for model weight factors) (see TS 22.261 [2]) |
| 8 | 300 ms (NOTE 10, NOTE 13) | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  | video, etc.) |
| 10 | 1100ms (NOTE 10,NOTE 13, NOTE 17) | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| 69 (NOTE 9, NOTE 12) | 60 ms (NOTE 7, NOTE 8) | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 70 (NOTE 12) | 200 ms (NOTE 7, NOTE 10) | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 79 | 50 ms (NOTE 10, NOTE 13) | V2X messages (see TS 23.287 [121]) |
| 80 | 10 ms (NOTE 5, NOTE 10) | Low Latency eMBB applications Augmented Reality |
| 82 | 10 ms (NOTE 4) | Discrete Automation (see TS 22.261 [2]) |
| 83 | 10 ms (NOTE 4) | Discrete Automation (see TS 22.261 [2]); V2X messages (UE - RSU Platooning, Advanced Driving: Cooperative Lane Change with low LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 84 | 30 ms (NOTE 6) | Intelligent transport systems (see TS 22.261 [2]) |
| 85 | 5 ms (NOTE 5) | Electricity Distribution- high voltage (see TS 22.261 [2]). V2X messages (Remote Driving. See TS 22.186 [111], NOTE 16, see TS 23.287 [121]). Split AI/ML inference - DL Split AI/ML image recognition, (see TS 22.261 [2]) |
| 86 | 5 ms (NOTE 5) | V2X messages (Advanced Driving: Collision Avoidance, Platooning with high LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 87 | 5 ms (NOTE 4) | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 88 | 10 ms (NOTE 4) | Interactive Service - Motion tracking data, (see TS 22.261 [2]), split AI/ML inference - UL Split AI/ML image recognition, (see TS 22.261 [2]) |
| 89 | 15 ms (NOTE 4) | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| 90 | 20 ms (NOTE 4) | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |

It is obvious that those traffic are mostly video/voice which are inter-active traffic (some are even highlighted as mission-critical and V2X (i.e., high-speed and safety related)), for which the UE is mostly kept in RRC_CONNECTED during the whole session, rather than being released to RRC_IDLE / RRC_INACTIVE (which requires several seconds level inter-packet time period).

- The UP resumption latency reduction from 5G RRC_INACTIVE (e.g., from 166.5ms/86.5ms to 44.85ms) lacks of real use case, since the real QoS requirement can either be satisfied by transition latency from RRC_IDLE, or cannot be satisfied if being released to RRC_INACTIVE (i.e., have to be kept in RRC_CONNECTED) due to short inter-packet time period.

On the one hand, the latency upon state transition is not ‘low’ enough, and this makes the 3rd RRC state less attractive, due to lack of use case for the this ‘medium’ transition latency. While as analysed above, the reactive mobility (via reactive UE context retrieve) still leads to latency for UE to resume UP activity upon data arrival.

On the other hand, one drawback of RRC_INACTIVE is the complexity for deployment, due to the dependency on Xn interface availability. Different from RRC_CONNECTED and RRC_IDLE, which can fully rely on N2 interface, Xn interface availability is essential for RRC_INACTIVE, which leads to difficulty to RNA area planning (i.e., it cannot be done purely based on geo-graphical area, but has to take Xn interface availability into account). And this complexity leads to difficulty of deployment, since the improvement over RRC_IDLE state, i.e., medium transition latency , lacks of real use case.

However, if one wants to improve the Xn based procedure via N2 based procedure, it is questionable whether that would remove the advantage of RRC_INACTIVE, i.e., CP latency reduction, due to the involvement of AMF.

- It is not clear whether AMF involvement, if change the Xn-based procedure to N2-based procedure, would lead to increase of CP latency, and thus remove the advantage of RRC_INACTIVE over RRC_IDLE.

Nevertheless, operator did not show good interest on the 5G RRC_INACTIVE state, given all the effort spent on the SDT (which is an even more narrow use case).

- There is quite limited commercialization of RRC_INACTIVE in 5G system, even though the huge specification effort 3GPP spent on it, due to 1) no real use case for the CP latency, 2) deployment complexity.

For 6G, we should start from the question of use case as well, before looking into RRC state design.

- For 6G INACTIVE mode, R2 identify the applicable use case first, i.e., large inter-packet time (so that it is feasible for UE to leave RRC_CONNECTED) and short PDB requirement (so that transition delay from RRC_IDLE is too large).

Given the analysis above, the 6G RRC modelling should aim at improvement on top of the 5G design, where approach-2 and approach-3 seems to be a good start point.

Table 4 Comparison between approach-2/3

| Index | When there is no data ongoing | Upon data arrival | Consumption when no data ongoing | CP latency upon data arrival | Xn Reliance |
| --- | --- | --- | --- | --- | --- |
| 2 | 5G RRC_ INACTIVE | 5G RRC_ CONNECTED | Low | Medium | Yes |
| 3 | 5G RRC_ CONNECTED | 5G RRC_ CONNECTED | Medium if tuning DRX properly (Due to measurement and signalling for mobility) | Low | No for L3 handover, but yes for 5G (C)LTM |

Comparing the two-starting point:

If one starts from approach-2, one should solve the latency issue and Xn reliance issue.

To reduce/remove the Xn reliance issue, there are generally two ways

- To reduce the size for the operating area of the concerned state/sub-state, so that the Xn availability is not a concern anymore, e.g., limit it to intra-gNB.

- To remove the Xn reliance from the root level, i.e., to operate using N2 interface directly. But there is a concern that this operation would increase the resumption latency, if a re-active UE context retrieval is adopted.

If one tends to handle the mode as 6G RRC_INACTIVE state, it needs to solve the Xn reliance issue and further improve the UP-resumption latency, while further improve energy efficiency, to help commercialization in 6G.

To optimize the latency upon data arrival (only if there is use case identified, which has large inter-packet delay + short PDB requirement), for which the key should be to solve the latency due to RACH procedure and the re-active UE context retrieval. Which can be in various forms, which can include:

- Enhancement to reduce the RACH latency, e.g., to extend the applicability of RACH-less functionality;

- Enable proactive context relocation, as we did in RRC_CONNECTED. But in order to keep the advantage of low consumption when no data on-going, this proactive context relocation has to be pre-configured / conditionally-configured.

- Early CSI acquisition/report, in order for quick bit-rate ramp-up upon UP resumption, rather than a slow-start.

Or if one starts from approach-3, the Xn reliance should be evaluated as well (as analysed above), to simplify the RAN deployment complexity.

Besides, to optimize the consumption when there is no data, the key should be to relax/remove the measurement and to simplify/remove the signalling for measurement report and mobility command. which can include:

- Conditional configuration, to save the unnecessary measurement and signaling exchange. Specifically

  - Enhance to save unnecessary L1/L3 measurement when there is no-data. For LTM, UE performs L1 measurement for each candidate cell upon reception of the LTM configuration, which aim to tracking the candidate cell and providing assistance info to NW for cell switch decision. While the UE measurement seems not useful from network perspective during no data period and would cause consumption for UE. Further enhancement to save the measurement overheads can be studied. e.g., remove/relax the L1/L3 measurements on the configured LTM candidate cell(s).

  - Enhance to save unnecessary signaling exchange when there is no data. In RRC_CONNECTED, to achieve per cell paging, UE is required to perform complete cell switch execution (HO command, HO confirm) to inform network of UE’s arrival. For no data period, there is a trade-off between the granularity of UE location tracking area and the overhead of the HO confirmation message. To be energy efficient, simplified UE location tracking and HO confirm can be studied to optimize the unnecessary signaling exchange overhead and enable fast resumption upon data arrival. And the measurement reporting overhead can also be reduced/removed considering the signaling overheads.

- More power-efficiency UE wake-up mechanism, e.g., via LPWUS/PDCCH signal directly instead of ‘paging-like’ message.

If one tends to handle the mode as 6G RRC_CONNECTED state, it needs to further improve energy efficiency, and solve the Xn reliance issue for LTM procedure.

- The potential enhancement / optimization of functionality during 6G INACTIVE/sub-state mode (regardless how to model it) includes
1) further reduced UP resumption latency upon data arrival,
2) methods to reduce the deployment complexity due to Xn reliance
3) while keeping / enhancing the benefit of low power consumption.

- For 6G INACTIVE/Sub-State mode, R2 study to identify solution candidates that improve energy efficiency , including 
1) Relax measurement and simplify/avoid signaling exchange during no-data period;
2) more power-efficient UE wake-up upon DL data arrival, e.g., to simplify the ‘paging’ operation.

For Xn reliance issue, anyway R3 should be the leading WG for the technical details.

And the latency increase due to N2-based procedure should be avoided, if one believes it is the advantage of RRC_INACTIVE.

- For 6G INACTIVE mode, R2 relies on R3 to study for solution candidates that reduce/remove Xn reliance to reduce deployment complexity, and if R3 decided to do so, based on the identified use case (in terms of inter-packet time and latency requirement), R2 evaluate the applicability of solution candidates in terms of CP latency.

Other than the two above which is more independent of modelling decision, another aspect for the latency is somehow dependent only modelling

- If goes for RRC_INACTIVE, the use case has to have large inter-packet time, so that leaving RRC_CONNECTED is feasible. And one can further evaluate whether there is an improved requirement on CP latency for those use case. If yes, enhancement can be considered.

- Otherwise, if goes for RRC_CONNECTED, there is no such concern, so that most interactive traffic of short inter-packet time + short latency can be served.

- For 6G INACTIVE mode, R2 decide whether to study for solution candidates that enable fast UP resumption upon data arrival, based on the identified use case (in terms of inter-packet time and latency requirement), including
1) pro-active UE context preparation and configuration,
2) Extended RACH-less access, and
3) Early CSI acquisition/reporting.

And the next step is to compare between the two modelling, i.e., to implement the related features using a sub-state of RRC_CONNECTED or via a third state like RRC_INACTIVE.

To do the comparison, the focus should be more on complexity rather than feasibility, since either way would be feasible. While the complexity can be evaluated in a way of comparing the similarity / difference of RRC_CONNECTED, and thus the additional effort / impact of implementing the functionalities as a sub-state / third-state.

Table 5 Comparison between per-CC cell modelling and multi-CC cell modelling

| Target | Solution | If model the functionality as a third RRC state (e.g., RRC_INACTIVE) | If model the functionality as a sub-state of RRC_CONNECTED | Comment |
| --- | --- | --- | --- | --- |
| Energy efficiency improvement | Conditional configuration to alleviate measurement reporting during no-data period | Can be already achieved by legacy cell (re)selection procedure in RRC_INACTIVE | Additional effort taking advantage of conditional mobility procedure | Feasible for both approaches, yet with smaller impact for the 1st approach |
|  | more power-efficient UE wake-up upon DL data arrival, e.g., to simplify the ‘paging’ operation | Further enhancement on the paging mechanism, possibility to align with the procedure for RRC_IDLE | Align with the scheduling procedure to be defined in RRC_CONNECTED | Feasible for both approaches |
| To reduce/remove Xn reliance to reduce deployment complexity | Reduce the size of the ‘area’ | Implementation-based scheme | Implementation-based scheme | Feasible for both approaches |
|  | To support the N2-based procedure | Brand-new mechanism, requires joint R2 and R3 work, and it is unclear whether the N2-based route would increase the state transition latency | Naturally supported by L3 HO procedure defined as in legacy. While LTM-type procedure defined in 5G is still limited to Xn | Feasible for both approaches, yet with smaller impact / concern for the 2nd approach |
| To enable fast UP resumption upon data arrival (NOTE: for RRC_INACTIVE, the premise is there is such use case of large inter-packet time + short latency requirement) | Pro-active UE context preparation and configuration, | Change the re-active UE context retrieval in 5G RRC_INACTIVE to pro-active UE context relocation. | Naturally supported by conditional configuration procedure defined in 5G (e.g., CHO, LTM, CLTM) | It is against the essential difference between RRC_CONNECTED (UE context preparation before mobility initiated by network) and RRC_INACTIVE (UE context retrieval after mobility initiated by UE) |
|  | RACH-less access | Enhance the RACH procedure of RRC resume | Naturally supported by mobility procedure defined in 5G (e.g., L3 HO and LTM) | Feasible for both approaches, yet with smaller impact for the 2nd approach |
|  | Early CSI acquisition/reporting | To add the early measurement (for CSI) and report procedure for RRC resume | Naturally supported by mobility procedure defined in 5G (e.g., L3 HO and LTM) | Feasible for both approaches, yet with smaller impact for the 2nd approach |

Please note the third state will anyway leads to additional effort for the state transition, not only at specification level but also at signaling level.

- Overall, the potential enhancement / optimization of functionality would cause less impact / concern if implemented as a sub-state for RRC_CONNECTED, compared with being implemented as a separate state of RRC_INACTIVE.

- For 6G INACTIVE/Sub-State mode, after identifying the functionality to support / optimize, R2 evaluate / compare the complexity of implementing those functionalities via an INACTIVE/Sub-State.

Conclusion

Based on the discussion in section2, we have following proposals:

Observation 1	For 6G INACTIVE/Sub-State mode, R2 concluded to discuss the targeted functionality for the mode before decision on modelling (i.e., whether treat it as a separate RRC_INACTIVE state or a sub-state of RRC_CONNECTED).

Observation 2	5G RRC_CONNECTED state was designed for fast UP delivery, but without sufficient optimization of the power / resource consumption during mobility when there is no data activity.

Observation 3	5G RRC_INACTIVE was designed to reach a trade-off between UP resumption latency and UE power consumption during no-data period.

Observation 4	The UP resumption latency reduction from 5G RRC_INACTIVE (e.g., from 166.5ms/86.5ms to 44.85ms) lacks of real use case, since the real QoS requirement can either be satisfied by transition latency from RRC_IDLE, or cannot be satisfied if being released to RRC_INACTIVE (i.e., have to be kept in RRC_CONNECTED) due to short inter-packet time period.

Observation 5	It is not clear whether AMF involvement, if change the Xn-based procedure to N2-based procedure, would lead to increase of CP latency, and thus remove the advantage of RRC_INACTIVE over RRC_IDLE.

Observation 6	There is quite limited commercialization of RRC_INACTIVE in 5G system, even though the huge specification effort 3GPP spent on it, due to 1) no real use case for the CP latency, 2) deployment complexity.

Observation 7	The potential enhancement / optimization of functionality during 6G INACTIVE/sub-state mode (regardless how to model it) includes 1) further reduced UP resumption latency upon data arrival, 2) methods to reduce the deployment complexity due to Xn reliance 3) while keeping / enhancing the benefit of low power consumption.

Observation 8	Overall, the potential enhancement / optimization of functionality would cause less impact / concern if implemented as a sub-state for RRC_CONNECTED, compared with being implemented as a separate state of RRC_INACTIVE.

Proposal 1	For 6G INACTIVE mode, R2 identify the applicable use case first, i.e., large inter-packet time (so that it is feasible for UE to leave RRC_CONNECTED) and short PDB requirement (so that transition delay from RRC_IDLE is too large).

Proposal 2	For 6G INACTIVE/Sub-State mode, R2 study to identify solution candidates that improve energy efficiency , including  1) Relax measurement and simplify/avoid signaling exchange during no-data period; 2) more power-efficient UE wake-up upon DL data arrival, e.g., to simplify the ‘paging’ operation.

Proposal 3	For 6G INACTIVE mode, R2 relies on R3 to study for solution candidates that reduce/remove Xn reliance to reduce deployment complexity, and if R3 decided to do so, based on the identified use case (in terms of inter-packet time and latency requirement), R2 evaluate the applicability of solution candidates in terms of CP latency.

Proposal 4	For 6G INACTIVE mode, R2 decide whether to study for solution candidates that enable fast UP resumption upon data arrival, based on the identified use case (in terms of inter-packet time and latency requirement), including 1) pro-active UE context preparation and configuration, 2) Extended RACH-less access, and 3) Early CSI acquisition/reporting.

Proposal 5	For 6G INACTIVE/Sub-State mode, after identifying the functionality to support / optimize, R2 evaluate / compare the complexity of implementing those functionalities via an INACTIVE/Sub-State.

Reference

[1]	3GPP TR 38.914 V0.2.0 (2025-09).