3GPP TSG-RAN WG2 Meeting #131		R2-250xxx

Bengaluru, India, 25th – 29th Aug. 2025

Title:	[Draft] Reply LS on paging capability loss issue

Response to:	R2-2505063/ S2-2506087

Release:	Rel-19

Work Item:	NR_LPWUS-Core, TEI

Source:	vivo [to be RAN2]

To:	SA2

Cc: 	RAN3

- Contact Person:

- Name:	             		 Chenli

E-mail Address:	 Chenli5g@vivo.com

- 1. Overall Description:

RAN2 would like to thank SA2 for the response to Reply LS on paging capability loss issue in [R2-2505063/ S2-2506087].

RAN2 wants to further clarify our intention on the method of UE radio paging capability loss mentioned in the previous LS [R2-2503172/S2-2504513]:

Alt2:

- Regarding the understanding from SA2 on “the certain area mentioned by RAN2 is the whole NR coverage of that PLMN.”, it is not the intention from RAN2.

  - RAN2 understands “certain area” could be also smaller area, e.g. NAS registration area, paging area, etc. If operators want to support LP WUS in the area (e.g. smaller area than whole PLMN), operator needs to upgrade gNB in this area. If the UE moves into this area from outside, there will be paging capability miss issue.

Regarding the solution investigated in SA2:

SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF.

RAN2 understands that it is up to RAN3 whether/how to address it.

2. Actions:

To SA2:

RAN2 kindly request SA2 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #131bis      13th Oct. – 17th Oct. 2025				Prague, Czech Republic

TSG RAN WG2 Meeting #132           17th Nov. – 21st Nov. 2025			Dallas，USA