3GPP TSG-RAN WG2 Meeting #131bis	Draft R2-2507734

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.12.2

Source:	Samsung

Title:	[AT131bis][203][MIMOevo] Remaining MAC issues

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

Introduction

- [AT131bis][203][MIMOevo] Remaining MAC issues (Samsung)

Intended outcome: Proposals in R2-2507734 for the remaining MAC issues.

Deadline: before Friday CB (Thursday 7pm local time)

Discussion

- [Issue-1]

UE behavior for the different caes, a) the TAT for mode-B PUSCH is expired while the TAT for PUCCH is running, and b) if the TAT for mode-B PUCCH is expired while the TAT for PUSCH is running, clears the CG for UEI

Monday progress:

- For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, the UE clears the Type1 CG PUSCH. (Adopt the TP in the Appendix 3 as baseline).

Chair: we will CB to the following on Friday session.

?? For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, UE releases the PUCCH for mode-B UEI reporting.

Offline discussion:

For case a) the TAT for mode-B PUSCH is expired while the TAT for PUCCH is running, release PUCCH:

RAN1 has specified the multiplexing/dropping rule between UEI reporting and legacy CSI reporting.

Agreement RAN1#121 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding priority rules for CSI report multiplexing/dropping, UEI beam report for both mode-A and mode-B is prioritized over Semi-persistent CSI reports on PUCCH and Periodic CSI reports on PUCCH - UEI beam report for mode-A > Aperiodic CSI report > UEI beam report (for mode-B) > Semi-persistent CSI reports on PUSCH Note-1: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. Note-2: How to capture the above is up to Editor.

Since PUCCH resource for UEI beam reporting is not dedicated for UEI reporting, the same PUCCH resource can be configured for UEI beam reporting and legacy P/SP-CSI reporting. If the PUCCH resource is released for UEI beam reporting, it will cause issue for UE to report legacy CSI on PUCCH in the following case.

- Cell-A CSI-ReportConfig with mode-B event1/2/7

type-1 CG on Cell A, TAT expired, type-1 CG is cleared for Cell A as legacy

PUCCH on Cell B, configured with BWP-Id and PUCCH-ResourceId. TAT running for Cell B, but PUCCH resource released as proposed.

- Cell-B CSI-ReportConfig with Periodic/Semi-Persistent CSI on PUCCH.

PUCCH on Cell B, configured with the same BWP-Id and PUCCH-ResourceId. TAT running for Cell B, but P/SP-CSI on PUCCH cannot be transmitted if PUCCH is released as proposed.

In addition, RAN1 has also specified the multiplexing/dropping rule between UEI reporting and HARQ feedback, which confirms the same PUCCH resource can be applied for UEI reporting and HARQ feedback. If the PUCCH resource is released for UEI beam reporting, it will cause issue for UE to transmit HARQ feedback.

- Cell-A CSI-ReportConfig with mode-B event1/2/7

type-1 CG on Cell A, TAT expired, type-1 CG is cleared for Cell A as legacy

PUCCH on Cell B, configured with BWP-Id and PUCCH-ResourceId. TAT running for Cell B, but PUCCH resource released as proposed.

- NW may not know the exact timing when UE releases the PUCCH on Cell B. HARQ feedback can be scheduled on the same PUCCH resource on Cell B.

UE cannot transmit the HARQ feedback on Cell B if PUCCH is released as proposed.

As the analysis above, UE autonomous releasing the PUCCH resource on the other cell will cause issues to legacy CSI reporting and HARQ feedback.

Following the current specification for TAT expiry and UEI reporting, UE transmits UEI Report Indication on PUCCH and does not transmit the actually report on PUSCH due to expired TAT. Actually, even without the actually L1-RSRP report on PUSCH, the Report Indication on PUCCH informs NW that the corresponding event is triggered, so NW can know the current beam becomes worse than the new beam or a threshold. This is different from cell DRX where NW cannot not receive outside active time.

Since the current specification is not broken, and release PUCCH will cause the above issues, Rapporteur suggests the following proposal to avoid causing more issues in correction phase.

Proposal: If the TAT (associated with a sTAG) for mode-B Type1 CG PUSCH is expired while the TAT for PUCCH is running, UE does not transit the mode-B PUSCH, UE can transmit UEIRI on PUCCH, no MAC spec. impact.

Q1: Do you agree the above Proposal?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | We don’t want to restrict NW configuration, also need to avoid causing more issues. |
| LGE | Yes | Agree with analysis of Rapporteur |
| ASUSTeK | No | The prioritization (multiplexing/dropping) between UEI beam report and legacy CSI report/HARQ feedback made by RAN1 is for time-domain overlapping between different UL resources, which does not imply PUCCH resource configuration sharing between UEIRI and HARQ/legacy CSI: #### 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH In the following, a UE is configured to transmit K PUCCHs for respective - K-L SRs in a slot, as determined by a set of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, and - L PUCCHs for respective L UEIRIs in a slot as determined by a set of pucch-ResourceId with SR transmission occasions or UEIRI transmission occasions that would overlap with a transmission of a PUCCH with HARQ-ACK information from the UE in the slot or with a transmission of a PUCCH with CSI report(s) from the UE in the slot.. In addition, we don’t think it is a reasonable network configuration to share a PUCCH resource between UEIRI and legacy CSI report or HARQ feedback. Even if we consider the normal scenario where the TAT is not expired: If HARQ feedback and UEIRI were to share a same PUCCH resource, when receiving a 1-bit signal on PUCCH, the network would not be able to know if this signal is HARQ feedback or UEIRI. If UEIRI and legacy CSI report were to share a same PUCCH resource, the network would have to blind decode the PUCCH received due to different payload size between UEIRI and legacy CSI report. In conclusion, we think the scenarios raised by rapporteur will not happen and we can keep the online conclusion. |
| Ofinno | No | - For the multiplexing/dropping rule, what RAN1 has specified is that when the PUCCH resource of HARQ-ACK and PUCCH resource of UEIRI overlaps in time, how to multiple them in the same PUCCH resource. In this case, both HARQ-ACK and UEIRI have their own PUCCH resources, since they overlap in time, they are multiplexed in one of the PUCCH resources. Therefore, if the PUCCH resource for mode-B UEIBR is released in this condition, the HARQ-ACK can be transmitted on its own PUCCH resource without any problem. - NW can know the exact timing when UE releases the PUCCH because the NW knows when the TAT expires for which cell. No issue on this point. - The proposal by the rapporteur is not correct. Based on the current RAN1 spec, if the UE transmits the UEIRI on PUCCH, the UE eventually needs to transmit the mode-B PUSCH (e.g., after the TAT/CG is reacquired). However, the problem is that since there is a long delay between the UEIRI on PUCCH and the actual CSI report on PUSCH in this case. The CSI report will be outdated. - We couldn’t understand why the UE releases the PUCCH is to restrict NW configuration. NW can configure what they want. Conversely, the UE releases the PUCCH allows the NW to reallocate these resources to other UEs, which is beneficial for the NW. |
| Sharp | No | It is preferred that UE does not transmit UEIRI on PUCCH since the report could not be transmitted. |

For case b) if the TAT for mode-B PUCCH is expired while the TAT for PUSCH is running, clears the CGI:

There is an issue for UE behavior in the following case (i.e., same type-1 CG resource are associated to PUCCH resources on different cells).

- CSI-ReportConfig #1 on Cell A, mode-B event1

PUCCH on Cell B, TAT expired, PUCCH resource released for Cell B as legacy

type-1 CG resource on Cell A, TAT running, type-1 CG cleared as the agreement

- CSI-ReportConfig #2 on Cell A, mode-B event2

PUCCH on Cell B, TAT expired, PUCCH resource released for Cell B as legacy

same type-1 CG resource on Cell A, TAT running, type-1 CG cleared as the agreement

- CSI-ReportConfig #3 on Cell A, mode-B event7

PUCCH on Cell C, TAT running, send UEIRI on PUCCH

same type-1 CG resource on Cell A, TAT running, but UE cannot send UEI report if type-1 CG is cleared due to CSI-ReportConfig #1&2

Note the only configuration restriction from RAN1 is that “For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH”.

This restriction does not preclude that CSI report configurations associated to different PUCCH resources can be associated to same type-1 CG PUSCH. So, the above scenario is possible.

In addition, NW may not know the exact timing when UE clears the type-1 CG PUSCH and when the resource can be reallocated for other use even if UE clears the resource.

Given these issues, Rapporteur suggests to reconsider whether to clear mode-B type-1 CG PUSCH. According to the RAN1 spec., since the PUCCH is not transmitted due to TAT expired, UE will not transit the second step on PUSCH. Since nothing is broken in the current specification and clearing type-1 CG will cause the above issues, the following way-forwards are proposed.

Way-forward 1: No MAC impact. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for mode-B Type1 CG PUSCH is running, according to the RAN1 spec., UE will not transit the mode-B PUSCH since the PUCCH is not transmitted.

Way-forward 2: Update the agreement. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, and if the Type1 CG is only associated to this PUCCH for mode-B UEI reporting, the UE clears the Type1 CG PUSCH.

Q2: Do you agree WF1 or WF2?

| Company | WF1/2 | Comments |
| --- | --- | --- |
| Samsung | WF1 |  |
| LGE | No strong view |  |
| ASUSTeK | WF2 with comment | We'd prefer to stick to the online agreement to keep NW flexibility to manage the resources. If RAN1 allows such configuration, we are ok to update the agreement for clarification. |
| Ofinno | Keep the agreement as it is. WF1 is not acceptable WF2 is acceptable if that configuration can happen (see our explanation). | - We don’t want to revert the agreement. Based on the online discussion, no company objected this, and the chair does not put CB for this agreement. WF1 is not acceptable. - The case raised by the rapporteur is not a reasonable configuration. Please allow us to explain it. If CSI report configuration 1 for event 1 using PUCCH resource 1 and CSI report configuration 2 using PUCCH resource 2 for event 7 are both triggered, and the UE transmits UEIRI on PUCCH resource 1 for event 1 and UEIRI on PUCCH resource 2 for event 7, and if these two CSI report configurations share the same type 1 CG PUSCH resource, the NW may not know which CSI report the UE will transmit via the Type 1 PUSCH (e.g., CSI report for event 1 or CSI report for event 2 or both). Also, since the size of CSI report for different events are different, the NW needs to apply different hypothesis to receive (e.g., CSI report 1 only, CSI report 2 only, both CSI reports are multiplexed in PUSCH). The NW needs to make sure this kind of configuration does not happen. - NW can know the exact timing when UE clears the type-1 CG because the NW knows when the TAT expires for which cell. No issue on this point. - The mode-B type 1 CG resource cannot be transmitted anymore if the corresponding PUCCH has been released according to RAN1 spec. It’s better to clear the CG resource to increase resource efficiency. Keeping CG resources that are no longer usable offers no benefits and only drawbacks. |
| Sharp | WF1 |  |

- [Issue-2]

TAT expired for any TCI state of multi-panel SDM for sDCImTRP 2TA

Monday progress:

R2-2506941	Discussion on remaining MAC issues	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3 [Issue-5]: If UE is configured with sDCI mTRP two TA and UL multi-panel transmission with SDM mode, UE clears the CG resource if at least one TCI state indicated by the DCI for the CG resource is associated with the expired TAT.

R2-2507199	Remaining MAC issues in MIMO	Ofinno	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 4	When a serving cell is configured with multipanelSchemeSDM and with two TAGs, to avoid unsuccessfully decoding due to only partial PUSCH transmission/MIMO layer/TRP has valid TA, RAN2 to discuss and down-select one of the following options:

•	Option 1: UE clears any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 5).

•	Option 2: UE does not transmit any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 6).

R2-2507498	MAC open issues	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3: For multipanelSchemeSDM, UE clear the CG/PUSCH for SP-CSI if all of the activated TCI state(s) for the CG/PUSCH for SP-CSI is associated with the TAG(s) of the expired TAT(s). This is aligned with legacy behaviour; no change is needed for this.

Proposal 4: For multi-panel SDM scheme, NW should ensure to indicate in DCI both two TCI states with running TAT(s) for multipanelSchemeSDM, so that UE can use the indicated TCI states to transmits all MIMO layers of the TB. Discuss whether to specify that “UE does not transmit any uplink transmission via multi-panel SDM scheme if any TCI state to be applied for the multi-panel SDM transmission is associated to a TAG for which the TAT is expired.”

Discussion

-	ZTE think O1 in Ofinno P4 is preferable.

-	Qualcomm not sure if this issue is real. And think if we need a solution then P1 is reasonable.

-	Ofinno think it is not easy for NW to always ensure like proposed by Samsung P4.

-	Samsung do not agree with O1 from Ofinno P4.

[CB on Friday]

- ?? The following is take as baseline: If UE is configured with sDCI mTRP two TA and UL multi-panel transmission with SDM mode, UE clears the CG resource if at least one TCI state indicated by the DCI for the CG resource is associated with the expired TAT.

Offline discussion:

For sDCI mTRP operation using type-1 CG,

- MAC CE is used to activate up to 8 pairs of TCI states, each pair maps to a codepoint in the DCI TCI field (3bits). Take joint unified TCI state as an example, for separate unified TCI state, 4 TCI states (2 for DL TCI states and 2 for UL TCI states) are mapped to a DCI codepoint.

- The DCI TCI field indicates one codepoint among the 8 values, to indicate one pair of the activated TCI states to be applied.

- According to the RRC parameter applyIndicatedTCI-State {first, second, both} in ConfiguredGrantConfig, UE applies either the first or the second or both of the indicated pair of TCI states.

The actually applied TCI states are called “indicated TCI states” in RAN1 specification. In MAC specification, we use “activated TCI states” to refer to the set of MAC CE activated TCI states. And for the actual applied “indicated TCI states” and MAC CE activated TCI states we have specified the following in MAC for R18 mDCI mTRP 2TA.

Clause 5.2: 3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 4> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG. ///omitting/// The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.

For R19 2TA, any of the TCI states (RRC configured or MAC CE activated or DCI indicated) can be associated with any one of the two TAGs as RAN1 agreed. The above MAC behavior is also applied for sDCI mTRP with 2TA as RAN2 agreed before, which means

- if applyIndicatedTCI-State is set to “first” for a CG/PUSCH, when all the “first” ones of the 8 pairs of MAC-CE activated TCI states are associated to an expired TAT, UE clears the CG/PUSCH, regardless the actual applied “first” TCI state dynamically indicated by DCI.

- if applyIndicatedTCI-State is set to “second” for a CG/PUSCH, when all the “second” ones of the 8 pairs of MAC-CE activated TCI states are associated to an expired TAT, UE clears the CG/PUSCH, regardless the actual applied “second” TCI state dynamically indicated by DCI.

- if applyIndicatedTCI-State is set to “both” for a CG/PUSCH, when all the “both” ones of the 8 pairs of MAC-CE activated TCI states are associated to an expired TAT, UE clears the CG/PUSCH, regardless the actual applied “both” TCI states dynamically indicated by DCI.

- UE shall not transmit PUSCH using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running. This applies to multi-panel SDM scheme for which “both” TCI states are used, i.e., the PUSCH will not be transmitted if one or both TCI states indicated by DCI are associated to a TAG with TAT expired.

Based on this analysis, the current spec works well with nothing broken. So far the resource release behavior are all specified based on MAC CE activated TCI states. If any applied TCI state indicated by DCI is associated to an expired TAT, UE shall not transmit the PUSCH according to the green highlighted part.

For CATT P3 above, if we take this way to specify resource release for multi-panel SDM according to the applied TCI state indicated in DCI, following this philosophy we would have to specify for all the three blue highlighted bullets above according to the applied TCI states indicated in DCI, even for R18 2TA. However, the DCI indicated TCI states can change even more dynamically than MAC CE, the higher layer resource allocation so far is not designed according to PHY. Such UE behavior change is not the intention and should be avoided.

For Ofinno P4-O1 above, it is not clear regarding “any of two activated TCI states”. If it refers to the activated TCI states in MAC CE, there can be up to 16/32 activated TCI states. If it refers to the two DCI indicated TCI states, it is the same as CATT P3.

Since the current specification works with nothing broken, Rapporteur suggests the following proposal to avoid further optimization in correction phase.

Proposal: As legacy UE shall not transmit PUSCH using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running. This applies for all UL transmission schemes including multi-panel SDM scheme using “both” DCI indicated TCI states for a CG/PUSCH, i.e., the PUSCH will not be transmitted if one or both TCI states indicated by DCI are associated to a TAG with TAT expired. No further optimization to clear CG/PUSCH resource.

Q3: Do you agree the above Proposal?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Don’t see any need to optimize specifically for multi-panel SDM scheme. RAN1 has designed various UL transmission schemes, where “both” DCI indicated TCI states are applied one way or another, we should avoid over optimization. |
| LGE | Yes | Agree with Rapporteur’s analysis |
| Ofinno | No | We disagree with the understanding from the rapporteur. Please allow us to explain more in details: Based on the current MAC behaviors, if applyIndicatedTCI-State is set to “both” for a CG/PUSCH, only “both” activated TCI states associated with both expired TATs/TAGs, the UE can clear the CG/PUSCH, i.e., the UE does not clear the CG/PUSCH if only “one” TCI state associated with one expired TAT case. Moreover, based on the green text, the UE can still transmit the CG/PUSCH even if only “one” TCI state is associated with a running TAT/TAG. For example, when the UL transmission is associated with TCI state 1 associated with TAG 1 with a running TAT and TCI state 2 associated with TAG 2 with an expired TAT, the green text specified that the UE does not transmit the UL using the TCI state 2 associated with TAG 2 with an expired TAT, but it does not preclude the UL transmission using TCI state1. Thus, this causes the problem in SDM mode that the UE may partially transmit the CG/PUSCH by partial MIMO layers via partial TCI state, which results in the NW cannot decode the transmission successfully. The new proposal proposed by the rapporteur is not a legacy behavior, which still needs the specification change. Therefore, it’s better to clear the CG/PUSCH resource if the CG/PUSCH resource is not useful, which can avoid the problem and further increase the resource efficiency. For our P4-O1, we are ok to remove “two” in our proposal to reduce the confusion from the rapporteur: - Option 1: UE clears any CG/PUSCH for SP-CSI if any of the activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 5). |
| Sharp | Yes |  |

- [Issue-3]

DRX active time for mode-A UEI reporting

Monday progress:

R2-2506941	Discussion on remaining MAC issues	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 4 [Issue-10]: For mode-A UEIBR, keep the current wording in the MAC spec that UE considers the DRX as active time while a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH.

R2-2507265	MAC Open issues	Nokia 	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2:  If a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI, UE stays in the active state until the next PUCCH ressource for transmitting UE Initiated Report Indication. No MAC spec change.

R2-2507209	Remaining issues on UEI beam reporting	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3. For Mode-A, RAN2 introduce a new timer to handle Active time based on UEI reporting.

Proposal 4. For Mode-A, the new timer is started upon UEIRI transmission and is stopped upon expiry of the timer.

Discussions

P3 in R2-2507209

-	CATT think for legacy SR we do not have similar mechanism. LG E think these are different cases.

-	Asusteck agree with P3.

-	Ericsson wonders what is the new timer and how does it work.

P2 in R2-2507265

-	Asusteck think this is fine but we need to change the MAC spec.

-	CATT also ok with P2.

-	Ericsson support as well.

-	LG E wonders what is the UE behaviour on the next PUCCH resource. Nokia think UE will transmit based on the trigger, nothing new.

-	LG E has concern if NW configured a very short PUCCH periodicity, the granularity could be very short (such as on the slot level), and in this case not sure how it works.

[CB on Friday]

?? The following is take as baseline: For mode-A UEIBR, keep the current wording in the MAC spec that UE considers the DRX as active time while a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH.

?? If a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI, UE stays in the active state until the next PUCCH resource for transmitting UE Initiated Report Indication. Can further check if there is any MAC spec change.

Offline discussion:

Based on the online discussion, there are so far two options.

Option 1: UE stays in active time until receiving PDCCH scheduling. No MAC impact.

Option 2: UE stays in active time until the next PUCCH resource occasion. This leads to MAC impact.

The concern for O1 is that UE will be stuck in active time forever if NW missed the PUCCH or UE missed the PDCCH. In Rapporteur’s view, this will not really happen. If the current beam becomes so poor that causes any miss of the transmission, the UEI event will be met continuously for new instances, and UE will send a UEIRI on PUCCH for each new event instance while staying in active time. This is similar to transmitting SR multiple times before getting UL grant. For the worse case, BFD/BFR will be triggered, which may lead to RA in the end, or RLF will be triggered leading to RLF recovery. There is always a fallback mechanism in work.

One concern raised for O2 is that the duration to the next PUCCH occasion can be very small. According to PUCCH periodicity it can be as small as 2 symbols, 6 or 7 symbols, 1 slot, 2 slots…

pucch-Resource-r19                       SEQUENCE {

periodicityAndOffset                     CHOICE {

sym2                                     NULL,

sym6or7                                  NULL,

sl1                                      NULL,                       -- Recurs in every slot

sl2                                      INTEGER (0..1),

sl4                                      INTEGER (0..3),

sl5                                      INTEGER (0..4),

sl8                                      INTEGER (0..7),

sl10                                     INTEGER (0..9),

sl16                                     INTEGER (0..15),

sl20                                     INTEGER (0..19),

sl40                                     INTEGER (0..39),

sl80                                     INTEGER (0..79),

sl160                                    INTEGER (0..159),

sl320                                    INTEGER (0..319),

sl640                                    INTEGER (0..639)

},

resource                                 PUCCH-ResourceId,

ul-BWP-Id-r19                            BWP-Id,

pucch-Cell-r19                           ENUMERATED {spCell, pucch-Scell}

}

If the configured PUCCH periodicity is very small, there may not be enough time for NW to transmit PDCCH or UE to receive PDCCH before the next PUCCH occasion.

Option 1: UE stays in active time until receiving PDCCH scheduling. No MAC impact.

Option 2: UE stays in active time until the next PUCCH resource occasion. Change the MAC impact.

Q4: Do you agree Option 1 or 2?

| Company | Option 1/2 | Comments |
| --- | --- | --- |
| Samsung | Option 1 | Don’t see a strong motivation to optimize this case, for which the existing fallback mechanisms can be applied. |
| LGE | Option 2 | There may be a case which should be considered. - PUCCH is transmitted by UEI event triggering and the UE starts Active time, but the beam is recovered. In this case, the UE may not cancel Active time according to the current specification. Thus, a new method is needed to handle the triggered Active time. |
| ASUSTeK | Option 2 | We believe that unlike SR, there will not always be a fallback for stopping the DRX active time regarding UEIRI. According to RAN1 spec, the UE transmits UEIRI to the network as long as the number of event instance reaches the threshold: If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH… The network can directly fix the current beam (e.g., according to legacy periodic beam report), and the counter for counting UEI report event instance is reset when indicated TCI state (current beam) is updated: The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated. An example is illustrated as above. The UE transmits UEIRI on PUCCH when event instance counter reaches eventInstanceCount (3 in this example). The NW provides indicated TCI state update (without the UE receiving DCI scheduling mode-A report). Since the current beam has been updated by the network, there will be no further UEI report triggered, let alone BFR/RLF. Therefore, having a clear stop condition for UEIRI DRX active is beneficial to prevent the UE from monitoring PDCCH endlessly. Regarding the short-duration issue for PUCCH, since the UE will transmit PUCCH as long as the counter meets the threshold and not yet reset, the DRX active time will be prolonged after each PUCCH transmission which should be long enough for DCI scheduling. |
| Ofinno | Option 2 | For Option 1, we are wondering if the UE transmits a fist PUCCH, and then the UE releases the PUCCH resource after that (e.g., when a TAT for the PUCCH expires). In this case, if the first PUCCH failed to transmit or missed by the NW, the UE will never have chance to transmit the PUCCH again and to receive the PUSCH scheduling. Until a certain fallback mechanism (e.g., BFR) is triggered, the UE should keep in active time based on the current spec wording, which may cause unnecessary power consumption. Somehow, a stop point for PDCCH monitoring is needed. For Option 2, we agree that the PUCCH period can be very small. However, the UE will keep transmitting the PUCCH, which can keep prolonging the time for active time to receive the possible PUSCH scheduling. |
| Sharp | Option 1 |  |

Conclusions

The following proposals are listed based on the above discussion.

…

References

- R2-2507702	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others		RAN2 Vice Chairman (CATT)