3GPP TSG-RAN WG2 #131bis	R2-250XXXX

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.5.2

Source:	Ericsson

Title:	[AT131bis][301][NES] RRC open issues (Ericsson)

Document for:	Discussion

1	Introduction

This document is to capture the outcome of the following discussion:

[AT131bis][301][NES] RRC open issues (Ericsson)

Scope: Continue the discussion on RRC open issues 3, 7, 9, 10

Intended outcome: summary of the offline discussion (in R2-2507761)

F2F time: Wednesday morning coffee break 10:30-11:00

Deadline for offline discussion summary: Thursday 2025-10-14 14:00

2	Discussion

2.1 Impact of od-ssb-PositionsInBurst on ssb-ToMeasure

R2-2506817	[C184] Impact of od-ssb-PositionsInBurst on ssb-ToMeasure	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: A clarification that OD-SSB-PositionsInBurst is configured as a subset of ssb-ToMeasure within the associated MeasObjectNR should be added to the field description of the OD-SSB-PositionsInBurst.

R2-2507334	Discussion on RILS E023, E024, E025, X201, O006, J002, H128, H129, J005, Z101, Z102, V503 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 6: RAN2 to agree that OD-SSB specific MO includes values for ssb-ToMeasure to match the OD-SSB activated by the MAC CE.

-	Apple and vivo think this is sufficient

Continue in offline 301

Our understanding is that the field Ssb-ToMeasure is used to define scheduling restriction and if MO defines only one value which is superset of all the OD-SSB-PositionsInBurst the operation of such SCell is not very optimal.

From 38.133

9.2.5.3 Scheduling availability of UE during intra-frequency measurements UE shall be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE, or the UE indicates no-gap-no-interruption for intra-frequency measurement, or the UE indicates no-gap-with-interruption for intra-frequency measurement. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included.

Based on the discussion so far, we suggest postponing this discussion so that companies can get better understanding.

Q1. Do you agree with the proposal from the rapporteur above (Yes/No)? Please suggest a way forward if you do not.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Apple | Postpone | Encourage company to coordiante with their RAN1 and RAN4 colleague for discusison in Dallas. |
| Ericsson | Postpone |  |
| Nokia |  | Good idea to ensure with relevant other WG delegates. Not a problem to delay decision till Dallas |
| Huawei, HiSilicon | Yes |  |

Conclusion: ???

???

2.2 Neighbourcell measurements

R2-2506879	handling of RRC open issues	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal: for a measObject (in the measurement identity list), if a SCell corresponding to the frequency of the measObject is configured with on demand SSB and always on/periodic SSB is not configured for the SCell and on demand SSB is not activated and this measObject is associated only with a measurement reporting of a type/event which needs measurements of this SCell: UE ignore the measObject and do not measure neighbor cells according to measObject.

Continue in offline 301

Comments from the offline session on Wednesday, October 15th:

- Samsung thinks that this is essential from UE standpoint. Apple thinks we should also consider handovers so such measurements can be necessary. Samsung thinks this should be purely for the SCell change. LG agrees with Apple. Xiaomi thinks that this proposal can be considered as optimization. Huawei thinks there is no need for RAN2 to specify such detailed measurement aspects.

=> No support for the proposal.

Not agreed.

2.3 SIB1 acquisition in connected mode

R2-2506966	Remaining CP open issues of NES	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: [V500] Remove the RRC state restriction before the UE acquires SIB1 upon reception of SI change and PWS notification in the OD-SIB1 cell, and adopt the TP1 in the Annex 5.1.

-	Samsung supports the proposal. Also Apple supports the proposal

The UE acquires SIB1 from broadcast upon reception of SI change and PWS notification in the OD-SIB1 cell

Please see below the text proposed by Vivo in the document above:

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.2x:

4> if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

5> acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];

4> else:

5>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

5>	if the UE is in RRC_CONNECTED while T311 is running:

6>	perform the actions as specified in clause 5.2.2.3.3x;

3>	else:

4>	perform the actions as specified in clause 5.2.2.5.

Q3. Do you agree with the changes proposed above (Yes/No)? Please comment if not.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Rapporteur | Yes |  |
| Apple | Yes |  |
| Ericsson | yes |  |
| Nokia | Yes |  |
| Vivo | Yes |  |

Conclusion: ???

???

2.4 [H127]

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H127 | NES | 1 | Which RA occasions to use when the configuration is absent in RACH-ConfigDedicated | R2-25xxxxx | Huawei (Lili) |  | V012 | ToDo |

[Description]: In legacy spec, for CFRA, if occasions is absent, the corresponding field in RACH-ConfigCommon applies. With the introduction of additional RA resources, both legacy RA occasions and additional RA occasions can be configured in the RACH-ConfigCommon, it needs to be made clear which RA occasion is used when the field is absent in RACH-ConfigDedicated. Considering RAN2 has agreed that RACH adaptation is not applied for L3 HO command, the simplest way could be clarifying that legacy RA occasions (i.e. RA occasions not configured in addlRACH-Config-Adaptation-r19) are used when the field occasions is absent in RACH-ConfigDedicated.

[Proposed Change]:

occasions

RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon (except the RA occasions configured in addlRACH-Config-Adaptation in RACH-ConfigCommon) in the first active UL BWP.

[Comments]:

[Apple] We agree with this change. From technique-wise, it is not clear whether the UE has received paging short message to activate additional RACH resource upon reception during HO. So, since it is not clear how it works, we prefer to not allow additional RACH during HO.

[Rapporteur]: RAN2 has agreed that RACH adaptation is not applied for L3 HO command, but the usecase for dedicated RACH Config is not limited to L3 HO, or? If not, there is no need to exclude additional RA resources here. This needs further discussion in the next meeting.

[Samsung]: All cases where RACH adaptation is not applied is clearly specified in MAC. No change in RRC is needed.

Comments from the offline session on Wednesday, October 15th:

- Apple agrees with the rapporteur. Huawei thinks that it would be good to capture this also in 38.331. Samsung thinks that this parameter is used in the MAC and thus there is no need to capture anything in 38.331. Nokia agrees with Samsung and Apple.

=> No support for the proposed change.

Not agreed.

2.5 SSB adaptation activation and [H130]

R2-2507614 Control Plane issues [N001[N002][X200] [N003]	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal X: Allow direct SSB adaptation activation with RRC configuration.

-	Xiaomi and ZTE thinks that SSB adaptation can be applicable to deactivated Scell without further spec changes (based on group DCI in the Pcell). LG agrees

-	Nokia and Apple think that at the moment there are no RAN4 requirements for SSB adaptation for deactivated SCell

Continue in offline 301

When discussing the proposal above, it was observed that it is related to [H130] below. The discussion on these cases are merged.

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H130 | NES | 1 | SSB adaptation for deactivated SCell | R2-25xxxxx | Huawei (Lili) |  | V034 |  |

[Description]:

According to 38.321, when an SCell is deactivated, the MAC entity shall not monitor PDCCH on the SCell or for the SCell:

1> if the SCell is deactivated: - 2> not transmit SRS on the SCell; - 2> not report CSI for the SCell; - 2> not transmit on UL-SCH on the SCell; - 2> not transmit on RACH on the SCell; - 2> not monitor the PDCCH on the SCell; - 2> not monitor the PDCCH for the SCell; - 2> not transmit PUCCH on the SCell;

The above PDCCH monitoring behaviour inherits from LTE R10. With this restriction, the SSB adaptation indicated by DCI cannot apply to deactivated SCell. Even though we think dynamically changing the SSB periodicity to a smaller value for fast SCell activation is a valid use case, we prefer not to change the PDCCH monitoring behaviour or to revert RAN2 previous agreement of not introducing new MAC CE. This fast SCell activation can be enabled by OD-SSB operation. Under OD-SSB discussion, it is allowed to use the OD-SSB MAC CE to dynamically change the OD-SSB configuration including SSB periodicity. Alternatively, the network can use RRC reconfiguration to update the SSB periodicity of a deactivated SCell.

[Proposed Change]:

[[

od-ssb-r19                      SetupRelease {OD-SSB-r19}                                       OPTIONAL,   -- Need M

adap-SSB-Config-r19             SetupRelease {Adap-SSB-Config-r19}                              OPTIONAL    -- Cond SSB-AdaptM

]]

| Conditional Presence | Explanation |
| --- | --- |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N: - in the masterCellGroup at - SCell addition, - reconfiguration with sync, - resume of an RRC connection. - in the secondaryCellGroup, when the SCG is not indicated as deactivated at: - - SCG activation while the SCG was previously deactivated, - - SCell addition, - - reconfiguration with sync. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |
| SCellSIB20-Opt | This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise. |
| SCG-Opt | The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise. |
| SSB-Adapt | The field is optionally present, Need M, for an activated SCell. It is absent otherwise. |

Q5. Do you agree with the proposal from Huawei in R2-2506936 above (Yes/No)? Please comment if not.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Ericsson | No | The PDCCH monitoring behaviour mentioned above is inherited from LTE R10. With this restriction, the SSB adaptation indicated by DCI cannot apply to deactivated SCell. Even though we think dynamically changing the SSB periodicity to a smaller value for fast SCell activation is a valid use case, we prefer not to change the PDCCH monitoring behaviour or to revert RAN2 previous agreement of not introducing new MAC CE. This fast SCell activation can be enabled by OD-SSB operation. Under OD-SSB discussion, it is allowed to use the OD-SSB MAC CE to dynamically change the OD-SSB configuration including SSB periodicity. Alternatively, the network can use RRC reconfiguration to update the SSB periodicity of a deactivated SCell.” We think that it would be better if we limit the impact to UE behavior regarding PDCCH monitoring if the SCell is deactivated, however the proposed change could be too restrictive considering that it may be beneficial to update the SSB periodicity dynamically for faster SCell activation. Therefore, it should be possible to activate SSB adaptatio via RRC configuration. We are open to discuss whether such activation can only be performed when SCell is deactivated. |
| OPPO | No | It is a bit confusing, even if one hold the view that UE shall not monitor DCI format 2-9 on/for deactivated Scell, why that leads to a restriction of RRC configuration based on the Scell (de)activated status (as proposed by proponent)? Our understanding was that the RRC configuration of SSB adaptation can be provided upon Scell configuration, which does not affect the UE monitoring / not-monitoring DCI format 2-9 based on the Scell (de)activated status, controlled by MAC-CE / timer. Furthermore, we understand that for DCI format 2-9, if it is carried by Pcell, UE will monitor it regardless of Scell (de)activated status, same as for DCI format 2-1. Is there any point we missed here? |
| Apple | No change for now (but open to discuss next meeting if Huawei identy issue. | Currently, we have same view as OPPO and Ercisson from RAN2 perspective. Nothing is broken. Meanwhile on whether SSB adaptation can be performed in deactivated SCell, I asked our RAN1 colleagues. They said RAN1 didn’t discuss it before and it was not cystal clear. So, we are open to further discuss in next meeting if Huawei can identify more issues to allow SSB adaptation in deactivated SCell. |
| Nokia | No (Huawei) Yes (Nokia) | In August meeting RAN4 agreed that SSB adaptationis only for activated SCell. Regarding Huawei proposal – Considering this it seems that change seems not needed or even incorrect. Even tough ssb adaptation can only happen on deactivated scell one should of course allow configuration of it to let NW later activate Scell and activate SSB adaptation Regarding Nokia proposal – if agreed one should not allow it to be done for deactivated scell i.e. only together with direct SCell activation or for already activated SCell. Main use case is to activate adaptation together with scell activation. Then NW can deactivate adaptation with DCI. Also this align behaviour with OD-SSB. |
| Vivo | No | We checked with our RAN1 colleague and share the similar view with OPPO. We understand the UE monitors DCI 2_9 on PCell or other scheduling cell regardless of the activation status of the target SCell to adapt SSB, as it is a common signalling and can indicate the SSB adaptation status not only for one Scell. Hence, we don’t observe the need to further restrict the presence condition. |
| Huawei, HiSilicon |  | We think the current wording “for the SCell“ is too broad. The UE should be able to monitor DCI 2_9 from Pcell to know the SSB adaptation status of Scell in this case, but it’s better to clarify at least in the chairman notes that this is not prevented from “not monitor PDCCH for the SCell“. |

Conclusion: ???

???

2.6 [C181]

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C181 | NES | 1 | Cell selection case is missing in section 5.2.2.4.2x Actions upon reception of SIBxx |  | Rui (CATT) |  | V008 | PropReject |

[Description]: In section 5.2.2.4.2x	Actions upon reception of SIBxx, Cell selection case is missing.

[Proposed Change]:

Upon receiving SIBxx, the UE shall:

1>	store the SIBxx;

1>	SIB1 request configuration in the SIBxx is valid for acquiring OD-SIB1 of this cell in accordance with clause 5.2.2.3.1;

1>	SIB1 request configuration of another cell in this stored SIBxx is valid for acquiring OD-SIB during (re)selection to that cell, and after (re)selection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1:

[Comments]: Nokia: looks valid proposal

[vivo] It’s not necessary. Both cell selection and cell reselection case has been embodied in 5.2.2.3.1, as the UE relies on kssb value to determine how it acquires SIB1, and thus ‘1>	SIB1 request configuration in the SIBxx is valid for acquiring OD-SIB1 of this cell in accordance with clause 5.2.2.3.1;’ is sufficient.

[Ericsson] Either way is ok. Maybe not necessary but may not harm either.

[Apple] We agree with this change because cell selection may be triggered in OD-SIB1 (i.e. RRC re-establishment). It is different from legacy OD-SIB which can’t trigger cell selection.

[Rapporteur]: The proposed change does not seem to be essential.

[Samsung]: Support this change. The change seems essential. Without this change, SIBxx validity for the RRC re-establishment case is unclear.

Comments from the offline session on Wednesday, October 15th:

- There is support in the room for this proposal.

=> The proposed change is agreed.

Agreed.

2.7 [H125]

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H125 | NES | 1 | Optionality of R19 PEI configurations |  | Huawei (Lili) |  | V012 | PropReject |

[Description]: The R19 PEI configurations should be made optional.

[Proposed Change]:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...,

[[

po-NumPerPEI-r19                          ENUMERATED {po1, po2, po4, po8}                        			OPTIONAL,  -- Need R

payloadSizeDCI-2-7-r19                    INTEGER (1..maxDCI-2-7-Size-r17)                        			OPTIONAL,  -- Need R

pei-FrameOffset-r19                       INTEGER (0..32)                        								OPTIONAL  -- Need R

]]

}

[Comments]: [OPPO] How to interpret the case where one specific parameter is absent (but the others are present?), considering [[]]] already provides optionality.

[Apple] Agree with OPPO.

[Rapporteur] This depends on whether it should be possible to configure PEI for Rel-19 NES UEs when PEI (Rel-17) is not configured. Based on the agreements below:

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

it should not be possible (although it was not explicitly captured) so the current implementation should be fine.

[Huawei] Even if the optionality is indicated by “[[ ]]” implicitly, the need code also needs to be provided. Basically all other fields added under “[[ ]]” are marked as OPTIONAL with a need code. If the concern is that these parameters are either all configured or all absent, then maybe we a can used a R19 field to group these parameters.

Comments from the offline session on Wednesday, October 15th:

- Huawei thinks it may be better to introduce a new Rel-19 IE with parameters in that group to be mandatory. Apple agrees.

=> Introduce a new Rel-19 IE to group po-NumPerPEI-r19, payloadSizeDCI-2-7-r19, pei-FrameOffset-r19, where each parameter is mandatory.

Introduce a new Rel-19 IE to group po-NumPerPEI-r19, payloadSizeDCI-2-7-r19, pei-FrameOffset-r19, where each parameter is mandatory.

2.8 [X202]

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X202 | NES | 1 | Classify parameters of OD-SIB1 | R2-25xxxxx | Xiaomi (Li Zhao) |  | V003 | PropReject |

[Description]: According to the parameter list from RAN1, some parameters related to OD-SIB1 should be configured per od-sib1-Config instead of per SIB1-RequestConfig, e.g., sib1-rsrp-ThresholdSSB-r19, locationAndBandwidth-r19, absoluteFrequencyPointA-r19, ul-FrequencyBandList-r19, ul-SubCarrierSpacing-r19, etc.

[Proposed Change]: RAN2 to discuss to move those parameters not related to SIB1-RequestConfig from SIB1-RequestConfig to od-sib1-Config.

[Comments]:[Nokia] I’m not sure on this one. RAN1 xls seems to be bit vague on this. So far to use current asn.1 seems Ok anyway as the xls points out that all the parameters in frequenciInfoUL are per WUS config. So we would not do this change until it is confirmed with Ran

[Huawei]: Agree with Xiaomi. We understand that RRC parameters are up to RAN2 to specify but we did not see the motivation for deviating from the RAN1 parameter list, in particular what was the reasoning of the current parameter placement in OD-SIB1-Config-r19 and SIB1-RequestConfig-r19.

[vivo] Maybe it’s safer to keep them as it is now, like the sib1-rsrp-ThresholdSSB-r19 and ul-SubCarrierSpacing-r19, which is per Uplink configured (NUL or SUL)?

[Ericsson] It is this way dues to RAN2 SUL agreement. RAN1 did not consider SUL and hence this was not reflected in their parameter excel. There is no functional difference in RAN1 perepective with the existing order of parameters since all is there in the highest level IE.

[Samsung]: Agree with Ericsson. These parameters are UL carrier specific (SUL/NUL).

[Xiaomi] I think SUL related parameter is rsrp-ThresholdSSB-SUL, which is not reflected in RAN1 parameter list and OK to be configured within SIB1-RequestConfig as it relates to the selection of carrier when transmitting SIB1 request. However, there are many parameters not related to SIB1 request transmission, e.g., sib1-rsrp-ThresholdSSB-r19, locationAndBandwidth-r19, absoluteFrequencyPointA-r19, ul-FrequencyBandList-r19, ul-SubCarrierSpacing-r19, etc., that should be moved out according to RAN1 guidance. Also, there are some parameters instructed to be configured per SIB1 request configuration, e.g., ss-PBCH-BlockPower-r19, ssb-PositionsInBurst-r19 and sib1-TDD-UL-DL-ConfigurationCommon-r19 but are configured within od-sib1-Config. Thus, we think we need to discuss whether/how to follow the guidance from RAN1 parameter list.

[Apple] We agree with Samsung and Ericsson on NUL/SUL carrier specific parameters. To support SUL in NES cell, it is not sufficient to only introduce rsrp-ThresholdSSB-SUL because SUL can configure a separate RACH resource different from NUL. It is same as legacy OD-SIB: in SI-SchedulingInfo, it has si-RequestConfig and si-RequestConfigSUL with same type si-RequestConfig. Thus, we prefer to keep the current formulation.

[Rapporteur] There seems to be nothing broken with the current formulation. Note that RAN1 excel sheet does not indicate how signalling should be designed, and this is up to RAN2.

Comments from the offline session on Wednesday, October 15th:

- Xiaomi confirms that there is nothing broken with the current text but thinks that the change would be better. Samsung thinks that the current text is more clear especially when it comes to separation of SUL and NUL and states that this is also the case in legacy for SUL and NUL. Apple agrees.

=> X202 is not supported.

X202 is not pursued.

2.9 [H131]

R2-2506936	[H126][L201][X200][A103][H128][H129][X201][H131][H130][H127] Control plane issues	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: On [X201][H131]: Clarify that in the following cases the legacy servingCellMO is used for deriving serving cell measurement results:

-       if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured

-       if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency

-       if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is not activated

-       if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and OD-SSB transmission is activated

In the following case servingCellMO-OD is used for deriving serving cell measurement results:

-       if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated

-       if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and OD-SSB transmission is activated

Continue in offline 301

Please note that the revision made in red are since RAN2 agreed to use servingCellMO-OD for OD-SSB case1

Comments from the offline session on Wednesday, October 15th:

- Xiaomi thinks that for the 2nd bullet, od-ssb is configured without od-ssb-absoluteFrequency.

Continue in offline 301

Q9. Do you agree with the proposal above (Yes/No)? Please comment if not.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Apple | Yes | Aligned with my understanding. |
| Ericsson | Yes |  |
| Nokia | Yes | Sorry for delay – during online session it was difficult to be certain but this looks good now after further review. |
| Vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |

Conclusion: ???

???

3	Conclusion

In this contribution we discuss the open issues related to the third objective. Based on the discussion in the previous section, we propose the following:

Proposal 1	???

Proposal 2	???

Proposal 3	???

Proposal 4	???

Proposal 5	???

Proposal 6	???

Proposal 7	???

Proposal 8	???

Proposal 9	???

References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.