3GPP TSG RAN WG 2 #131-bis										R2-2507774

Prague, Czech Republic, October 13th-17th, 2025

Agenda Item:	7.0.2.17

Source:	CSCN

Title:	Summary of the offline discussion [AT131bis][310][R18 NR NTN]  Event D2 (CSCN)

WID/SID:	NR_NTN_enh-Core

Document for:	Discussion/Decision

- Introduction

This is summary report of the following offline:

[AT131bis][310][R18 NR NTN] Event D2 (CSCN)

Scope: Continue the discussion on correction for Event D2

Intended outcome:  summary of the offline discussion and revised CRs (in R2-2507774, 75, 76,77)

Deadline for offline: Thursday 2025-10-16 11:00

- Discussion

  - Online conclusion in Monday session

R2-2506990	Correction on eventD2 with multiple moving reference locations	CSCN, ZTE Corporation, Sanechips, Huawei, Hisilicon, Xiaomi, CATT, OPPO	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1:	RAN2 can further discuss and down select from the following options to address the case when multiple neighbor cells with moving reference locations fulfil eventD2 and would trigger measurement report:

−	Option 1: For multiple moving reference locations on the same frequency, the network should configure separate MeasObjectNR for each reference location, where only one MO can perform SSB measurements.

−	Option 2: Similar as the eventA processing mechanism, use a cellsTriggeredList to include cells with moving reference locations that meet the entering/leaving condition for eventD2 during the "Measurement report triggering" and "Measurement reporting" procedures. Add information in MeasResults to indicate which neighbour cell (s) triggered event D2 is FFS.

−	Option 3: Add the moving reference location(s) for eventD2 to the ReportConfigNR.

- Apple thinks this is a valid issue, and prefers option 2 (if we go for option 3 we need to add more).

- Vivo thinks we can leave this to NW implementation to only configure one, alternatively we can go for option 1. QC agrees

- QC thinks there could be a problem also for multiple configuration of D1 events, for privacy considerations.

- Nokia thinks in case we can go for option 1

- Ericsson thinks none of the options are necessary and a NW implementation could address this case

- CSCN thinks we could rule out option 3 and could accept option 1

- Xiaomi prefers option 1.

  - Proposed way-forward

When multiple moving reference locations configured in MeasObjectNR, current procedure cannot support the UE triggering more than one measurement report when the eventD2 is met for multiple neighbor cells with different moving reference locations. Majority confirm this is a valid issue in R18.

Three options are introduced in R2-2506990:

− Option 1: For multiple moving reference locations on the same frequency, the network should configure separate MeasObjectNR for each reference location, where only one MO can perform SSB measurements.

− Option 2:Use a cellsTriggeredList to include cells with moving reference locations that meet the entering/leaving condition for eventD2 during the "Measurement report triggering" and "Measurement reporting" procedures. Add information in MeasResults to indicate which neighbour cell (s) triggered event D2 is FFS.

− Option 3: Add the moving reference location(s) for eventD2 to the ReportConfigNR.

According to the online discussion, we give the updates based on each option in R2-2507775, R2-2507776 and R2-2507777. Please note that in option 3a, it’s a new option with much less impact on the specification based on the compromise of option 2 and option 3. The options are listed in Table.1, we need to further down-selection for corrections on eventD2 with multiple moving reference locations.

Based on the online discussion, updates have been provided for each option outlined in R2-2507775, R2-2507776, and R2-2507777. It should be noted that Option 3a is a newly introduced alternative, which has significantly reduced impact on the specification by incorporating a compromise between Option 2 and Option 3. The options are summarized in Table 1, and further down-selection is required to address corrections related to eventD2 involving multiple moving reference locations.

Table 1: Options for evnetD2 correction

| Option | Description | Impact |
| --- | --- | --- |
| Option 1 (draft R2-2507775) | 1.In clause 5.5.2.1, clarify eventD2 that each MeasObjectNR associated with CellsToAddModListExt-v1800 can only configure one moving reference location. When multiple moving reference locations need to be configured on the same frequency, the network needs to configure MeasObjectNR separately for each reference location. Each MeasObjectNR configuration includes one moving reference location associated with eventD2. | 1.if the network supports the change and the UE does not, UE is able to report measurement results of more than one neighboring cells that fulfil eventD2 entering/leaving condition. 2.if the UE supports the change and the network does not, network would configure more than one moving reference locations per MO and expect measurement report of at most one cell that fulfills eventD2 entering/leaving condition. |
| Option 2 (draft R2-2507776) | 1.In clause 5.5.4.1, modify the eventD2 measurement procedure with a cellsTriggeredList to track the state changes of moving reference locations that meet the entering/leaving condition during the "Measurement report triggering" and "Measurement reporting" procedures 2.In clause 5.5.5.1, add a note "It is up to UE implementation how to choose the best neighbouring cells for eventD2." | 1.if the network supports the change and the UE does not, UE is unable to report measurement results of more than one neighboring cell that fulfil eventD2 entering/leaving condition. 2.if the UE supports the change and the network does not, UE can report measurement results of more than one neighboring cell fulfil eventD2. |
| Option 3a (draft R2-2507777) | 1. In clause 5.5.4.15a, add a clarification that if eventD2PhysCellId is configured in reportConfigNR, only the movingReferenceLocation associated with this eventD2PhysCellId is used. 2. In clause 6.3.2, add the eventD2PhysCellId-r18 with conditional presence in ReportConfigNR. | 1.if the network supports the change and the UE does not, UE is unable to report measurement results of more than one neighboring cell that fulfil eventD2 entering/leaving condition. 2.if the UE supports the change and the network does not, UE is unable to report measurement results of more than one neighboring cell that fulfil eventD2 entering/leaving condition. |

Question 1: Companies are asked to provide comments on which option(s) they support in Rel-19.

| Company | Preferred option(s) | Comments |
| --- | --- | --- |
| xxx | option 1, option 2,option 2 |  |

Proposal 1: xxxxxx.

- Conclusion

- Reference

- TS 38.331, Radio Resource Control (RRC); Protocol specification, v18.6.0